# ANNEX 1

# Annex 1: Key sources

Methodology

IPCC Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories

Chapter 7 Methodological Choice and Recalculation

The Tier 1 method to identify key source categories assesses the impacts of various source categories on the level and, if possible, the trend, of the national emissions inventory. When the national inventory estimates are available for several years, it is good practice to assess the contribution of each source category to both the level and trend of the national inventory. If only a single year’s inventory is available, only a Level Assessment can be performed.

The Tier 1 method to identify key source categories can be readily completed using a spreadsheet analysis.

Level Assessment – TIER I

The contribution of each source category to the total national inventory level is calculated according to

Equation 7.1

Source Category Level Assessment = Source Category Estimate / Total Estimate

Lx,t = Ex,t / Et

Where:

Lx,t is the Level Assessment for source x in year t

Source Category Estimate (Ex,t ) is the emission estimate of source category x in year t

Total Estimate (Et) is the total inventory estimate in year t

Trend Assessment - TIER I

The contribution of each source category’s trend to the trend in the total inventory can be assessed if more than one year of inventory data are available, according to

Equation 7.2:

Source Category Trend Assessment = (Source Category Level Assessment)| (Source Category Trend – Total Trend) |

Tx,t = Lx,t *| {[(Ex,t – EX,0) / Ex,t] – [(Et – E0) / Et]} |

Where:

Tx,t is the contribution of the source category trend to the overall inventory trend, called the Trend Assessment. The Trend Assessment is always recorded as an absolute value, i.e. a negative value is always recorded as the equivalent positive value.

Lx,t is the Level Assessment for source x in year t (derived in Equation 7.1)

Ex,t and Ex,0 are the emissions estimates of source category x in years t and 0, respectively

Et and E0 are the total inventory estimates in years t and 0, respectively

The Source Category Trend is the change in the source category emissions over time, computed by subtracting the base year (year 0) estimate for source category x from the current year (year t) estimate and dividing by the current year estimate.

The Total Trend is the change in the total inventory emissions over time, computed by subtracting the base year (year 0) estimate for the total inventory from the current year (year t) estimate and dividing by the current year estimate.

Table 1: GPG Table 7.A1 (Tier I approach)

| rank | CRF | IPCC Source Category |  | Direct | Gg CO2 eq. | Gg CO2 eq. | level 2004 | trend | trend | KS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| level 2004 | Sector |  |  | GHG | 1986 | 2004 | % | equ. 7.2 | % | trend |
| 1 | 1A | 1. Energy Industries | a. Public Electricity and Heat Production | CO2 | 6533,661 | 6282,900 | 31,50 | 0,913 | 2,43 | yes |
| 2 | 1A | 3. Transport | b. Road Transportation | CO2 | 1902,131 | 4053,451 | 20,32 | 11,008 | 29,29 | yes |
| 3 | 1A | 4. Other Sectors | b. Residential | CO2 | 1100,171 | 1643,005 | 8,24 | 2,811 | 7,48 | yes |
| 4 | 1A | 2. Manufacturing Industries and Construction | f. Other (as specified in table 1.A(a) sheet 2) | CO2 | 1775,287 | 1102,934 | 5,53 | 3,311 | 8,81 | yes |
| 5 | 1A | 4. Other Sectors | a. Commercial/Institutional | CO2 | 612,091 | 815,864 | 4,09 | 1,066 | 2,84 | yes |
| 6 | 1A | 2. Manufacturing Industries and Construction | d. Pulp, Paper and Print | CO2 | 649,264 | 501,226 | 2,51 | 0,715 | 1,90 | yes |
| 7 | 2 | Industrial Processes | 1. Cement Production | CO2 | 514,615 | 485,581 | 2,43 | 0,119 | 0,32 | no |
| 8 | 6 | A. Solid Waste Disposal on Land | 1. Managed Waste Disposal on Land | CH4 | 298,801 | 415,491 | 2,08 | 0,608 | 1,62 | yes |
| 9 | 4 | D. Agricultural Soils (2) | 1. Direct Soil Emissions | N2O | 405,148 | 368,236 | 1,85 | 0,165 | 0,44 | no |
| 10 | 4 | A. Enteric Fermentation | 1. Non-Dairy Cattle | CH4 | 301,880 | 342,779 | 1,72 | 0,224 | 0,60 | yes |
| 11 | 4 | D. Agricultural Soils (2) | 3. Indirect Emissions | N2O | 333,371 | 302,951 | 1,52 | 0,136 | 0,36 | no |
| 12 | 1A | 2. Manufacturing Industries and Construction | a. Iron and Steel | CO2 | 1141,242 | 297,064 | 1,49 | 4,216 | 11,22 | yes |
| 13 | 1B | Fugitive Emissions from Fuels (1.Solid Fuels) | a. Coal Mining and Handling | CH4 | 358,906 | 270,654 | 1,36 | 0,428 | 1,14 | yes |
| 14 | 4 | A. Enteric Fermentation | 1. Dairy Cattle | CH4 | 429,314 | 263,424 | 1,32 | 0,817 | 2,17 | yes |
| 15 | 1A | 4. Other Sectors | c. Agriculture/Forestry/Fisheries | CO2 | 426,782 | 225,711 | 1,13 | 0,996 | 2,65 | yes |
| 16 | 2 | Industrial Processes | 3. Aluminium Production | CO2 | 89,402 | 210,560 | 1,06 | 0,619 | 1,65 | yes |
| 17 | 1A | 2. Manufacturing Industries and Construction | c. Chemicals | CO2 | 98,036 | 199,435 | 1,00 | 0,519 | 1,38 | yes |
| 18 | 1A | 2. Manufacturing Industries and Construction | e. Food Processing, Beverages and Tobacco | CO2 | 247,741 | 177,707 | 0,89 | 0,341 | 0,91 | yes |
| 19 | 4 | B. Manure Management | 8. Swine | CH4 | 232,219 | 162,685 | 0,82 | 0,340 | 0,90 | yes |
| 20 | 4 | B. Manure Management | 13. Solid Storage and Dry Lot | N2O | 259,261 | 153,826 | 0,77 | 0,520 | 1,38 | yes |
| 21 | 1A | 3. Transport | b. Road Transportation | N2O | 17,243 | 149,576 | 0,75 | 0,672 | 1,79 | yes |
| 22 | 4 | B. Manure Management | 1. Non-Dairy Cattle | CH4 | 93,492 | 136,647 | 0,69 | 0,224 | 0,60 | yes |
| 23 | 4 | B. Manure Management | 1. Dairy Cattle | CH4 | 167,074 | 129,233 | 0,65 | 0,183 | 0,49 | yes |
| 24 | 2 | Industrial Processes | 3. Aluminium Production | PFC | 276,291 | 120,008 | 0,60 | 0,777 | 2,07 | yes |
| 25 | 6 | B. Waste Water Handling | 2. Domestic and Commercial Waste Water | CH4 | 112,561 | 95,227 | 0,48 | 0,082 | 0,22 | no |
| 26 | 1A | 4. Other Sectors | b. Residential | CH4 | 134,677 | 90,541 | 0,45 | 0,216 | 0,58 | yes |
| 27 | 2 | Industrial Processes | 2. Lime Production | CO2 | 220,206 | 87,976 | 0,44 | 0,658 | 1,75 | yes |
| 28 | 6 | B. Waste Water Handling | 1. Industrial Wastewater | CH4 | 96,116 | 85,700 | 0,43 | 0,048 | 0,13 | no |
| 29 | 1B | Fugitive Emissions from Fuels (1.Solid Fuels) | c. Other (SO2 scrubbing)) | CO2 |  | 72,752 | 0,36 | 0,369 | 0,98 | yes |
| 30 | 2 | Industrial Processes | 1. Refrigeration and AC Equipment | HFC |  | 65,634 | 0,33 | 0,333 | 0,89 | yes |
| 31 | 6 | B. Waste Water Handling | 2. Domestic and Commercial Waste Water | N2O | 58,716 | 58,253 | 0,29 | 0,001 | 0,00 | no |
| rank | CRF | IPCC Source Category |  | Direct | Gg CO2 eq. | Gg CO2 eq. | level 2004 | trend | trend | KS |
| level 2004 | Sector |  |  | GHG | 1986 | 2004 | % | equ. 7.2 | % | trend |
| 32 | 1A | 2. Manufacturing Industries and Construction | b. Non-Ferrous Metals | CO2 | 440,216 | 57,119 | 0,29 | 1,918 | 5,10 | yes |
| 33 | 2 | Industrial Processes | 2. Ferroalloys Production | CO2 | 57,635 | 54,721 | 0,27 | 0,012 | 0,03 | no |
| 34 | 4 | D. Agricultural Soils (2) | 2. Pasture, Range and Paddock Manure (3) | N2O | 23,753 | 50,162 | 0,25 | 0,135 | 0,36 | no |
| 35 | 2 | Industrial Processes | 4. Carbide Production | CO2 | 44,985 | 43,001 | 0,22 | 0,008 | 0,02 | no |
| 36 | 3 | Solvent and Other Product Use | D. 1. Use of N2O for Anaesthesia | N2O | 81,903 | 39,246 | 0,20 | 0,212 | 0,56 | yes |
| 37 | 1A | 3. Transport | c. Railways | CO2 | 68,182 | 37,003 | 0,19 | 0,154 | 0,41 | no |
| 38 | 1B | Fugitive Emissions from Fuels (2.Oil and NG) | b. Natural Gas | CH4 | 56,090 | 33,514 | 0,17 | 0,111 | 0,30 | no |
| 39 | 2 | Industrial Processes | 1. Iron and Steel Production | CO2 | 40,149 | 29,774 | 0,15 | 0,050 | 0,13 | no |
| 40 | 1A | 4. Other Sectors | c. Agriculture/Forestry/Fisheries | N2O | 49,388 | 27,172 | 0,14 | 0,110 | 0,29 | no |
| 41 | 1A | 1. Energy Industries | a. Public Electricity and Heat Production | N2O | 26,201 | 25,889 | 0,13 | 0,000 | 0,00 | no |
| 42 | 1A | 4. Other Sectors | b. Residential | N2O | 20,936 | 20,556 | 0,10 | 0,001 | 0,00 | no |
| 43 | 4 | A. Enteric Fermentation | 3. Sheep | CH4 | 4,242 | 20,036 | 0,10 | 0,080 | 0,21 | no |
| 44 | 4 | A. Enteric Fermentation | 8. Swine | CH4 | 23,103 | 18,464 | 0,09 | 0,022 | 0,06 | no |
| 45 | 1A | 2. Manufacturing Industries and Construction | f. Other (as specified in table 1.A(a) sheet 2) | N2O | 35,133 | 18,417 | 0,09 | 0,083 | 0,22 | no |
| 46 | 2 | Industrial Processes | 8. Electrical Equipment | SF6 | 10,241 | 18,312 | 0,09 | 0,041 | 0,11 | no |
| 47 | 1A | 3. Transport | b. Road Transportation | CH4 | 10,681 | 12,723 | 0,06 | 0,011 | 0,03 | no |
| 48 | 4 | B. Manure Management | 12. Liquid Systems | N2O | 6,764 | 8,138 | 0,04 | 0,007 | 0,02 | no |
| 49 | 2 | Industrial Processes | 4. Soda Ash Production and Use | CO2 | 10,290 | 7,321 | 0,04 | 0,014 | 0,04 | no |
| 50 | 4 | A. Enteric Fermentation | 6. Horses | CH4 | 5,498 | 6,380 | 0,03 | 0,005 | 0,01 | no |
| 51 | 2 | Industrial Processes | 3. Limestone and Dolomite Use | CO2 | 20,305 | 5,489 | 0,03 | 0,074 | 0,20 | no |
| 52 | 4 | B. Manure Management | 9. Poultry | CH4 | 16,446 | 5,353 | 0,03 | 0,055 | 0,15 | no |
| 53 | 2 | Industrial Processes | 5. Other (Methanol) | CH4 | 2,929 | 5,190 | 0,03 | 0,012 | 0,03 | no |
| 54 | 1A | 3. Transport | c. Railways | N2O | 8,651 | 4,668 | 0,02 | 0,020 | 0,05 | no |
| 55 | 1A | 2. Manufacturing Industries and Construction | d. Pulp, Paper and Print | N2O | 1,383 | 4,068 | 0,02 | 0,014 | 0,04 | no |
| 56 | 1A | 1. Energy Industries | c. Manufacture of Solid Fuels and Other EI | CO2 | 104,710 | 3,395 | 0,02 | 0,508 | 1,35 | yes |
| 57 | 1A | 2. Manufacturing Industries and Construction | f. Other (as specified in table 1.A(a) sheet 2) | CH4 | 6,676 | 2,884 | 0,01 | 0,019 | 0,05 | no |
| 58 | 1A | 2. Manufacturing Industries and Construction | d. Pulp, Paper and Print | CH4 | 1,060 | 2,471 | 0,01 | 0,007 | 0,02 | no |
| 59 | 4 | A. Enteric Fermentation | 4. Goats | CH4 | 1,050 | 2,418 | 0,01 | 0,007 | 0,02 | no |
| 60 | 1A | 4. Other Sectors | a. Commercial/Institutional | CH4 | 15,279 | 2,389 | 0,01 | 0,064 | 0,17 | no |
| 61 | 1A | 3. Transport | a. Civil Aviation | CO2 | 0,622 | 1,891 | 0,01 | 0,006 | 0,02 | no |
| 62 | 1A | 4. Other Sectors | a. Commercial/Institutional | N2O | 5,087 | 1,860 | 0,01 | 0,016 | 0,04 | no |
| 63 | 1A | 1. Energy Industries | a. Public Electricity and Heat Production | CH4 | 1,595 | 1,454 | 0,01 | 0,001 | 0,00 | no |
| 64 | 1A | 1. Energy Industries | b. Petroleum Refining | CO2 | 62,181 | 0,983 | 0,00 | 0,307 | 0,82 | yes |
| 65 | 1A | 2. Manufacturing Industries and Construction | c. Chemicals | N2O | 0,248 | 0,743 | 0,00 | 0,003 | 0,01 | no |
| rank | CRF | IPCC Source Category |  | Direct | Gg CO2 eq. | Gg CO2 eq. | level 2004 | trend | trend | KS |
| level 2004 | Sector |  |  | GHG | 1986 | 2004 | % | equ. 7.2 | % | trend |
| 66 | 4 | B. Manure Management | 14. Other AWMS | N2O | 1,322 | 0,660 | 0,00 | 0,003 | 0,01 | no |
| 67 | 1A | 2. Manufacturing Industries and Construction | c. Chemicals | CH4 | 0,107 | 0,614 | 0,00 | 0,003 | 0,01 | no |
| 68 | 1A | 4. Other Sectors | c. Agriculture/Forestry/Fisheries | CH4 | 1,436 | 0,560 | 0,00 | 0,004 | 0,01 | no |
| 69 | 1A | 2. Manufacturing Industries and Construction | a. Iron and Steel | CH4 | 2,057 | 0,522 | 0,00 | 0,008 | 0,02 | no |
| 70 | 2 | Industrial Processes | 2. Foam Blowing | HFC |  | 0,504 | 0,00 | 0,003 | 0,01 | no |
| 71 | 4 | B. Manure Management | 6. Horses | CH4 | 0,428 | 0,496 | 0,00 | 0,000 | 0,00 | no |
| 72 | 4 | B. Manure Management | 3. Sheep | CH4 | 0,101 | 0,476 | 0,00 | 0,002 | 0,01 | no |
| 73 | 2 | Industrial Processes | 7. Other (Glass Production) | CO2 |  | 0,387 | 0,00 | 0,002 | 0,01 | no |
| 74 | 1A | 2. Manufacturing Industries and Construction | a. Iron and Steel | N2O | 3,270 | 0,323 | 0,00 | 0,015 | 0,04 | no |
| 75 | 1A | 2. Manufacturing Industries and Construction | e. Food Processing, Beverages and Tobacco | N2O | 0,615 | 0,313 | 0,00 | 0,001 | 0,00 | no |
| 76 | 1A | 2. Manufacturing Industries and Construction | e. Food Processing, Beverages and Tobacco | CH4 | 0,178 | 0,201 | 0,00 | 0,000 | 0,00 | no |
| 77 | 2 | Industrial Processes | 3. Fire Extinguishers | HFC |  | 0,182 | 0,00 | 0,001 | 0,00 | no |
| 78 | 4 | B. Manure Management | 11. Anaerobic Lagoons | N2O | 0,948 | 0,140 | 0,00 | 0,004 | 0,01 | no |
| 79 | 1A | 2. Manufacturing Industries and Construction | b. Non-Ferrous Metals | CH4 | 0,691 | 0,088 | 0,00 | 0,003 | 0,01 | no |
| 80 | 1A | 2. Manufacturing Industries and Construction | b. Non-Ferrous Metals | N2O | 1,252 | 0,062 | 0,00 | 0,006 | 0,02 | no |
| 81 | 4 | B. Manure Management | 4. Goats | CH4 | 0,025 | 0,058 | 0,00 | 0,000 | 0,00 | no |
| 82 | 1A | 3. Transport | c. Railways | CH4 | 0,078 | 0,042 | 0,00 | 0,000 | 0,00 | no |
| 83 | 2 | Industrial Processes | 2. Nitric Acid Production | N2O | 0,000 | 0,020 | 0,00 | 0,000 | 0,00 | no |
| 84 | 1A | 3. Transport | a. Civil Aviation | N2O | 0,005 | 0,016 | 0,00 | 0,000 | 0,00 | no |
| 85 | 1A | 1. Energy Industries | c. Manufacture of Solid Fuels and Other EI | N2O | 0,226 | 0,009 | 0,00 | 0,001 | 0,00 | no |
| 86 | 1A | 1. Energy Industries | b. Petroleum Refining | N2O | 0,069 | 0,002 | 0,00 | 0,000 | 0,00 | no |
| 87 | 1A | 1. Energy Industries | c. Manufacture of Solid Fuels and Other EI | CH4 | 0,201 | 0,002 | 0,00 | 0,001 | 0,00 | no |
| 88 | 1A | 1. Energy Industries | b. Petroleum Refining | CH4 | 0,094 | 0,001 | 0,00 | 0,000 | 0,00 | no |
| 89 | 1A | 3. Transport | a. Civil Aviation | CH4 | 0,000 | 0,001 | 0,00 | 0,000 | 0,00 | no |
| 90 | 1B | Fugitive Emissions from Fuels (2.Oil and NG) | a. Oil | CH4 | 0,422 | 0,000 | 0,00 |  |  | no |
| 91 | 2 | Industrial Processes | 4. Carbide Production | CH4 | 0,783 | 0,000 | 0,00 |  |  | no |