# ANNEX 4

# Annex 4: CO2 reference approach and comparison with sectoral approach, and relevant information on the national energy balance

## Total Emission of CO2 - Reference approach and comparison with sectorial approach

Table 1: Emissions of CO2 in Slovenia (reference approach) for the Period 1986 to 2004

| Gg CO2 | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Crude Oil | 679 | 710 | 1610 | 3 | 2 | 0 | 0 | 0 |
| Lubricants | NE | NE | NE | NE | NE | NE | NE | 30 |
| LPG | 123 | 97 | 99 | 230 | 251 | 243 | 252 | 239 |
| Gasoline | 1330 | 1654 | 2149 | 2509 | 2523 | 2403 | 2346 | 2095 |
| Jet Kerosene, Kerosene | 1 | 3 | 2 | 3 | 2 | 2 | 2 | 2 |
| Gas Oil | 1001 | 937 | 1166 | 1529 | 1623 | 1746 | 1938 | 2298 |
| Fuel Oil | 1486 | 1815 | 1799 | 2693 | 2635 | 2491 | 2321 | 2260 |
| Petroleum Coke | 74 | 137 | 94 | 74 | 84 | 77 | 84 | 126 |
| Total Liquid Fuels | 4682 | 5342 | 6912 | 7026 | 7130 | 6947 | 6924 | 7053 |
| Other Bitoumnus Coal | 123 | 1 | 2 | 0 | 0 | 54 | 134 | 110 |
| Lignite and Brown Coal | 7977 | 6577 | 5589 | 5490 | 6189 | 6423 | 6025 | 6159 |
| Gas Oven/Gas Coke | 695 | 216 | 176 | 146 | 265 | 265 | 189 | 185 |
| Anthracite | 154 | 88 | 54 | 0 | 0 | 0 | 0 | 0 |
| Solid Fuel Totals | 8949 | 6882 | 5821 | 5636 | 6454 | 6742 | 6349 | 6448 |
| Natural gas | 1407 | 1627 | 1469 | 1642 | 1682 | 1640 | 1798 | 1813 |
| Gaseous Fuel Totals | 1407 | 1627 | 1469 | 1642 | 1682 | 1640 | 1798 | 1813 |
| Biomass |  |  |  |  |  |  |  |  |
| Fuel wood | 1320 | 1872 | 1819 | 1959 | 1659 | 1687 | 1796 | 1644 |
| Biomass Total | 1320 | 1872 | 1819 | 1959 | 1659 | 1687 | 1796 | 1644 |
| TOTAL (w/o biomass) | 15174 | 13993 | 14377 | 14577 | 15530 | 15580 | 15381 | 15314 |
| Stored carbon: |  |  |  |  |  |  |  |  |
| Natural gas | 125 | 131 | 173 | 258 | 275 | 236 | 291 | 257 |
| Lubricants | 10 | 11 | 6 | 14 | 14 | 14 | 19 | 15 |
| TOTAL (w/o biomass) | 15039 | 13852 | 14198 | 14304 | 15240 | 15329 | 15071 | 14907 |

Note: Some data in the CRF Reference aproach table were found to be missing and therefore there is a small difference between this table and totals from CRF.

Year 2001 - in CRF Table – Reference approach Export and Stock changes of Residual Fuel Oil are missing.

The total difference of CO2 emissions between the sectoral approach and the reference approach in 2004 amounted to less than 0.6% which is deemed satisfactory. The sectoral approach gives somewhat higher emissions for solid fuels, which are a consequence of coke used as reduction agent and incineration of semi-finished products in industrial stokers, which are, however, not treated in chapter 6.C Waste Incineration.

A comparison of energies between the two approaches shows a bigger difference, amounting to 2%. This is chiefly a consequence of deviations in the consumption of gaseous fuels (+9.4%), since the reference approach covers also non-energy consumption of natural gas for the production of methanol.

Table 2: Differences in energy consumption (Reference approach/National Approach)

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| liquid | -1.15 | 3.48 | 1.91 | 0.07 | 0.80 | -0.36 | -0.18 | 0.41 |
| solid | 0.24 | 1.76 | 0.95 | 0.92 | 2.38 | 1.77 | 2.10 | 1.34 |
| gaseous | 4.46 | 4.52 | 11.11 | 15.85 | 16.55 | 14.34 | 16.08 | 9.42 |
| total | 0.29 | 2.93 | 2.92 | 2.80 | 3.76 | 2.59 | 3.26 | 2.03 |

Table 3: Differences in CO2 emissions (Reference approach/National Approach)

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| liquid | -1.42 | 3.62 | 1.63 | -0.07 | 0.63 | -0.51 | -0.38 | -0.33 |
| solid | 0.17 | 1.72 | 0.85 | 0.96 | 2.45 | 1.90 | 2.21 | 0.83 |
| gaseous | -4.05 | -3.24 | -0.56 | 0.20 | 0.23 | -0.01 | -0.02 | -4.13 |
| total | -0.82 | 1.76 | 1.01 | 0.26 | 1.27 | 0.52 | 0.64 | -0.56 |

## Reference approach – by fuel

### Crude Oil

The only importer and processor of crude oil in Slovenia is Nafta Lendava, therefore data on production, import and export of crude oil are taken from its balance sheets.

Table 4: Crude Oil Consumption and Calculation of Emission of CO2 from Oil Consumption in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Production (Mg) | 2569 | 2506 | 859 | 782 | 963 | 0 | 0 |
| Import (Mg) | 107031 | 128357 | 144598 | 0 | 0 | 0 | 0 |
| - From other YU Rep. (Mg) | 369532 | 412044 | 0 | 0 | 0 | 0 | 0 |
| Import+ YU Rep. (Mg) | 476563 | 540401 | 0 | 0 | 0 | 0 | 0 |
| Export (Mg) | 269181 | 304012 | 0 | 0 | 1004 | 0 | 0 |
| Stock Change (Mg) | -8018 | 10982 | 0 | 26 | -41 | 0 | 0 |
| Consumption (Mg) | 217969 | 227913 | 859 | 756 | 0 | 0 | 0 |
| NCV (MJ/kg) | 42.9 | 42.9 | 42.9 | 42.9 | 42.9 | 42.9 | 42.9 |
| Consumption (TJ) | 9350 | 9777 | 37 | 32 | 0 | 0 | 0 |
| Carbon content.(t C/TJ) | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Emission (Gg C) | 187 | 196 | 1 | 1 | 0 | 0 | 0 |
| Oxidised Fraction | 0,99 | 0,99 | 0,99 | 0,99 | 0,99 | 0,99 | 0,99 |
| Emission (Gg C) | 185 | 194 | 1 | 1 | 0 | 0 | 0 |
| Conversion factor C - CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 679 | 710 | 3 | 2 | 0 | 0 | 0 |

Data source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Tg/5

Emission Factor Source: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

In IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.16 -I.20), the suggested conversion factors from natural units to Joules range from 40.4 to 43.0 TJ/1000 t, for former Yugoslavia 42.75 TJ/1000 tonnes. LEG documents specify for all those years a conversion factor of 42.9 TJ/1000 t. (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Tg/5). Calculations take into account the emission factor of Ministry of the Economy.

For the year 2004 we have obtained data from SORS in electronic format (file name E_8_2004.xls).

Emission of CO2 has been calculated on the basis of the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of a fuel as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low. For emission factors, the uncertainties are somewhat higher. The emission factors applied are taken from IPCC methodology (and are not obtained by measurements performed in Slovenia).

Table 5: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author's estimate

### Liquefied Petroleum Gas  (LPG)

The calculation of consumption of LPG in Slovenia has been done on the basis of data taken from Table Zb/1, Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy. For the year 2004 we have obtained data from SORS in electronic format (file name E_8_2004.xls).

Table 6: Consumption of LPG and Calculation of Emission of CO2 from Consumption of LPG in Slovenia in the Period 1986 to 2004

| (LPG) | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Production (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import (Mg) | 41000 | 400 | 82000 | 85400 | 86400 | 87818 |  |
| - From other YU Rep. (Mg) | 0 | 33100 | 0 | 0 | 0 | 368 |  |
| Import+ YU Rep. (Mg) | 41000 | 33500 | 82000 | 85400 | 86400 | 88186 | 86153 |
| Export (Mg) | 0 | 0 | 0 | 0 | 0 | 5 | 144 |
| Stock Change (Mg) | -1800 | -400 | 2000 | -1800 | 2000 | 488 | 2775 |
| Consumption (Mg) | 42800 | 33900 | 80000 | 87200 | 88400 | 88669 | 83234 |
| NCV (MJ/kg) | 46.05 | 46.05 | 46.05 | 46.05 | 46.05 | 46.05 | 46.05 |
| Consumption (TJ) | 1971 | 1561 | 3684 | 4016 | 4071 | 4083 | 3833 |
| Carbon content.(t C/TJ) | 17.2 | 17.2 | 17.2 | 17.2 | 17.2 | 17.2 | 17.2 |
| Emission (Gg C) | 34 | 27 | 63 | 69 | 70 | 70 | 70 |
| Oxidised Fraction | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| Emission (Gg C) | 34 | 27 | 63 | 68 | 69 | 70 | 66 |
| Conversion factor C - CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 123 | 97 | 230 | 251 | 254 | 255 | 239 |

Data source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Zb/1

Emission Factor Source: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.21) suggest a conversion factor from natural units to Joules 47.3 TJ/1000 t. LEG documents specify for all these years a conversion factor of 46.05 TJ/1000 tonnes

Emission of CO2 has been calculated on the basis of the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of fuels as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low. For emission factors, the uncertainties are somewhat higher. The emission factors applied are specified in IPCC methodology (and are not a result of measurements in Slovenia).

Table 7: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty Of Emission Factors | 10% |

Source: Author's estimate

### Gasoline

Preparing consumption balances for this report took a relatively long period, 17 years. During this period, the modes of presentation and aggregation of data on the consumption of gasoline (motor gasoline and primary gasoline), jet kerosene (kerosene), fuel oil, and gas oil have in part undergone a change. Consequently, this necessitated a different approach to the calculation of consumption for individual years.

Balances of consumption for those fuels have been done on the basis of the following presumptions:

PERIOD 1986 to 1990

Gasoline consumption is split into two groups, one comprising auto and primary gasoline, the other aviation gasoline for turboprop and piston engine aircraft.

1. Under the category Purchase for motor gasoline, quantities reported in LEG-85 to 90 are the sum of purchases in other republics of the former Yugoslavia and total production in Slovenia. The balances on pages Tg/1 (LEG) are illogical, since they make it appear as if there was no production of oil products in Slovenia for the period 1986 to 90, which is not correct. It is evident that the category “Purchase” includes the total production of Nafta Lendava (both quantities sold in Slovenia as well as internal consumption and sales in other republics of the former Yugoslavia) from the reports for the period 1991 to 1996, when the entire production has been presented in the category “Production” (compare Tables Tg/1 in LEG 1986 - 1990 to tables Tg/1 in LEG 1991 - 2003).

Not so for the primary gasoline: quantities produced in Nafta Lendava have not been reported in the category Purchase. Consequently, these quantities are not subtracted later (item 3).

2. This is how total available quantities of an individual refined petroleum product in Slovenia have been obtained.

3. From the total available quantity of an individual refined petroleum product in Slovenia, the total final production of Nafta-Lendava, which has been taken into account in category Purchase in LEG, Table Tg/5, is subtracted. Thus:

– production of auto gasoline sold in Slovenia

– production of auto Gasoline exported.

The production of primary gasoline, which is reported under items Processing and Internal Consumption is not taken into account, since these quantities are processed further into other products.

Outline of the Calculation of Consumption of Oil Products 1986 to 1990

| Purchase | Purchase in other republics of the former Yugoslavia. This category for the period 1986 to 1990 includes also production in Slovenia |
| --- | --- |
| +Import |  |
| =Available quantities |  |
| - Production in Nafta Lendava (i.e. production in Slovenia) | Subtract all quantities of motor gasoline produced in Nafta Lendava (sold in Slovenia, in other republics of the former Yugoslavia or used for internal consumption). These quantities are subtracted because they are reported in the category Purchase (and should not be). Primary gasoline is not reported in the category Purchase, and is therefore not subtracted here (!!!!!!). |
| Real Import | Thus are obtained quantities which are available in Slovenia and the emissions of which are taken into account in the processing of crude oil. |
| - Real Export | It is necessary to subtract quantities which are not used in Slovenia and which have been taken into account for emissions from the processing of crude oil. This is the quantity that is exported – that part of the production of refined petroleum products of Nafta Lendava, which is sold to other republics of the former Yugoslavia: motor gasoline and primary gasoline. Export is also that quantity of motor gasoline that is sold to other republics of the former Yugoslavia as reported by Elektrogospodarstvo (LEG, Tg/1 and Tg/2) – i.e. sales of Istrabenz in other republics of the former Yugoslavia. For gasoline for the period 1986 to 1991, the sum of both exports is taken into account (from tables Tg/1 and Tg/5). Taken into account is also the export of motor gasoline (emissions from its consumption will be reported by the country which will retail this gasoline) and export of raw gasoline (which will be processed into motor gasoline in the exporting country and reported within the framework of the consumption of motor gasoline in that country). In the same way, the export of fuel oil and gas oil is calculated. |
| - (+) Stock Change |  |
| Consumption in Slovenia | The result are quantities consumed in Slovenia which have not been taken into account for the processing of crude oil, minus quantities of refined petroleum products, which are exported. |

4. Quantities obtained are the actual import and purchase from other republics of the former Yugoslavia. The produced quantities of auto gasoline are subtracted because CO2 emissions from them have already have been taken into account in the processing of crude oil. Should they not be subtracted, then they are presented twice: in the category Oil Products (gasoline, gas oil and fuel oil) and Crude Oil Consumption.

5. Subsequently all exported values must be subtracted since these emissions do not arise in Slovenia,

– export or sales outside the Republic of Slovenia, as reported by Nafta-Lendava (Table Tg/5 – for gasoline the export of motor gasoline and primary gasoline is taken into account).

– values that are reported as export by Elektrogospodarstvo: these are the quantities, which are exported by Istrabenz to its agencies abroad.

Export of oil products (gasoline, gas oil and fuel oil) must therefore be subtracted twice: firstly because the quantities consumed have already been reported in the category Purchase, and secondly, because emissions have already been taken into account in the consumption of crude oil.

Stock change is obtained by subtracting the stocks for the end of the period from stock at the beginning of the period.

6. The obtained quantities are taken into account for the calculation of emissions of CO2.

Data calculated as the consumption of oil products for 1990 in this report differ from those specified in Seljak 1998, where those quantities of refined petroleum products which had been sold by Istrabenz in other republics of the former Yugoslavia or abroad have not been taken into account as export.

YEAR 1991

In calculating the consumption of refined petroleum products in Slovenia for 1991, an error has occurred in LEG. Production of Nafta Lendava has been reported twice: under the category Purchase (as was the case in LEG 1986-1990) and under the category Production (as reported since 1992). Consequently, the quantities of motor gasoline, gas oil, fuel oil and residual fuel oil, which were produced in Nafta Lendava, have been subtracted from the category Purchase (or import from other republics of the former Yugoslavia) of motor gasoline (or gas oil / fuel oil).

Table 8: Calculation of Purchase of Oil Products in other Republics of the Former Yugoslavia in 1991

|  | Quantities reported in LEG (TG/1)- tonnes | Sales in Slovenia – LEG /Tg/5 | Real Purchase in other Republics of the Former Yugoslavia |
| --- | --- | --- | --- |
| Motor Gasoline | 324000 | 113932 | 210068 |
| Gas oil | 251401 | 104717 | 146684 |
| Fuel Oil and Residual Fuel Oil | 494219 | 235106 | 259113 |
| Fuel Oil | 302524 | 143885 | 158577 |
| Residual Fuel Oil | 191695 | 91221 | 100536 |

The ratio has been calculated on the basis of reported quantities in column: Reported Quantities in LEG (TG/1): 38.8 % residual fuel oil – 61.2 % fuel oil.

The export of oil products in 1991 is obtained by summing up the export as reported by Nafta Lendava (motor gasoline and primary gasoline) and export as reported by Elektrogospodarstvo (sales of Istrabenz in other republics of the former Yugoslavia).

PERIOD 1992-2003

1. In LEG 1992-1996 under the category Purchase, only those values were reported which had actually been purchased in other republics of the former Yugoslavia (since 1994 these quantities are no more). Under the category Production, the quantities produced in Slovenia (Nafta Lendava) are reported. Calculations must not take into account the oil products that were produced in Slovenia (since both calorific values as well as emissions of CO2 have already been taken into account for the consumption of crude oil). Hence, further calculations take into account only quantities either purchased in other republics of the former Yugoslavia or imported.

2. Subtract all export from the obtained quantities. This is the value that was provided by Elektrogospodarstvo as sales into other republics of former Yugoslavia (until 1993) as well as export (since 1994).

Stock changes are obtained by subtracting the stocks for the end of the period from the stocks for the beginning of the period.

3. Thus are obtained the quantities taken into account in calculating emissions of CO2.

Outline of Consumption of Oil Products 1992-2000

| Purchase | Under this category in LEG-91 to 96 the quantities purchased in other republics of the former Yugoslavia have been reported |
| --- | --- |
| +Import | Import from other countries |
| Real Import | Quantities which have been available in Slovenia and which have not been taken into account in processing of crude oil. |
| - Real Export | Minus quantities which have not been consumed in Slovenia and which have been taken into account for emissions from the processing of crude oil. This quantity is exported. For gasoline, the export of both motor gasoline as well as primary gasoline is subtracted. |
| - (+) Stock Change |  |
| Consumption in Slovenia | Quantities which have been consumed in Slovenia and which have been taken into account for the processing of crude oil, minus quantities of refined petroleum products that have been exported. |

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.21) suggest a conversion factor from natural units to Joules of 44.8 TJ/1000 t. The factors specified in LEG for individual years ranged from 42.87 to 43.14 TJ/1000 t (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy, Table Zb/1). Based on measurements, the emission factor of the Ministry of the Economy has been taken into account in calculations for individual years. For the year 2004 we have obtained data from SORS in electronic format (file name E_8_2004.xls).

The emission of CO2 has been calculated on the basis of the formula and emission factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Table 9: Consumption of Gasoline and Calculation of Emission of CO2 from Consumption of Gasoline in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Purchase (Mg) | 445600 | 497300 | 34000 | 0 | 0 | 0 | 0 |
| Production of Motor Gasoline (Mg) |  |  |  |  |  |  |  |
| Import (Mg) | 5500 | 78000 | 868000 | 805900 | 777800 | 781935 | 717856 |
| Available Quantity in Slovenia (Mg) | 451100 | 575300 | 902000 | 805900 | 777800 | 781935 | 717856 |
| - Production of Motor Gasoline by Lendava, sold in Slovenia | 17248 | 40204 |  |  |  |  |  |
| - Export of Gasoline by Lendava |  |  |  |  |  |  |  |
| - Internal Consumption of Gasoline | 0 | 0 |  |  |  |  |  |
| Total production of Motor Gasoline in Slovenia | 17248 | 40204 |  |  |  |  |  |
| Real Import (Mg) | 433852 | 535096 | 902000 | 805900 | 777800 | 781935 | 717856 |
| Export, as reported by Elektrogospodarstvo - Sales of Istrabenz Abroad (Mg) | 6500 | 8800 |  |  |  |  |  |
| Export, as Reported by Nafta Lendava (Mg) |  |  |  |  |  |  |  |
| Export – Total (Mg) | 6500 | 8800 | 78000 | 29100 | 20800 | 26858 | 29905 |
| Stock Change (Mg) | 5300 | -223 | 26000 | -25400 | -7000 | 9213 | 21814 |
| Consumption in Slovenia minus Production of Lendava (Mg) | 422052 | 526519 | 798000 | 802200 | 764000 | 745864 | 666137 |
| Factor MJ/kg | 43.19 | 43.07 | 43.08 | 43.08 | 43.08 | 43.08 | 43.08 |
| Consumption (TJ) | 18228 | 22677 | 34378 | 34559 | 32913 | 32132 | 28697 |
| Emission Factor (t C/TJ) | 20.10 | 20.09 | 20.11 | 20.11 | 20.11 | 20.11 | 20.108 |
| Emission (Gg C) | 366 | 456 | 691 | 695 | 662 | 646 | 577 |
| Oxidised Fraction | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| Emission (Gg C) | 363 | 451 | 684 | 688 | 655 | 640 | 571 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 1330 | 1654 | 2509 | 2523 | 2403 | 2346 | 2095 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Tg/1, Tg/5

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13, estimates of an expert group.

Basic sources do not specify any uncertainties. The utilized input data are calculated on the basis of two balances: the balance sheets of Nafta Lendava and Elektrogospodarstvo Slovenije. With regard to the fact that many categories of consumption and of production have not been harmonized, the estimated total uncertainty of input data amounts to 10 %. The uncertainty of input data differs for individual years:

- 5% for the period 1993 to 2004,

- 10 % for the period 1986 to 1990, 1992

- 20 % for year 1991.

Calculations account for the factor of calorific value of fuels issued by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low. For emission factors, the uncertainties are somewhat higher. The emission factors applied are taken from IPCC methodology (and are not a result of measurements in Slovenia).

Table 10: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data for the Period 1993 to 2004 | 5 % |
| --- | --- |
| Estimate of Uncertainty of Input Data for the Period 1986 to 1990, 1992 | 10% |
| Estimate of Uncertainty of Input Data for 1991 | 20 % |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author's estimate

### Jet kerosene, Kerosene

Calculation of consumption of jet kerosene and kerosene in Slovenia has been done on the basis of data from the tables Tg/1 and Tg/5, Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy. For the year 2004 we have obtained data from SORS in electronic format (file name E_8_2004.xls).

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.21) suggest a factor of 44.6 TJ/1000 tonnes. LEG documents specify for all those years a conversion factor of 42.8 TJ/1000 tonnes (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Zb/1). Based on measurements, the emission factor of the Ministry of the Economy has been taken into account in calculations.

Emission of CO2 has been calculated on the basis of the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Most jet kerosene is consumed outside the territory of Slovenia in international transport in the so-called international bunkers, and that is why these quantities are not included in the calculation of national emissions for Slovenia. Quantities, which are specified in LEG under the category of aviation gasoline for turboprop engines, are considered as being consumed in international transport, while quantities under the Aviation Gasoline for Piston Engine Aircraft category is considered as consumption in Slovenia. After independence, the consumption of jet kerosene in international transport fell strongly (likewise the number of transported passengers diminished from 1.7 million in 1987 to 0.55 million in 1995 – Source: Statistical Office of the Republic of Slovenia: Letni pregled prometa in zvez, Ljubljana: Statistical Office of the Republic of Slovenia 1995).

Table 11: Consumption of Jet Kerosene and Calculation of Emission of CO2 from Consumption of Jet Kerosene in Slovenia for the Period 1986 to 2004

|  | 1986 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- |
| Purchase (Mg) | 22300 | 0 | 0 | 0 | 0 | 0 |
| Import (Mg) | 10300 | 24622 | 27458 | 33342 | 28422 | 22864 |
| Real Import (Mg) | 32600 | 24622 | 27458 | 33342 | 28422 | 22864 |
| Real Export (Mg) | 0 | 237 | 922 | 5943 | 2367 | 610 |
| Stock Change (Mg) | 600 | 231 | -427 | -363 | 232 | 1954 |
| Consumption Abroad (Mg) | 31800 | 23230 | 26207 | 27009 | 25050 | 19679 |
| Consumption in Slovenia (Mg) | 200 | 924 | 756 | 753 | 772 | 621 |
| Factor MJ/kg | 43.5 | 43.5 | 43.5 | 43.5 | 43.5 | 43.5 |
| Consumption (TJ) | 9 | 40 | 33 | 33 | 34 | 27 |
| Emission Factor (t C/TJ) | 19.5 | 19.5 | 19.5 | 19.5 | 19.5 | 19.5 |
| Emission (Gg C) | 0 | 1 | 1 | 1 | 1 | 1 |
| Oxidised Fraction | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| Emission (Gg C) | 0 | 1 | 1 | 1 | 1 | 1 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 1 | 3 | 2 | 2 | 2 | 2 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Tg/2, Tg/4

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of a fuel as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low. For emission factors, the uncertainties are somewhat higher. The emission factors applied are specified in IPCC methodology (and are not a result of measurements in Slovenia).

Table 12: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author's estimate

### Gas oil

The calculation of consumption of gas oil in Slovenia has been done on the basis of data from tables Tg/1 and Tg/5, Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy. Consumption has been calculated on the basis of presumptions, which for secondary fuels are presented in the chapter on gasoline. For the year 2004 we have obtained data from SORS in electronic format (file name E_8_2004.xls).

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I. 21) specify a factor of 43.3 TJ/1000 t. For all those years, a conversion factor of 42.7 TJ/1000 tonnes has been specified in LEG reports (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy, Table Zb/1). Based on measurements, the emission factor of the Ministry of the Economy has been taken into account in calculations.

Emission of CO2 has been calculated on the basis of the formula and emission factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Table 13: Consumption of Gas Oil and Calculation of Emission of CO2 from Consumption of Gas Oil in Slovenia for the Period 1986 to 2003

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Purchase (Mg) | 336000 | 348700 | 30000 | 0 | 0 | 0 | 0 |
| Production (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import (Mg) | 45200 | 7000 | 519000 | 578318 | 630100 | 688670 | 829622 |
| Purchase +Import (Mg) | 381200 | 355700 | 549000 | 578318 | 630100 | 688670 | 829622 |
| - Production of Lendava, sold in Slovenia | 53469 | 48451 |  |  |  |  |  |
| - Export Lendava |  |  |  |  |  |  |  |
| - Internal Consumption (Mg) | 0 | 169 |  |  |  |  |  |
| Total Production in Slovenia (Mg) | 53469 | 48620 | 0 | 0 | 0 | 0 | 0 |
| Real Import (Mg) | 327731 | 307080 | 549000 | 578318 | 630100 | 688670 | 829622 |
| Export, as reported by Elektrogospodarstvo - Sales of Istrabenz abroad (Mg) | 4100 | 9400 |  |  |  |  |  |
| Export, as reported by Nafta Lendava (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Export - Total (Mg) | 4100 | 9400 | 87000 | 74079 | 73298 | 49280 | 60699 |
| Stock Change (Mg) | 5500 | -180 | -24000 | -11699 | 1759 | 23175 | 38407 |
| Consumption in Slovenia minus Production of Lendava (Mg) | 318131 | 297860 | 486000 | 515938 | 555043 | 616215 | 730516 |
| Factor MJ/kg | 42.7 | 42.7 | 42.7 | 42.7 | 42.7 | 42.7 | 42.7 |
| Consumption (TJ) | 13584 | 12719 | 20752 | 22031 | 23700 | 26312 | 31193 |
| Emission Factor (t C/TJ) | 20.29 | 20.3 | 20.3 | 20.3 | 20.3 | 20.3 | 20.3 |
| Emission (Gg C) | 276 | 258 | 421 | 447 | 481 | 534 | 633 |
| Oxidised Fraction | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| Emission (Gg C) | 273 | 255 | 417 | 443 | 476 | 529 | 627 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 1001 | 937 | 1529 | 1623 | 1746 | 1938 | 2298 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Tg/1, Tg/5

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Basic sources do not specify any uncertainties. The utilized input data are calculated on the basis of two sources: the balance sheets of Nafta Lendava and Ministry of the Economy, hence the input data uncertainties are higher. Calculations take into account the factor of calorific value of fuels specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are low. For emission factors, the uncertainties are higher. The emission factors applied are specified in IPCC methodology (and are not a result of measurements in Slovenia).

Table 14: Estimate of Uncertainty of Utilized of Data (in %)

| Estimate of Uncertainty of Input Data for the Period 1993 to 1996 | 5 % |
| --- | --- |
| Estimate of Uncertainty of Input Data for the Period 1986 to 1990, 1992 | 10% |
| Estimate of Uncertainty of Input Data for 1991 | 20 % |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author's estimate

### Fuel Oil

Table 15: Consumption of Fuel Oil and Calculation of Emission of CO2 from Consumption of Fuel Oil in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Purchase (Mg) | 264900 | 370800 |  |  |  |  |  |
| Production (Mg) |  |  | 31000 |  |  |  |  |
| Import (Mg) | 0 | 37800 | 762000 | 787440 | 746743 | 704867 | 741583 |
| Purchase +Import (Mg) | 264900 | 408600 | 793000 | 787440 | 746743 | 704867 | 741583 |
| – Production Lendava, sold in Slovenia |  |  |  |  |  |  |  |
| – Export Lendava |  |  |  |  |  |  |  |
| – Internal Consumption (Mg) |  |  |  |  |  |  |  |
| Total Production in Slovenia (Mg) |  |  |  |  |  |  |  |
| Real Import (Mg) | 264900 | 408600 | 793000 | 787440 | 746743 | 704867 | 741583 |
| Export as reported by Elektrogospodarstvo - Sales of Istrabenz abroad (Mg) | 1800 | 4100 |  |  |  |  |  |
| Export as reported by Nafta Lendava (Mg) |  |  |  |  |  |  |  |
| Export - Total (Mg) | 1800 | 4100 | 43000 | 47006 | 13502 | 6114 | 21620 |
| Stock Change (Mg) | 2100 | 3400 | 22000 | -5784 | 23400 | 3786 | -15761 |
| Consumption in Slovenia minus Production of Lendava (Mg) | 261000 | 401100 | 728000 | 746218 | 709841 | 694968 | 735724 |
| Factor MJ/kg | 41.82 | 41.87 | 41.9 | 41.9 | 41.9 | 41.9 | 41.77 |
| Consumption (TJ) | 10915 | 16794 | 30503 | 31267 | 29742 | 29119 | 30731 |
| Emission Factor (t C/TJ) | 20.2 | 20.2 | 20.2 | 20.2 | 20.2 | 20.2 | 20.26 |
| Emission (Gg C) | 220 | 339 | 616 | 632 | 601 | 588 | 623 |
| Oxidised Fraction | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| Emission (Gg C) | 218 | 336 | 610 | 625 | 595 | 582 | 616 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2 | 800 | 1232 | 2237 | 2293 | 2181 | 2135 | 2260 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Tg/1, Tg/5 For the year 2004 we have obtained data from SORS in electronic format (file name E_8_2004.xls).

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Because of the differing definitions for individual categories within the category “Fuel Oil” (extra light, light, residual) a direct presentation of separate data for the production of fuel oil and residual fuel oil is not possible. Those data after 1992 are not even necessary for the presentation of consumption in the reference approach. For the period 1986 to 1991, an estimation is necessary. LEG (Table TG/5) brings only data for the total production of fuel oil and residual fuel oil in Nafta Lendava. The Statistical Office of the Republic of Slovenia (Statistical Office of the Republic of Slovenia: SL-86 to 91) publishes data on the production of fuel oil in its publications. However, data on the production of residual fuel oil are reported under the category Fuel Oil (cf. data on production for the period 1992 to 2002 in LEG and SL-96, p. 287). For the period 1986 to 1991 data have been estimated:

### Residual Fuel Oil

Table 16: Consumption of Residual Fuel Oil and Calculation of Emission of CO2 from Consumption of Residual Fuel Oil in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Purchase (Mg) | 306900 | 257700 |  |  |  |  |  |
| Production (Mg) |  |  | 42000 |  |  |  |  |
| Import (Mg) | 80700 | 65300 | 109000 | 111085 | 97300 | 62745 | 55884 |
| Purchase +Import (Mg) | 387600 | 323000 | 151000 | 111085 | 97300 | 62745 | 55884 |
| – Production Lendava sold in Slovenia | 162492 | 99244 | 0 | 0 | 0 | 0 | 0 |
| – Export Lendava |  |  |  |  |  |  |  |
| – Internal Consumption (Mg) |  |  |  |  |  |  |  |
| Total Production in Slovenia (Mg) | 162492 | 99244 | 0 |  |  |  |  |
| Real Import (Mg) | 225108 | 223756 | 151000 | 111085 | 97300 | 62745 | 55884 |
| Export as reported by Elektrogospodarstvo - Sales of Istrabenz abroad (Mg) |  |  |  |  |  |  |  |
| Export as reported by Nafta Lendava (Mg) |  |  |  |  |  |  |  |
| Export – Total (Mg) | 0 | 0 | 3000 | 515 | 0 | 0 | 1838 |
| Stock Change (Mg) | 0 | 32300 | -1000 | -997 | -4000 | 2188 | -1360 |
| Consumption in Slovenia minus Production Lendava (Mg) | 225108 | 191456 | 149000 | 111567 | 101300 | 60557 | 50395 |
| Factor MJ/kg | 39.74 | 39.8 | 40 | 40 | 40 | 40 | 40 |
| Consumption (TJ) | 8946 | 7620 | 5960 | 4463 | 4052 | 2422 | 2016 |
| Emission Factor (t C/TJ) | 21.1 | 21.1 | 21.1 | 21.1 | 21.1 | 21.1 | 21.1 |
| Emission (Gg C) | 189 | 161 | 126 | 94 | 85 | 51 | 43 |
| Oxidised Fraction | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| Emission (Gg C) | 187 | 159 | 124 | 93 | 85 | 51 | 42 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 685 | 584 | 457 | 342 | 310 | 186 | 154 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Tg/1, Tg/5

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Basic sources do not specify any uncertainties. The utilized input data are calculated on the basis of two sources: the balance sheets of Nafta Lendava and Ministry of the Economy, hence the input data uncertainties are higher. Calculations take into account the factor of calorific value of fuels specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are low. For emission factors, the uncertainties are higher. The emission factors applied are specified in IPCC methodology (and are not a result of measurements in Slovenia).

Table 17: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data for the Period 1993 to 2004 | 5 % |
| --- | --- |
| Estimate of Uncertainty of Input Data for the Period 1986 to 1990 | 10% |
| Estimate of Uncertainty of input data for years 1991, 1992 | 20 % |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author's estimate

### Bitumen and Lubricants

IPCC methodology (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.21) enables reporting the consumption of secondary fuels or products only in the case of their being imported. In Slovenia, there are no exact data on the import of bitumen and lubricants. This section takes into account the presumption that all consumed quantities of bitumen have been imported (or produced from imported raw materials), since there are no reports on the production of bitumen in Slovenia (Statistical Office of the Republic of Slovenia: Annual Industry Survey 1986-2003). The consumption of imported bitumen has no influence on the final emissions of CO2. The imported quantities of bitumen have no influence on emissions of CO2: they have to be added up in the chapter on the consumption of bitumen and subtracted in the chapter on carbon, which was stored in products.

For the consumption of lubricants, only imported quantities should be specified, since emissions of other quantities have already been taken into account for the processing of crude oil. Since these data (on import) are available in Slovenia only for 2004 in satisfactory quality, the quantities on the consumption of lubricants are not specified for other years. Imported quantities of lubricants have no influence on emissions of CO2: they have to be added up in the chapter on the consumption of lubricants and subtracted in the chapter on stored carbon. The chapter on stored carbon therefore reports only the consumed quantities of lubricants that have been produced in Slovenia.

### Petroleum Coke

The calculation of consumption of petroleum coke in Slovenia has been done on the basis of data from the research of the Institute of Metals and Technology (Breskvar, Torkar 1999) and the report of the Treibacher Schleifmittel d.o.o. (Nekrep 1999). No adequate data are available for Slovenia, which would enable presentation similar to that for other secondary liquid fuels. Most petroleum coke is consumed in Talum, Kidričevo for the production of electrodes, whereas until 1994 small quantities were used in the production of silicon carbide by Tovarna Dušika Ruše. The balance for Slovenia has been done on the basis of the reports of those two enterprises. For the year 2004 we have obtained data from SORS in electronic format (file name E_8_2004.xls).

In IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13 and I. 23), the suggested conversion factor from natural units to Joules amounts to 31.00 TJ/1000 t, and the emission factor amounts to 27.5 t C/TJ.

The emission of CO2 has been calculated on the basis of the formula and emission factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Table 18: Consumption of Petroleum Coke and Calculation of Emission of CO2 from Consumption of Petroleum Coke in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Production (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import (Mg) | 24019 | 44273 | 23931 | 27124 | 24799 | 27228 | 40654 |
| Purchase (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import + Purchase (Mg) | 24019 | 44273 | 23931 | 27124 | 24799 | 27228 | 40654 |
| Export (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Stock Change (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Consumption (Mg) | 24019 | 44273 | 23931 | 27124 | 24799 | 27228 | 40654 |
| Factor MJ/kg | 31.00 | 31.00 | 31.00 | 31.00 | 31.00 | 31.00 | 31.00 |
| Consumption (TJ) | 745 | 1372 | 742 | 841 | 769 | 844 | 1260 |
| Emission Factor (t C/TJ) | 27.5 | 27.5 | 27.5 | 27.5 | 27.5 | 27.5 | 27.5 |
| Emission (Gg C) | 20 | 38 | 20 | 23 | 21 | 23 | 35 |
| Oxidised Fraction | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 | 0.99 |
| Emission (Gg C) | 20.3 | 37.4 | 20.2 | 22.9 | 20.9 | 23.0 | 34 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 74 | 137 | 74 | 84 | 77 | 84 | 126 |

Data Source:

Breskvar B., Torkar M.: Določitev emisij toplogrednih plinov v proizvodnji aluminija, železa in jekla ter ferozlitin, Institute of Metals and Technology, Ljubljana 1999

Nekrep K: CO2 Emisije v proizvodnji SiC (working material), Treibacher Schleifmittel, Ruše 1999

Reports from enterprises

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13, 1. 23

Basic sources do not specify any uncertainties. All utilized input data are taken from reports of the enterprises using petroleum coke; hence, the uncertainty is somewhat higher. The calorific values of fuels and emission factors are specified in IPCC methodology (and are not a result of measurements in Slovenia); hence, the uncertainty is also somewhat higher than for other secondary fuels.

Table 19: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 10% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 10% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author's estimate

### Hard Coal

The calculation of the consumption of hard coal in Slovenia has been done on the basis of data from Table P/1, Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy.

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.14 -I.20) suggest a conversion factor from natural units to Joules from 18.00 to 33.5 TJ/1000 t, for the former Yugoslavia from 23.6 to 30.7 TJ/1000 tonnes. LEG reports specify for different years emission factors from 25 to 27.86 TJ/1000 tonnes (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Zb/1). Based on measurements, the emission factor of the Ministry of the Economy has been taken into account in calculations for individual years.

Table 20: Consumption of Black Coal and Calculation of Emission of CO2 from Consumption of Black Coal in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Production (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import (Mg) | 62700 | 500 | 0 | 0 | 21817 | 54200 | 44197 |
| Purchase (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import + Purchase (Mg) | 62700 | 500 | 0 | 0 | 21817 | 54200 | 44197 |
| Export (Mg) | 15100 | 0 | 0 | 0 | 0 | 0 | 0 |
| Stock Change (Mg) | -600 | 0 | 0 | 0 | 0 | 0 | 0 |
| Consumption (Mg) | 48200 | 500 | 0 | 0 | 21817 | 54200 | 44197 |
| Factor MJ/kg | 27.57 | 27.57 | 25.00 | 25.00 | 25.00 | 25.00 | 25.00 |
| Consumption (TJ) | 1329 | 14 | 0 | 0 | 545 | 1355 | 1105 |
| Emission Factor (t C/TJ) | 25.8 | 25.8 | 25.8 | 25.8 | 27.6 | 27.6 | 27.6 |
| Emission (Gg C) | 34 | 0 | 0 | 0 | 15 | 37 | 30 |
| Oxidised Fraction | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 |
| Emission (Gg C) | 33.6 | 0.3 | 0.0 | 0.0 | 14.8 | 36.7 | 30 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 123 | 1 | 0 | 0 | 54 | 134 | 110 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Pr/1

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Emission of CO2 has been calculated based on the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of a fuel as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low. For emission factors, the uncertainties are somewhat higher. The emission factors applied are taken from IPCC methodology (and are not a result of measurements in Slovenia).

Table 21: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author’s estimate

### Lignite and Brown Coal

The calculation of consumption of brown coal and lignite in Slovenia has been done on the basis of data from Table Zb/1. , Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy. For the year 2004 we have obtained data from SORS in electronic format (file nameS:  E_10_2004.xls, E_11_2004.xls).

Lignite (CRF) 			=  Lignite (LEG) + Domestic Brown Coal (LEG)

Sub bitumnus coal (CRF) 	=  Imported Brown Coal (LEG)

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.14 -I.20) suggest a conversion factor from natural units to Joules from 7.8 to 20.0 TJ/1000 t, for former Yugoslavia from 8.9 to 16.7 TJ/1000 t.

LEG reports for individual years specify a calorific value from 9.39 to 10.66 TJ/1000 t for lignite, from 11.25 to 12.76 for domestic brown coal, whereas no factors are specified for imported brown coal. (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Tables Zb/1, Pr/1 and EL/7-0).

Calorific values for domestic coal have been taken from SORS.

For imported brown coal (from America, Russia, Indonesia and the Czech Republic), calorific values have been calculated on the basis of data from main importers.

The imported coal has thus been disaggregated into three consumption segments. Data for the biggest consumer of imported coal, the combined heat and power plant Ljubljana, have been obtained directly from this consumer.

In industry, imported coal is practically used by only one enterprise; consequently, data for the net calorific value (NCV) as reported by that enterprise have been taken into account. For lesser quantities of coal, consumed in general consumption, average net calorific values as reported by the providers to the Statistical Office of the Republic of Slovenia have been considered.

### Lignite (CRF)

Table 22: Consumption of Lignite (CRF) and Calculation of Emission of CO2 from Consumption of Lignite in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Production (Mg) | 6790500 | 5582500 | 4479500 | 4818000 | 4663038 | 4829513 | 4809058 |
| Import (Mg) | 160400 | 20300 | 0 | 0 | 0 | 0 | 804 |
| Purchase (Mg) | 780600 | 210200 | 0 | 0 | 0 | 0 | 0 |
| Import + Purchase (Mg) | 941000 | 230500 | 0 | 0 | 0 | 0 | 0 |
| Export (Mg) | 167800 | 66400 | 0 | 700 | 900 | 0 | 0 |
| Stock Change (Mg) | -15400 | -308700 | -33805 | 78607 | -369042 | 71433 | 1616 |
| Consumption (Mg) | 7579100 | 6055300 | 4513304,9 | 4738693 | 5031180 | 4758080 | 4808246 |
| Factor MJ/kg | 10,00 | 10,53 | 10,36 | 10,71 | 10,47 | 10,32 | 10,502 |
| Consumption (TJ) | 75781 | 63751 | 46779 | 50755 | 52672 | 49081 | 50494 |
| Emission Factor (t C/TJ) | 29.29 | 28.71 | 27.90 | 28.86 | 28.59 | 28.45 | 28.45 |
| Emission (Gg C) | 2220 | 1830 | 1305 | 1465 | 1506 | 1397 | 1437 |
| Oxidised Fraction | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 |
| Emission (Gg C) | 2175.4 | 1793.6 | 1279 | 1436 | 1476 | 1369 | 1408 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 7977 | 6577 | 4690 | 5265 | 5412 | 5019 | 5162 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy, Table Zb/1

Source of Emission Factors: National emission factors for domestic lignite for the period 1986-2003, Elektroinštitut Milan Vidmar, 2004

All factors for Lignite (CRF) totals have been calculated on the basis of the weighted arithmetic mean for Lignite (LEG) and Imported brown coal (LEG).

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of a fuel as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low.

Table 23: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author’s estimate

### Sub bituminous coal

Table 24: Consumption Sub bituminous coal and Calculation of Emission of CO2 from Sub bituminous coal in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Production (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import (Mg) |  |  | 455000 | 437900 | 534100 | 514152 | 583882 |
| Purchase (Mg) |  |  | 0 | 0 | 0 | 0 | 0 |
| Import + Purchase (Mg) |  |  | 455000 | 437900 | 534100 | 514152 | 583882 |
| Export (Mg) | 0 | 0 | 0 | 0 | 0 | 10757 | 49026 |
| Stock Change (Mg) | 0 | 0 | 31952 | -48824 | -16900 | -42617 | 0 |
| Consumption (Mg) |  |  | 423048 | 486723 | 551000 | 546012 | 534856 |
| Factor MJ/kg |  |  | 19.06 | 19.15 | 18.51 | 18.587 | 18.676 |
| Consumption (TJ) |  |  | 8064 | 9322 | 10197 | 10149 | 9989 |
| Emission Factor (t C/TJ) |  |  | 27.6 | 27.6 | 27.6 | 27.6 | 27.6 |
| Emission (Gg C) |  |  | 223 | 257 | 281 | 280 | 276 |
| Oxidised Fraction |  |  | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 |
| Emission (Gg C) |  |  | 218.1 | 252.2 | 275.8 | 274.5 | 270.2 |
| Conversion of C to CO2 |  |  | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) |  |  | 800 | 925 | 1011 | 1007 | 991 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy, Table Zb/1

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of a fuel as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low.

Table 25: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author’s estimate

### Coke

The calculation of the consumption of coke in Slovenia was done on the basis of data from Table Pr/1, Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy. This chapter takes into account the total consumed quantities of coke: both those consumed in production processes as well as those consumed in power generation.

In IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.14 -I.20) the suggested conversion factors from natural units to Joules range from 20.8 to 30.1 TJ/1000 t, for former Yugoslavia 26.9 TJ/1000 t. LEG reports specify for all those years a conversion factor of 29.3 TJ/1000 t (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Zb/1). Since the conversion factor is specified on the basis of measurements, the calculations take into account the factor of the Ministry of the Economy. For the year 2004 we have obtained data from SORS in electronic format (file name E_10_2004.xls).

Table 26: Consumption of Coke and Calculation of Emission of CO2 from Consumption of Coke in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Production (Mg) | 0 | 0 | 0 | 0 | 0 |  |  |
| Import (Mg) | 6700 | 0 | 48095 | 81700 | 85200 | 57012 | 59600 |
| Purchase (Mg) | 218700 | 67600 | 0 | 0 | 0 |  |  |
| Import + Purchase (Mg) | 225400 | 67600 | 48095 | 81700 | 85200 | 57012 | 59600 |
| Export (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Stock Change (Mg) | 1600 | -2000 | 1000 | -3600 | -100 | -3973 | -21 |
| Consumption (Mg) | 223800 | 69600 | 47095 | 85300 | 85300 | 60985 | 59621 |
| Factor MJ/kg | 29.3 | 29.3 | 29.31 | 29.31 | 29.31 | 29.31 | 29.31 |
| Consumption (TJ) | 6557 | 2039 | 1380 | 2500 | 2500 | 1787 | 1747 |
| Emission Factor (t C/TJ) | 29.5 | 29.5 | 29.5 | 29.5 | 29.5 | 29.5 | 29.5 |
| Emission (Gg C) | 193 | 60 | 41 | 74 | 74 | 53 | 52 |
| Oxidised Fraction | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 |
| Emission (Gg C) | 189.6 | 59.0 | 39.9 | 72.3 | 72.3 | 51.7 | 51.5 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 695 | 216 | 146 | 265 | 265 | 189 | 185 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy, Table Pr/1

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Emission of CO2 has been calculated on the basis of the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, whilst calorific values are in the majority of cases the results of measurements. For the consumption of coke, the major problem is the consumption of petroleum coke, which in some inventories is accounted for while in others it is not. Consequently, the uncertainty is higher than for other solid fuels. The emission factors applied have been taken from IPCC methodology (and are not a result of measurements in Slovenia).

Table 27: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 10% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author's estimate

### Anthracite

The calculation of the consumption of anthracite in Slovenia has been done on the basis of data from Table Pr/1, Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy. This chapter takes into account the total consumed quantities of anthracite: both those consumed in production processes, as well as those consumed in power generation.

In IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.14 -I.20) the suggested conversion factors from natural units to Joules range from 20.8 to 30.1 TJ/1000 t, for former Yugoslavia 26.9 TJ/1000 t. Emission factors specified in LEG range from 29.3 to 31.9 TJ/1000 t (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy, Table Zb/1). Based on measurements, the emission factor of the Ministry of the Economy has been taken into account in calculations. For the year 2004 we have obtained data from SORS in electronic format (file name E_10_2004.xls).

Emission of CO2 has been calculated on the basis of the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Table 28: Consumption of Anthracite and Calculation of Emission of CO2 from Consumption of Anthracite in Slovenia for the Period of 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Production (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import (Mg) | 45000 | 20800 | 0 | 0 | 0 | 0 | 0 |
| Purchase (Mg) | 0 | 1200 | 0 | 0 | 0 | 0 | 0 |
| Import + Purchase (Mg) | 45000 | 22000 | 0 | 0 | 0 | 0 | 0 |
| Export (Mg) | 0 | 0 | -0,1 | -0,1 | -0,1 | -0,1 | 0 |
| Stock Change (Mg) | -9500 | -9200 | 0 | 0 | 0 | 0 | 0 |
| Consumption (Mg) | 54500 | 31200 | 0,1 | 0,1 | 0,1 | 0,1 | 0 |
|  | 38428 | 19721 |  |  |  |  |  |
|  | 16072 | 11479 | 0,1 | 0,1 | 0,1 | 0,1 | 0 |
| Factor MJ/kg | 29.3 | 29.3 | 29.3 | 29.3 | 29.3 | 29.3 | 29.3 |
| Consumption – Total (TJ) | 1594 | 913 | 0 | 0 | 0 | 0 | 0 |
| Emission Factor (t C/TJ) | 26.8 | 26.8 | 26.8 | 26.8 | 26.8 | 26.8 | 26.8 |
| Emission (Gg C) | 43 | 24 | 0 | 0 | 0 | 0 | 0 |
| Oxidised Fraction | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 |
| Emission (Gg C) | 41.9 | 24.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 154 | 88 | 0 | 0 | 0 | 0 | 0 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy, Table Pr/1

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of a fuel as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low. For emission factors, the uncertainties are somewhat higher. The emission factors applied are taken from IPCC methodology (and are not a result of measurements in Slovenia).

Table 29: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author’s estimate

### Natural Gas

The calculation of the consumption of natural gas in Slovenia has been done on the basis of data from Table Z/1, Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003, Ljubljana: Ministry of the Economy. For the year 2004 we have obtained data from SORS in electronic format (file name E_9_2004.xls).

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996) have required from the very beginning inventories to be compiled in Joules and therefore suggested no conversion factors from natural units to Joules. The factors of calorific value specified in LEG range from 33.5 to 34.1 TJ/1000 t (Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2002, Ljubljana: Ministry of the Economy, Table Zb/1). Based on measurements, the emission factor of the Ministry of the Economy has been taken into account in calculations.

Table 30: Consumption of Natural Gas and Calculation of Emission of CO2 from Consumption of Natural Gas in Slovenia for the Period of 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Production (kSm3) | 7371 | 24800 | 6000 | 6100 | 5600 | 4919 | 5040 |
| Import (kSm3) | 824211 | 880100 | 1007000 | 1037400 | 994900 | 1108876 | 1099000 |
| Purchase (kSm3) | 0 | 8300 | 0 | 0 | 0 | 0 | 0 |
| Import + Purchase (kSm3) | 824211 | 888400 | 1007000 | 1037400 | 994900 | 1108876 | 1099000 |
| Export (kSm3) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Stock Change (kSm3) | 0 | -23900 | 0 | 0 | 0 | 0 | 0 |
| Consumption (kSm3) | 831582 | 937100 | 1013000 | 1043500 | 1000500 | 1113795 | 1104040 |
| Factor MJ/Sm3 | 33.5 | 34.10 | 34.08 | 34.08 | 34.08 | 34.08 | 34.08 |
| Consumption (TJ) | 27858 | 31955 | 34523 | 35562 | 34097 | 37958 | 37626 |
| Emission Factor (t C/TJ) | 15.074 | 15.074 | 15.074 | 15.074 | 15.074 | 15.074 | 15.070 |
| Emission (Gg C) | 420 | 482 | 520 | 536 | 514 | 572 | 497 |
| Oxidised Fraction | 0.995 | 0.995 | 0.995 | 0.995 | 0.995 | 0.995 | 0.995 |
| Emission (Gg C) | 417.8 | 479.3 | 517.8 | 533.4 | 511.4 | 569.3 | 494 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 1532 | 1758 | 1899 | 1956 | 1875 | 2088 | 1813 |

Data Source: Ministry of the Economy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy, Table Zb/1

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Emission of CO2 has been calculated on the basis of the formula from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30). Emission factors have been taken from the documents of the Institute of Energy Industries (Gasperič M., Dornik M.: Določitev emisijskega faktorja CO2 pri energetski izrabi ter emisijskega faktorja CH4 pri transportu in distribuciji zemeljskega plina, Ljubljana: Institute of Energy Industries, 1998).

Basic sources do not specify any uncertainties. All utilized input data are taken from official inventories, while calculations take into account the factor of calorific value of a fuel as specified by the Ministry of the Economy of the Republic of Slovenia, hence the uncertainties are supposed to be low. For emission factors, the uncertainty is low since they are the result of measurements in Slovenia.

Table 31: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 5% |

Source: Author’s estimate

### Gas Works Gas and Mixed Gas

Production and consumption of gas works gas and mixed gas in Slovenia is reported here for the period 1986 to 1993 (Tables Pg/1, Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy; Letopisi energetskega gospodarstva 1986-1996). After 1994 there was no consumption of gas works gas and mixed gas in Slovenia.

All consumed quantities of gas works gas and mixed gas in Slovenia was produced in Slovenia (and belong to the group of secondary fuels), therefore they do not contribute to total emissions of CO2 according to IPCC methodology. In IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996) there is no suggested special category of fuels for gas works gas and mixed gas.

Consequently, the reference approach will not present quantities of gas works gas and mixed gas that are consumed in Slovenia. They will only be taken into account in the sectoral approach in the Consumer Goods category.

### Biomass

There are no precise data on the consumption of fuel wood for the monitored period in Slovenia. Various expert estimates for individual areas have been done. Here, data from the following sources have been used:

- For the consumption of biomass in general consumption (residential and commercial/institutional): Gasperič M., Dornik M.: Disagregacija porabe goriv na komercialno institucionalni sektor in gospodinjstva, Ljubljana: Institute of Energy Industries, 1998;

- For the consumption of biomass in industry for the period 1986-1999: Simončič P., Kobler A., Robek R., Žgajnar L.: Ocena emisij oz. ponora TGP za gozdarstvo ter spremembe rabe zemljišč, Slovenian Forestry Institute, Ljubljana 1999 and Statistical office of the Republic of Slovenia (JQ Renewables) from 1999 on.

- For the consumption of biomass in power cogeneration plants: Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy; Letopisi energetskega gospodarstva 1986-1999 (Tables Dt/1-0). and Statistical office of the Republic of Slovenia (JQ Renewables) from 1999 on.

- Desegregation and total consumption of biomass for year 2000-2004, Statistical office of the Republic of Slovenia - JQ Renewables

Table 32: Consumption of Biomass in Slovenia for the Period of 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Power Heat Plants (Mg) | 13000 | 13000 | 26869 | 26048 | 24651 | 25801 | 35320 |
| Industry (Mg) | 414000 | 287000 | 347000 | 267954 | 293509 | 388907 | 257355 |
| Residential (Mg) | 1115283 | 1115283 | 1115283 | 1115283 | 1115283 | 1115283 | 1107985 |
| Commercial/Institutional (Mg) | 182000 | 182000 | 182000 | 6327 | 5752 | 2301 | 2286 |
| Total Wood Consumption (Mg) | 1724283 | 1597283 | 1671153 | 1415612 | 1439195 | 1532293 | 1402946 |

IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996) suggest no conversion factors from natural units to Joules. Sources of data for the consumption of biomass specify emission factors for different type of fuel wood from 9 to 18 MJ/kg of wood. In this report, the emission factor of 12.17 MJ/kg of dry matter has been applied, calculated as the weighted arithmetic mean of the consumption of various types of biomass in Slovenia for 1994, taken from the following source: Žgajnar L., Bitenc B.: Nekateri pomembnejši kazalci rabe lesa v slovenski energetiki, in Biomasa - vir energije za Slovenijo (anthology of papers from a symposium), Ministry of the Economy, Ministry of Agriculture and Forestry, Ministry of the Environment and Physical Planning, Brdo pri Kranju 1996.

Table 33: Consumption of Biomass and Calculation of Emission of CO2 from Consumption of Biomass in Slovenia for the Period 1986 to 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ton | ton | ton | ton | ton | ton | ton |
| Production (Mg) | 1724283 | 1597283 | 1671153 | 1415612 | 1439195 | 1532293 | 1402946 |
| Import (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Purchase (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Import + Purchase (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Export (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Stock Change (Mg) | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| Consumption (Mg) | 1724283 | 1597283 | 1671153 | 1415612 | 1439195 | 1532293 | 1402946 |
| Factor MJ/kg | 12.2 | 12.2 | 12.2 | 12.2 | 12.2 | 12.2 | 12.2 |
| Consumption (TJ) | 20985 | 19439 | 20338 | 17228 | 17515 | 18648 | 16644 |
| Emission Factor (t C/TJ) | 26.8 | 26.8 | 26.8 | 26.8 | 26.8 | 26.8 | 26.8 |
| Emission (Gg C) | 562 | 521 | 545 | 462 | 469 | 500 | 446 |
| Oxidised Fraction | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 | 0.98 |
| Emission (Gg C) | 551.1 | 510.5 | 534.2 | 452.5 | 460.0 | 489.8 | 437.1 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Emission (Gg CO2) | 2021 | 1872 | 1959 | 1659 | 1687 | 1796 | 1603 |

Data Source:

Gasperič M., Dornik M.: Disagregacija porabe goriv na komercialno institucionalni sektor in gospodinjstva, Ljubljana: Institute of Energy Industries, 1998,

Simončič P., Kobler A., Robek R., Žgajnar L.: Ocena emisij oz. ponora TGP za gozdarstvo ter spremembe rabe zemljišč, Slovenian Forestry Institute, Ljubljana 1999,

Statistični letopis energetskega gospodarstva republike Slovenije 1986-1996, Ljubljana: Ministry of the Economy; Letopisi energetskega gospodarstva 1986-1996 (Tables Dt/1-0)

Statistical office of the Republic of Slovenia – IEA Renewables Questionaire/Woodweg

There are no comparable data on the import and export of biomass in Slovenia in the monitored period. Therefore, it is assumed in this report that there was no import or export of biomass in the period in question.

Emission of CO2 has been calculated on the basis of the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

Source of emission factors: Žgajnar L., Bitenc B.: Nekateri pomembnejši kazalci rabe Lesa v slovenski energetiki, in Biomasa - vir energije za Slovenijo (anthology of papers from a symposium), Ministry of the Economy, Ministry of Agriculture and Forestry, Ministry of the Environment and Physical Planning, Brdo pri Kranju 1996.

Source of Emission Factors: Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p.1.13

Basic sources do not specify any uncertainties. Utilized input data are the result of estimates and this author's calculation, consequently, the uncertainty is relatively high.

Table 34: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 30% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 20% |
| Estimate of Uncertainty of Emission Factors | 10% |

Source: Author’s estimate

### Estimate of Stored Carbon

Minor quantities of fuels are used as raw materials for the production of certain non-energy products. The carbon stored in these products is not emitted right away, but in a longer time. Consequently, the quantities of carbon stored in such products are supposed to be subtracted from the total emitted quantities calculated in previous items. In Slovenia, data on the consumption of fuels for the production of non-energy feedstocks is available only for some products.

Products or raw materials, which are consumed in their production, will be included in calculations, in cases when there are data on the basis of which it is possible to estimate these quantities.

### Natural Gas

No data on the amount of natural gas consumed for the production of various products (particularly methanol) in which the carbon is stored, are available. An utmost conservative presumption would be that these are all the quantities of natural gas which have been consumed in the sector of production of basic chemicals and which are reported in publications of the Statistical Office of the Republic of Slovenia as consumption of natural gas as a feedstock (and not for the consumption in power cogeneration plants, public heat plants or thermal power plants).

The problem of adequate inventories occurs particularly for the consumption of Nafta Lendava. Nafta Lendava reports its consumption of natural gas partly in the production of refined petroleum products sector and partly in production of basic chemicals sector. Data on the consumption of natural gas in that factory differ greatly for different sources. As the fundamental source (according to this author's and the expert group's estimates) the Statistical Office of the Republic of Slovenia has been chosen since by definition the category of consumption of natural gas as a feedstock is closest to the category which is required by IPCC for the consumption of products with stored carbon. (Source: Statistical Office of the Republic of Slovenia: Annual Industry Survey (research results), Ljubljana: Statistical Office of the Republic of Slovenia, 1986-2004).

Emission of CO2 has been calculated on the basis of the formula and factors from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.13., I. 30)

CO2 stored in products has been calculated on the basis of the formula from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.28., I. 23, I.13).

Table 35: Consumption of Natural Gas as a Raw Material and Stored Carbon for the Period 1986 – 2004.

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Consumption (kSm3) | 67666 | 69524 | 136740 | 146069 | 125528 | 154302 | 136356 |
| Factor (MJ/Sm3) | 33.50 | 34.10 | 34.08 | 34.08 | 34.08 | 34.08 | 34.08 |
| Consumption (TJ) | 2266.81 | 2371 | 4660 | 4978 | 4278 | 5259 | 4647 |
| Emission Factor (t C/TJ) | 15.075 | 15.075 | 15.075 | 15.075 | 15.075 | 15.075 | 15.075 |
| Emission (Gg C) | 34 | 36 | 70 | 75 | 64 | 79 | 70 |
| Fraction of Carbon Stored | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Stored carbon (Gg C) | 34 | 36 | 70 | 75 | 64 | 79 | 70 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Stored CO2 (Gg CO2) | 125.3 | 131.0 | 257.6 | 275.2 | 236.5 | 290.7 | 256.9 |
| Source of AD | LPI-86,str.372 | LPI-90,str.192 | LEG Tg/6-3 | LEG Tg/6-3 | LEG Tg/6-3 | LEG Tg/6-3 | LEG Tg/6-3 |

Table 36: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 20% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 5% |

Source: Author’s estimate

### Lubricants

In the chapter on the consumption of lubricants (Chapter 1.7) only the quantities, which have been imported, should be specified, since emissions of other quantities have been already accounted for in the processing of crude oil. Data on import and export of lubricants in Slovenia are not available in satisfactory quality. Consequently, those quantities are not specified in the chapter on the consumption of lubricants. Therefore, it is not necessary to specify all quantities of lubricants consumed in Slovenia in this chapter, but on the contrary, only those quantities that are both produced and consumed in Slovenia. With regard to the fact that the quantities, which are produced in Slovenia, are relatively small, it is considered that lubricants are not being exported from Slovenia.

Therefore, as the consumption of lubricants which are produced (and the emissions of which are taken into account in the emissions from the production or processing of crude oil) and consumed in Slovenia, those quantities of engine oils and lubricants have been specified, which have been produced in the Production of Oil Products sector (Source: Statistical Office of the Republic of Slovenia: Annual Industry Survey (research results), Ljubljana: Statistical Office of the Republic of Slovenia, 1986-2004).

Stored CO2 has been calculated on the basis of the formula from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.28., I. 23, I.13).

Table 37: Consumption of Engine Oils and Other Lubricating Oils and Stored Carbon for the Period 1986 - 2004

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Consumption (tonnes) | 6592 | 7201 | 9805 | 9701 | 9363 | 12636 | 28623 |
| Factor TJ/1000 tonnes | 40.2 | 40.2 | 40.2 | 40.2 | 40.2 | 40.2 | 40.2 |
| Consumption (TJ) | 265 | 289 | 394 | 390 | 376 | 508 | 1151 |
| Emission Factor (t C/TJ) | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Emission (Gg C) | 5 | 6 | 8 | 8 | 8 | 10 | 10 |
| Fraction of Carbon Stored | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Stored Carbon (Gg C) | 2.65 | 2.89 | 3.94 | 3.90 | 3.76 | 5.08 | 14.91 |
| Conversion of C to CO2 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 | 3.6667 |
| Stored CO2 (Gg ) | 9.7 | 10.6 | 14.5 | 14.3 | 13.8 | 18.6 | 54.7 |
| Source 1: | LPI-86,str.190 | LPI-90, str.74 | SORS | SORS | SORS | SORS | SORS |
| Source 2: | IPCC, str. I.13, I.23 | IPCC, str. I.13, I.23 | IPCC, str. I.13, I.23 | IPCC, str. I.13, I.23 | IPCC, str. I.13, I.23 | IPCC, str. I.13, I.23 | IPCC, str. I.13, I.23 |

Table 38: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 20% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 5% |

Source: Author’s estimate