SLOVENIA’S NATIONAL INVENTORY REPORT 2006

Submission under the United Nations Framework Convention on Climate Change 2006

Ljubljana, October 2006

PREFACE

The Environmental Agency of the Republic of Slovenia is in accordance with the Slovenian legislation charged with both the overall coordinating of activities that are necessary for the development of emission inventories, as well as with implementing inventories for the purposes of reporting to the United Framework Convention on Climate Change (UNFCCC) and to the European Commission. The Republic of Slovenia is as a party to the convention obligated to make annual GHG emission inventories and to report them.

In 2004, Slovenia has established a uniform and sustainable system for making inventories that will enable a timely fulfilment of reporting duties, including the filling in of CRF tables as well as making the reports. Thus in 2004, the Republic of Slovenia has for the first time submitted data on emissions in the laid-down format (CRF) to the Secretariat, and in calculating emissions, most of remarks of the UNFCCC review processes given so far, have been taken into account. This NIR is therefore the third after 1996, which both in terms of its contents and its form is harmonized with the recommendations of the Secretariat.

Notwithstanding the efforts made so far to improve the inventories, we are aware that there are areas that will call for upgrading and improving in the future. In order to improve the process of developing inventories, in 2005, a QA/QC plan and a Manual of Procedures have been prepared. Within the framework of preparing inventories for 2003, we have already marked both activity data and emission factors that will have to be completed and upgraded in the future and actions to this effect are already under way.

For the support and completion of this National Inventory Report we would also like to thank to the GEF Project “Capacity Building for Improving GHG Inventories”

This project has been lead by:

Matej Gasperič and Tajda Mekinda Majaron

Specific responsibilities for the NIR 2006 have been as follows:

Executive Summary		Tajda Mekinda Majaron

Introduction			Tajda Mekinda Majaron,

Trends in GHG emissions	Tajda Mekinda Majaron,

Energy				Tajda Mekinda Majaron,

Industrial processes		Tajda Mekinda Majaron

Solvent use			Tajda Mekinda Majaron

Agriculture			Jože Verbič D.Sc., Tajda Mekinda Majaron

LULUCF			Nike Krajnc D.Sc., Mitja Piškur M.Sc, Primož Simončič D.Sc.

Waste				Tajda Mekinda Majaron

Recalculations			Tajda Mekinda Majaron

CFR Tables			Tajda Mekinda Majaron, Martina Logar D.Sc.

TABLE OF CONTENTS

EXECUTIVE SUMMARY	8

ES.1. Background information on greenhouse gas inventories and climate change	8

ES.2. Summary of national emissions and removal related trends	9

ES.3. Overview of source and sink category emission estimates and trends	10

ES.4. Overview of Emission Estimates and Trends of Indirect GHGs and SO2	12

1.	INTRODUCTION	13

1.1	Background information on greenhouse gas inventories and climate change	13

1.2	A description of the institutional arrangement for inventory preparation	14

1.3	Brief description of the process of inventory preparation	17

1.4	Brief general description of methodologies and data sources used	18

1.5	Brief description of key source categories	21

1.6	Information on the QA/QC plan including verification and treatment of confidentiality issues where relevant	23

1.7	General uncertainty evaluation, including data on the overall uncertainty for the inventory totals	24

1.8	General assessment of the completeness	24

2	TRENDS IN GREENHOUSE GAS EMISSIONS	27

2.1	Description and interpretation of emission trends for aggregated greenhouse gas emissions	27

2.2	Description and interpretation of emission trends by gas	28

2.3	Description and interpretation of emission trends by source	30

2.4	Description and interpretation of emission trends for indirect greenhouse gases and SO2	32

3	ENERGY (CRF sector 1))	33

3.1	Overview of sector	33

3.1.1	Comparison of the sectoral approach with the reference approach	34

3.1.2	International bunker fuels	35

3.1.3	Feedstock and non-energy use of fuels	35

3.1.4	CO2 capture from flue gases and subsequent CO2 storage	37

3.2	Fuel combustion (CRF 1.A)	38

3.2.1	Country-specific issues	38

3.2.2	Country-specific EFs	38

3.2.3	Oxidation factor	39

3.2.4	Energy Industries	39

3.2.5	Manufacturing Industries and Construction (IPCC: I A 2)	48

3.2.6	Transport (IPCC: I A 3)	57

3.2.7	Other sector (IPCC: I A 4)	64

3.3	Fugitive emissions from solid fuels and oil and natural gas (CRF 1.B)	71

3.3.1	Solid Fuels (IPCC: I B I)	71

3.3.2	Oil  (IPCC: I B 2)	74

3.3.3	Natural Gas (IPCC: I B 2)	76

4	INDUSTRIAL PROCESSES (CRF sector 2)	84

4.1	Cement Production	85

4.1.1	Source category	85

4.1.2	Methodological issues	85

4.1.3	Uncertainties and time-series consistency	87

4.1.4	Source specific recalculations	87

4.2	Lime Production	87

4.2.1	Source category description	87

4.2.2	Methodological issues	87

4.2.3	Uncertainties and time-series consistency	88

4.2.4	Source specific recalculations	88

4.2.5	Source-specific planned improvements	88

4.3	Limestone and Dolomite Use	89

4.3.1	Source category description	89

4.3.2	Methodological issues	89

4.3.3	Uncertainties and time-series consistency	89

4.3.4	Source-specific planned improvements	89

4.4	Soda ash production and use	89

4.4.1	Source category description	90

4.4.2	Methodological issues	90

4.4.3	Uncertainties and time-series consistency	90

4.4.4	Recalculations	90

4.4.5	Future improvements	90

4.5	Production and use of Miscellaneous Mineral Products	90

4.5.1	Source category description	90

4.5.2	Methodological issues	90

4.5.3	Uncertainties and time-series consistency	91

4.5.4	Recalculations	91

4.5.5	Future improvements	92

4.6	Nitric acid Production	92

4.6.1	Source category description	92

4.6.2	Methodological issues	92

4.6.3	Recalculations	92

4.6.4	Future improvements	92

4.7	Carbide Production	92

4.7.1	Source category description	92

4.7.2	Methodological issues	93

4.7.3	Uncertainties and time-series consistency	94

4.7.4	Recalculations	94

4.7.5	Future improvements	94

4.8	Production of Other Chemicals	94

4.8.1	Source category description	94

4.8.2	Methodological issues	94

4.8.3	Uncertainties and time-series consistency	95

4.8.4	Recalculations	95

4.8.5	Future improvements	95

4.9	Iron and Steel Production	96

4.9.1	Source category description	96

4.9.2	Methodological issues	96

4.9.3	Uncertainties and time-series consistency	97

4.9.4	Recalculations	97

4.9.5	Future improvements	98

4.10	Ferroalloys Production	98

4.10.1	Source category description	98

4.10.2	Methodological issues	98

4.10.3	Uncertainties and time-series consistency	99

4.10.4	Recalculations	99

4.10.5	Future improvements	99

4.11	Aluminium Production	99

4.11.1	Source category description	99

4.11.2	Methodological issues	100

4.11.3	Uncertainties and time-series consistency	101

4.11.4	Recalculations	102

4.11.5	Future improvements	102

4.12	Other Metal Production (IPCC: 2 C 5)	102

4.13	Pulp and Paper Industries	102

4.13.1	Source category description	102

4.13.2	Methodological issues	102

4.14	Food and Drink	102

4.14.1	Source category description	102

4.14.2	Methodological issues	103

4.14.3	Uncertainties and time-series consistency	103

4.14.4	Recalculations	103

4.14.5	Future improvements	103

4.15	Emissions Related to Consumption of Halocarbons and Sulphur Hexafluoride	103

4.15.1	Source category description	103

4.15.2	Methodological issues	106

4.15.3	Source-specific planned improvements	109

4.15.4	Recalculations	110

5	SOLVENT AND OTHER PRODUCT USE (CRF sector 3)	111

5.1	Solvent use	111

5.1.1	Source category description	111

5.1.2	Methodological issues	111

5.2	Other products use	113

5.2.1	Uncertainties and time-series consistency	114

5.2.2	Source-specific planned improvements	114

5.2.3	Source-specific recalculations	114

6	AGRICULTURE (CRF sector 4)	115

6.1	CH4 Emissions from Enteric Fermentation	115

6.1.1	Source category description	116

6.1.2	Activity data and methodological issues	116

6.1.3	Uncertainties and time-series consistency	122

6.1.4	Source-specific recalculations	123

6.1.5	Future improvements	123

6.2	CH4 Emissions from Manure Management	123

6.2.1	Source category description	123

6.2.2	Methodological issues	125

6.2.3	Uncertainties and time-series consistency	127

6.2.4	Source-specific recalculations	128

6.2.5	Future improvements	128

6.3	N2O Emissions from Manure Management	128

6.3.1	Source category description	128

6.3.2	Methodological issues	128

6.3.3	Uncertainties and time-series consistency	130

6.3.4	Source-specific recalculations	130

6.3.5	Future improvements	130

6.4	Emissions from Agricultural Soils	131

6.4.1	Direct N2O Emissions from Agricultural Soil	131

6.4.2	Recalculations	134

6.4.3	Future improvements	134

6.4.4	Nitrous oxide emissions from grazing animals	135

6.4.5	Recalculations	135

6.4.6	Future improvements	135

6.4.7	Indirect N2O emissions from Agricultural Soil	135

6.4.8	Recalculations	137

6.4.9	Future improvements	137

7	LULUCF (CRF sector 5)	138

7.1	Source category description	138

7.2	Methodological issues	139

7.2.1	Changes in Forest and Other Woody Biomass Stocks	140

7.2.2	Regrowth of Abandoned Managed Lands	142

7.2.3	CO2 Emission Due to Land-use Change	144

7.3	Uncertainties and time-series consistency	145

7.4	Source-specific planned improvements	145

8	WASTE (CRF sector 6)	147

8.1	CH4 Emissions from Solid Waste Disposal sites	147

8.1.1	Source category description	147

8.1.2	Methodological issues	148

8.1.3	Uncertainties and time-series consistency	155

8.1.4	Source specific recalculations	155

8.1.5	Future improvements	155

8.2	Emissions from Wastewater Handling	155

8.2.1	Source category description	155

8.2.2	Methodological issues	156

8.2.3	Uncertainties and time-series consistency	163

8.2.4	Source-specific recalculations	163

8.2.5	Source-specific planned improvements	163

9	OTHER	163

10	RECALCULATIONS AND IMPROVEMENTS	164

10.1	Explanations and justifications for recalculations	164

10.2	Implications for emission level	164

10.3	Recalculations, including in response to the review process and planned improvements to the inventory	164

11	SOURCES AND LITERATURE	166

## EXECUTIVE SUMMARY

## ES.1. Background information on greenhouse gas inventories and climate change

The methodology for the calculation of greenhouse gases kept developing all the time and is even today a project under development. In the IPCC inventory of greenhouse gases for Slovenia, the 1996 version was applied (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996), which in many parts also takes into account the IPCC 2000 Guidelines (Good Practice Guidance and Uncertainty Management)

The guidelines for the implementation of the inventory of greenhouse gases contain prescribed methods for calculation of emissions, providing a unified framework for reporting and documenting sources for all inventories. One of the main aims of this method is to ensure comparability of data gathered in individual states and that calls for a definition of at least a minimum scope of equal methods, criteria, and estimating procedures. These guidelines come in three volumes, which together with GPG are a complete set of recommended methods for estimating emissions.

With regard to the activities that are sources of pollution, the guidelines are divided into 6 areas:

- Energy

- Industrial processes

- Solvent and other product use

- Agriculture

- Forestry and land-use change

- Waste.

The degree of completion of IPCC guidelines for the calculation of emissions varies for individual gases. Guidelines for the calculation of emissions of CO2 have been laid down for almost all activities, while for other gases methods have been laid down only for major sources. This is a sign of the global nature of IPCC methodology – including particularly those gases and methods, which are major polluters on a global level – inventories that are important on a national level should be constructed by individual states themselves.

ABBREVIATIONS USED:

CORINAIR- the Atmospheric Emission Inventory for Europe

DEE – National Emission Inventories

DGP – Working Materials of the Statistical Office of the Republic of Slovenia

EMEP - Co-operative Programme for Monitoring and Evaluation of the Long Range Transmission of Air Pollutants in Europe

EC – EMEP-CORINAIR (jointly published publication)

HMZ – Hydrometeorological Institute of the Republic of Slovenia

IEA – International Energy Agency

IPCC - Intergovernmental Panel on Climate Change

LEG – Annual Energy Statistics of the Energy Sector of the Republic of Slovenia

LPI – Annual Industry Survey

MOP – Ministry of the Environment and Physical Planning

OECD – Organisation for Economic Co-operation and Development

SL – Statistical Yearbook of the Republic of Slovenia

UNEP – United Nations Environmental Programme

## ES.2. Summary of national emissions and removal related trends

In 2004, total GHG emissions accounted for 19.946 Tg CO2 eq.). The biggest fraction among gases belongs to CO2 (82.2%), followed by CH4 with 10.6%, N2O with 6.2% and F–gases with 1.0% of GHG emissions. The largest fraction in emissions among the sectors belongs to the energy sector (80.9%), agriculture accounts for 9.9% of total GHG emissions, industrial processes for 5.7%, waste for 3.3% and solvent and other product use for 0.2%. The most important emission source in energy sector with 97.7% of emissions is the fuel combustion sub-sector, which is split further into energy supply (40.0%), transport (27.0%), Other (17.9%), and Industry and Construction sectors (15.0%). CO2 emission sinks due to the Land Use Change and Forestry are assumed to amount to 5.644 Tg.

Compared to the base year, the greatest fraction increase has been that of CO2 emissions, namely from 80.1% in 1986 to 82% in 2004.

As a result of the restructuring of economy and the process of gaining independence, the lowest emission level was reached in 1992. The first peak was reached in 1997 because of low fuel prices in Slovenia, which increased motor fuel purchases by foreigners, while the second peak was reached in 2002, due to lower electricity production in hydroelectric power plants, which was made up for by increased electricity production in thermal power plants. In 2004, GHG emissions were 1.08% below the emissions in 1986.

Table 0.1: Slovenian GHG emissions and removals by gas

| GREENHOUSE GAS EMISSIONS | Base year ( 1986 ) | 1990 | 1995 | 2000 | 2003 | 2004 | Change from base to latest reported year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | (%) |
| CO2 emissions including net CO2 from LULUCF | 14,570.654 | 11,447.009 | 9,960.025 | 9,940.814 | 10,630.518 | 10,753.530 | -26.197 |
| CO2 emissions excluding net CO2 from LULUCF | 16,159.907 | 14,632.756 | 14,865.266 | 15,115.975 | 15,948.746 | 16,397.250 | 1.469 |
| CH4 | 2,376.290 | 2,289.374 | 2,118.178 | 2,136.056 | 2,123.174 | 2,108.716 | -11.260 |
| N2O | 1,340.892 | 1,218.707 | 1,170.751 | 1,293.830 | 1,283.838 | 1,235.306 | -7.874 |
| HFCs | NA,NO | NA,NO | 28.957 | 31.127 | 56.913 | 66.321 | 100.000 |
| PFCs | 276.291 | 257.444 | 285.685 | 105.612 | 118.990 | 120.008 | -56.565 |
| SF6 | 10.241 | 10.303 | 11.522 | 15.738 | 17.915 | 18.312 | 78.810 |
| Total (including net CO2 from LULUCF) | 18,574.367 | 15,222.838 | 13,575.119 | 13,523.178 | 14,231.349 | 14,302.192 | -23.000 |
| Total (excluding net CO2 from LULUCF) | 20,163.620 | 18,408.584 | 18,480.360 | 18,698.339 | 19,549.577 | 19,945.913 | -1.080 |

## ES.3. Overview of source and sink category emission estimates and trends

In accordance with UNFCCC Reporting Guidelines, inventories are grouped into six emission categories: Energy, Industrial Processes, Solvent use, Agriculture, Land-Use, Land Use Change and Forestry, and Waste.

Table 0.2: Slovenian GHG emissions and removals by  sector

Of all sectors, the most important sector is Energy with a lion’s share of 80.9% of all emissions of GHG. Within this sector, the biggest fractions are Electricity and Heat Production with 39.1%, and Traffic with 26.4%, where road traffic accounts for 99% of all traffic emissions, followed by Households and Other Commercial Usage with 17.5%, and Industry and Construction with 14.7%. The remainder (2.3%) was fugitive emissions.

The Energy sector shows a growing trend, emissions in 2004 have risen compared to the previous year by 2.7%, while in the 1999-2002 period they have risen by as much as 5,6%, which is chiefly a consequence of increased consumption of fuels in thermal power plants and cogeneration plants and an increase in the consumption of liquid fuels in traffic.

The strong increase of emission in the Electricity and Heat Production is an indication of the dependence of the production of electrical energy on fossil energy sources (in the Republic of Slovenia particularly from coal).

Traffic emissions have since 1986 increased already by 112,2%, mainly due to a large increase of the vehicle stock as a consequence of a rise in purchasing power of the population, and an increased need for transportation of goods as a consequence of permanent economic growth in the period 1991-2004. The growth trend in 2002 levelled off, as GHG emissions, compared to the previous year 2001, have risen only by 0.6%. However, GHG emissions have risen in 2003 by 3.6% and by 3.7% in 2004 when Slovenia joined the EU.

Emissions from Households and Other Commercial Use are stable. The 1990-2004 is characterized in particular by the growth of consumption of primary energy and substitution of solid fuels with natural gas. This substitution process is finished, since the quantities of solid fossil fuels consumed in this sector in year 2004 were negligent.

Emissions from Industrial Processes which represent 5.7% of total emissions have in the last year grown by 2.7%. This growth was mostly due to the emissions from the sector of Consumption of Halocarbons (main emissions driver is the Air Conditioning Equipment sub sector), which have increased by 13.1%. The consumption of solvents in the Republic of Slovenia represents a mere 0.2% of total GHG emissions and has shown practically no change over the years. Considering their negligible fraction, these emissions have been estimated in a very rough manner and with a considerable uncertainty.

Emissions from agriculture represent 9.9% of total GHG emissions and 51.6% of methane emissions in the Republic of Slovenia. Emissions from agriculture have, compared to the base year, diminished by 14.4%.

Emissions from waste have in the 1986-2004 period increased by 15.6%, comparing to the last year by 5%. Methane emissions from the Solid Waste Disposal on Land (SWD) with represents 63.5% of total emissions have increased by 39.1% compared to the base year and by 5.6% in reported year. The increase is a consequence of increase of amount of waste disposed to the solid waste disposal sites.  On the other hand emissions from wastewaters have decreased by 10,6% in the period 1986-2004 due to the reduction of industrial wastewater.

The sink in Land-Use, Land Use Change and Forestry sector are still not estimated fully in accordance with 2003 GPG for LULUCF. According to the estimates, calculated only on aggregated level sink amounted to 5.644 Mt of CO2, which is considerably more than the amount allocated to Slovenia (1.32 Mt). The improved estimation of the sink is foreseen for the next NIR.

Note: Values between 1986 and 1990 were interpolated.

Figure 0.1: GHG Emissions in Slovenia by sector: 1986-2004

## ES.4. Overview of Emission Estimates and Trends of Indirect GHGs and SO2

The Republic of Slovenia has in 1992 ratified the Convention on Long-Range Transboundary Air Pollution in Europe, within the framework of which, parties undertake to reduce emissions of carbon monoxide (CO), nitrogen oxides (NOx), non-methane volatile organic compounds (NMVOCs), and sulphur dioxide (SO2). Slovenia has in 2004 also ratified the Gothenburg Protocol and has committed itself to significantly reduce emissions of above-enumerated pollutants and, consequently, contribute to reduce the formation of tropospheric ozone.

Table 0.3: Emission of pollutants under CRLTAP Convention in period 1990-2004 in ton.

|  | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO | 80565 | 91427 | 99014 | 93208 | 87651 | 80776 | 82329 |
| NOx | 63432 | 66591 | 59673 | 59095 | 58222 | 55997 | 57498 |
| NMVOC | 43652 | 44337 | 50772 | 48830 | 47962 | 45871 | 46206 |
| SOx | 196266 | 124730 | 98738 | 68312 | 71003 | 65612 | 54121 |

The main sources of NOx emissions are traffic and thermal power plants. NOx emissions in the Republic of Slovenia are diminishing at a slower rate than expected, particularly because of the ever increasing fraction of diesel-fuelled motor vehicles and, in part, because of the deviation of real life emissions from laid-down emissions of heavy-duty trucks. Recently, even a slight rise has been recorded, which is chiefly a consequence of increased production of electric energy from fossil fuels.

SO2 emissions in Slovenia have since 1990 diminished by as much as 72% compared to 1990. This reduction was achieved in part through flue gas desulphurisation and in part also by substituting domestic coal for imported coal with low sulphur content.

Further reduction of SO2 is expected in 2005 when the new wet flue gas desulphurisation unit installed at TE Trbovlje will be operational.

Reducing SO2 emissions has significantly improved air quality in Slovenia. Beside the improvement of the quality of solid fuels, in this period there was a crucial reduction of the permitted contents of sulphur in liquid fuels, in residual fuel oil, extra light fuel oil, as well as in diesel oil. The chief remaining source, after improving the quality of liquid fuels, is actually the domestic coal.

NMVOC emissions, encompassing more than a hundred organic components, are emitted chiefly in traffic, solvent use, and technological processes. A great fraction of NMVOCs are emissions due to the consumption of woody biomass in households. Compared to 2000, emissions in 2004 diminished by 9%.

Chief sectors for CO emissions are particularly commercial, institutional, and residential boilers as well as traffic. CO emissions from traffic are diminishing mainly because of modernising the vehicle stock and the diminishing fraction of gasoline-fuelled vehicles.

NMVOC and CO inventories have been in 2000 complemented by emissions of NMVOC and CO from the consumption of woody biomass in households. Since the recalculation for the previous period has not been performed yet, a direct comparison of emission prior to and after 2000 is not possible.

## INTRODUCTION

## Background information on greenhouse gas inventories and climate change

At the Second World Climate Conference in Geneva in October and November 1990, a clear need was expressed for standard methodology for monitoring emissions of greenhouse gases, which was to enable comparing and enhancing inventories in individual countries. Under the auspices of OECD and International Energy Agency and with the support of the United States of America, United Kingdom, and Norway, a draft methodology was set up. That document comprised six direct and indirect greenhouse gases: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), nitrogen oxides (NOx), carbon monoxide (CO), and non-methane volatile organic compounds (NMVOCs). The methodology was adopted in Paris in March 1991 at the Fifth Session of the Intergovernmental Panel on Climate Change (IPCC) and it became the starting point for individual states in creating their national inventories of greenhouse gases.

The methodology for the calculation of greenhouse gases kept developing all the time and is even today a project under development. In the IPCC inventory of greenhouse gases for Slovenia, the 1996 version was applied (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996), which in some parts also takes into account the emissions of direct greenhouse gases that have been encompassed by the Kyoto Protocol (CF4, C2F6, PFCs, HFCs and SF6).

The guidelines for the implementation of the inventory of greenhouse gases contain prescribed methods for calculation of emissions, providing a unified framework for reporting and documenting sources for all inventories. One of the main aims of this method is to ensure comparability of data gathered in individual states and that calls for a definition of at least a minimum scope of equal methods, criteria, and estimating procedures. These guidelines come in three volumes, which together form a complete set of recommended methods for estimating emissions.

The report presents estimates for the 6 gaseous air pollutants included in Annex A to the Kyoto Protocol: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), hydro fluorocarbons (HFCs), per fluorocarbons (PFCs) and sulphur hexafluoride (SF6), as well as estimates for indirect GHGs, including carbon monoxide (CO), nitrogen oxides (NOx), and non-methane volatile organic compounds (NMVOC). Data are also reported for sulphur oxides (SOx).

Global warming potential

The GWP of a greenhouse gas is defined as the ratio of the time-integrated radiative forcing from the instantaneous release of 1 kg of a trace substance relative to that of 1 kg of a reference gas (IPCC 2001). Direct radiative effects occur when the gas itself is a greenhouse gas. The reference gas used is CO2, and therefore GWP-weighted emissions are measured in Tg of CO2 equivalents (Tg CO2 eq.). 9 All gases in this executive summary are presented in units of Tg CO2 equivalent.

While any time period may be selected, this report uses the 100-year GWPs recommended by the IPCC, and adopted by the UNFCCC for reporting purposes (IPCC 1996). GWP values are listed below in Table 03.

Table 1.1: Global Warming Potentials (100 Year Time Horizon) Used in this Report

Gas 			GWP

Carbon dioxide (CO2)	 1

Methane (CH4)*	 21

Nitrous oxide (N2O) 	310

HFC-23 		11,700

HFC-32 		650

HFC-125 		2,800

HFC-134a 		1,300

HFC-143a 		3,800

HFC-152a 		140

HFC-227ea 		2,900

HFC-236fa 		6,300

HFC-4310mee 	1,300

CF4 			6,500

C2F6 			9,200

C4F10 			7,000

C6F14 			7,400

SF6 			23,900

Source: IPCC (1996)

* The methane GWP includes the direct effects and those indirect effects due to the production of tropospheric ozone and stratospheric water vapour. The indirect effect due to the production of CO2 is not included. Global warming potentials are not provided for CO, NOx, NMVOCs, SO2 or aerosols because there is no agreed upon method to estimate the contribution of gases that are short-lived in the atmosphere, spatially variable, and have only indirect effects on radiative forcing (IPCC 1996).

## A description of the institutional arrangement for inventory preparation

In the Republic of Slovenia, the institution charged with the responsibility for making GHG inventories is the Environmental Agency of the Republic of Slovenia. In accordance with its tasks and obligations to international institutions, the Environmental Agency is charged with making inventories of GHG emissions as well as emissions that are defined in the Convention on Long Range Transboundary Air Pollution within the laid-down time-limit. To this effect, the Environmental Agency has increased the number of its staff. In making the inventories, the Environmental Agency cooperates with numerous other institutions and administrative bodies which relay the necessary activity data and other necessary data for making the inventories.

The chief sources of data are the Statistical Office of the Republic of Slovenia and the Ministry of Environment, Spatial Planning and Energy; however, the Environmental Agency obtains much of its data through other activities, which it performs under the Environmental Protection Act. Emissions from two sectors are calculated by two external institutions: emissions from Agriculture are calculated by the Slovenian Agriculture Institute, sinks in the Land Use Change and Forestry sector by the Slovenian Forestry Institute.

Slovenia has to comply with the following obligations:

Slovenia's annual obligation under the UNECE Convention on Long-range Transboundary Air Pollution (CLRTAP) and its Protocols (1979), comprising the annual reporting of national emission data on SO2, NOX, NMVOCs, NH3, CO, TSP, PM10, and PM2,5 as well as on the heavy metals Pb, Cd and Hg and persistent organic hydrocarbons (PAHs), dioxins and furans and hexa-chlor-benzene (HCB).

Slovenia's annual obligation under the Decision No 280/2004/EC of the European Parliament and of the Council of 11 February 2004 concerning a mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol.

Implementing of this decision is described in Commission Decision of 10 February 2005 laying down rules implementing Decision No 280/2004/EC of the European Parliament and of the Council concerning a mechanism for monitoring Community greenhouse gas emissions and for implementing the Kyoto Protocol (2005/166/EC).

Slovenia’s obligation under the United Nations Framework Convention on Climate Change (UNFCCC, 1992).

Relevant COP Decisions and Guidelines are:

- Decision 3/CP.5 Guidelines for the Preparation of National Communications by Parties included in Annex I to the Convention, Part I: UNFCCC Reporting Guidelines on Annual Inventories (referring to Document FCCC/CP/1999/7).

- Decision 4/CP.5 Guidelines for the Preparation of National Communications by Parties included in Annex I to the Convention, Part II: UNFCCC Reporting Guidelines on National Communications (referring to Document FCCC/CP/1999/7).

- Document FCCC/CP/1999/7 Review of the Implementation of Commitments and of other Provisions of the Convention – UNFCCC Guidelines on Reporting and Review.

- Decision 11/CP.4 National communications from Parties included in Annex I to the Convention.

- Document FCCC/CP/2001/13/Add.3 Report of the Conference of the Parties on its seventh session, held at Marrakech from 29 October to 10 November 2001, Addendum, Part two: Action taken by the Conference of the Parties, Volume III (Decision 20/CP.7: Guidelines for national systems under Article 5, paragraph 1, of the Kyoto Protocol; Decision 21/CP.7: Good Practice Guidance and Adjustments under Article 5, paragraph 2, of the Kyoto Protocol; Decision 22/C.7: Guidance for the preparation of the information required under Article 7 of the Kyoto Protocol; Decision 23/CP.7: Guidelines for review under Article 8 of the Kyoto Protocol).

Slovenia's obligation according to Article 15 of the European IPPC Directive 1996/61/EC is to implement a European Pollutant Emission Register (EPER). Article 15 of the IPPC Directive can be associated with Article 6 of the Aarhus Convention (United Nations: Aarhus, 1998), which refers to the right of the public to access environmental information and to participate in the decision-making process of environmental issues.

Table 1.2: Inventory Institutional Arrangements and Data Sources

| IPCC category | IPCC sub-category | Sources of data |
| --- | --- | --- |
| CRF 1 A – Energy. Fuel Combustion | CRF 1A1 - Energy Industry |  Large Combustion Plants (LCP)  Ministry of Energy, Directorate of Energy (DGE): annual energy statistics  Energy Agency  Statistical Office of the Republic of Slovenia: Joint Questionnaires, Energy Balances |
|  | CRF 1A2 - Manufacturing Industries and Construction |  Statistical Office of the Republic of Slovenia:  Environmental Agency of the Republic of Slovenia |
|  | CRF 1A3 – Transport |  Statistical Office of the Republic of Slovenia: Joint Questionnaires, Energy balances  Ministry of Transport, Directorate for National Roads (DRSC)  Ministry of Internal affairs (vehicle stock) |
|  | CRF 1A4 – Other Sectors |  Statistical Office of the Republic of Slovenia: |
| CRF 1 B – Fugitive Emissions from Fuels |  |   Ministry of Energy, Directorate of Energy (DGE)  Agency of Energy  Statistical Office of the Republic of Slovenia: |
| CRF 2 – Industrial Processes | CRF 2A – Mineral Products |  Statistical Office of the Republic of Slovenia:  Environmental Agency of the Republic of Slovenia |
|  | CRF 2B – Chemical Industry |  Statistical Office of the Republic of Slovenia:  Environmental Agency of the Republic of Slovenia |
|  | CRF 2C – Metal Production |  Statistical Office of the Republic of Slovenia:  Environmental Agency of the Republic of Slovenia |
|  | CRF 2D – Other Production |  Statistical Office of the Republic of Slovenia:  Environmental Agency of the Republic of Slovenia |
|  | CRF 2F – Consumption of Halocarbons and SF6 |  Environmental Agency of the Republic of Slovenia |
| CRF 3 – Solvent and Other Product Use |  |  Statistical Office of the Republic of Slovenia  Environmental Agency of the Republic of Slovenia |
| CRF 4 – Agriculture |  |  Agricultural Institute of Slovenia  Statistical Office of the Republic of Slovenia |
| CRF 5 – Land Use Change and Forestry |  |  Slovenian Forestry Institute  Statistical Office of the Republic of Slovenia |
| CRF 6 – Waste | CRF 6A – Solid Waste Disposal on Land |  Environmental Agency of the Republic of Slovenia |
|  | CRF 6B – Wastewater Handling |  Statistical Office of the Republic of Slovenia  Environmental Agency of the Republic of Slovenia |

## Brief description of the process of inventory preparation

Owing to the ever increasing obligations of the Republic of Slovenia with regard to reporting, the Environmental Agency of the Republic of Slovenia has decided to implement a unified system of collecting data for the purposes of making inventories, as well as secure reliable financing in accordance with the annual program of its work. The ability to fulfil its obligations with regard to reporting was also improved by the participation of Environmental Agency in the GEF project “Capacity building for improving GHG inventories”, which has finished in June 2006, and thus the Republic of Slovenia has in due time made the inventories and sent them in the required form to the UNFCCC Secretariat.

A Memorandum of Understanding has been concluded with institutions that participate in the preparation, binding these institutions to submit quality and verified data to the Environmental Agency in due time, because the time limits for making inventories and the NIR have shortened with the entry of Slovenia into EU, since inventories and part of the NIR for the year before last must be made until 15 January with ability for corrections and final submission of NIR until 15 March. In view of this, an agreement has been reached with the participating institutions for them to shorten the time limits for submitting data. For reasons of complexity, attention was mostly focused on Joint Questionnaires of the Statistical Office of the Republic of Slovenia, on the basis of which the Statistical Office produces the energy balance of the Republic of Slovenia, wherein the most important date on energy sector are to be found.

Figure 1.1: Data flow of Slovenian Inventory System

The year 2003 saw the end of the process of harmonization of data collection among the Directorate of Energy, Ministry of Environment, Spatial Planning and Energy, and the Statistical Office of the Republic of Slovenia. An end was put to previous parallel double collecting of data. The competence of collecting data has in accordance with the law passed to the Statistical Office of the Republic of Slovenia, which checks the data and eliminates potential reporting errors, and submits consolidated data to the Directorate of Energy, which publishes data annually in its Energy Yearbook of the Republic of Slovenia. In terms of its contents, data are identical as those submitted in Joint Questionnaires to the IEA.

All data from external institutions are submitted in the same form. Data are stored on a network server. Access to files is limited in accordance with the security policy. Backup copies on server are done at regular intervals in accordance with the requirements of the information system. The archive is stored at several locations and on several types of media. All support research projects are stored both in digital form as well as in hard-copy form.

Subsequent to the making of an inventory, all support files are frozen so that they may not be changed anymore. CRF tables are stored both in digital form as well as a hard copy.

For submitting reports to different institutions, various report formats have been made, since the database is used to report to UNFCCC, as well to EEA, EC, and CLRTAP. All external reports of the Environmental Agency of the Republic of Slovenia are conducted in accordance with ISO 9001 via the Agency's reporting service, which keeps inventories of reports. Parallel to that, emission data are submitted to the Statistical Office of the Republic of Slovenia, which publishes emission data in its publications and submits them to EUROSTAT and IEA. Establishing a single reporting system in 2003 has ensured above all the uniformity and traceability of reporting.

## Brief general description of methodologies and data sources used

Inventories of GHG emissions were presented on the basis of the IPCC (IPCC 1996, GPG 2000) methodology for all the gases and sectors, except in a few cases where they were presented separately. Some emission sources that were registered and assessed were not covered by the IPCC methodology. Due to the importance of the source and accessible data, different approaches (tiers) from within the IPCC methodology were used. National emission factors were used for assessment of emissions from domestic coal and natural gas (Tier 2), while for other fuels, mainly default IPCC emission factors were used.

The quantities of fuels and consumed fuel energy values were taken from energy annual inventories, prepared by Ministry of Economy, Directorate of Energy, based on the data of the Statistical Office of the Republic of Slovenia. Additional data on the energy use of some types of waste (waste tires, oils and solvents) were acquired. Data on the fuel consumption in agriculture and forestry refer to mobile sources only, while the rest of the fuel consumption of those sub-sectors is included in the public and service sub-sector. Default IPCC emission factors and oxidation fractions were used for energy consumption of liquid fossil fuels. Due to the greater fraction of methane in natural gas used in Slovenia, the more appropriate, slightly lower CH4 emission factor than the default emission factor was used for the whole period. CH4 and N2O emissions in road transport were determined according to a more specific methodology and CORINAIR emission factors. From the potential emissions for diesel-fuelled vehicles, we subtracted carbon that was included in emitted solid parts, while for gasoline vehicles a 100% fuel oxidation was assumed. Under CO2 fugitive emissions in the Energy sector, those emissions were considered that were released during flue gas desulphurization in thermal power plants and were calculated on the basis of data on the consumption of calcium carbonate. CO2 emissions in post-mining activities were not assessed due to the lack of data on desorption. However, they appeared greater than those that were released directly during coal excavation. Emission factors for fugitive emissions of CH4 in mining activities were determined on the basis of measurements of methane concentrations in ventilation shafts in mines and estimated quantities of released methane. The emission factor that was determined in this manner was lower than the default IPCC emission factor. The regional default IPCC emission factor for transmission and distribution of natural gas does not correspond to the conditions in Slovenia; consequently, in calculating CH4 emissions due to the distribution of natural gas, data of the companies that manages the distribution and transportation network were used. Distribution losses were estimated according to the length of individual types of transmission pipelines with regard to the pipe type, applying specific losses per unit of length, as presented in the German Inventory, and this appears a sensible solution considering the level of maintenance and low average age of the distribution network.

Emissions from industrial processes were mostly determined on the basis of statistical data on production and consumption of raw materials and by applying the country specific emission factors. After 1997, the Statistical Office of the Republic of Slovenia partly changed the manner of collecting and presenting these data, and therefore some data were obtained from individual companies. The consumption of anodes and other reducing agents was taken into account for the production of metals and ferroalloys. Therefore, the consumption of coke was also included in the steel and iron production since 1997, while for the production of ferroalloys, the consumptions of coke and petrol coke were used. Emissions from primary aluminium production were estimated from anode consumption and from PFC emissions, which were determined on the basis of the number and duration of anode effects. When determining the actual emissions caused by the use of HFC, data were obtained from companies that use or sell those materials, as well as data on the export and import of refrigerators. For SF6 emissions, the release of this gas from gas insulated switchgear in the Energy sector was assessed.

NMVOC emissions from the consumption of solvents and diluents were estimated mostly on the basis of the CORINAIR methodology. This category also includes emissions that arise from evaporation during the use of N2O, mostly for anaesthesia.

In agriculture, methane emissions from enteric fermentation in bovine animals were elaborated in great detail and the Tier 2 approach was upgraded by disaggregating livestock into 18 categories according to the intensity of breeding. For emissions from manure management, the Tier 2 approach was used for pig production and bovine animal production. The Tier 1 approach was used for other animals that represent a smaller fraction in methane emissions. Input data for N2O emissions from manure handling and from indirect emissions from fertilisation with animal fertilisers were obtained in the process of estimating methane emissions. For N2O emissions, default IPCC factors, determining the conversion of nitrogen into N2O, were used.

Methane emissions from solid waste handling were determined by FOD method which take into account the time dynamics of methane release. Emissions of N2O from wastewater were determined according to the consumption of proteins in human nutrition and production data in food and paper industry.

Table 1.3:SUMMARY REPORT FOR  METHODS AND EMISSION FACTORS USED iz CRF

| GREENHOUSE GAS SOURCE AND SINK | CO2 |  | CH4 |  | N2O |  | HFCs |  | PFCs |  | SF6 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CATEGORIES | Method applied | Emission factor | Method applied | Emission factor | Method applied | Emission factor | Method applied | Emission factor | Method applied | Emission factor | Method applied | Emission factor |
| 1. Energy | M,T1 | CS,D | M,T1 | CS,D | M,T1 | CS,D |  |  |  |  |  |  |
| A. Fuel Combustion | M,T1 | CS,D | M,T1 | CS,D | M,T1 | CS,D |  |  |  |  |  |  |
| 1. Energy Industries | T1 | CS,D | T1 | D | T1 | D |  |  |  |  |  |  |
| 2. Manufacturing Industries and Construction | T1 | D | T1 | D | T1 | D |  |  |  |  |  |  |
| 3. Transport | M,T1 | D | M,T1 | CS,D | M,T1 | CS,D |  |  |  |  |  |  |
| 4. Other Sectors | T1 | CS,D | T1 | D | T1 | D |  |  |  |  |  |  |
| 5. Other | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| B. Fugitive Emissions from Fuels | T1 | D | T1 | CS,D | NA | NA |  |  |  |  |  |  |
| 1. Solid Fuels | T1 | D | T1 | CS | NA | NA |  |  |  |  |  |  |
| 2. Oil and Natural Gas | NA | NA | T1 | CS,D | NA | NA |  |  |  |  |  |  |
| 2. Industrial Processes | CR,CS, D,T2 | CS,D, OTH,PS | D | D | D | D | T2 | D | T3 | PS | T2 | D |
| A. Mineral Products | CR,CS,D,T2 | CS,D,OTH | NA | NA | NA | NA |  |  |  |  |  |  |
| B. Chemical Industry | CR,D | D,OTH | D | D | D | D |  |  |  |  | NA | NA |
| C. Metal Production | D,T2 | PS | NA | NA | NA | NA | NA | NA | T3 | PS | NA | NA |
| D. Other Production | NA | NA |  |  |  |  |  |  |  |  |  |  |
| E. Production of Halocarbons and SF6 |  |  |  |  |  |  | NA | NA | NA | NA | NA | NA |
| F. Consumption of Halocarbons and SF6 |  |  |  |  |  |  | T2 | D | NA | NA | T2 | D |
| G. Other | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |
| 3. Solvent and Other Product Use | CR | CR |  |  | D | D |  |  |  |  |  |  |
| 4. Agriculture |  |  | T1,T2 | CS,D | D, T1a,T1b | CS,D |  |  |  |  |  |  |
| A. Enteric Fermentation |  |  | T1,T2 | CS,D |  |  |  |  |  |  |  |  |
| B. Manure Management |  |  | T1,T2 | CS,D | D | D |  |  |  |  |  |  |
| C. Rice Cultivation |  |  | NA | NA |  |  |  |  |  |  |  |  |
| D. Agricultural Soils |  |  | NA | NA | D,T1a, T1b | CS,D |  |  |  |  |  |  |
| E. Prescribed Burning of Savannas |  |  | NA | NA | NA | NA |  |  |  |  |  |  |
| F. Field Burning of Agricultural Residues |  |  | NA | NA | NA | NA |  |  |  |  |  |  |
| G. Other |  |  | NA | NA | NA | NA |  |  |  |  |  |  |
| 5. Land Use, Land-Use Change and Forestry | T2 | CS,D | NA | NA | NA | NA |  |  |  |  |  |  |
| A. Forest Land | T2 | CS,D | NA | NA | NA | NA |  |  |  |  |  |  |
| B. Cropland | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| C. Grassland | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| D. Wetlands | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| E. Settlements | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| F. Other Land | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| G. Other | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| 6. Waste | NA | NA | T1,T2 | D | T1 | D |  |  |  |  |  |  |
| A. Solid Waste Disposal on Land | NA | NA | T2 | D |  |  |  |  |  |  |  |  |
| B. Waste-water Handling |  |  | T1 | D | T1 | D |  |  |  |  |  |  |
| C. Waste Incineration | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| D. Other | NA | NA | NA | NA | NA | NA |  |  |  |  |  |  |
| 7. Other (as specified in Summary 1.A) | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA | NA |

## Brief description of key source categories

The analysis of key source categories has been done on the basis of the sectoral distribution and using the Tier 1 approach. This approach has been used both for the base year and for the year 2004.

Level assessment was undertaken for years 1986 and 2004 and trend assessment was performed for year 2004.

On the basis of the analysis, 26 categories were selected, which represent 95.2% of emissions in 2004 according to level assessment and 7 more were chosen which are key category according to trend assessment.  As many as 22 categories are key sources according to level and trend key source analyse.

In previous submissions also Tier 2 KS analyze had been done. We have not done Tier 2 analyze for this year submission as uncertainty estimates have not been updated yet.

Table 1.4: Brief description of IPCC KS Categories for 2004

## Information on the QA/QC plan including verification and treatment of confidentiality issues where relevant

The Republic of Slovenia has not yet fully developed and implemented a Quality Assurance and Quality Control plan as recommended by IPCC Good Practice Guidelines (IPCC 2000). Quality control procedures are described in the Manual of Procedures which has already been elaborated.

Certain data control procedures covered by the Manual of Procedures are already in use in developing inventories. The items verified are input data at the level of sectoral activity data, the appropriateness of chosen emission factors, the applied methodology as well as intermediate and final calculations of emissions where deviations between real life emission factors and factors as calculated from the CRF table are reviewed, too.

In 2006 additional quality control check point was introduced by forwarding assessment of verified emission reports from installations included in National Allocation Plan to the Statistical Office of the Republic of Slovenia (SORS). Role of SORS is to compare data from installations included in EU-ETS with data from their reporting system and to propose, if necessary, correctional measures. Outcome of data consistency checks is used as preliminary information for the Ministry of the Environment and Spatial Planning to perform on site inspections. However first iteration of data comparison showed that differences are significantly lower than 5%.

Together with a team who is preparing other pollutants inventory (CLRTAP) and our information management service we are in the middle of establishing the joint database. This will finally enable us to prepare all necessary reports to conventions and EU from the same set of data. The database is going to contain all activity data, emission factors and other parameters including description of sources from 1980 on for other pollutants, and from 1986 on for GHG emissions. On defined control points QC procedures will be included and documented. At a final stage we are going to develop a direct bulk import file from database to CRF Reporter. Majority of database will be prepared until end of 2006, next submission calculations will be prepared on both ways what will enable us to check for possible errors. The whole process is planned to be finished in March 2007.

After finishing this project a lot of time consuming manual work for preparing GHG inventory will not be required any more. The staff working on GHG and other pollutant inventory will join together to perform some common work and common QC procedures like: raw data collection, acquisition, and handling (e.g., completeness, accuracy of data sources and units, consistency of activity data trend, etc.); data input into database (e.g., transcription errors, accuracy of units, etc.) and uncertainty estimates of AD.

QA generally consists of independent third-party review activities to ensure that the inventory represents the best possible estimates of emissions and removals and to support the effectiveness of the QC program. In the past we have performed only one peer-review. In 2006 during preparation of our Forth NC Report, we have got many useful comments from team preparing our Forth National Communication Report. Although those comments were not presented as official report, we have accepted many suggestions and corrected some mistakes. We are planning to prepare official peer review for 2007 inventory.

## General uncertainty evaluation, including data on the overall uncertainty for the inventory totals

In 2003, the uncertainties of activity data and emission factors in sector 1.A.1.a Public Electricity and Heat Production have been reduced. In this sector, because of the introduction of CO2 emission trading within the EU, we have re-verified the input data and the procedure of determining the consumption of fuels. In calculating emissions from this sector, the national emission factors based on coal sampling and ascertaining the carbon contents have also been applied. All analyses have been done in the accredited laboratory in accordance with the EN ISO 17025 (“General requirements for the competence of testing and calibration laboratories”). Public power plants have presented their plan of measures to ensure that fuel consumption will be measured without intermediate storage before combustion in the installation applying measuring devices resulting in a maximum permissible uncertainty of less than +/- 2.5% for the measuring process.

The combined uncertainty was derived from Tier 1 method. The uncertainties of individual activity data and emission factors are mostly based on expert judgment. Since expert judgments of individual experts are at variance, the highest individual uncertainties have been taken into account. The total uncertainties have been derived both for Level Uncertainty as well as for Trend Uncertainty.

Table 1.5: Combined Uncertainty of GHG inventories

|  | 1986 | 2002 | 2003 |
| --- | --- | --- | --- |
| Level Assessment | 16.0% | 13.1% | 12.0% |
| Trend Assessment | - | 4.0% | 3.0% |

- In UNFCCC review report of previous year submission the ERT recommended that Slovenia revise the uncertainty assessment as the method of assigning uncertainties to the source categories seems not to be in line with the IPCC good practice guidance, and to add information on the methods used and descriptions on the expert judgements applied. As this task is rather exacting we didn’t made this improvement in 2006 submission yet but we are planning to update uncertainty estimates and include more descriptions in the future submission. For this reason we didn’t calculate new combined uncertainty for 2004 inventory and we didn’t perform a Tier 2 KS analyze either. Uncertainty estimates from previous submission are presented in Annex 7.

## General assessment of the completeness

CRF Table 9 (Completeness) has not been fill in for this submission because of some problems with CRF Reporter and very time consuming way of putting this information in appropriate cell. We hope that this problem will be solved in next year when automatic filling of CRF Reporter is going to be restored.

An assessment of completeness for each sector may be found in the Sector Overview part of the corresponding subchapters, here some aggregated information is presented.

Sources and sinks

All sources of direct GHG gases, included in the IPCC Guidelines, are covered in inventory. There are some gaps in estimations of sinks considering LULUCF sector. We are going to fill this gaps in 2007 at the end of the research project, which has already started in 2005. There are also some gaps in estimations of NMVOC emissions fin Solvent Use Sector.

Gases

The direct GHGs as well as the postulated precursor gases are covered by the Slovenian inventory. After submitting our CRF tables we realized that precursor gases have not been transferred from CRF Reporter to CRF tables. We are going to solved this problem after new version of software will be released.

Geographic coverage

The geographic coverage is complete. No territory in Slovenia has been left uncovered by the inventory.

Notation keys

IE (included elsewhere):

The consumption of fuel in water-borne navigation and for military purposes has not been estimated at the state level, due to its small size. The fuel used for this purposes is included in other sectors (road transportation, institutional sector and civil aviation). Slovenia doesn’t have any jet fighter airplanes and Navy and there is no possibility that emissions from these sources would be included in international bunkers and therefore excluded from national totals.

Slovenia has no waste incineration plant yet. Very small amount of waste is combusted in cement plants and reported in Energy sector/Manufacturing Industry and Construction.

NE (not estimated):

The highest number of source categories marked with NE is found in LULUCF sector, which is in right now in the process of improvement.

There are also many categories marked with NE because methodologies for estimating GHG emissions are not available in IPCC manuals from 1996 or in GPG from 2000. These sources are:

  - CO2 emissions from underground mines

  - GHG emissions in solvent use sector

  - CH4 from enteric fermentation from poultry

  - CH4 from agricultural soil

  - CO2 from solid waste disposal sites

  - N2O from industrial waste water

For some GHG  emissions there are checkups in progress to see if they actually are “NO”. As a part of the program to improve the inventory, it is planned that those sources or sinks be either estimated, marked with “IE” or allocated as “NO”. These are:

  - fugitive GHG emissions from Venting and Flaring

  - CO2 emissions from iron and steel plant considering sintering process

  - N2O emissions from use of N2O in Solvent use sector

In agriculture sector all categories in Field Burning of agriculture residues are marked as NE.

Although this practice is not allowed in Slovenia same burning did happened on very small scale.

The  potential emissions of F-gases have also not been estimated, because data on import and export are not available separately for individual gas bat only together with some other gasses. Solution of this problem is in the hands of relevant institutions.

NA (not applicable):

The increase of this number is due to improved completeness of the CRF- tables.

NO (not occurring)

The highest number of source categories marked with NO is found in agriculture, but there are some in industrial processes and energy industries.

C (confidential)

Statistical low considering confidentiality is very strict in Slovenia. All data which are gathered from three or less reporting unit are confidential. It is good practise in national statistic that this boundary is even higher (five unit). As Slovenia is a small country almost all relevant categories from industrial processes sector and in less extend also in energy sector are confidential. Nevertheless no data in our report is marked with C. The confidentiality problem in activity data has been solved on individual level with each relevant plant. In the future, data gathered from verified reports from installations included in ETS, are going to resolve this problem generally for most cases.

# TRENDS IN GREENHOUSE GAS EMISSIONS

## Description and interpretation of emission trends for aggregated greenhouse gas emissions

The total emissions of GHG in 2004, sinks not considered, amounted to 19,946 Mt CO2 eq., which represents a 1,08% reduction of emissions compared to 1986 base year. In the period 1986-1991, a reduction of emissions was recorded due to the economic conditions at that time and the Republic of Slovenia gaining its independence. In the period 1992-1997, a strong increase of emissions was recorded, which was a consequence of increasing economic growth and revival of industrial production. In the second half of that period, the increased emissions were a consequence of “gasoline tourism” (25% of the total sale of motor fuels in the Republic of Slovenia), since the prices of motor fuels in the Republic of Slovenia were appreciably lower than in the neighbouring countries.

In the period 1998-1999, emission decreased due to the measures undertaken by the neighbouring countries to curb the ”gasoline tourism” and due to the increased supply of electrical energy from the Krško Nuclear Power Plant. In the 2000-2002 period, emission kept increasing again due to the increased obligatory export of electrical energy from the Krško Nuclear Power Plant to the Republic of Croatia. Simultaneously, due to very dry and hot summers the consumption of electrical energy increased and the production of electrical energy in hydroelectric power stations in the Republic of Slovenia decreased. Consequently, the thermal power plants had to make up for both the deficit in the production of electrical energy in hydroelectric power plants as well as the increased consumption of electrical energy, however, in 2004, the situation stabilised and more electricity was produced from hydro power plants.

Figure 2.1: Slovenian GHG emission trend. (Values between 1986 and 1990 were interpolated.)

## Description and interpretation of emission trends by gas

CO2 emissions in 2004 represented 82.2% of overall emissions of greenhouse gases. CO2 emissions followed the consumption of energy and with regard to their fraction exerted a major influence on total emissions. Compared to 1986, in 2004 they increased by 1.5%. CH4 emissions represented 10.6% of total emissions in 2004 (11.8% in 1986) and were lower than in 1986 by 11.2%. Compared to the previous year, N2O emissions have decreased by 3.8% and represented 6.2% of total emissions, but were nevertheless lover of N2O emissions in 1986 by 7.9%. F-gasses represent 1% of total emissions and some of them (HFC and SF6) have shown significant increase since 1986.

Table 2.2:GHG emission trends by gas

| GREENHOUSE GAS EMISSIONS | Base year ( 1986 ) | 1990 | 1995 | 2000 | 2003 | 2004 | Change from base to latest reported year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | (%) |
| CO2 emissions including net CO2 from LULUCF | 14,570.654 | 11,447.009 | 9,960.025 | 9,940.814 | 10,630.518 | 10,753.530 | -26.197 |
| CO2 emissions excluding net CO2 from LULUCF | 16,159.907 | 14,632.756 | 14,865.266 | 15,115.975 | 15,948.746 | 16,397.250 | 1.469 |
| CH4 | 2,376.290 | 2,289.374 | 2,118.178 | 2,136.056 | 2,123.174 | 2,108.716 | -11.260 |
| N2O | 1,340.892 | 1,218.707 | 1,170.751 | 1,293.830 | 1,283.838 | 1,235.306 | -7.874 |
| HFCs | NA,NO | NA,NO | 28.957 | 31.127 | 56.913 | 66.321 | 100.000 |
| PFCs | 276.291 | 257.444 | 285.685 | 105.612 | 118.990 | 120.008 | -56.565 |
| SF6 | 10.241 | 10.303 | 11.522 | 15.738 | 17.915 | 18.312 | 78.810 |
| Total (including net CO2 from LULUCF) | 18,574.367 | 15,222.838 | 13,575.119 | 13,523.178 | 14,231.349 | 14,302.192 | -23.000 |
| Total (excluding net CO2 from LULUCF) | 20,163.620 | 18,408.584 | 18,480.360 | 18,698.339 | 19,549.577 | 19,945.913 | -1.080 |

Carbon dioxide – CO2

CO2 emissions in the period 1986-2004 may be split into five segments. In the first segment, 1986-1991, emissions diminished due to reduction of industrial production and the war for independence in 1991. Emissions rose strongly in the 1991-1997 period, when emissions increased also due to gasoline tourism. Then came a short period of emission reduction as a consequence of the reduction of gasoline tourism and decreased consumption of fossil fuels for the production of electrical energy. After 1999, emissions have risen again, mainly as a consequence of the production of electrical energy. CO2 emissions in 2002 thus amounted to 16.15 Mt of CO2, which is nearly the same as in the 1986 base year. However, in 2003, emission of CO2 decreased by 1.1%, which was mainly due to lower emission from Energy Industries and in 2004 increased again by 2.8%, which is mainly due to traffic.

In that entire period of time, the strongest increase of CO2 emissions was in traffic emissions, namely by as much as 108%, from 1,97 Mt CO2in year 1986 to 4.09 Mt CO2 in 2004.

Methane – CH4

Methane emissions have in the 1986-2004 period constantly kept diminishing from 2.38 Mt CO2 eq. in 1986 to 2.11 Mt CO2 eq. in 2004. CH4 emissions have diminished by 11.3% in spite of the increase of emissions from waste by 17.5%, compared to the base year. This reduction was mainly due to Agriculture (-14.6%) and Energy sector (-29.1%)

Nitrous oxide - N2O

N2O emissions have been reduced from 1.34 Mt CO2 eq. in 1986 to 1.24 Mt CO2 eq. in 2004. In agriculture, which is the main source of N2O emissions, emissions diminished chiefly due to reduced number of animals and the reduced extent of arable crop production, particularly legumes and papilionaceous plants. Partly, this reduction is due to the changed manner of manure storage, since the fraction of straw based systems is diminishing on account of the increasing fraction of accommodation on slatted floor. Recently, an increase of the fraction of traffic in total N2O emissions has been observed, this fraction rising from 1.9% in 1986 to 12.5% in 2004.

Hydro-fluorocarbons - HFC

HFC emissions keep growing from year to year. In 2004, emissions increased by 16.5% compared to the previous year, which is mostly the consequence of increasing fraction of air conditioners in motor vehicles.

Per-fluorocarbons - PFC

The only source of PFC in the Republic of Slovenia is the primary production of aluminium. Improving the technology in the production of aluminium since 1995 almost halved the then emissions, which diminished from 276 kt CO2 eq. to 120 kt CO2 eq. A more detailed description of the reduction of emissions is given in chapter 4.11, Aluminium Production

Sulphur-hexafluoride - SF6

The main source of SF6 emissions is high-voltage gas insulated switchgear and circuit breakers. SF6 emissions represent only 0.1% of total GHG emissions.

Note: Values between 1986 and 1990 were interpolated.

Figure 2.2: GHG Emissions in Slovenia by gas

## Description and interpretation of emission trends by source

According to the UNFCCC Reporting Guidelines, emissions estimates are grouped into six IPCC categories: Energy, Industrial Processes, Solvent use, Agriculture, Land Use, Land-Use Change and Forestry, and Waste.

Table 2.3: GHG emissions and removals in Slovenia by sector: 1986-2004

| GREENHOUSE GAS SOURCE AND SINK CATEGORIES | Base year ( 1986 ) | 1990 | 1995 | 2000 | 2003 | 2004 | Change from base to latest reported year |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | CO2 eq. (Gg) | (%) |
| 1. Energy | 15,922.253 | 14,266.356 | 14,721.030 | 14,953.070 | 15,721.719 | 16,144.772 | 1.398 |
| 2. Industrial Processes | 1,287.833 | 1,291.991 | 1,109.289 | 969.709 | 1,104.261 | 1,134.663 | -11.894 |
| 3. Solvent and Other Product Use | 81.903 | 43.400 | 17.251 | 42.729 | 33.332 | 39.246 | -52.082 |
| 4. Agriculture | 2,305.437 | 2,214.916 | 2,090.863 | 2,136.370 | 2,066.484 | 1,972.562 | -14.439 |
| 5. Land Use, Land-Use Change and Forestry(7) | -1,589.253 | -3,185.747 | -4,905.241 | -5,175.161 | -5,318.228 | -5,643.721 | 255.118 |
| 6. Waste | 566.194 | 591.921 | 541.928 | 596.461 | 623.780 | 654.670 | 15.626 |
| 7. Other | NA | NA | NA | NA | NA | NA | 0.000 |
| Total (including LULUCF) | 18,574.367 | 15,222.838 | 13,575.119 | 13,523.178 | 14,231.349 | 14,302.192 | -23.000 |
| Total (excluding net CO2 from LULUCF) | 20,163.620 | 18,408.584 | 18,480.360 | 18,698.339 | 19,549.577 | 19,945.913 | -1.080 |

By far the most important sector is Energy, which in 2004 accounted for 80.9% of the total GHG emissions. Emissions in this sector have increased by 1,4%, compared to the base year. Within this sector, in the 1986-2004 period, GHG emissions from Energy Industry, as the biggest sub-sector, have been reduced by 6.2%. In the most recent period, 2000-2004, steep growth (+12.2%) has been recorded which is due to the increased consumption of electrical energy. Undoubtedly, the highest increase of GHG emissions has been in traffic sector, by as much as 112.2%, due to an increase in road transportation, while emissions from other kinds of traffic have been slightly reduced. Increased emissions have been recorded also in general consumption (Other Sectors), namely by 19.5%. There was an appreciable reduction of GHG from industry in 1986-2000 period (-52%). In the time after 2000, a stabilisation of emission has been observed.

GHG emissions from industrial processes have since 1986 at first been falling sharply to reach their lowest value in 1993, but then started to rise again, yet remaining all the time below the base year, in 2004 by 11.9%. The most important GHG of this sector was carbon dioxide with 81.5% of emissions from this category, followed by PFCs with 10.6%, HFC with 5.8%, SF6 with 1.6%, and CH4 with 0.5%. N2O emissions in this sector have been negligible.

The main source is Mineral Production, of which the production of cement and lime alone contribute almost half of emissions in this sector.

The Solvent and other Product Use sector represents 0.2% of total emissions. Emissions in this sector keep diminishing, since the Republic of Slovenia has ceased to have any production in which GHG emissions could arise. Thus, GHG emissions have been reduced from 82kt CO2 eq. to 39 kt CO2 eq., only from N2O emissions.

In Agriculture as the second most important sector, emissions in 2004 amounted to 1972.6 Gg, which represented 9.9% of all emissions. Agriculture represents the main source of methane and N2O emissions, namely 51.5% of all methane emissions and 71.6% of all N2O emissions. Within the agricultural sector, N2O emissions account for 54.8% of emissions, CH4 emissions account for 55.2% of emissions.

GHG Emissions from agriculture show small oscillations for individual years, but the general trend is on the decrease. In 2004, emissions were 14.4% below the base year.

The most important sub-sector are emissions from agricultural soils, which contribute 36.6% of all emissions from agriculture, followed by emissions from enteric fermentation with 33.1%, the rest is contributed by emissions of methane and N2O from animal manure (30.3).

Emissions from waste

Methane emissions from the Waste sector are the second largest source of methane and represent 28.3% of all emissions of methane in Slovenia. The fraction of methane emissions in this sector amounts to 91.1%, the remaining part representing N2O. Solid-waste handling contributes 63.5% to the total emissions from this sector, municipal wastewater 23.4%, and industrial wastewater 13.1%.

Compared to the base year, emissions have risen by 15.6%, which is mostly due to emissions from SWDS that show an increase of 39.1%. The increase of emissions from this source is a consequence of the increase in the amount of disposed municipal waste and the application of FOD method for calculating emissions. Emissions from wastewaters are lower than in the base year by 10.6%, which is mostly due to the recovery of gas in wastewater treatment plants and the decrease of industrial production.

Note: Values between 1986 and 1990 were interpolated.

Figure 2.3: GHG Emissions in Slovenia by sector

## Description and interpretation of emission trends for indirect greenhouse gases and SO2

The highest contribution to the total emission of SO2 is that of thermal power plants and power cogeneration plants. In 1995, SO2 emission was reduced considerably, mostly because of the operation of the device for desulphurisation of flue gases on unit 4 of the Šoštanj Thermal Power Plant, as well as due to lower sulphur contents in liquid fuels, as set by the Directive on the Quality of Liquid Fuels with regard to the contents of sulphur, lead, and benzene. A further considerable reduction was noticeable after 2000, when the device for desulphurisation of flue gases on unit 5 of the Šoštanj Thermal Power Plant was put into operation. In the last year, emissions have risen slightly due to increased consumption of domestic coal for the production of electrical energy. Further considerable decrease of SO2 is expected in 2006 due to operation of wet flue gas desulphurisation in TE Trbovlje.

The biggest contribution to the overall NOx emissions is that of mobile sources (road traffic). After 1992, NOx emissions began to grow, mainly because of the ever-increasing traffic density; the growth has been extremely strong, despite of ever-greater number of vehicles with catalytic converters. After 1997, NOx emissions declined markedly, due to decreased consumption of fuels in the Road Traffic sector and increased fraction of vehicles with catalytic converters.

Road traffic accounts for nearly half of total NMVOC emissions, but overall emissions from traffic keep diminishing and have from 1986 to 2004 been reduced due to the modernization of the vehicle stock and a growing fraction of diesel-fuelled motor vehicles.

Table 2.3: Emissions of CO, NOx, SO2 and NMVOC in Slovenia (ton)

|  | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO | 80565 | 91427 | 99014 | 93208 | 87651 | 80776 | 82329 |
| NOx | 63432 | 66591 | 59673 | 59095 | 58222 | 55997 | 57498 |
| NMVOC | 43652 | 44337 | 50772 | 48830 | 47962 | 45871 | 46206 |
| SO2 | 196266 | 124730 | 98738 | 68312 | 71003 | 65612 | 54121 |

NMVOC and CO inventories have been in 2000 complemented with emissions of NMVOC and CO from the consumption of woody biomass in households. Since the recalculation for the previous period has not been performed yet, a direct comparison of emissions prior to and after year 2000 is not possible.

# ENERGY (CRF sector 1))

## Overview of sector

The energy sector is the most important sector of GHG emissions in the Republic of Slovenia, since it accounts for 80.9% of overall CO2 eq. emissions (w/o considering LULUCF). Emissions from this sector arise from fuel combustion, accounting for 97.3% emissions from the energy sector, and as fugitive emissions from fuels, accounting for 2.7% of emissions.

Figure 3.3: Emissions of GHG in Energy Sector by sectors in 2004

GHG emissions from fuel combustion have, compared to 2003, increased by 2.8% and were thus 1.7% higher than in the 1986 base year. The most important sub-sector is Energy Industries with 40.1% of emission of CO2 eq., followed by Traffic with 27%. For traffic, virtually all emissions are accounted for by road traffic and within that category particularly noticeable is the growth of the fraction of emissions from goods transport, since the goods transport in transit through Slovenia is since 2000, annually increasing by more than 10%.

In the Other sector consumption, which accounts for 17.9% of emissions, Commercial and Residential sectors prevail. The strongest reduction of emissions was recorded in Manufacturing Industries and Construction, which accounted for 15% of emissions. Emissions in this sector have, chiefly due to substitution of coal and liquid fuels, diminished by as much as 46.3% since 1986.

Emissions from the production of electrical energy and heat (IPCC 1.A.1.a) vary in accordance with the production of electrical energy. It has to be taken into consideration that in the Republic of Slovenia in 2004, 4.1 TWh (i.e. 26.8%) of electrical energy was produced in public hydroelectric power plants, 5.4 TWh (i.e. 35,7%) in the Krško Nuclear Power Plant, and 5.3 TWh in public thermal power plants (i.e. 35.0%), while the remaining 0.4 TWh (i.e. 2.4%) was produced by auto producers. The structure changes slightly from year to year, depending mostly on the changes in the hydrology of the Slovenian rivers.

Table 3.4: Emissions from Energy sector by sources in Gg CO2 eq.

Fugitive emissions in the Republic of Slovenia are of minor importance. The biggest fraction in the structure of these emissions of methane is coal mining in underground mines, yet emissions keep constantly diminishing due to ever-smaller excavation of coal. In inventories, CO2 emissions from flue gas desulphurisation (SO2 scrubbing) are entered as Fugitive Emissions from solid fuels, i.e. under 1.B.1.c Other, where it is indicated that these emissions are emissions from flue gas desulphurisation. Based on the in-country review and recommendations, Slovenia have developed country-specific emission factors for fugitive emissions from transport and distribution of natural gas.

CO2 emissions from biomass have been computed as well, but have not been included in the calculation; however, all other greenhouse gases (CH4, N2O) have been included in accordance with the methodology.

### Comparison of the sectoral approach with the reference approach

The total difference of CO2 emissions between the sectoral approach and the reference approach in 2004 amounted to less than 0.6% which is deemed satisfactory. The sectoral approach gives somewhat higher emissions for solid fuels, which are a consequence of coke used as reduction agent and incineration of semi-finished products in industrial stokers, which are, however, not treated in chapter 6.C Waste Incineration.

A comparison of energies between the two approaches shows a bigger difference, amounting to 2%. This is chiefly a consequence of deviations in the consumption of gaseous fuels (+9,4%), since the reference approach covers also non-energy consumption of natural gas for the production of methanol.

More detail description of comparison with data sources is available in Annex 4.

Table 3.5: Difference of the Reference Approach over the National approach: Energy Consumption.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| liquid | -1.15 | 3.48 | 1.91 | 0.07 | 0.80 | -0.36 | -0.18 | 0.41 |
| solid | 0.24 | 1.76 | 0.95 | 0.92 | 2.38 | 1.77 | 2.10 | 1.34 |
| gaseous | 4.46 | 4.52 | 11.11 | 15.85 | 16.55 | 14.34 | 16.08 | 9.42 |
| total | 0.29 | 2.93 | 2.92 | 2.80 | 3.76 | 2.59 | 3.26 | 2.03 |

Table 3.6: Difference of the Reference Approach over the National approach: CO2 emissions

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| liquid | -1.42 | 3.62 | 1.63 | -0.07 | 0.63 | -0.51 | -0.38 | -0.33 |
| solid | 0.17 | 1.72 | 0.85 | 0.96 | 2.45 | 1.90 | 2.21 | 0.83 |
| gaseous | -4.05 | -3.24 | -0.56 | 0.20 | 0.23 | -0.01 | -0.02 | -4.13 |
| total | -0.82 | 1.76 | 1.01 | 0.26 | 1.27 | 0.52 | 0.64 | -0.56 |

### International bunker fuels

The Republic of Slovenia had no international marine bunkers in 2004. A small quantity of fuel for small boats and yachts has been sold on four petrol stations at Adriatic coast (Izola pier, Lucija Pier, Marina Koper and Piran - Pier.

(http://www.omv-slovenija.si/omvib/OMVServisFrame_tri.asp ). As those stations are selling fuel to road vehicle also a division between road and marine traffic is not possible. For this reason we have reported all fuel in sub sector road traffic. We have one international port “Luka Koper” but all ships are refuelled in international waters by Italian ships (under Panama flags).

The entire consumption of jet kerosene was counted as aviation bunker fuel, since there are no commercial domestic flights in Slovenia. In spite of negligible quantities of emissions, the entire consumption of aviation gasoline for piston engine aircraft was counted as consumption and emission in Slovenia, since it was assumed that this was fuel for small jet aircraft, which fly between smaller regional airports in Slovenia.

Table 3.7: International Aviation Bunkers

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| fuel in TJ | 1364 | 1109 | 805 | 994 | 1123 | 1156 | 1105 | 842 |
| Gg CO2 eq. | 98 | 80 | 58 | 72 | 81 | 83 | 80 | 61 |

Fuel consumption for military purposes is negligible. Emissions from Army are included in road transportation (diesel), in Institutional sector (heating oil) and in civil aviation (for aviation gasoline). Slovenia doesn’t have any jet fighter airplanes and Navy and there is no possibility that emissions would be included in international aviation and therefore excluded from national totals.

### Feedstock and non-energy use of fuels

Natural gas

The biggest fraction of non-energy usage of fuels is the consumption of natural gas for the production of methanol, amounting to 136 Sm6 of natural gas in 2004. Smaller quantities of liquid fuels were consumed for manufacturing lubricating oils and greases, accounting for a consumption of 12.6 kt of refined petroleum products.

Source of activity:	from 1985 to 1995 Statistical office, LPI

from 1995-1996, Statistical office, DGP

from 1997-2003 LEG, Table Tg/6-3

from 2004 on SURS excel file

Figure 3.2:  Methanol production in Nafta Lendava.

In Slovenia low-pressure Lurgi technology is used. The methanol is produced from synthesis gas obtained from natural gas and steam in reactor.

CH4 + H2O → CO + 3 H2

This reaction, commonly called steam-methane reforming or SMR, is endothermic and the heat transfer limitations place limits on the size of the catalytic reactors used. The carbon monoxide and hydrogen then react on a second catalyst to produce methanol

The exothermal methanol reactor with three main reactions:

CO + 2H2 ® CH3OH

CO2 + 3H2 ® CH3OH + H2O

CO2 + H2 ® CO + H2O

is operated at high pressure and unconverted gas is recycled.

Stored CO2 has been calculated on the basis of the formula from IPCC guidelines, p. I.28., I. 23, I.13. We have assumed that all methane used for methanol production is stored in the

product or in CO in emitted gas.

Table 3.5: Natural gas

|  |  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Natural Gas | 1000 m3 | 67666 | 69524 | 91577 | 136740 | 146069 | 125528 | 154302 | 136356 |
| Fraction of C stored |  | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| Carbon EF | t C/TJ | 15,075 | 15,075 | 15,075 | 15,075 | 15,075 | 15,075 | 15,075 | 15,075 |
| Stored CO2 | Gg | 125,3 | 131,0 | 172,6 | 257,6 | 275,2 | 236,5 | 290,7 | 256,9 |

Table 3.6: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 5% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 5% |

Source: Author’s estimate

Oil and Lubricants

Source of activity:	from 1985 to 1995 Statistical office, LPI

from 1995-1996, Statistical office, DGP

from 1997-2003 Statistical office information

from 2004 on SURS excel file

Emission factor:

Stored CO2 has been calculated on the basis of the formula from IPCC guidelines,1996, p. I.28., I. 23, I.13.

Table 3.7: Oil and Lubricants

|  |  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oil and Lubricants | t | 6592 | 7201 | 4178 | 9805 | 9701 | 9363 | 12636 | 28623 |
| NCV | TJ/kt | 40.2 | 40.2 | 40.2 | 40.2 | 40.2 | 40.2 | 40.2 | 40.2 |
| Fraction of C stored |  | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 | 0,5 |
| Carbon EF | t C/TJ | 20 | 20 | 20 | 20 | 20 | 20 | 20 | 20 |
| Stored CO2 | Gg | 9,7 | 10,6 | 6,2 | 14,5 | 14,3 | 13,8 | 18,6 | 54,7 |

Table 3.8: Estimate of Uncertainty of Utilized Data (in %)

| Estimate of Uncertainty of Input Data | 20% |
| --- | --- |
| Estimate of Uncertainty of Calorific Values | 5% |
| Estimate of Uncertainty of Emission Factors | 5% |

Source: Author’s estimate

Coke and petroleum coke

Coke and petroleum coke, used in industry as reduction agent or feedstock have been subtracted  from energy sector and emissions from these fuels are presented in industrial processes sector.

Before 1997, amount of coke, used for production of  iron and steel, ferroalloys and carbide was reported as fuel consumption in relevant sectors. After 1997 this fuel started to be collected separately, but it takes a while that all non-energy used fuel has been reported correctly. From 2000 in statistical data energy and non-energy use of fuel in industry are presented separately.

To avoid double counting, we have subtracted all coke used in  iron and steel, ferroalloys and carbide production from energy sector except coke in iron production in the base year 1986. In that time pig iron was still produced and the disaggregation into the consumption of fuel as an additive and the consumption of fuel as an energy product was impossible. For consumption of coke, the decision was taken to attribute all coke, which is consumed in the production of iron and steel in this year, to the energy sector as fuel consumption and no emissions from coke used in iron and steel are presented in industrial processes.

Amount of petrol coke used for anode production in aluminium production is also account as process emissions. As this fuel has never been reported to SORS under category fuel consumption there is no need for subtraction from energy sector.

### CO2 capture from flue gases and subsequent CO2 storage

There are no plants for recovery and storage of CO2 in Slovenia.

## Fuel combustion (CRF 1.A)

### Country-specific issues

An interesting feature of inventories of greenhouse gases for Slovenia is the fact that the chosen 1986 base year goes back to the time when Slovenia was still a part of Yugoslavia. This fact notwithstanding, Slovenia has already at that time had its own electrical energy statistics and annual reports, which have been published annually without any interruptions ever since 1955. Due to the stable functioning system of data collection and economic conditions (no commercially sensitive data) it is correct to say that the energy statistics in particular was exceptionally good and centralized, and the data reliable and trustworthy.

The number of key reporting units prior to 1992 was exceptionally small, since only one enterprise imported natural gas, two enterprises refined petroleum products, while coal import was transacted within the framework of three thermal power plants.

From 1986 to 2004, the terminology in publications has undergone some changes, since after 1991 “Sale to other republics” became „Export”, while „Purchase from other republics” became „Import”. The terminology related to coal remains somewhat special. In national publications, “Lignite” is used only for coal excavated in the pit of Velenje. The coal from other pits is entered as „brown coal” in spite of virtually the same net calorific value (NCV). This brown coal is combined with imported coals that have a considerably higher net calorific value and, in terms of methodology, truly belong to brown coals.

To avoid erroneous interpretations in international comparisons of inventories, we have decided to combine the entire production of domestic coal in the CRF table 1.A(b) – Sectoral background data for Energy (Reference Approach) on the basis of net calorific value under „Lignite”.

After 1996, the Statistical Office of the Republic of Slovenia changed the Unified classification of activities for the Standard classification of activities, and that has caused a slight alteration of emissions within the sector Manufacturing Industries and Construction.

### Country-specific EFs

Lignite

With regard to the need to upgrade GHG emissions inventories, in 2004, national CO2 emission factors for domestic coal were developed. CO2 emission factors were obtained on the basis of determined carbon contents in the fuel. Data on carbon contents in fuel for the entire period 1986-2004 are available only for coal from the biggest pit in Slovenia, the Šoštanj Lignite Pit. The carbon contents of lignite were verified by supplementary chemical analyses of coal samples from this pit in an accredited laboratory in accordance with EN ISO 17025. (further information: A Review of the Chemical Analysis of Coal from the Pit of Šoštanj).

Table 3.9:  National CO2 emission factor for domestic lignite from Velenje pit.

|  | 1986 | 1990 | 1991 | 1992 | 1993 | 1994 | 1995 | 1996 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| t CO2/TJ | 107.76 | 104.83 | 99.76 | 99.07 | 99.44 | 94.74 | 98.97 | 99.81 |
|  | 1997 | 1998 | 1999 | 2000 | 2001 | 2002 | 2003 | 2004 |
| t CO2/TJ | 99.78 | 97.55 | 98.08 | 100.51 | 104.58 | 103.33 | 102.80 | 103.74 |

Natural gas

We use slightly modified EF also for natural gas. CO2 emission factors were obtained on the basis of determined carbon contents in the fuel. A detail results are in study from year 1998, which is available only in Slovene language (Določitev emisijskega faktorja CO2 pri energetski izrabi zemeljskega plina). Because EF don’t vary a lot between years we have used value 55 t CO2/TJ for all years. The value is already multiplied with default  oxidation factor for gaseous fuels.

### Oxidation factor

For inventory purpose we have used default oxidation factor from IPCC Reference Manual, 1996.  They are as follows:

Solid fuels:		0.980

Liquid fuels:	 	0.990

Gaseous fuels: 	0.995

All values of CO2 EF in this report are already multiply by default oxidation factors with no exception. For this reason oxidation factors would not be mention in further chapters.

### Energy Industries

This chapter presents the consumption of fuels and emissions of greenhouse gases in:

- Public Electricity and Heat Production (CRF 1A1a)

- Petroleum Refining (CRF 1A1b)

- Manufacture of solid fuels and Other energy Industries (CRF 1A1c)

Figure 3.4: GWP of GHG emissions from Energy Industries

Public electricity and heat production is the most important category in this sub-sector with 97,5% share in the base year and almost 100% share in 2004. Other two categories have consisted mainly from fuel consumption in one refinery and in fuel consumption for coal mining activities.

Table 3.10: GWP of GHG emissions from Energy Industries

#### Public Electricity and Heat Production (CRF 1A1a)

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 32.40 |  | 1 |
| 2004 | Level, Trend | CO2 | 31.50 | 2.43 | 1 |

In this sector, there are three big point sources in the Republic of Slovenia, which represent the backbone of the production of electrical energy from thermal power plants. All three plants use coal for the production of electrical energy. Two of these thermal power plants (the Šoštanj Thermal Power Plant - TEŠ and the Trbovlje Thermal Power Plant - TET) are located beside coal pits. Since 2003, CHP Ljubljana – TE-TOL uses exclusively imported coal with high net calorific value and low sulphur contents for the production of electrical energy and heat.

Table 3.11: Public electricity and Combined Heat and Power Plants in Slovenia

| Power plant | Location | Unit | Year | Power (MW) | Main fuel type |
| --- | --- | --- | --- | --- | --- |
| TEŠ | Šoštanj | A/1 (1) | 1956 | 105.0 | Lignite |
| TEŠ | Šoštanj | A/2 (2) | 1956 | 105.0 | Lignite |
| TEŠ | Šoštanj | A/3 (3) | 1960 | 125.0 | Lignite |
|  | Šoštanj | A/3 (4) | 1971 | 125.0 | Lignite |
| TEŠ | Šoštanj | B/4 (5) | 1977 | 740.0 | Lignite |
| TEŠ | Šoštanj | C/5 (6) | 1977 | 920.0 | Lignite |
| TE-TOL | Ljubljana | D/1 (1) | 1967 | 137.0 | Imported coal |
| TE-TOL | Ljubljana | D/2 (2) | 1967 | 137.0 | Imported coal |
| TE-TOL | Ljubljana | D/3 (3) | 1984 | 207.0 | Imported coal |
| TET | Trbovlje | F/4 (4) | 1968 | 350.0 | Domestic coal |

Besides these three thermal power plants we have also one small plant Brestanica – TEB which use natural gas and operate mainly as back up plant when more electricity is needed or when any other plant is on refit.

Methodology

To estimate emissions from Public Electricity and Heat Production, the following methodology has been adopted.

Quantity of Fuel Combusted x Net Calorific value x EF per energy of Fuel x (oxidation factor) = Emissions

Activity data

The main source of data for all energy industries in the Republic of Slovenia for the period 1986-2003 is LEG – Annual Energy Statistics of the Energy Sector of the Republic of Slovenia.

As LEG is not published early enough to enable us to calculate GHG inventory on time in 2005 we have for the firs time received data for 2004 directly from Statistical Office of the Republic of Slovenia in electronic format before they are published. This excel sheets are going to be our source of data for all fuel consumption in the future.

For four power plants we have taken data from official reports which are yearly prepared by Electroinstitut Milan Vidmar.

To make it more clear, the sub-sector Public Electricity and Heat Plants has in national inventories been disaggregated into:

- Public power plants 		(TE Šoštanj, TE Trbovlje, TE Brestanica)

- Public CHP			(TE-TO Ljubljana)

- Public heat plants 		(Heat Plants listed in LEG Dt/1)

a) Public power plant

For the period 1986-2003, data on the consumption of fuels for individual public power plants are collected in LEG Table EL/9-0 or Table EL/7-0, respectively. For 2004 data have been taken from official reports which are yearly prepared for individual plant by Electroinstitut Milan Vidmar.

In LEG-94, an error has occurred in addition of categories with regard to the consumption of natural gas: the consumption in Other Industry section amounted to 55597 and not to 98459 thousand Nm3 (total consumption amounted to 710567 thousand Nm3 – established on the basis of addition of consumption in industry).

For the period 1995-2003, the consumption of fuels in the Public Power Plants group is obtained by subtracting the consumption of fuels in the Ljubljana Power Cogeneration Plant for the generation of electric energy from the item Public Power Plants from LEG, and by adding the consumption of fuels in the TE-Šoštanj for the production of heat (LEG, Table EL/7-0). The consumption of fuels in the Ljubljana Power Cogeneration Plant for the production of heat has already been taken into account in the category Power Cogeneration Plants and Public Heat Plants (LEG: Table ZB/3). A new category was introduced in 1995, Transformation (instead of Electric Utilities) that comprises the consumption of public power plants, industrial power plants as well as power cogeneration plants and public heat plants.

b) Public CHP (combined heat and power generation plants)

The Ljubljana Power Cogeneration Plant consumes approximately 45 % of fuel for the production of heat; consequently, it is classified in the group of combined heat and power generation plants. For the period 1986-2003, data on the consumption of fuels for TE-TOL are collected in LEG Table EL/7-0. For 2004 data have been taken from official reports which are yearly prepared for individual plant by Electroinstitut Milan Vidmar.

c) Power cogeneration plants and public heat plants

The category comprises all power cogeneration plants and public heat plants. For 1986-2003 data have been taken from LEG Table Dt/1. In 1986, only data for JP Energetika, Ljubljana exist, for 1996 for instance data already included fuel consumption in 15 public heat plants.

For 2004 individual data for 6 Power cogeneration plants and 28 public heat plants were obtained for the first time in electronic format. (E2LP_04.xls and E3L_04.xls). Data in this format are going to be available also in the future.

Data on the consumption of fuels in power cogeneration plants and public heat plants only started to be published on a regular basis after 1987, therefore for 1986 only data on the consumption of natural gas in 1986 have been taken into account (all consumption in the Electricity Generating Industries sector is attributed to the consumption in power cogeneration plants), for other fuels, data for 1987 have been taken (the first successive year when they were available). With regard to small quantities of consumed fuels, the estimated uncertainty is small.

Net calorific values

Net calorific values have been taken from SORS, The values for solid fuel varies from year to year but for  the liquid and gaseous fuel almost the same values have been used  for the entire period as these types of fuel don’t change a lot from year to year.

Table 3.12:  NCVs for the fuel used in energy industry.

| Year | Lignite (Velenje) | Sub-bituminous Coal - domestic | Sub-bituminous Coal - imported | Gas Oil | Residual Fuel Oil | LPG | Natural Gas | Wood and Other Biomass |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/Mm3 | TJ/kt |
| 1986 | 9.390 | 11.880 |  | 41.820 | 39.740 | 46.000 | 33.500 | 12.170 |
| 1990 | 9.810 | 12.760 |  | 41.870 | 39.800 | 46.000 | 34.100 | 12.170 |
| 1991 | 9.980 | 12.879 |  | 41.880 | 39.800 | 46.000 | 34.100 | 12.170 |
| 1992 | 10.260 | 12.589 |  | 41.900 | 39.900 | 46.000 | 34.100 | 12.170 |
| 1993 | 10.070 | 13.351 |  | 41.900 | 39.800 | 46.000 | 34.100 | 12.170 |
| 1994 | 9.960 | 12.666 |  | 41.900 | 39.860 | 46.000 | 34.100 | 12.170 |
| 1995 | 10.220 | 10.000 | 15.546 | 41.900 | 40.000 | 46.000 | 34.100 | 12.170 |
| 1996 | 9.690 | 11.300 | 16.107 | 41.900 | 40.000 | 46.000 | 34.100 | 12.170 |
| 1997 | 9.610 | 11.300 | 16.422 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |
| 1998 | 10.010 | 11.230 | 16.924 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |
| 1999 | 9.690 | 11.110 | 16.649 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |
| 2000 | 10.170 | 11.230 | 16.308 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |
| 2001 | 10.660 | 10.660 | 17.416 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |
| 2002 | 10.350 | 11.220 | 17.927 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |
| 2003 | 10.138 | 11.560 | 18.057 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |
| 2004 | 10.301 | 11.680 | 18.676 | 41.900 | 40.000 | 46.050 | 34.080 | 12.170 |

Emission factors

We have used country specific CO2 EFs for domestic lignite and natural gas. More detailed description is in chapter 3.2.2..

Emission factors for all other fuels have been taken from IPCC Reference Manual. 1996.

Table 3.13:  Emission factors used for all period

|  | Unit | Lignite (Velenje) | Sub-bituminous Coal | Gas Oil | Residual Fuel Oil | LPG | Natural Gas | Wood and Other Biomass |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 EF | t/TJ | Table 3.9 | 99.2 | 73.3 | 76.6 | 62.4 | 55.0 | 107.4 |
| CH4 EF | t/TJ | 0.001 | 0.001 | 0.003 | 0.003 | 0.002 | 0.001 | 0.03 |
| N2O EF | t/TJ | 0.0014 | 0.0014 | 0.0006 | 0.0006 | 0.0006 | 0.0001 | 0.004 |

Recalculations

No recalculations have been performed in this category.

Future improvements

- Emission factors

Beginning in 2005. all thermal power plants in the Republic of Slovenia will carry out regular coal sampling and determine the carbon contents. which in accordance with the Monitoring guidelines for monitoring and reporting of greenhouse gas emissions pursuant to Directive 2003/87/EC of European Parliament and of the Council is necessary for CO2 emission trading in the period 2005-2007 on the territory of the European Union.

- Activity data

At a monthly level. we will verify the consumption of fuel in 1986 in correlation with the production of electrical energy for the Šoštanj Thermal Power Plant. because we have received a notice from the Šoštanj Thermal Power Plant that there is a possibility of an error in the reported consumption of fuels for LEG 1986. which according to the archive data of the Šoštanj Thermal Power Plant should be higher by 118,000 tons.

#### Petroleum Refining (CRF 1A1b)

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | no | CO2 | 0.31 |  | 33 |
| 2004 | Trend | CO2 | 0.00 | 0.82 | 64 |

This chapter presents the entire consumption of fuels in oil industry; however. the main representative of this sector is only one company – the Nafta Lendava Refinery. According to statistical methodology. in the period 1986-1996 this sector also included quantities of fuels that were consumed for the production of electric energy in this sector.

Methodology

To estimate emissions from Public Electricity and Heat Production, the following methodology has been adopted.

Quantity of Fuel Combusted x Net Calorific value x EF per energy of Fuel x (oxidation factor) = Emissions

Activity data

Data on the consumption of fuels in this sector for the period 1986-2003 have been collected in LEG – Annual Energy Statistics of the Energy Sector of the Republic of Slovenia.

Data for 2004 have been from SORS and are in file E_PE-M 2004.xls.

Consumptions for individual energy products, collected in tables LEG as follows:

For the period 1986-1996 	under „Oil Industry”

From 1997 onwards 	under „DF–Production of coke, refined petroleum products and nuclear fuel”

-  For the consumption of liquid fuels		Table Tg/3 or Table Pg/6 for LPG

- For the consumption of solid fuels 		Table Pr/6

- For the consumption of gaseous fuels 	Table Pg/6

For the period 1992 to 1994, LEG reported substantial consumption of residual fuel oil and gas oil in the category Internal Consumption and Losses (LEG Table Zb/1). These quantities were supposed to have been consumed in the Nafta Lendava Refinery. consequently. they were added to the consumption in the oil industry. For the years 1986, 1990, 1991, 1995, and 1996 in this category (Internal Consumption and Losses), the consumption of residual fuel oil and gas oil has not been reported. The consumption of residual fuel oil in the oil industry amounted to:

For 1992: 6180+ 15100=21280 t

For 1993: 5713+ 8000=13713 t

For 1994: 3125+ 8200=11325 t

In 1995 and 1996. the values of consumption of natural gas from tables Zb/1 and Zb/3 in LEG show a discrepancy. The difference is the consumption of natural gas in Nafta Lendava. Consequently, these quantities (which in the reference approach have been subtracted within the framework of the chapter Stored Carbon) have not been taken into account in sectoral approach (theoretically it would be correct to first add them – because they were actually consumed in Slovenia - and subsequently subtract them – because they have not been used as fuel but as feedstock for the production of methanol).

After 1996, data on the consumption in this sector have been included in the industrial sector DF – Production of coke. refined petroleum products. and nuclear fuel. With regard to the fact that in the Republic of Slovenia there is neither any production of coke nor nuclear fuel, data for period 1997-2004 are comparable to the period 1986-1996.

Net calorific values

Net calorific values have been taken from SORS, and are presented in the following table.

Table 3.14:  NCVs for the fuel used in energy industry.

| Year | Gas Oil | Residual Fuel Oil | Diesel | LPG | Natural Gas |
| --- | --- | --- | --- | --- | --- |
|  | TJ/kt | TJ/kt | TJ/kt | TJ/kt | MJ/Sm3 |
| 1986 | 41.82 | 39.74 | 42.70 | 46.00 | 33.50 |
| 1990 | 41.87 | 39.80 | 42.70 | 46.00 | 34.10 |
| 1991 | 41.88 | 39.80 | 42.70 | 46.00 | 34.10 |
| 1992 | 41.90 | 39.90 | 42.70 | 46.00 | 34.10 |
| 1993 | 41.90 | 39.80 | 42.70 | 46.00 | 34.10 |
| 1994 | 41.90 | 39.86 | 42.70 | 46.00 | 34.10 |
| 1995 | 41.90 | 40.00 | 42.70 | 46.00 | 34.10 |
| 1996 | 41.90 | 40.00 | 42.70 | 46.00 | 34.10 |
| 1997 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 1998 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 1999 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2000 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2001 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2002 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2003 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2004 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |

Emission factors

We have used country specific CO2 EF for natural gas. More detailed description is in chapter 3.2.2..Emission factors for all other fuels have been taken from IPCC Reference Manual. 1996.

Table 3.15:  Emission factors used for all period

| Year | Unit | Gas Oil | Residual Fuel Oil | Diesel | LPG | Natural Gas |
| --- | --- | --- | --- | --- | --- | --- |
| CO2 EF | t/TJ | 73.3 | 76.6 | 73.7 | 62.4 | 55.0 |
| CH4 EF | t/TJ | 0.002 | 0.002 | 0.002 | 0.002 | 0.005 |
| N2O EF | t/TJ | 0.0006 | 0.0006 | 0.0006 | 0.0006 | 0.0001 |

Recalculations

No recalculations have been performed in this category.

Future improvements

No improvements is planned for this category.

#### Manufacture of solid fuels and Other energy Industries (CRF 1A1c)

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 0.52 |  | 25 |
| 2004 | Trend | CO2 | 0.02 | 1.35 | 56 |

This sector covers the consumption of fuels reported in LEG under “Coal-mining” or after 1997 under CA – Production of energy commodities. only

Methodology

To estimate emissions from Manufacturing of solid fuels and Other energy Industries. the following methodology has been adopted.

Quantity of Fuel Combusted x Net Calorific value x EF per energy of Fuel x (oxidation factor) = Emissions

Activity data

Consumptions according to individual energy products are collected in LEG tables as follows:

For the period 1986-1996 under „Coal-mining”

From 1997 onwards under „CA–Production of energy commodities”

- For the consumption of liquid fuels		Table Tg/3 or Table Pg/6 for LPG

- For the consumption of solid fuels 		Table Pr/6

- For the consumption gaseous fuels	Table Pg/6

Data for 2004 have been from SORS and are in file E_PE-M 2004.xls.

Net calorific values

Net calorific values have been taken from SORS, and are presented in the following table.

Table 3.16:  NCVs for the fuel used in energy industry.

| Year | Sub-bituminous Coal - domestic | Sub-bituminous Coal - imported | Gas Oil | Residual Fuel Oil | Diesel | LPG | Natural Gas |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/Mm3 |
| 1986 | 11.880 |  | 41.82 | 39.74 | 42.70 | 46.00 | 33.50 |
| 1990 | 12.760 |  | 41.87 | 39.80 | 42.70 | 46.00 | 34.10 |
| 1991 | 12.879 |  | 41.88 | 39.80 | 42.70 | 46.00 | 34.10 |
| 1992 | 12.589 |  | 41.90 | 39.90 | 42.70 | 46.00 | 34.10 |
| 1993 | 13.351 |  | 41.90 | 39.80 | 42.70 | 46.00 | 34.10 |
| 1994 | 12.666 |  | 41.90 | 39.86 | 42.70 | 46.00 | 34.10 |
| 1995 |  | 17.404 | 41.90 | 40.00 | 42.70 | 46.00 | 34.10 |
| 1996 |  | 16.353 | 41.90 | 40.00 | 42.70 | 46.00 | 34.10 |
| 1997 |  | 17.712 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 1998 |  | 20.664 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 1999 |  | 20.806 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2000 |  | 20.782 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2001 |  | 20.947 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2002 |  |  | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2003 |  |  | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |
| 2004 |  |  | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 |

Emission factors

We have used country specific CO2 EF for natural gas. More detailed description is in chapter 3.2.2..Emission factors for all other fuels have been taken from IPCC Reference Manual. 1996.

Table 3.17:  Emission factors used for all period

| Year | Unit | Sub-bituminous Coal | Gas Oil | Residual Fuel Oil | Diesel | LPG | Natural Gas |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 EF | t/TJ | 99.2 | 73.3 | 76.6 | 73.7 | 62.4 | 55.0 |
| CH4 EF | t/TJ | 0.010 | 0.002 | 0.002 | 0.002 | 0.002 | 0.005 |
| N2O EF | t/TJ | 0.0014 | 0.0006 | 0.0006 | 0.0006 | 0.0006 | 0.0001 |

Recalculations

No recalculations were performed in this sector.

Future improvements

No improvement is planned for this sector

### Manufacturing Industries and Construction (IPCC: I A 2)

| Iron and Steel | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 5.66 |  | 4 |
| 2004 | Level, Trend | CO2 | 1.49 | 11.22 | 12 |

| Non-ferrous Metals | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 2.18 |  | 9 |
| 2004 | Trend | CO2 | 0.29 | 5.10 | 32 |

| Chemicals | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 0.49 |  | 26 |
| 2004 | Level, Trend | CO2 | 1.00 | 1.38 | 17 |

| Pulp, Paper and Print | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 3.22 |  | 6 |
| 2004 | Level, Trend | CO2 | 2.51 | 1.90 | 6 |

| Food, Beverages and Tobacco | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 1.23 |  | 19 |
| 2004 | Level, Trend | CO2 | 0.89 | 0.91 | 18 |

| Other | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 8.80 |  | 3 |
| 2004 | Level, Trend | CO2 | 5.53 | 8.81 | 4 |

This chapter presents the consumption of fuels and emissions of greenhouse gases in five specific types of industry, all other are hidden under other industry where also fuel for construction industry is included. For this reason, that in “other” a big number of enterprises are included this is the most important for GHG emissions  The most important industry under other is non-metal production which including cement and lime production as two of the most important fuel consumers. There was an appreciable reduction of GHG from industry in 1986-1997 In the last years a stabilisation of emission has been observed.

Table 3.18: GWP of GHG emissions from Manufacturing Industries and Construction

Figure 3.5: GWP of GHG emissions from Manufacturing Industries and Construction

Methodology

Emissions from combustion in manufacturing industries and construction were estimated using the Tier 1 methodology described in IPCC 96. The following basic formula was used:

Quantity of Fuel Combusted x Net Calorific value x EF per energy of Fuel x (oxidation factor) = Emissions

The total emission for this sub/sector is the sum of different industrial activities, using diverse fuels and combustion technologies.

Activity data

PERIOD 1986-1996

The consumption in both categories has to be disaggregated in accordance with the classification of activities applied in IPCC guidelines. The classification applied in LEG has been taken as the basis.

Table 3.19: Conversion Table between national energy statistics (LEG) and CRF

| ISIC – CATEGORY | LEG Classification (1986-1996) |
| --- | --- |
| Iron and Steel | Iron and Steel Production |
| Non-Ferrous Metals | Non-Ferrous Metals |
| Chemicals | Chemical Industry |
| Pulp. Paper and Print | Pulp and Paper Industry Print Industry |
| Food Processing. Beverages and Tobacco | Food Processing Industry Tobacco Industry |
| Other | Other |

In this report, the group “Other” is a sum of activity data in the following categories:

Table 3.20: Conversion Table between national energy statistics (LEG) and CRF

| ISIC- CATEGORY | LEG Classification (1986-1996) |
| --- | --- |
| Other | Metal Industry |
|  | Non-metal industry |
|  | Shipbuilding |
|  | Electrical Industry |
|  | Construction |
|  | Timber Industry |
|  | Textile Industry |
|  | Leather Industry |
|  | Rubber Industry |
|  | Recycling |
|  | Other Industry |

PERIOD 1997-2003

In 1997, LEG began to publish data according to the Standard Classification of Activities, which in some categories differs from the classification, which had been used until 1996. Most activities are defined in a similar manner, but for certain activities, this is not possible. The next table shows the distribution of activities in accordance with the IPCC classification.

Table 3.21: Conversion Table between national energy statistics (LEG) and CRF

| ISIC – CATEGORY | LEG Classification – SCA category |
| --- | --- |
| Iron and Steel | DJ - Production of metals and metal products |
| Non-Ferrous Metals |  |
| Chemicals | DG - Production of chemicals |
| Pulp. Paper and Print | DE - Production of fibres, pulp, paper, and cardboard |
| Food Processing. Beverages and Tobacco | DA – Production of food, beverages, and tobacco products |
| Other | Other |

In this section, the group “Other” is a sum of activity data in the following categories:

Table 3.22: Conversion Table between national energy statistics (LEG) and CRF

| ISIC- CATEGORY | LEG Classification |
| --- | --- |
| Other | DI - Production of non-metal mineral products |
|  | DB - Production of textiles |
|  | DC - Production of leather and leather goods |
|  | DD – Wood-processing and woodworking |
|  | DH - Production of rubber products |
|  | DK - Production of machines and devices |
|  | DL - Production of electrical and optical equipment |
|  | DM – Production of vehicles and vessels |
|  | DN - Production of furniture. not included elsewhere |
|  | F - Construction |

For consumption in individual industrial sectors there are detailed (disaggregated) data, the values of which in Slovenia are strongly dependant on the mode of reporting and features of individual industrial sectors characterized by high concentration (values depending on the consumption in one or two factories). Data from basic sources hint at some relatively big changes in the consumption of fuels in some sectors. Therefore, it is necessary to direct the attention to some of the most important changes, particularly with regard to the consumption of natural gas:

Values for the consumption of natural gas in the pulp, paper and print industry – other consumption were relatively low for 1990. The consumption of natural gas in that sector for the period 1991 to 1996 ranged from 150 to 175 million m3, but in 1990 amounted to only 58 million m3. The difference is probably hidden in the sector “Other” (in 1990 the reported consumption in that sector amounted to 102 million m3) (LEG 90-96, Tables ZB-3).

The consumption of natural gas in power cogeneration plants in 1990, 1992, 1993, 1994 is presented in LEG (Zb/1) under the balance item Other Industry, in 1986 under the category Electric utilities – Electricity Generating Industries, in 1991 it has not been shown, in 1995 and in 1996 under the sector Power Cogeneration Plants. Consequently, the consumption of natural gas in power cogeneration plants must be subtracted from this category – to avoid double counting of emissions.

In LEG-94 an error has occurred in addition of categories with regard to the consumption of natural gas: the consumption in Other Industry sector amounted to 55597 and not 98459 thousand Nm3.

Consumption of natural gas in the Metal Industry sector – Other Consumption amounted to 39 million m3 in 1993, rose to 74 million m3 in 1994 only to fell to 40 million m3 in 1995 (LEG93-95. Tables ZB-3)

Consumption of natural gas in the Oil Industry sector – Consumption in Industrial Power Plants in 1993 amounted to 26.5 million m3. in 1994 to 16.4 million m3 and in 1995 to 32.8 million m3 (calculations of the Institute of Energy Industries. 1998. p. 3)

Consumption of natural gas in the Non-Ferrous Metals sector – Other Consumption in 1994 amounted to 101.7 million m3. in 1995 to 55.6 million m3 and in 1996 to 70.5 million m3 (LEG93-95. Tables ZB-3).

In the chapter on industry (sectoral approach), it is necessary to subtract the natural gas which has been consumed in the production of methanol and which has also been subtracted in the reference approach (in the chapter on stored carbon) from the quantities reported in LEG for the period 1986 to 1994. These quantities have been presented in the category Internal Consumption and Losses (Table Zb/1 in LEG), partly in category Chemical Industry, partly (only for 1986) in the Other Industry sector. For the two years (1992 and 1993) when the losses were higher than the consumption for the production of methanol. these quantities have been added to the consumption in the Other Industry sector.

For 1995 and 1996, this calculation is not necessary, because the summary table Zb/1 LEG takes into account the quantities of natural gas (data used for the calculation in the reference approach) that have been consumed as a feedstock in Nafta Lendava, but are not specified in Table Zb/3 (data used in the calculation for sectoral approach).

YEAR 2004

For the year 2004 we have obtained very detailed data about fuel consumption in industry in electronic format (E_PE_M 2004.xls). The list of fuel collected including 24 different fuels. The non-energy and energy use of fuels are reported separately. Data about fuel consumption and NCV are reported on the lowest level of disaggregation possible.

For this reason from 2004 on a fuel consumption in iron and steel industry and in non-ferrous metals industry can be separated according to the rules presented in the following table.

Table 3.23: Table for disaggregation of fuel in DJ sector (manufacture of basic metals and fabricated metal products)

| SCA category | ISIC category | Description |
| --- | --- | --- |
| DJ 27.1 | Iron and Steel | Manufacture of basic iron and steel and of ferrous alloys |
| DJ 27.2 | Iron and Steel | Manufacture of tubes |
| DJ 27.3 | Iron and Steel | Other first processing of iron and steel |
| DJ 27.4 | Non-ferrous Metal | Manufacture of basic precious and non-ferrous metals |
| DJ 27.510 | Iron and Steel | Casting of iron |
| DJ 27.520 | Iron and Steel | Casting of steel |
| DJ 27.530 | Non-ferrous Metal | Casting of light metal |
| DJ 27.540 | Non-ferrous Metal | Casting of other non-ferrous metal |
| DJ 28 | Other industry | Manufacture of fabricated metal products, except machinery and equipment |

Inclusion of auto producers into Manufacturing Industries sector

In accordance with IPCC Reference manual, the item Industry reports the consumption of fuels in the group industrial power plants (auto producers – enterprises that generate electric energy for internal consumption and/or heat for sale) as well as other consumption in industry (except in production processes) .

In the 1986 -1996 period, the consumption of fuels by auto producers in LEG was recorded under Electric utilities – Industry, and in the period 1997- 2003 under Conversion – Auto producers.

Period 1986-2000

Because there are no published of data on auto producers at the level of industrial branches for the period 1986-2000, on the basis of which it would be possible to assign the consumption of fuel to each individual industrial branch, for each kind of fuel a different (most appropriate) approach was used.

- Lignite

Total consumption is attributed to pulp and paper industry. The paper mill in Krško uses lignite in its power cogeneration plant. In the documents of the Statistical Office of the Republic of Slovenia, the total consumption is attributed to the consumption in thermal power plants, while in LEG one half of the consumption is attributed to the consumption in industry, the other half to industrial thermal power plants. In this report, half is reported as consumption in pulp and paper industry (heat), half as consumption in industrial power plants in pulp and paper industry. Consumption of lignite in other sectors has not been reported.

- Brown Coal

Consumption of brown coal in industrial power plants in the monitored period was reported only in 1986. Since quantities are quite small (1272 t), consumption is reported in the sector “Other”.

- Residual Fuel Oil

Consumption of residual fuel oil in industrial power plants in the monitored period was low (from 0 to 10176 t). Since quantities are quite small. consumption is reported in sector “Other”.

- Gas Oil and Natural Gas

The majority of industrial thermal power plants use gas oil or natural gas. Total quantities of consumed gas oil and natural gas are disaggregated according to the produced quantities of electric energy in those power plants according to the following procedure:

- Determine which power plants use gas oil or natural gas

- Add up the quantities of electric energy produced in those power plants

- Allocate fractions of consumed residual fuel oil or natural gas, respectively. according to produced quantities of energy in individual power plants

- Define the sector to which individual power plants belong.

Period 2000-2004

Recently, we have commenced to treat auto producers individually, since the Statistical Office of the Republic of Slovenia, which prepares data for LEG, has completed its database. Now, aggregated data on the consumption of fuels by auto producers at the level of industrial branches are available, where the sums of individual fuels correspond to the consumption of auto producers from LEG.

Net calorific values

Net calorific values have been taken from SORS, The values for solid fuel varies from year to year but for  the liquid and gaseous fuel almost the same values have been used  for the entire period as these types of fuel don’t change a lot from year to year.

Table 3.24:  NCVs for the fuel used in manufacturing industry and construction.

| Year | Lignite (Velenje) | Sub-bituminous Coal - domestic | Sub-bituminous Coal - imported | Other Bituminous Coal | Anthracite | Coke | Petroleum coke |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt |
| 1986 | 9.390 | 11.880 |  | 27.57 | 29.25 | 29.30 | 31.000 |
| 1990 | 9.810 | 12.760 |  | 27.57 | 29.25 | 29.30 | 31.000 |
| 1991 | 9.980 | 12.879 |  | 25.00 | 29.25 | 29.30 | 31.000 |
| 1992 | 10.260 | 12.589 |  | 25.00 | 29.25 | 29.30 | 31.000 |
| 1993 | 10.070 | 13.351 |  | 25.00 | 29.25 | 29.30 | 31.000 |
| 1994 | 9.960 | 12.666 |  | 25.00 | 29.25 | 29.30 | 31.000 |
| 1995 | 10.220 |  | 17.404 | 25.00 | 29.31 | 29.31 | 31.000 |
| 1996 | 9.690 |  | 16.353 | 25.00 | 29.31 | 29.31 | 31.000 |
| 1997 | 9.610 |  | 17.712 | 25.00 | 29.31 | 29.31 | 31.000 |
| 1998 | 10.010 |  | 20.664 | 25.00 | 29.31 | 29.31 | 31.000 |
| 1999 | 9.690 |  | 20.806 | 25.00 | 29.31 | 29.31 | 31.000 |
| 2000 | 10.170 |  | 20.782 | 25.00 | 29.31 | 29.31 | 31.000 |
| 2001 | 10.660 |  | 20.947 | 25.00 | 29.31 | 29.31 | 31.000 |
| 2002 | 10.350 |  | 21.000 | 21.00 | 29.31 | 29.31 | 31.000 |
| 2003 | 10.138 |  | 21.570 | 21.00 | 29.31 | 29.31 | 31.000 |
| 2004 | 10.301 |  |  |  |  |  | 29.927 |
| Year | Gas Oil | Residual Fuel Oil | Diesel | LPG | Natural Gas | Wood and Other Biomass |  |
|  | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/Mm3 | TJ/kt |  |
| 1986 | 41.82 | 39.74 | 42.70 | 46.00 | 33.50 | 12.17 |  |
| 1990 | 41.87 | 39.80 | 42.70 | 46.00 | 34.10 | 12.17 |  |
| 1991 | 41.88 | 39.80 | 42.70 | 46.00 | 34.10 | 12.17 |  |
| 1992 | 41.90 | 39.90 | 42.70 | 46.00 | 34.10 | 12.17 |  |
| 1993 | 41.90 | 39.80 | 42.70 | 46.00 | 34.10 | 12.17 |  |
| 1994 | 41.90 | 39.86 | 42.70 | 46.00 | 34.10 | 12.17 |  |
| 1995 | 41.90 | 40.00 | 42.70 | 46.00 | 34.10 | 12.17 |  |
| 1996 | 41.90 | 40.00 | 42.70 | 46.00 | 34.10 | 12.17 |  |
| 1997 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |
| 1998 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |
| 1999 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |
| 2000 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |
| 2001 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |
| 2002 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |
| 2003 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |
| 2004 | 41.90 | 40.00 | 42.70 | 46.05 | 34.08 | 12.17 |  |

Table 3.25:  NCVs for the solid fuel used in manufacturing industry and construction in 2004.

| Industry - 2004 | Sub-bituminous Coal - imported | Other Bituminous Coal | Coke |
| --- | --- | --- | --- |
| Iron and steel | 18.129 |  | 30.031 |
| Pulp. Paper and Print | 20.400 |  |  |
| Food Processing. Beverages and Tobacco |  |  | 29.000 |
| Other | 19.908 | 29.400 |  |

Emission factors

We have used country specific CO2 EF for domestic lignite and natural gas. More detailed description is in chapter 3.2.2.. Emission factors for all other fuels have been taken from IPCC Reference Manual, 1996.

Table 3.26:  EFs for the fuel used in manufacturing industry and construction.

| Year | Unit | Lignite (Velenje) | Sub-bituminous Coal - domestic | Sub-bituminous Coal - imported | Other Bituminous Coal | Anthracite | Coke | Petroleum coke |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 EF | t/TJ | Table 3.9 | 99.2 | 99.2 | 92.7 | 96.3 | 106.0 | 99.80 |
| CH4 EF | t/TJ | 0.010 | 0.010 | 0.010 | 0.010 | 0.010 | 0.010 | 0.0100 |
| N2O EF | t/TJ | 0.0014 | 0.0014 | 0.0014 | 0.0014 | 0.0014 | 0.0014 | 0.0040 |
| Year | Unit | Gas Oil | Residual Fuel Oil | Diesel | LPG | Natural Gas | Wood and Other Biomass |  |
| CO2 EF | t/TJ | 73.3 | 76.6 | 73.7 | 62.4 | 55.0 | 107.4 |  |
| CH4 EF | t/TJ | 0.002 | 0.002 | 0.002 | 0.002 | 0.005 | 0.030 |  |
| N2O EF | t/TJ | 0.0006 | 0.0006 | 0.0006 | 0.0006 | 0.0001 | 0.0040 |  |

Waste incineration

In industry, particularly in cement industry, beside common used fuel also same wastes are incinerated because very high temperature in own.

We have very detailed data about amount and composition of waste from one cement plant where the main process of waste incineration in Slovenia was occurring. We have this data from 1996 on.

For 2004 we obtained data from pulp and paper industry about consumption of black liquor. This year 283137 t have been consumed, NCV was 6,4 TJ/kt. For GHG calculation we have used the same EF as we used for wood.

Table 3.27:  Amount of waste incinerated in cement plant.

|  | waste industrial oils | waste cooking fat | waste cooking oils | waste tyres | other waste |
| --- | --- | --- | --- | --- | --- |
|  | t | t | t | t | t |
| 1996 | 1058 | 0 | 0 | 1649 | 725 |
| 1997 | 1629 | 0 | 0 | 1390 | 760 |
| 1998 | 1526 | 0 | 0 | 2695 | 800 |
| 1999 | 3459 | 0 | 0 | 2744 | 835 |
| 2000 | 1854 | 0 | 0 | 4551 | 619 |
| 2001 | 2382 | 2124 | 0 | 2014 | 957 |
| 2002 | 1626 | 2214 | 0 | 3138 | 230 |
| 2003 | 2229 | 3319 | 0 | 4346 | 110 |
| 2004 | 3598 | 5225 | 12 | 6051 | 0 |

Table 3.28:  NCVs for waste incinerated in the cement plant.

|  | waste industrial oils | waste cooking fat | waste cooking oils | waste tyres | other waste |
| --- | --- | --- | --- | --- | --- |
|  | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt |
| 1996 | 37.00 |  |  | 27.21 | 11.00 |
| 1997 | 37.00 |  |  | 27.21 | 11.00 |
| 1998 | 37.00 |  |  | 27.21 | 11.00 |
| 1999 | 37.00 |  |  | 27.21 | 11.00 |
| 2000 | 37.00 |  |  | 27.21 | 11.00 |
| 2001 | 37.00 | 39.20 |  | 27.21 | 11.00 |
| 2002 | 37.00 | 39.20 |  | 27.21 | 11.00 |
| 2003 | 37.00 | 39.20 |  | 27.21 | 11.00 |
| 2004 | 41.90 | 40.41 | 40.00 | 27.00 |  |

Table 3.29:  EFs for waste incinerated in the cement plant.

|  | waste industrial oils | waste cooking fat | waste cooking oils | waste tyres | other waste |
| --- | --- | --- | --- | --- | --- |
|  | t/TJ | t/TJ | t/TJ | t/TJ | t/TJ |
| CO2 EF | 73.30 | 70.01 | 70.01 | 70.01 | 70.01 |
| CH4 EF | 0.0020 | 0.0300 | 0.0300 | 0.0300 | 0.0300 |
| N2O EF | 0.0006 | 0.0040 | 0.0040 | 0.0040 | 0.0040 |

Recalculations

Many improvements have been made in this sector which influence on calculation of GHG emissions in year 1986 and period 1990-2003. This improvements are:

Coke reported to be used in iron and steel industry have been subtracted from energy sector for period 1990-2000.

In cement industry a petroleum coke has been used in all years. After obtaining individual data from cement plants we have to made the following recalculations. Coke reported in non-metal industry has been found out that is petroleum coke indeed. We have subtracted this amount from coke and from solid fuel and started to use this new category of fuel among liquid fuel. We have to add new data until 1995 as no consumption of petroleum coke have been reported before and transform from coke to petroleum coke.

For the period 1997-2003 we didn’t have data about fuel consumption in iron and steel and in non-ferrous metals separately. For this reason for this period we have reported all emissions under iron and steel and for non-ferrous metals notation key IE has been used. In this submission we have used actual data about sharing each fuel between these two categories for division of fuel for previous years.

### Transport (IPCC: I A 3)

Traffic is an important source of emissions of greenhouse gases, mostly carbon dioxide and nitrous oxide, and also an important source of emissions which cause problems in terms of air quality, such as sulphur oxides (SOx), nitrous oxides (NOx), carbon monoxide (CO), non volatile organic compounds (NMVOC), and consequently are indirectly responsible for the formation of ozone (O3) in lower troposphere.

Table 3.30: GHG emissions from Transport in Gg CO2 eq..

Figure 3.6: GWP of GHG emissions from Transport

#### Road transport

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 9.43 |  | 2 |
| 2004 | Level, Trend | CO2 | 20.32 | 29.29 | 2 |
| Base Year | no | CO2 | 0.09 |  | 45 |
| 2004 | Level, Trend | N2O | 0.75 | 1.79 | 21 |

Clearly, the most important source is road traffic, which accounts for 99% of all traffic emissions; GHG emissions have more than doubled, compared to the base year. Due to the ever-increasing fraction of motor vehicles equipped with catalytic converters, in the 1986-2004 period, N2O emissions rose more than 8 times, thus becoming a key source.

Methodology

For the compilation of GHG inventories, the CORINAIR methodology and the COPERT III emission factors for calculation of emissions of CO2, N2O, CH4, SO2, NOx, NMVOC, NH3, and PM have been applied. While the COPERT III model is used in the Republic of Slovenia for calculating GHG emissions, it is also used for calculating emissions of pollutants that are being reported by the Republic of Slovenia to the Convention on Long-Range Transboundary Air Pollution.

According to CORINAIR, the pollution and the consumption of fuels in road transport are determined as a product of mileage and emission factors, which define emissions and consumption of fuels in units of mass per km of mileage. Within the framework of CORINAIR, there are two approaches for estimating emissions of road traffic. The fundamental, simpler methodology for individual category of vehicles and type of pollutant adopts a universal emission factor, which does not take into account detailed driving characteristics, which influence emissions. The detailed methodology takes into account the dependence of emission factors on road conditions. Particularly detailed is the treatment of road traffic of passenger cars where emission factors for major pollutants are directly dependent on the average driving speed. Cold start over-emissions are treated separately, as they are dependent on average trip length in urban traffic as well as climate conditions. Emissions of hydrocarbons due to fuel evaporation are also covered.

The results of the more detailed variant of CORINAIR methodology give a better description of traffic pollution and are particularly useful as a starting point for planning measures for the reduction of pollution. The precondition for applying the more detailed methodology is the appropriate quality and quantity of data on traffic.

In Slovenia, quality data on traffic on state roads are available and that has made it possible to apply a more detailed methodology. Therefore, the CORINAIR disaggregation into urban, rural and highway driving has been upgraded to:

- Motorways (AC)

- High-speed roads (HC)

- Main roads, 1st class (G1)

- Main roads, 2nd class (G2)

- Regional roads, 1st class (R1)

- Regional roads, 2nd class (R2)

- Regional roads, 3rd class (R3)

- Tourist roads (RT)

- Local urban roads (LC_URB)

- Local non-urban roads (LC_NURB)

CORINAIR classifies road vehicles into four main categories:

- Passenger cars;

- Light-duty trucks;

- Heavy-duty trucks;

- Two-wheelers.

The basic classification of passenger cars is based on the type of fuel:

- Gasoline;

- Gas Oil (Diesel);

- LPG;

- Two stroke gasoline for two-stroke vehicles.

The main categories of vehicles are further divided into subcategories with regard to the type of fuel. engine volume. EURO type. and mass.

Emission factors

Emission factors for greenhouse gases have been taken from CORINAIR. COPERT III model for each vehicle type and driving conditions

Table 3.31: Weighted emission factor for gasoline and diesel

| Emission Factor (t/TJ) | Gasoline |  |  | Diesel |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CO2 | CH4 | N2O | CO2 | CH4 | N2O |
| 1986 | 73.0 | 0.027 | 0.002 | 73.7 | 0.0042 | 0.0026 |
| 1990 | 73.0 | 0.028 | 0.002 | 73.7 | 0.0045 | 0.0027 |
| 1991 | 73.0 | 0.029 | 0.002 | 73.7 | 0.0043 | 0.0026 |
| 1992 | 73.0 | 0.029 | 0.002 | 73.7 | 0.0041 | 0.0026 |
| 1993 | 73.0 | 0.030 | 0.002 | 73.7 | 0.0042 | 0.0026 |
| 1994 | 73.0 | 0.028 | 0.003 | 73.7 | 0.0039 | 0.0026 |
| 1995 | 73.0 | 0.027 | 0.005 | 73.7 | 0.0038 | 0.0025 |
| 1996 | 73.0 | 0.026 | 0.006 | 73.7 | 0.0039 | 0.0027 |
| 1997 | 73.0 | 0.023 | 0.006 | 73.7 | 0.0040 | 0.0033 |
| 1998 | 73.0 | 0.019 | 0.007 | 73.7 | 0.0041 | 0.0040 |
| 1999 | 73.0 | 0.016 | 0.009 | 73.7 | 0.0041 | 0.0047 |
| 2000 | 73.0 | 0.016 | 0.009 | 73.7 | 0.0042 | 0.0054 |
| 2001 | 73.0 | 0.017 | 0.010 | 73.7 | 0.0044 | 0.0061 |
| 2002 | 73.0 | 0.017 | 0.011 | 73.7 | 0.0045 | 0.0069 |
| 2003 | 73.0 | 0.017 | 0.011 | 73.7 | 0.0045 | 0.0068 |
| 2004 | 73.0 | 0.017 | 0.011 | 73.7 | 0.0045 | 0.0068 |

Activity data

Fuel consumption

To obtain the consumption of fuel in road traffic it is necessary to subtract from the quantities of sold fuels the consumption in other modes of transport and the consumption of motor fuels that is not linked to traffic.

Accordingly, the consumption of gasoline and diesel fuel in road traffic has been calculated in accordance with the following formula:

Consumption in road traffic (LEG)

- Fuel consumption in agriculture

- Fuel consumption in forestry

- Fuel consumption in construction

+ Inappropriate use of fuel (due to tax differentiation)

= Consumption in road traffic

Table 3.32: Consumption of Gasoline for Road Transport (000 ton)

| Gasoline (tons) | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Road Transport (LEG) | 443926 | 564999 | 798492 | 799543 | 764044 | 745864 | 666137 |
| - Agriculture | 12016 | 9227 | 3626 | 3631 | 3603 | 3631 | 3495 |
| - Forestry | 1680 | 1172 | 808 | 835 | 806 | 902 | 867 |
| - Construction | 17000 | 18000 | 4330 | 4292 | 4778 | 4000 | IE |
| = Road Transport | 413230 | 536600 | 789728 | 790785 | 754857 | 737331 | 661775 |

Table 3.33: Consumption of Diesel for Road Transport (000 ton)

| Diesel (tons) | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Road Transport (LEG) | 349831 | 325831 | 473890 | 507946 | 542684 | 604581 | 695140 |
| - Agriculture | 108326 | 85255 | 62596 | 62678 | 62199 | 62688 | 60328 |
| - Forestry | 15120 | 10548 | 7272 | 7515 | 7254 | 8118 | 7803 |
| - Construction | 62000 | 38500 | 45072 | 37154 | 32946 | 30523 | IE |
| + Other sector (inapp. use) | 36121 | 96020 | 0 | 0 | 0 | 0 | 0 |
| = Road Transport | 200506 | 287548 | 358950 | 400599 | 440285 | 503252 | 627009 |

The quantities of motor fuels sold in the Republic of Slovenia are gathered in LEG Table Tg/1, the quantities of fuel for railway transportation are taken from LEG Table Tg/4. Data on the consumption of fuels in agriculture originate from the annual studies of the Slovenian Agriculture Institute. Data on the consumption of fuel in forestry and construction have been obtained from the Statistical Office of the Republic of Slovenia.

COPERT III Model is a bottom-up model and was calibrated to the obtained quantity of consumed refined petroleum products in the Republic of Slovenia (top-down) in such a way as to ensure the correctness of the calculation of emissions.

Vehicle Fleet Composition

The COPERT methodology requires a detailed knowledge of the structure of the vehicle fleet composition. Data on vehicles are collected by the Ministry of the Interior. Accordingly, for every licensed motor vehicle, 26 parameters are available. Each category of vehicles has been assigned consumption and appropriate emission factors with regard to the performed transport work. In the Republic of Slovenia, traffic on state roads is being counted on more than 200 counting spots, of which at 74 of them the vehicles are not only counted according to their type, but also their speed is measured, which is particularly important for the calculation of NOx emissions.

Table 3.34: Vehicle fleet structure in Slovenia in 2003

Table 3.35: Emissions of GHG from Road Transport

| year | Fuel Consumption (ton) | Fuel Consumption (PJ) | CO2 Emissions (000 tons) | CH4 Emissions (ton) | N2O Emissions (ton) |
| --- | --- | --- | --- | --- | --- |
| 1986 | 604082 | 25.992 | 1902.13 | 509 | 56 |
| 1990 | 824408 | 35.401 | 2593.09 | 696 | 79 |
| 1991 | 774874 | 33.327 | 2441.38 | 689 | 72 |
| 1992 | 807495 | 34.707 | 2540.17 | 758 | 74 |
| 1993 | 936932 | 40.278 | 2947.38 | 911 | 87 |
| 1994 | 1034730 | 44.469 | 3254.16 | 965 | 134 |
| 1995 | 1138062 | 48.903 | 3579.18 | 1011 | 194 |
| 1996 | 1321703 | 56.784 | 4156.76 | 1084 | 275 |
| 1997 | 1343746 | 57.703 | 4226.11 | 954 | 281 |
| 1998 | 1159309 | 49.801 | 3646.05 | 721 | 291 |
| 1999 | 1102497 | 47.367 | 3467.36 | 592 | 360 |
| 2000 | 1148678 | 49.349 | 3612.60 | 624 | 385 |
| 2001 | 1191385 | 51.173 | 3746.94 | 644 | 435 |
| 2002 | 1195142 | 51.319 | 3758.80 | 638 | 473 |
| 2003 | 1240583 | 53.253 | 3901.80 | 637 | 481 |
| 2004 | 1288785 | 55.283 | 4053.45 | 606 | 483 |

Future Improvements

- Data for the local roads are in need of improvement, particularly data on speed on local roads, since that exerts a decisive influence on CO emissions;

- Ensure better data on the performed transport work of light duty vehicles;

- Improve the estimate of cold start and evaporative emissions parameters

- Update emission calculation with updated emission factors.

These modifications will mostly affect emissions of pollutants, such as NOx, NMVOC, CO, and PM, but will have less implication in emissions of CH4 and N2O, and no implications in total national carbon dioxide emissions.

#### Railway

Key sector - Base year: 	no

Key sector - Year 2004:	no

Methodology

To estimate emissions from the railway. the following methodology has been adopted.

Quantity of Fuel used x Net Calorific value x EF per energy of Fuel = Emissions

Activity data

The main source of emissions is a consumption of gas oil. The specified data are based on the consumption in the railway transport sector (Ministry of Energy: Statistical Yearbook of Energy Sector in the Republic of Slovenia 1986-2004. Ljubljana: Ministry of Energy. Table Zb/3).

The consumption of brown coal in railway transportation is negligibly small (from 0 to 646 t) and emissions of CH4 and N2O are therefore not included in the following calculations. This coal is used in only one ‘’archaic’’ steam driven locomotive which is almost 100 years old. According to information from Railway Company, they are trying to avoid using hard coal, due to safety reasons, durability and preservation this piece of history.

Net calorific values

We have used value 42.7  TJ/1000t for gas diesel oil and 12.76 TJ/1000t for brown coal.

Emission factors

In calculating emissions of individual gases, emission factors from IPCC guidelines 1996 have been applied. They are shown in the following table.

Table 3.36: EFs for fuel used in railways.

| EF | Unit | Gas Oil | Brown coal |
| --- | --- | --- | --- |
| CO2 EF | t/TJ | 73.300 | 99.2 |
| CH4 EF | t/TJ | 0.004 |  |
| N2O EF | t/TJ | 0.030 |  |

Recalculations

No changes have been made in this source sector.

#### Aviation

Key sector - Base year: 	no

Key sector - Year 2004:	no

Methodology

Most quantities of jet kerosene are consumed outside the territory of Slovenia in international transport, i.e. in the so-called international bunkers, therefore these quantities are excluded from the calculations of national emissions for Slovenia.

To estimate emissions from Aviation, the following methodology has been adopted.

Quantity of Fuel used x Net Calorific value x EF per energy of Fuel = Emissions

Activity data

Most quantities of jet kerosene are used outside the territory of Slovenia in international transport, i.e. in the so-called international bunkers, therefore these quantities are excluded from the calculations of national emissions for Slovenia. As quantities consumed in international air transport in Slovenia those quantities are taken into account, which are reported in LEG under the category Aviation Gasoline for Turboprop Engines – these quantities are not included in total national emissions for Slovenia. As consumption in Slovenia, the category Aviation Gasoline for Piston Engine Aircraft is taken into account. The calculation of the consumption of jet kerosene and kerosene in Slovenia has been done on the basis of data and calorific values from Tables Tg/1 and Tg/5. Statistični letopis energetskega gospodarstva Republike Slovenije 1985-2003, Ljubljana: Ministry of the Economy.

Difference between IEA data and inventory data occurs due to rounding. In IEA/EUROSTAT statistics all values are rounded and expressed in 000 tons.

Net calorific values

We have used value 43.5  TJ/1000t for aviation gasoline.

Emission factors

For the calculation of emissions and individual gases, the emission factors from IPCC guidelines from 1996 have been applied: 71.5 t CO2/TJ, 0.001 t CH4/TJ and 0.002 t N2O/TJ.

Recalculations

No changes have been made on this source sector

Future Improvements

In the present inventories, the consumption of fuel in LTOs is counted as international bunker. The improvement will, as a consequence, negligibly increase the emissions of the Republic of Slovenia, but inventories will be more appropriate from the methodological point of view. In as much as data for 1986 will be accessible, we shall also perform a recalculation for the entire period 1986-2004.

### Other sector (IPCC: I A 4)

This chapter presents the consumption of fuels and emissions of greenhouse gases in:

- Commercial / Institutional sector 	(IPCC: I A 4 a)

- Residential sector  			(IPCC: I A 4 b)

- Agriculture and forestry 		(IPCC: I A 4 c)

Figure 3.7: GWP of GHG emissions from Other Sector.

Table 3.37: GWP of GHG emissions from Other Sector.

#### Commercial/Institutional Sector (IPCC: I A 4 a) and

#### Residential Sector (IPCC: I A 4 b)

| Commercial/ Institutional | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 3.04 |  | 7 |
| 2004 | Level, Trend | CO2 | 4.09 | 2.84 | 5 |

| Residential | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 5.46 |  | 5 |
| 2004 | Level, Trend | CO2 | 8.24 | 7.48 | 5 |
| 2004 | Level, Trend | CH4 | 0.45 | 0.58 | 23 |

Methodology

Emissions were estimated using Tier 2 methodology described in IPCC 96. The following basic formula was used:

Emissions = (EFabc x Activityabc)

Where:

EF = Emission Factor (kg/TJ);

Activity = Energy Input (TJ);

a = Fuel type;

b = Sector-activity; and

c = Technology type.

Activity data

The consumption of fuels in the commercial sector and households has been in our basic source of data (Statistical Yearbook of Electricity Generating Industries) combined under “Široka potrošnja”. Disaggregation into these two categories has been done within the framework of the research project done at the end of the year by the Institute of Energy Industries (Gasperič, Dornik 1998). Data from that research project have been corrected in the following points:

Table 3.38: Estimates of Inappropriate Consumption of Fuel Oil in Commercial Sector and Households

|  | Other Consu. (LEG) | Estimate of “Inappropriate” Use of Res.Fuel Oil. subtracted from Other Cons (LEG) & added to Road Transport | Other Consumption (NIR) | Split of “Inappropriate” Use of Fuel Oil |  | Actual Consumption of Res. Fuel Oil in Other Consumption |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Resid. | Comm./ Institution | Resid. (NIR) | Comm./Inst (NIR) |
|  | A | B | C=A-B | D=0.8 x B | E=0.2 x B | F+G=C |  |
|  | (tonnes) | (tonnes) | (tonnes) | 80% | 20% | (tonnes) | (tonnes) |
| 1986 | 157835 | 36121 | 121714 | 28897 | 7224 | 72117 | 49597 |
| 1990 | 310342 | 96020 | 214322 | 76816 | 19204 | 121803 | 92519 |
| 1991 | 327577 | 80733 | 246844 | 64586 | 16147 | 145063 | 101781 |
| 1992 | 300726 | 59608 | 241118 | 47686 | 11922 | 144779 | 96339 |
| 1993 | 411782 | 744 | 411038 | 595 | 149 | 262945 | 148093 |
| 1994 | 432591 | 1141 | 431450 | 913 | 228 | 275945 | 155505 |
| 1995 | 512171 | -23212 | 535383 | - | - | 342645 | 192738 |
| 1996 | 625621 | -79731 | 705352 | - | - | 451425 | 253927 |
| 1997 | 697066 | 0 | 697066 | - | - | 446084 | 250982 |
| 1998 | 718587 | 0 | 718587 | - | - | 459896 | 258691 |
| 1999 | 755417 | 0 | 755417 | - | - | 483467 | 271950 |
| 2000 | 674464 | 0 | 674464 | - | - | 431657 | 242807 |
| 2001 | 684636 | 0 | 684636 | - | - | 438167 | 246469 |
| 2002 | 658761 | 0 | 658761 | - | - | 421607 | 237154 |
| 2003 | 633476 | 0 | 633476 | - | - | 405425 | 228051 |
| 2004 | 620586 | 0 | 620586 | - | - | 397175 | 223411 |

- All quantities of LPG specified in LEG under the category “Široka Potrošnja – Other Consumption” are in this report presented as consumption in households.

- Quantities of fuel oil which have been consumed in road transport as gas oil and which have been estimated in the research project ”Assessment of Emissions of Greenhouse Gases in Road Traffic” (Institute of Transport Technology, 1999) are subtracted from the sector “Široka Potrošnja”, namely 80 % from sector Consumption in Households and 20 % from Consumption in Commercial Sector

All quantities of residual fuel oil that are reported in LEG as consumed in Other consumption, are in this report presented as consumption in the commercial/institutional sector. In the energy statistics of Slovenia, this item is a balance category; consequently, it will be positive in some years, negative in other years. Quantities used in calculating emissions for this report (either positive or negative values) have been taken from LEG.

Net calorific values

Net calorific values have been taken from SORS, The values for solid fuel varies from year to year but for  the liquid and gaseous fuel almost the same values have been used  for the entire period as these types of fuel don’t change a lot from year to year.

Table 3.39: NCVs for the fuel used in Commercial Sector and Households.

| Year | Lignite (Velenje) | Sub-bituminous Coal - domestic | Sub-bituminous Coal - imported | Gas Oil | Residual Fuel Oil | LPG | Natural Gas | Wood and Other Biomass |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/kt | TJ/Mm3 | TJ/kt |
| 1986 | 9.390 | 11.880 |  | 41.82 | 39.74 | 46.00 | 33.50 | 12.17 |
| 1990 | 9.810 | 12.760 |  | 41.87 | 39.80 | 46.00 | 34.10 | 12.17 |
| 1991 | 9.980 | 12.879 |  | 41.88 | 39.80 | 46.00 | 34.10 | 12.17 |
| 1992 | 10.260 | 12.589 |  | 41.90 | 39.90 | 46.00 | 34.10 | 12.17 |
| 1993 | 10.070 | 13.351 |  | 41.90 | 39.80 | 46.00 | 34.10 | 12.17 |
| 1994 | 9.960 | 12.666 |  | 41.90 | 39.86 | 46.00 | 34.10 | 12.17 |
| 1995 | 10.220 |  | 17.404 | 41.90 | 40.00 | 46.00 | 34.10 | 12.17 |
| 1996 | 9.690 |  | 16.353 | 41.90 | 40.00 | 46.00 | 34.10 | 12.17 |
| 1997 | 9.610 |  | 18.203 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |
| 1998 | 10.010 |  | 18.531 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |
| 1999 | 9.690 |  | 18.563 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |
| 2000 | 10.170 |  | 17.983 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |
| 2001 | 10.660 |  | 18.834 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |
| 2002 | 10.350 |  | 19.000 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |
| 2003 | 10.138 |  | 19.000 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |
| 2004 | 10.301 |  | 19.000 | 41.90 | 40.00 | 46.05 | 34.08 | 12.17 |

Emission factors

We have used country specific CO2 EF for domestic lignite and natural gas. More detailed description is in chapter 3.2.2. Emission factors for all other fuels have been taken from IPCC Reference Manual, 1996.

Table 3.40: EFs for the fuel used in Commercial Sector and Households.

| Year | Unit | Lignite (Velenje) | Sub-bituminous Coal - domestic | Gas Oil | Residual Fuel Oil | LPG | Natural Gas | Wood and Other Biomass |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CO2 EF | t/TJ | Table ? | 99.2 | 73.3 | 76.6 | 62.4 | 55.0 | 107.4 |
| CH4 EF | t/TJ | 0.010 | 0.010 | 0.002 | 0.002 | 0.002 | 0.005 | 0.030 |
| N2O EF | t/TJ | 0.0014 | 0.0014 | 0.0006 | 0.0006 | 0.0006 | 0.0001 | 0.0040 |

Recalculations

No changes have been made on this source sector

Future Improvements

In the next report, the disaggregation of fuels between Households and Other Commercial/Institutional usage will be renewed,  but that will not effect the total emissions of GHG from this sector.

Our future plans is also to improve estimate of the consumption of woody biomass in households. Uncertainty is considerable, both as regards the quantity of consumed biomass as well as the average net calorific value (NCV) of woody biomass.

#### Agriculture and Forestry (IPCC: I A 4 c)

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 2.12 |  | 11 |
| 2004 | Level, Trend | CO2 | 1.13 | 2.65 | 15 |

This chapter should present all consumption of fuel in agriculture, forestry, and fishing. However, only the consumption of fuel for mobile sources in these sectors is presented here. Not enough data are available for the consumption of fuel in stationary sources in Slovenia; consequently, these quantities are included in the Commercial / Institutional sector.

Methodology

Emissions for all pollutants emissions are estimated by means of the following formula:

Emission (p.y) = EF(p) * ConsFuel(y)

Where

Emission (p.y) - Emission of pollutant p in year y (ton/yr);

EF(p) - Quantity of pollutant p emitted (ton/TJ);

ConsFuel(y) - consumption of gas oil in agriculture machines and off-road vehicles during in year y (ton/yr).

Activity data

The consumption of fuels till year 2000 in agriculture is taken from the research project ”The Consumption of Fuels in Agriculture” by the Slovenian Agriculture Institute, October 2003. For estimation of fuel consumption in Agriculture from year 2000 onwards, we used the same energy intensity (fuel consumption/ha of land) as it was in year 2000.

Table 3.41: Estimate of Consumption of Gasoline in Agriculture.

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Cultivated Land in State owned Agriculture ent. (1000 ha) | 70 | 77 | 62 | - | - | - | - |
| Total Cultivated Land (1000 ha) | 647 | 653 | 509 | 510. | 506 | 510 | 491 |
| Consumption of Gasoline in State owned Agriculture ent. (1000 t) | 1.3 | 1.088 | 0.7 | - | - | - | - |
| Consumption of Gasoline per Hectare of Cultivated Land (t/1000 ha) | 18.6 | 14.1 | 10.5 | 7.1 | 7.1 | 7.1 | 7.1 |
| Estimated Consumption of Gasoline in Total Agriculture (1000 t) | 12.0 | 9.2 | 3.63 | 3.63 | 3.60 | 3.63 | 3.50 |

Table 3.42: Estimate of Consumption of Diesel in Agriculture.

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Consumption of Diesel in State owned Agriculture ent. (1000 t) | 14.6 | 12.6 | - | - | - | - | - |
| Consumption of Diesel per Hectare of Cultivated Land in State owned Agriculture ent. (t/1000 ha) | 208.6 | 163.5 | 123 | 123 | 123 | 123 | 123 |
| Estimated consumption of Diesel Fuels in Total Agriculture (1000 t) | 135 | 107 | 62.596 | 62.678 | 62.199 | 62.688 | 60.328 |

The consumption of fuels in the entire forestry is estimated on the basis of the consumption of fuel in the state-owned logging enterprises.

For the state-owned sector, data are available for the consumption of fuel and cut, for private sector only data on cut. First, the consumption per m3 of cut in state owned logging enterprises (4.8 tonnes /1000 m3) is estimated. Based on these estimates and data on total cut, the estimate of consumption in the whole of forestry is calculated. For forestry, there are no separate data on the consumption of gasoline and gas, only the total consumption. Consequently, the split is done considering the split in agriculture (10 % gasoline, 90 % gas oil), presuming that the same amount of fuels is consumed per m3 of felled wood in private forestry as in social forestry.

Data on consumption of fuel in state owned forest are available for 1990 and for the period 1994-2003. In other years the research has not been conducted, hence the consumption of fuel per felled quantities in other years has been estimated by linear interpolation (4.8 tons per 1000 m3 of cut). The estimated error is marginal, since quantities do not undergo essential changes

Table 3.43: The Calculation of the Consumption of Fuels in State Owned Forest

|  | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- |
| Consumption of Fuel in State owned Forest (tons) | 5922 | 2808 | 2782 | 2777 | 3911 | 3292 |
| Cut in State owned Forest (1000 m3) | 1230 | 907 | 871 | 912 | 1147 | 1123 |
| Consumption of Fuel per Cut Quantities (tons per 1000 m3) | 4.8 | 3.1 | 3.2 | 3.0 | 3.4 | 2.9 |
| Total Cut (1000 m3) | 2435 | 2609 | 2614 | 2646 | 3007 | 2958 |
| Total Consumption of Fuel in Forestry (1000 t) | 11.7 | 8.080 | 8.350 | 8.060 | 10.25 | 8.67 |
| Gasoline (10 %) (tones) | 1.172 | 0.808 | 0.835 | 0.806 | 1.025 | 0.867 |
| Diesel (90 %) (tones) | 10.548 | 7.272 | 7.515 | 7.254 | 9.225 | 7.803 |

Source of activity data:

Data needed for estimation of consumption of fuels in Agriculture and Forestry is available for years from 1986 to 2004 (Statistical Office of the Republic of Slovenia: Statistical Yearbook RS, Statistical Office of the Republic of Slovenia, Ljubljana).

Source of calorific values: Ministry of Energy: Statistični letopis energetskega gospodarstva republike Slovenije 1986-2003. Ljubljana: Ministry of Energy. Table Zb/3. Table Zb/1

Net calorific values

We have used value 43.8  TJ/1000t for gasoline and 42.7 TJ/1000t for gas diesel oil. Until 1998 a heavy fuel oil has been used in agriculture wit NCV 40 TJ/1000t.

Emission factors

In calculating emissions, the emission factors, recommended in IPCC guidelines, have been used, category: Agriculture and Forestry (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996, p. I.91): for gas oil, category: Diesel Engines, for gasoline, category: Gasoline 4-stroke engines.

Table 3.44: EFs of the Consumption of Fuels in Agriculture and Forestry.

| Year | Unit | gasoline | Gas/Diesel Oil | Heavy Fuel Oil |
| --- | --- | --- | --- | --- |
| CO2 EF | t/TJ | 71 | 73 | 76.6 |
| CH4 EF | t/TJ | 0.08 | 0.004 | 0.1 |
| N2O EF | t/TJ | 0.002 | 0.03 | 0.0006 |

## Fugitive emissions from solid fuels and oil and natural gas (CRF 1.B)

This chapter presents the fugitive emissions of greenhouse gases from:

- Solid fuels 	(IPCC 1.B.1) Coal Mining. Coal Handling. SO2 scrubbing

- Oil		(IPCC 1.B.2) Production. Processing. Storage

- Natural Gas 	(IPCC 1.B.2) Production. Transmission. Distribution and Leakages

Table 3.45: GWP of Fugitive emissions of GHGs

Figure 3.8: GWP of Fugitive emissions of GHGs

### Solid Fuels (IPCC: I B I)

#### Coal Mining (IPCC: I B I a) + Coal Handling (IPCC: I B I b)

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 1.78 |  | 13 |
| 2004 | Level, Trend | CH4 | 1.36 | 1.14 | 13 |

This chapter encompasses emissions arising from the production, processing, and storage of coal. The most important component of those emissions is CH4 emissions that arise in mining and post-mining activities.

Methodology

Methane emission = (EF1(m3 CH4/t)+ EF2(m3 CH4/t))* excavated coal (t/year )* 0.67

EF1 = Methane emission factor in coal excavation (m3 CH4/t)

EF2 = Methane emission factors in post-mining activities for coal (m3 CH4/t)

Activity data

Data on excavated quantities of coal according to individual coalmines are taken from LEG Table Pr/2 and LEG Table Pr/3

Table 3.46: Excavation of Coal in Slovenia 1986 – 2004

| Pit | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Velenje | 5000.5 | 4210.0 | 3743.1 | 3448.1 | 4047.7 | 4221.7 | 4197.7 |
| Trbovlje - Hrastnik | 1241.6 | 904.6 | 736.6 | 685.0 | 638.7 | 607.8 | 611.3 |
| Zagorje | 314.6 | 244.1 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Senovo | 120.0 | 108.0 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Kanižarica | 126.2 | 94.4 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Laško | 25.0 | (closed) | (closed) | (closed) | (closed) | (closed) | (closed) |
| Total Coal Excavation (Gg) | 6827.9 | 5561.1 | 4479.7 | 4133.1 | 4686.4 | 4829.5 | 4809.1 |

Emission factors

Estimates of emission factors for individual coalmines in Slovenia were done at the Ecological Research Institute (Zapušek A., Orešnik K., Avberšek F: Assessment of methane emission factors in coal excavation in 1986 and in the period 1990-1996, Velenje: ERICo - Ecological Research Institute, 1999).

Due to rather small emissions from this sector for the period 1996-2004, no special research project has been done so that for that period an average emission factor from the past period has been assumed.

Table 3.47: Emission Factors for CH4 in Coal Excavation 1986 – 2004 (m3 CH4/t coal)

| Pit | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Velenje | 0.95 | 0.33 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Trbovlje - Hrastnik | 0.62 | 0.88 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Zagorje | 1.39 | 3.59 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Senovo | 0.57 | 0.63 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Kanižarica | 0.33 | 0.45 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Laško | 3.82 | (closed) | (closed) | (closed) | (closed) | (closed) | (closed) |

Table 3.48: Emission Factors for CH4 in Post Mining Activities 1986 – 2004 (m3 CH4/t coal)

| Pit | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Velenje | 3.05 | 3.67 | 1.00 | 1.00 | 1.00 | 1.00 | 1.00 |
| Trbovlje - Hrastnik | 2.38 | 2.12 | 3.00 | 3.00 | 3.00 | 3.00 | 3.00 |
| Zagorje | 1.61 | 2.00 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Senovo | 2.43 | 2.37 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Kanižarica | 2.67 | 2.52 | (closed) | (closed) | (closed) | (closed) | (closed) |
| Laško | 0.18 | (closed) | (closed) | (closed) | (closed) | (closed) | (closed) |

Table 3.8: Emission of CH4 from Mining & Post Mining Activities 1986 – 2004 (Gg CH4)

| (000 ton) | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Mining Activities | 4.13 | 2.12 | 8.02 | 7.39 | 8.56 | 8.89 | 8.85 |
| Post-Mining Activities | 12.96 | 12.30 | 3.99 | 3.69 | 4.00 | 4.05 | 4.04 |
| Total | 17.09 | 14.42 | 12.01 | 11.08 | 12.56 | 12.94 | 12.89 |

Recalculations

No changes have been made in this source sector.

#### Scrubbing (IPCC: I B I c)

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | no |  |  |  |  |
| 2004 | Trend | CH4 | 0.36 | 0.98 | 29 |

Using the technology for the reduction of SO2 emissions in the process of consumption of coal is causing emissions of CO2. According to IPCC methodology, these emissions belong to the category of emissions arising from the consumption of coal.

Methodology

CO2 emissions from scrubbing = consumption of additive CaCO3 * 0.44

Activity data

Data on CaCO3 consumption are taken from the documents of Milan Vidmar Electroinstitute (Šuštaršič A. et all.: Estimate of Emissions and the Results of Measurements of Emission Concentrations in the Šoštanj Thermal Power Plant in 2003, Milan Vidmar Electroinstitute, Ljubljana 1995-2004). Prior to 1995, there were no wet flue gas desulphurisation units installed for reducing emission of SO2 in Slovenia.

Emission factors

29/01/2004/EC Guidelines for the monitoring and reporting of greenhouse gas emissions pursuant to Directive 2003/87/ of the European Parliament and of the Council

Emission factor is based on use of stoichiometric ratios of conversions of carbonates [t CO2/t dry carbonate] as shown in below table. This value shall be adjusted for the respective moisture and gangue content of the applied carbonate material.

Table 3.50: Stoichiometric emission factors for SO2 scrubbing

| Carbonate | Emission factor [t CO2/t Ca-. Mg- or other Carbonate] | Remarks |
| --- | --- | --- |
| CaCO3 | 0.440 |  |
| MgCO3 | 0.522 |  |
| general: XY(CO3)Z | Emission factor = [MCO2] / {Y * [Mx] + Z *[MCO32-]} | X = alkali earth or alkali metal Mx = molecular weight of X in [g/mol] MCO2 = molecular weight of CO2 = 44 [g/mol] MCO3- = molecular weight of CO32- = 60 [g/mol] Y = stoichiometric number of X = 1 (for alkali earth metals) = 2 (for alkali metals) Z = stoichiometric number of CO32- = 1 |

Table 3.9: Consumption of additive and CO2 emissions from scrubbing

|  | 1986 | 1990 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CaCO3 Consumption (000 ton) | NO | NO | 83.5 | 137.2 | 132.1 | 158.5 | 165.3 |
| CO2 emissions (Gg) | NO | NO | 36.7 | 60.4 | 58.1 | 69.8 | 72.8 |

### Oil  (IPCC: I B 2)

Key sector - Base year: 	no

Key sector - Year 2004:	no

Owing to negligible quantities of produced crude oil (963 tons in 2002, and no production since than), methane emissions from the production of crude oil and refined petroleum products were insignificant in 2002 and did not occur in 2003 and 2004.

Methodology

CH4 emissions = CH4 emission1 + CH4 emission2 + CH4 emission3

CH4 	emission1:

Emission in the production of crude oil = production of crude oil (ton) x calorific value x emission factor (kgCH4/TJ)

CH4 emission2:

Emission in the processing of crude oil = quantities processed in Slovenia (ton) x calorific value x emission factor (kgCH4/TJ)

CH4 emission3:

Emission in storage of crude oil = quantities processed in Slovenia (ton) x calorific value x emission factor (kgCH4/TJ)

Activity data

Data on production. processing (processing and internal consumption). and calorific values of oil have been taken from LEG (Table Zb/1. Statistical Yearbook of the Energy Sector in the Republic of Slovenia 1985-2003. Ljubljana: Ministry of the Economy). No data are available on stored quantities. In this section. it is assumed that all quantities of processed oil are also stored in Slovenia.

In distribution of gasoline (at refinery dispatch stations. in transport. at depots. and at service stations) NMVOCs emissions arise. Calculations take into account the quantities of gasoline that are sold in Slovenia (Statistical Yearbook of Energy Sector in the Republic of Slovenia 1985-2003. Ljubljana: Ministry of the Economy. Tables Zb/1).

Emission factors

Emission factors for the emission of methane arising in the production. processing. and storage of oil have been calculated on the basis of the arithmetic mean from IPCC guidelines (Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Workbook. UNEP-OECD-IEA-IPCC. Bracknell 1995. p. I.21) from the data for former Soviet Union. Central and Eastern Europe. Emission factors are given in a relatively wide interval (for production from 300 to 5000 kg CH4/PJ. and for processing from 90 to 1400 kg CH4/PJ).

Emiss. Factor=300+50002= 2650 kg CH4/PJ

For oil production:

Emiss. Factor=90+14002= 745 kg CH4/PJFor oil processing:

Emiss. Factor=20+2502= 135 kg CH4/PJ

For oil storage:

Table 3.10: Fugitive emissions from Oil

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | From 2003 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Production (Gg) | 0.0003 | 0.0003 | 0.0002 | 0.0001 | 0.0001 | 0.0001 | 0.0000 |
| Processing (Gg) | 0.0170 | 0.0170 | 0.0165 | 0.0046 | 0.0000 | 0.0000 | 0.0000 |
| Storage (Gg) | 0.0028 | 0.0031 | 0.0030 | 0.0008 | 0.0000 | 0.0000 | 0.0000 |
| Total (Gg) | 0.0201 | 0.0204 | 0.0197 | 0.0056 | 0.0001 | 0.0001 | 0.0000 |

### Natural Gas (IPCC: I B 2)

Key sector - Base year: 	no

Key sector - Year 2004:	no

CH4 emissions = Production + Transport + Distribution + Leakages at consumers

Table 3.11: Fugitive emissions from Natural gas in Gg CH4

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 0.010 | 0.033 | 0.024 | 0.008 | 0.008 | 0.007 | 0.007 | 0.007 |
| transport | 0.849 | 0.880 | 0.815 | 0.484 | 0.443 | 0.366 | 0.355 | 0.276 |
| distribution | 1.632 | 1.615 | 1.441 | 1.207 | 1.146 | 1.090 | 1.051 | 0.997 |
| gas use | 0.181 | 0.205 | 0.289 | 0.349 | 0.345 | 0.343 | 0.331 | 0.317 |
| total | 2.671 | 2.733 | 2.570 | 2.048 | 1.942 | 1.807 | 1.743 | 1.596 |

Production

Emissions in the production of natural gas = Produced quantities (thousand m3)* Net calorific value (NCV) * Emission factor (kg/PJ)

Table 3.53: Fugitive emissions from production of Natural gas  in 1000 Sm3 of CH4.

Activity data

Quantity of natural gas in Slovenia is very small, in 2004 it was 10,080 m3.

Data on the production and calorific value of natural gas have been taken from LEG (Table Zb/1, Statistical Yearbook of Energy Sector in the Republic of Slovenia 1985-2004, Ljubljana: Ministry of the Economy).

Emission factors

Emission factor for the emission of methane from the Production of natural gas have been taken from GPG, Table 2.18. We have used medium value 0.2% from net gas production. The yearly variation of implied EF expressed in kg/PJ is because of difference yearly variation of the net calorific values..

Transport, distribution and use of natural gas

Gasification of Slovenia started in the early 1970s. The newly discovered fields of natural gas in the former Soviet Union and Algeria and increasing demand for energy following the economic growth in Europe, actualized natural gas as an economic and practical source of energy. A possibility to connect to the newly constructed pipelines in Austria and Italy, enabled realization of natural gas delivery to Slovenia, as the country has no natural gas reserves of its own.

In 1974, the company Petrol-Zemeljski plin was established with the purpose to realize the »gasification program in Slovenia«. After three years of intensive preparations, construction of the backbone of the main pipeline system started, divided into the following three phases:

Ceršak – Rogatec – Vodice – Ljubljana,

Vodice – Jesenice,

Ljubljana – Nova Gorica – Anhovo.

In 1978, the newly constructed pipeline system enabled transportation of Russian natural gas for Croatia and delivered gas to the first two consumers in Slovenia, namely, Tovarna lesovine in lepenke Ceršak and Sladkogorska Sladki vrh. Till 1980, 630 km of mostly main pipelines were constructed and the Dispatcing Centre in Ljubljana as well as Maintenance Centres in Ljubljana and Maribor were built.The pipeline system has been expanded later and by the end of 1980s, the main lines to Novo mesto and Radeče were finished as well. Now, the total length of the pipeline system, owned by Geoplin, comes to almost 1.000 km and is still being extended.

From the very beginning, during the pipeline construction all environmental requirements have been taken into consideration. Alongside the whole pipeline route, the original state of the ground has been reinstated after concluded construction works, so that only the characteristic markings and meter-regulating stations show that under the surface there are kilometers and kilometers of pipes.

In 1992, natural gas deliveries from the second source, namely, from Algeria, started, what esentially increased reliability of supply and enabled growth of natural gas consumption also in households and commercial use. Since 2001, natural gas from the third source, that is Austria, has been delivered as well.

Social, political and economic changes in the past influenced several status transformations of the company. Since 1995, the company has been operating under the name of Geoplin d.o.o. Ljubljana. In 2004, in line with the terms of Energy Act, Geoplin established a daughter company Geoplin plinovodi and transferred complete energy infrastructure to it. The company Geoplin plinovodi started to perform its activity of natural gas transmission system operator on 1 January 2005.

Legislation

Before 1974 companies have used different standards. The material for pipelines has been made according to the JUS (Yugoslav standard), which has been transferred from DIN (West Germany standard) to a high degree. On same domain also east  German standard TGL (Technische Gute und Lieferbedinungen) has been used.

In 1974 all companies together with body of inspectors made an agreement to use west German standards (DVGW, DIN, VDI) because more than 90% gas devices have been made according to these standards, particularly DVGW standard (Deutsche Vereinigung des Gas und Wasserfaches e.V.). Until 2002 when new regulation has been passed, DVGW was the main directive for planning, construction, operation and maintenance of pipeline system. Now standard SIST EN 12007 is used in Slovenia which is completely in line with  CEN (standard of European Committee for Standardization).

The in-country review concluded that current emissions were far too low and needed to be further analysed and investigated for possible errors. After many consultations with expert from gas distribution companies we decided to use EF for transport and distribution which are presented in the paper from J. Reichert and M. Schoen: Methanemisionene durch den Einsatz von Gas in Deutchland von 1990 bis 1997 mit einem Ausblick auf 2010, Karlsruhe, Mai 2000. After uniting W and E part in one country, Germany was confronted with problem of reporting fugitive emissions from natural gas because two different types of standards were used in the history. Although Slovenia has never used east European standard and was in this and in many other fields more west oriented, we decided to use EF for E Germany for the base year and gradually lower this EF until present. Although standards were west European from beginning of our pipeline network we believe that emissions were higher in the base year as they are now.  In former Yugoslavia system of control wasn’t as rigorous as it is nowadays. Also the conscious of people involved in the building and maintenance of pipelines is on more high level in independent Slovenia as it was in the past. For this, mainly human and not legislation reason our emissions in the past were higher than now, but even in the worst case they were never so huge as GPG suggests. In GPG for former Yugoslavia the same EF are suggested as for former USSR, which had very different legislation and also different geographic and economic circumstances than Yugoslavia and particularly than Slovenia.

Figure 3.7: Network of pipes for transportation of natural gas.

Transport

For estimation of fugitive emissions from distribution of natural gas we were taking in account the following leakages:

  - from transport pipelines

  - from metering and regulation stations

  - from damaged pipelines

  - from shifting of pipelines because of highway building

  - from compressor station

Emissions are presented in the table below.

Table 3.54: Fugitive emissions from transport of Natural gas in 1000 Sm3 of CH4.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pipelines | 950 | 1007 | 864 | 515 | 465 | 420 | 379 | 343 |
| pneumatic stations | 124 | 124 | 165 | 120 | 109 | 98 | 87 | 66 |
| M&R stations | 198 | 187 | 132 | 0 | 0 | 0 | 0 | 0 |
| damaged pipelines | 2 | 3 | 2 | 30 | 30 | 30 | 5 | 5 |
| AC building |  |  | 60 | 60 | 60 |  | 60 |  |
| compressor station |  |  |  |  |  | 0.16 | 0.16 | 0.178 |
| TOTAL | 1274 | 1320 | 1222 | 725 | 664 | 548 | 531 | 413 |

Activity data

Data on the length of pipeline and some leakages  from 1986 to 1996 have been taken from the research project of the Institute of Energy Industries (Gasperič M., Dornik M.: Determining the CO2 Emission Factor in Energy Use and CH4 Emission Factor in Transport and Distribution of Natural Gas, Ljubljana: Institute of Energy Industries, 1998). From 1997 to 2004 data have been obtained directly from company Geoplin plinovodi.

Figure 3.8: Compressor station in Kidričevo.

Table 3.55: Length of transport pipelines in km.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| pipelines | 740 | 784 | 927 | 948 | 951 | 954 | 957 | 960 |

Company Geoplin plinovodi had 960 km of pipelines in 2004. They are build from longitudinally welded steel pipes, which are protected with anticorrosive isolative material and dug in the soil approximately 1.5 m deep. Main three branches are working under pressure of 50 bar thus one branch has pressure 67 bars.

As demand from natural gas is increasing, the compressor station in Kidričevo was start to work in 2002. Its maximum amount of compressed natural gas is 260.000 Sm3/h with 50 bars. In station there are two compressors both with 3.2 MW, but only one is operating in the time while the second is only for backup.

Emission factors

Emission factors for the emission of methane from the pipelines has been taken from German article. For emissions from pipeline for period 1986 to 1992 EF 1284 m3/km for east Germany has been used. From 1993 the 10% reduction of emissions annually has been taken in account.

Table 3.56: EFs for fugitive emissions from transport pipelines.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EF in 1000 Sm3/km | 1,284 | 1,284 | 932 | 544 | 489 | 440 | 396 | 357 |

All data on other losses from transport of natural gas have been obtained from company Geoplin Plinovodi. Losses from metering and regulation station are zero from 2000 on, because there is no mechanical metering stations any more. There was some losses because of landslides and building of highways. According to company measurements methane emissions from compressor station are negligible.

Distribution

For estimation of fugitive emissions from distribution of natural gas we were taking in account the following leakages:

  - from distribution pipelines

  - from metering and regulation stations

  - during construction of new pipelines and maintenance of existing

  - from inspection of gas meters

  - from service pipelines (pipelines between main distribution network and households)

Emissions are presented in the table below.

Methane emissions from distribution of natural gas in 1000 Sm3 CH4.

Table 3.57: Fugitive emissions from distribution of Natural gas in 1000 Sm3 of CH4.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| distribution pipelines | 1162.5 | 1055.0 | 913.1 | 698.1 | 656.2 | 618.1 | 587.7 | 552.6 |
| M&R stations | 7.5 | 9.7 | 12.7 | 24.6 | 26.9 | 27.7 | 29.3 | 30.5 |
| construction… | 0.4 | 0.5 | 1.3 | 2.7 | 2.8 | 3.0 | 3.7 | 3.3 |
| gas meters | 0.02 | 0.02 | 0.03 | 0.05 | 0.05 | 0.04 | 0.04 | 0.04 |
| service pipelines | 1278.5 | 1358.5 | 1233.2 | 1083.7 | 1031.8 | 984.8 | 954.3 | 907.4 |
| total | 2448.9 | 2423.7 | 2160.4 | 1809.2 | 1717.8 | 1633.6 | 1575.0 | 1493.8 |

Activity data

Activity data for distribution of natural gas have been taken from the research project, made by the Economic Interest Association of Natural Gas Distributors.  Data are including the following:

  - type and length of distribution pipelines,

  - number of M&R stations,

  - number and type of inspected gas meters

  - typical volume of gas meters

  - leakages during construction and maintenance work

  - type and length of service pipelines

Table 3.58: Length of distribution pipelines in km.

Table 3.59:  Number of meter-regulation stations

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| M&R stations | 37 | 48 | 63 | 122 | 133 | 137 | 145 | 151 |

Table 3.60:  Number of inspected gas meters

| gas meter | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| G4 | 9062 | 8414 | 12228 | 19992 | 21036 | 18355 | 16077 | 15162 |
| G6 DO G10 | 56 | 71 | 80 | 101 | 100 | 106 | 148 | 110 |
| over G10 | 110 | 135 | 231 | 381 | 378 | 312 | 399 | 416 |
| total | 9228 | 8620 | 12539 | 20474 | 21514 | 18773 | 16624 | 15688 |

A volume of gas meters are: type G4 has 0.002 m3, from G6 to G10 we have taken average of 0.006 m3 and for bigger meters 0.016 m3.

Table 3.61: Length of service pipelines in km.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PE and PVC | 46 | 95 | 251 | 756 | 858 | 964 | 1094 | 1203 |
| steel (old) | 98 | 117 | 117 | 92 | 85 | 78 | 71 | 64 |
| steel (new) | 0 | 0 | 4 | 9 | 9 | 11 | 16 | 19 |
| cast iron | 21 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |
| total | 166 | 212 | 373 | 858 | 952 | 1054 | 1181 | 1286 |

Emission factors

Emission factors for the emission of methane from the distribution pipelines has been taken from German article.  For emissions from pipeline for period 1986 to 1992 EF for east Germany has been used. From 1993 the 10% reduction of emissions annually has been taken in account.

Table 3.62: EFs for fugitive emissions from distribution (low pressure)  pipelines.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PE and PVC | 445 | 445 | 324 | 192 | 172 | 155 | 140 | 126 |
| steel (old) | 4945 | 4945 | 3605 | 2129 | 1916 | 1724 | 1552 | 1397 |
| steel (new) | 247 | 247 | 247 | 247 | 247 | 247 | 247 | 247 |
| cast iron | 8396 | 8396 | 6121 | 3614 | 3253 | 2928 | 2635 | 2371 |

For emission from M&R stations EF 202 m3/station has been applied for all years.

Considering emissions from gas meters we have assumed that during inspection all methane in the meter escape. Emissions were therefore calculated as product between number of inspected gas meters and volume depending on gas meter type.

We have also obtained data about leakages during construction and maintenance.

Emission factors for the emission of methane from the service pipelines has been taken from German article.  For emissions from pipeline for period 1986 to 1992 EF for east Germany has been used. From 1993 the 10% reduction of emissions annually has been taken in account.

Table 3.63: EFs for fugitive emissions from service pipelines.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PE and PVC | 2101 | 2101 | 1532 | 904 | 814 | 733 | 659 | 593 |
| steel (old) | 9890 | 9890 | 7210 | 4257 | 3832 | 3448 | 3104 | 2793 |
| steel (new) | 742 | 742 | 742 | 742 | 742 | 742 | 742 | 742 |
| cast iron | 9890 | 9890 | 7210 | 4257 | 3832 | 3448 | 3104 | 2793 |

Leakages from households

These are CH4 emissions which arise in the households sector during lighting of gas ranges.

Table 3.64: Methane emissions from gas used in households in 1000 Sm3 CH4.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| households | 271 | 308 | 433 | 523 | 517 | 515 | 496 | 474 |

Activity data

Activity data for emissions from this source are number of households using natural gas. This data have been obtained from research project, made by the Economic Interest Association of Natural Gas Distributors.

We have assumed that in the beginning only one appliance has been used in each household while in the 2005 there is approximately 1.8 appliances in household. This is very rough estimate but having no better data we taking in account this presumption.

Table 3.65: Number of households and appliances using natural gas.

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| households | 54,282 | 61,570 | 78,368 | 112,431 | 117,400 | 124,798 | 129,569 | 135,564 |
| appliances | 54,282 | 61,570 | 101,878,4 | 174,268 | 187840 | 205,917 | 220,267 | 237,237 |

Emission factor

Emission factor for the emission of methane from gas appliances has been taken from GPG, Table 2.18. We have used medium value 5 m3/appliance/yr for period 1986-1992 and low value 2 m3/ appliance/yr for 2004. Values between 1992 and 2004 were interpolated.

Table 3.66: Methane emission factors for fugitive emissions from gas used (households).

|  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Emission factor | 5 | 5 | 4.25 | 3 | 2.75 | 2.5 | 2.25 | 2 |

Future Improvements

No future improvements are planning for this category.

Recalculations

According to the findings in the in-country review, the applied emission factor for methane fugitive emissions from gas transport and distribution of natural gas is by an order of magnitude lower than the lowest applied emission factor among Annex I countries. Slovenia was therefore obliged to preparing a detailed study regarding national fugitive emissions from natural gas.

In present submission we have recalculated all emissions from transport, distribution and use of natural gas, using emission factors from German study and supplemented activity data from companies.

# INDUSTRIAL PROCESSES (CRF sector 2)

Industrial activities not related to energy, produce various GHGs emissions. Emission sources are industrial production processes in which raw materials are chemically or physically transformed. In this transformation, many different GHGs can be released, such as CO2, CH4, N2O, and PFCs. Some industrial sources also produce NOx, NMVOCs, CO, and SO2.

Some fluorinated compounds (HFCs, PFCs and SF6) are consumed in industrial processes or used in different applications as substitutes for ozone depleting substances (ODS). They have also been considered in the inventory.

Due to the intertwined nature of procedures in industry and characteristics of individual reported units, it is in certain cases difficult to distinguish if certain emissions originate from the consumption of fuels for energy purposes or from the consumption of raw materials in industrial processes. The main criterion is the purpose for which a raw material or fuel is used.

Emissions from industrial processes in Slovenia account for nearly 6% of total national GHG emissions, excluding LULUCF. They amounted to 1,288 Gg CO2 equivalent in the base year and to 1,135 Gg CO2 equivalent in 2004. The main source of emissions is mineral industry with 51.7% of emissions, followed by metal production with 36.6% of emissions. Significantly lower are contributions from chemical industry (4.2%) as well as from the consumption of F-gases (7.5%).

The main source of emissions from this sector is cement industry, which is responsible for 42.8% of GHG emissions. The second largest source is aluminium production with 29.1% of GHG emissions.

Figure 4.0.1: Process emissions of GHG from different types of industries in CO2 eq./year.

The in-country review of our 2003 inventory recommended that we should obtain data for period 1987-1989 and fill in CRF tables for that period as well. As we have not collected all necessary data yet, we were unable to fill in the CRF tables. But in some cases we already use those data in the NIR in our tables and graphs to make our time series more clear.

MINERAL INDUSTRY

## Cement Production

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 2.55 |  | 8 |
| 2004 | Level | CO2 | 2.43 | 0.32 | 7 |

### Source category

Carbon dioxide emissions arising in the production of cement are a major industrial-process source of emissions of greenhouse gases. There are two producers of cement in Slovenia, producing mostly Portland cement.

The basic raw material for the production of cement is marl, which is a homogeneous mixture of limestone and clay and which originated in past geological periods through sedimentation. As there is no longer enough natural marl for mass production, the cement production mix, which must contain 75-78% of calcium carbonate (CaCO3), is prepared by mixing limestone and clay components: from such with 35% of CaCO3 to limestone with more than 95% of CaCO3. The limestone, which is a source of CaO, normally has an admixture of dolomite, which introduces MgO into the system. Clay components are bearers of SiO2, Al2O3, and Fe2O3. Blast furnace slag, silica sand, bauxite, and gypsum are added to the homogenized mix during grinding.

Raw meal powder is fed into the cement kiln through a heat exchange unit. Natural gas, fuel oil, petroleum coke, coal dust, waste oils, and tyres are used as fuels in the clinker calcination process.

Carbon dioxide emissions from cement production result from the conversion of CaCO3, the main constituent of limestone, to lime (CaO), while CO2 as a by-product is let out into atmosphere.

Sulphur oxides emissions result from sulphur, which is present both in fuel and in some constituent materials such as clay. Contrary to what occurs with CO2, most of the SO2 that is formed during calcination will usually be absorbed and long-term immobilized in clinker and later in cement.

### Methodological issues

CARBON DIOXIDE EMISSIONS

Separate emissions are estimated from carbon originally present in fuel and carbon present in raw materials, although they are in fact emitted at same place and are inseparable in concept.

CO2 from carbon in fuel has been estimated from the fuel consumption for each fuel type. Emissions of this kind have already been included under source sector 1A2 - Energy Combustion in Industry.

Activity data are data on the annual production of clinker that were obtained from the Statistical Office of the Republic of Slovenia (DGP). For the purposes of emission trading, activity data to calculate process emissions in the production of clinker for the period 1999-2003 were obtained directly from both cement works. The differences in values from statistical data varied from 1.15% in 2001 to less than 0.01% in 2002. The necessity to ensure time consistency meant that emissions have been calculated from statistical data even for this period of time.

Since cement production is a key source category, according to the IPCC GPG, the Tier 2 method must be applied in calculating emissions. As mentioned, we obtained data that are more detailed for the period 1999-2003, which enabled us to determine our own emission factor. It turned out higher than that set by the IPCC guidelines, CO2 emissions are not increased by the CaCO3 in the raw material alone, but also by the presence of MgCO3.

The IEF average for the last five years is 541 kg CO2/t of clinker. As the location of quarries are the same as in the base year, we have applied this emission factor for calculating emissions for the whole period.

Carbon in cement kiln dust (CKD) is not accounted in emission calculation as in both cement factories CKD is returned in process.

Figure 4.1.9: Cement and clinker production in ton/year.

SO2 EMISSIONS

Emissions of SO2 in the production of cement arise from sulphur, which is present in the fuels, as well as from clay. Emissions of SO2 from the consumption of fuels are presented in the chapter on emissions from energy sources. The chapter on industrial processes presents emissions from the consumption of clay, which is calculated on the basis of produced quantities of cement. The data on the production of cement have likewise been obtained from the Statistical Office of the Republic of Slovenia (DGP). Emissions have been calculated by applying the recommended emission factor of 0.3 kg SO2/ton of cement, which is laid down in the IPCC Guidelines for National GHG Inventories, 1996.

Emissions of SO2 from 2001 on were reported on the basis of measurements that were performed in cement works and reported to EPER. Again, process emissions and emissions due to combustion of fuel were separated, Emissions of SO2 were significantly higher from those calculated according to the laid-down methodology, because one of the cement works uses raw materials excavated in its vicinity with a very high content of sulphur.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 2%.

Uncertainty of emission factor amounts to 2%.

### Source specific recalculations

No recalculation has been done for this source.

## Lime Production

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CO2 | 1.09 |  | 21 |
| 2004 | Trend | CO2 | 0.44 | 1.75 | 27 |

### Source category description

CO2 emissions from the production of lime are the third most important process-source of greenhouse gas emissions and they belong to the key source categories (level in base year and trend in 2004). In Slovenia, there are three lime producers.

Lime is generated by heating the input raw material, i.e. limestone, to high temperature (900-1200°C). During this process, limestone is converted into CaO and emits CO2.

### Methodological issues

CARBON DIOXIDE EMISSIONS

CO2 emission was calculated according to IPCC methodology. For this report the national emission factor was calculated and used for the first time. As mentioned, we obtained data that are more detailed for the period 1999-2003, but in case of lime this data was not as good as from cement producers. In March 2006 all operators of installations included in EU ETS had to submit the verified report for emissions during the preceding years to the competent authority (Ministry of Environment and Spatial Planning). These reports together with data for NAP 1 enabled us to determine our own emission factor for lime production. The limestone used for lime production has small amount of dolomite, which next to CaCo3 consists also of MgCO3. We have considered both carbonates as input taking into account that calcination in the kiln is not full. We have estimated country specific EF to be 749 kg CO2/ton of lime and  applied this emission factor for calculating emissions for the whole period.

Activity data are data on annual production of raw lime, obtained from the Statistical Office of the Republic of Slovenia (DGP). For the purposes of emission trading, to calculate process emissions, activity data on the production of lime for the period 1999-2003 were obtained directly from producers. The values differed from statistical data by as much as 12.23% in 2000 to as little as 0.74% in 2001. As in the production of cement, the need to ensure time series consistency called for all emissions to be calculated from statistical data.

SO2 EMISSIONS

The methodology for calculating SO2 emissions from the production of lime is not given in IPCC guidelines. It is recommended not to calculate emissions until a more detailed methodology is ready.

Figure 4.2.10: Lime production in ton/year.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 10%.

Uncertainty of emission factor amounts to 5%.

### Source specific recalculations

All emissions in the reporting period have been recalculated using CS EF.

### Source-specific planned improvements

We will investigate possibilities to obtain detailed data on the consumption of lime in construction and subtract that emission from the total emission arising in the production of lime. In construction, lime is used to produce mortar, and it is also used for whitewashing. In drying, a reverse process to that in the production occurs, since CaO binds CO2 and converts to CaCO3. It is estimated that construction consumes between 10 to 20% of all lime.

## Limestone and Dolomite Use

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

Limestone and dolomite are used in many industries. During heating to high temperature, carbonates convert to oxides, emitting CO2. Most limestone and dolomite are thus consumed in the production of cement and lime, as described above. Along with other carbonates, they are also used in the production of metals and mineral products. The major sources of such emissions in Slovenia are the production of iron and steel and glass industry.

### Methodological issues

CARBON DIOXIDE EMISSIONS

CO2 emission has been calculated according to IPCC methodology. Default emission factors, 440 kg CO2/ton limestone, and 477 kg CO2/ton dolomite have been applied. Since no detailed data on the purity of input raw materials were available, the default factor for fractional purity (f) has been applied as well.

The Statistical Office of the Republic of Slovenia has been until 1997 gathering data on dolomite consumption and reporting it entirely under Non-Metal Processing (glass), while gathering and reporting all data on lime consumption as consumption in construction (cement and lime production). After that year no statistical data on limestone and dolomite consumption were available, therefore those data were obtained directly from iron, steel, and glass producers.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor amounts to 20%.

### Source-specific planned improvements

In order to calculate emissions for obtaining carbon coupons for emission trading, we acquired data on the consumption of limestone, dolomite, and other carbonates as well as data on fractional purity for these types of production as well as for the production of bricks and tiles.

If it should prove possible to obtain such data for the entire period, they will be taken into account in the future submission.

## Soda ash production and use

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

Soda ash (Na2CO3) is used as a raw material in numerous industrial processes: production of glass, soap and detergent, production of paper. CO2 emissions arise both in production as well as in consumption of soda ash. Production of soda ash in Slovenia is negligible; consequently, in this chapter only emissions arising in its consumption will estimated.

### Methodological issues

Emissions from consumption have been calculated according to IPCC methodology, applying an emission factor of 415 kg CO2/ton of Na2CO3.

Until 1997, consumption data were obtained from the Statistical Office of the Republic of Slovenia (DGP), later these data were not available anymore, and were therefore calculated from the data on import and export (http://bsp1h.gov.si/D2300.kom/komstart.html), and production (DGP). Stock changes were disregarded; however, it is our estimate that the error is not substantial.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor amounts to 20%.

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

## Production and use of Miscellaneous Mineral Products

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

There are no direct CO2 emissions in this category; however, there are emissions of indirect GHGs and SO2. This report covers emissions from the following sources: asphalt roofing production, road paving with asphalt and glass manufacturing.

### Methodological issues

Asphalt roofing production

This group includes production of asphalt roofing, which is mainly used for roof tiling and other construction work. Emissions are estimated on the basis of total annual produced quantities, obtained from the Statistical Office (DGP).

IPCC guidelines specify two processes and two different emission factors in the production of asphalt roofing: saturation with spray and saturation without spray. Herein the calculations of CO and NMVOC emissions are based on emission factors for saturation without spray.

IPCC guidelines also specify that emissions of NMVOCs arise also in the process of asphalt blowing and that it may be assumed that all of the asphalt used for non-paving use will be blown. The recommended emission factors range from 0.1 to 30 kg NMVOCs/ ton of bitumen products and depend on the process applied. In this report, the arithmetic mean has been taken and the emission factor of 15 kg NMVOCs per ton of asphalt roofing has been applied.

Road Paving with Asphalt

Road paving with asphalt is used for paving roads and other surfaces. Emissions of greenhouse gases arise in asphalt plants during manufacture, road-surfacing operations, and subsequently from the road surface. The applied emission factors for NMVOCs are taken from IPCC guidelines for emissions from the production of asphalt and the road surface.

As for the emission factor for NMVOCs from the road surface, an error has occurred in IPCC guidelines: the suggested emission factor is stated as 320 kg NMVOCs/ton of road surface. The correct value is 320 g NMVOCs/ton of road surface.

IPCC guidelines also give emission factors for emissions of NOx, SO2, and CO from production of asphalt roofing.

Activity data have been taken from the Statistical Office of the Republic of Slovenia (DGP).

Production of glass

When collecting data for NAP 1 we obtain also data about carbonate use in glass production.

As emissions from limestone and dolomite use are reported in that sub sector and soda ash use in separate sub sector, only very small amount of carbonates used in glass production are remaining to be included in this sector. Those carbonates are: potash (K2CO3) and barium carbonate (BaCO3). For  calculation the CO2 emissions the stoichiometric emission factors have been used (0.318 and 0.223 t CO2/t of carbonate respectively).

As we investigated glass production in our country, we realised that in Slovenia no substances are used which lead to NMVOC emissions.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor amounts to 50%.

### Recalculations

NMVOCs emissions oxidise in a relatively short period and therefore in Consumption of Coatings and in Industrial Processes. In previous submissions NMVOCs emissions have been transformed to CO2 emissions. In these two chapters there was deliberate double counting of emissions. A mass unit of NMVOCs contributes 3.19 mass units of CO2.

During review process we couldn’t make any explanation how this factor was derived. As we also didn’t find any method for calculation of CO2 from NMVOC in IPCC guidelines, we excluded this source from whole inventory for all period.

### Future improvements

No improvements is planned for this category.

CHEMICAL INDUSTRY

## Nitric acid Production

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

The production of nitric acid (HNO3) generates nitrous oxide (N2O) as a by-product of the high temperature catalytic oxidation of ammonia (NH3). In Slovenia, nitric acid is produced since 1997, the quantities produced are small, and N2O emissions are insignificant.

### Methodological issues

NITROUS OXIDE EMISSIONS

Emissions have been estimated according to IPCC methodology, applying an emission factor of 5.5 kg N2O/ton nitric acid. Input data have been obtained in the Statistical Office of the Republic of Slovenia (DGP).

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

## Carbide Production

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

There is only one carbide producer in Slovenia that continues to produce calcium carbide, and has discontinued the production of silicon carbide in 1995. CO2 emissions arise both in the production of calcium carbide as well as in its consumption. Calcium carbide (CaC2) is produced by heating calcium carbonate and subsequently reducing CaO with carbon. Both steps lead to emissions of CO2. In Slovenia, calcium carbide is not produced from limestone but from lime, hence CO2 emissions arise only in the reduction with carbon. CO2 emissions arise also in the consumption of calcium carbide.

Silicon carbide

In the production of silicon carbide (SiC), CO2 is released as a by-product. Petrol coke is used as a source of carbon. Data on the consumption of petrol coke in the production of silicon carbide were provided by the enterprise Treibacher Schleifmittell d.o.o. (Nekrep 1999). Consumption of petroleum coke was reported in Tovarna dušika Ruše. From 1995 on there was no production of silicon carbide in Slovenia. The emission has been estimated on the basis of IPCC methodology using input data of the Treibacher Schleifmittel d.o.o. (Nekrep 1999) about petrol coke used and carbon content in coke and product.

Methane emissions are calculated using suggested EF 10.2 kg/t petrol coke.

### Methodological issues

CARBON DIOXIDE EMISSIONS

IPCC guidelines specify that the production process using reducing agent emits 1090 kg CO2/ton of calcium carbide.

Figure 4.7.11: Production of calcium carbide according to data from the Statistical Office and according to data from producer in ton/year and use of calcium carbide.

The input data were the data on annual production of calcium carbide. Until 1997, those data were obtained from the Statistical Office of the Republic of Slovenia (DGP), but then SURS changed their methodology of gathering and presenting carbides, and therefore production data were obtained from the producer for the entire period.

Emission arises also in consumption. IPCC Guidelines recommend applying an emission factor of 1100 kg CO2/ton of calcium carbide.

Data on consumption were until 1997 obtained from The Statistical Office of the Republic of Slovenia (DGP), later those data were not available anymore, and therefore we calculated them from data from Statistical Office of the Republic of Slovenia on import and export (http://bsp1h.gov.si/D2300.kom/komstart.html) and production (DGP). Doing so meant ignoring stock changes. For these reasons, consumption in individual years changed a lot. To obtain smoother time series, consumption was estimated using moving averages.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor amounts to 20%.

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

## Production of Other Chemicals

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

This chapter describes sources of other greenhouse gases, ozone precursors, and SO2. In Slovenia, there are no other industrial sources of N2O besides those described above, while methane arises in the production of methanol.

The only ozone precursor that arises in Slovenia in chemical industry is NMVOC. It arises in the production of formaldehyde, paints, lacquers, adhesives, and rubber as well as in the production and processing of various plastics.

The source of SO2 is the productions of sulphuric acid and titanium dioxide.

### Methodological issues

METHANE EMISSIONS

The source of emissions is the production of methanol. In calculating emissions, the emission factor of 2 kg CH4/t methanol was applied.

Production data have been taken from the Statistical Office of the Republic of Slovenia (DGP).

NMVOC EMISSIONS

This includes emission from adhesives, lacquers and enamels, rubber products, PVC and other plastics as well as formaldehyde. The emission factors for formaldehyde and polypropylene have been taken from IPCC Guidelines. Other emission factors have been obtained from the EMEP/CORINAIR Guidebook. In calculating emissions from all other sources, the emission factors of 10 kg NMVOC/ton product and for processing plastics, 5 kg NMVOC/ton product have been applied.

The quantity of annual production was obtained from the Statistical Office of the Republic of Slovenia (DGP).

SO2 EMISSIONS

Emissions of SO2 in the Other Chemicals category arise in the production of sulphuric acid (H2SO4) and titanium dioxide (TiO2). The emission factors applied have been taken from IPCC guidelines.

For the 2000 - 2004 period, the reported SO2 emissions were based on measurements that were performed and reported to EPER.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor for CH4 amounts to 20%

Uncertainty of emission factor for NMVOC amounts to 50%

Uncertainty of emission factor for SO2 amounts to 30%

### Recalculations

NMVOCs emissions oxidise in a relatively short period and therefore in Consumption of Coatings and in Industrial Processes. In previous submissions NMVOCs emissions have been transformed to CO2 emissions. In these two chapters there was deliberate double counting of emissions. A mass unit of NMVOCs contributes 3.19 mass units of CO2.

During review process we couldn’t make any explanation how this factor was derived. As we also didn’t find any method for calculation of CO2 from NMVOC in IPCC guidelines, we excluded this source from whole inventory for all period.

### Future improvements

No improvements is planned for this category.

METAL PRODUCTION

## Iron and Steel Production

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

In Slovenia, there are three iron and steel producers. Primary production from ore existed only in the 1986 base year, and after 1990, steel production is based on utilization of scrap iron and steel.

Figure 4.9.12: Production of steel in ton/year.

The production of steel is a multiphase process, and some phases give rise to emissions of CO2. Most emissions occur in smelting iron scrap in EAF (electric arc furnace). The furnace is first filled with steel scrap, and then limestone and/or dolomite are added to allow the slag to form. The furnace utilizes electric heating through graphite electrodes. For increased productivity in the initial phase of melting, oxygen lances and a carbon injection system are used. From a metallurgical point of view, oxygen is used to reduce the carbon content in the molten metal and for removing other undesired elements.

Decarburising is performed also in secondary phases in a ladle furnace.

### Methodological issues

CARBON DIOXIDE EMISSIONS

CO2 emissions originating in the consumption of limestone and dolomite have been dealt with in chapter Mineral Industry, as laid down by the IPCC Guidelines.

In the 1986 base year, when pig iron was still produced, the disaggregation into the consumption of fuel as an additive and the consumption of fuel as an energy product was impossible. For consumption of coke, the decision was taken to attribute all coke, which is consumed in the production of iron and steel, to the energy sector as fuel consumption.

As now it is more appropriate to include the coke consumption among process emissions, this was done for the entire period 1990 – 2004.

Here, as a source of process emission, the consumption of graphite electrodes is also presented.

As mentioned, we obtained data that are more detailed for the period of the last five years, which enabled us to determine our own emission factor.

Figure 4.9.2: Consumption of coke and graphite electrode in comparison of steel produced in the years 1999-2004.

Data on the amount and carbon content in input and output material were obtained from producers. In our case, input materials were mostly coke (including FAT coke), graphite electrodes, and scrap iron. Using mass balance approach, we have calculated CO2 emissions and the average EF for this period has been 47 kg CO2/t of steel. This emission factor has been applied for the period 1990-1998. For the last five years, actual data were taken.

This CS EF is not appropriate for the base year because of the different type of production of steel (from ore).

OTHER EMISSIONS

IPCC guidelines mention emissions of SO2, NOx, CO, and NMVOCs from production of steel. However, the methods have not been finalized yet and have therefore not been taken into account here.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 10%.

Uncertainty of emission factor amounts to 10%

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

## Ferroalloys Production

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

Iron is smelted with other elements, such as silicon, manganese, chromium, molybdenum, vanadium, or tungsten, forming alloys that have specific material characteristics requirements. Usually, alloy formation occurs in electric arc furnaces (EAF) and, as described for steel production, carbon monoxide and carbon dioxide emissions occur from oxidation of carbon that is still present in coke - used as raw material - and from the consumption of graphite electrodes.

Slovenia has one producer of ferroalloys, producing mostly FeSi and FeSi inoculants, SiCa, as well as some other ferroalloys.

Figure 4.10.13: Ferroalloys production in ton/year.

### Methodological issues

CARBON DIOXIDE EMISSIONS

CO2 emissions have been calculated from the consumption of fuels and anodes. Like in the production of steel, the presentation of fuels was split, namely by considering emissions from consumption of coal and natural gas in Energy Sector/Consumption of Fuels in Industry, and other fuels in process emissions. CO2 emissions from the consumption of wood chips, which are biomass, have not been added to total emissions. Emissions have been calculated according to IPCC methodology by applying the defined emission factors.

Input data on fuel consumption for the entire period have been obtained from the producer, The producer has also supplied data on the quantities and type of ferroalloys and has thus enabled us to verify them by calculating emissions in accordance with another method. A comparison of the two methods has yielded very similar results.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor amounts to 20%

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

## Aluminium Production

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | no | CO2 | 0.44 |  | 29 |
| 2004 | Level, Trend | CO2 | 1.06 | 1.65 | 16 |
| Base Year | Level | PFC | 1.37 |  | 17 |
| 2004 | Level, Trend | PFC | 0.60 | 2.07 | 24 |

### Source category description

Aluminium is produced in two phases. Firstly, alumina (Al2O3) is extracted from bauxite ore. Aluminium is then produced in the second phase in an electrochemical process in the electrolysis cells, where alumina disintegrates into its components: aluminium and oxygen. Molten aluminium gathers at the cathode while oxygen reacts with carbon in the anode, causing the consumption of anodes, which have to be replaced.

Beside CO2, PFCs arise in the production of aluminium. This occurs during anode effect when the alumina content of the electrolyte falls below 1-2% and a gas film is formed at the anode. This stops the production of the metal and increases the cell voltage. Factors that influence the generation of PFCs are the frequency and duration of anode effects and the operating current of the cell.

In Slovenia, there is one aluminium producer. It is the second largest source of technological emissions of GHG. Since the base year, the production of aluminium has undergone numerous modernisations, resulting in reduced GHG emissions from this source in spite of increased production.

In 1986, Talum had two electrolysis units, A and B, both using Søderberg anode reduction cells. The annual production of aluminium in electrolysis unit A amounted to 21220 t, in electrolysis unit B to 23180 t, the total annual production amounted to 44400 t of aluminium. In 1986, the production of aluminium included the production of alumina, but that was discontinued in 1991 for reasons of economy and ecology, and since then alumina has been purchased on foreign markets. In 1991, the production in electrolysis unit A was discontinued as well.

In 1988, the first phase of the modernization of aluminium production has been carried out and concluded in 1999: a new electrolysis unit C with an annual production capacity of 40,000 t of aluminium has been built and its electrolysis technology was taken from Aluminium Pechiney.

Simultaneously, reduction cells in unit B were reconstructed to use prebaked anodes, thus increasing its annual production capacity to 35,000 t of aluminium.

The first phase of modernisation of aluminium production in electrolysis units was also a technological improvement in terms of reducing the number of anode effects per pot/day as well as their duration, which is a great contribution towards reducing GHG emissions as well as the consumption of electric energy: the new electrolysis unit C, featuring computerised voltage control and alumina feeding has on the average 0.2 anode effects per pot day, the upgraded electrolysis unit B (Søderberg anodes replaced by prebaked anodes) operates on the average with 2 anode effects per pot day.

Towards the end of the year 2000, Talum embarked upon the implementation of the second phase of upgrading the aluminium production that includes the construction of the second half of the electrolysis unit C with an annual production capacity of 40,000 t of aluminium. In 2004, the annual production of aluminium amounted to 120,666 t.

### Methodological issues

CARBON DIOXIDE EMISSIONS

Figure 4.11.14: Primary aluminium production in ton/year and a consumption of anodes in ton/year.

Data on emissions of GHGs and other gases are on a regular basis submitted to us by the Talum expert service. Data gathered by the Statistical Office of the Republic of Slovenia are unusable for our purposes since they include the entire Slovenian production of aluminium and not only the primary production. CO2 emissions are most precisely estimated from the consumption of anodes. Their consumption in 2004 amounted to 431 kg/ton Al. The emission factor is the same as recommended by IPCC, namely 3.6 t CO2/ton anodes.

CO2 emission includes anode burn-off from anode baking.

Implied EF in the year 2003 was 1.74 t CO2/t Al.

PFC EMISSIONS

Data on emissions of CF4 and C2F6 have likewise been obtained from Talum. Talum calculates emissions of both gases from its anode-effect inventory. The CF4 emission factor kept falling from the base year till now from 0.74 kg CF4/ton Al to 0.13 kg CF4/ton Al, which is a logical consequence of the modernisation of its technology. The C2F6 emission factor fell from 0.07 kg C2F6/ton Al in the base year to 0.014 kg C2F6/ton Al today.

Figure 4.11.15: CF4 and C2F6 emitted during anode effect in aluminium production in Gg CO2 eq./year.

OTHER EMISSIONS

Emissions of NOx and CO have been calculated on the basis of the data on the production of aluminium. The CO emission factor amounts to 129 kg CO/ton Al, the NOx emission factor to 0.15 kg NOx/ton Al, SO2 emissions have been measured. The re-calculation gives an emission factor of 16.6 kg SO2/ ton Al for electrolysis and 0.4 kg SO2/ ton Al for anode baking.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 10%.

Uncertainty of emission factors amounts to 10%

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

## Other Metal Production (IPCC: 2 C 5)

IPCC guidelines include processes in the production of other metals that may cause emissions of CO2 because of the consumption of reducing agents. However, emissions in these processes are low and dependent on the utilized processes. This is particularly the case for emissions from the production of chromium, lead, magnesium, nickel, silicon, titanium, and zinc. IPCC guidelines do not recommend any emission factors for calculating emissions from production of other metals.

OTHER PRODUCTION

## Pulp and Paper Industries

### Source category description

The production of paper may involve any of the following production processes: kraft pulping, acid sulphite pulping, and neutral sulphite semi-chemical process. The most widely used process is the kraft pulping process. There is only one pulp producer in Slovenia.

### Methodological issues

OZONE PRECURSORS and SO2 EMISSIONS

To calculate emissions of ozone precursors, emission factors recommended in IPCC Guidelines for pulp production according to kraft pulping process have been applied, SO2 emissions have been obtained on the basis of reports on measurements for EPER.

Activity data have been taken from the Statistical Office of the Republic of Slovenia (DGP)

## Food and Drink

### Source category description

In the production of food and drink, emissions of NMVOCs arise. Emissions from the following types of production have been calculated: production of alcoholic beverages, production of bread and pastry, production and processing of meat and fish, production of sugar, production of roasted coffee, production of margarine and cooking fats and production of animal feed.

### Methodological issues

NMVOC EMISSIONS

The emission factors used in calculating the emissions have been recommended in the National Emission Inventories (Rode 1998. p. 3) and IPCC guidelines (IPCC 1996). For emissions of NMVOCs in the production of wine, the arithmetic mean of emission factors for white wine and red wine, for spirits the emission factor for drinks classified as »brandy« has been applied.

Activity data have been taken from the Statistical Office of the Republic of Slovenia (DGP)

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor amounts to 50%

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

## Emissions Related to Consumption of Halocarbons and Sulphur Hexafluoride

### Source category description

Scope of application of HFC

HFCs began to be used in Slovenia in 1993 as a substitute for CFCs, which are ozone-depleting substances. They asserted themselves in particular in those fields where no other, more appropriate alternatives (e.g. hydrocarbons, CO2 etc.) were available.

The Slovenian chemical industry does not produce HFCs/PFCs and therefore these substances are imported. Major users generally import them on their own, lesser users buy them from distributors. Minor quantities will occasionally be re-exported.

HFC-134a began to be used in the production of household refrigerating/freezing appliances in 1993. It became predominant in two areas, the production of insulating foam and as a refrigerant. In 1995, Gorenje, the biggest Slovenian producer of refrigerating/freezing appliances, started to use cyclopentane in the insulation of appliances and discontinued using HFC-134a for this purpose in 1996. The other Slovenian producer of refrigerating/freezing appliances, LTH, did not use HFC-134a for insulating household appliances. The use of HFC-134a as a refrigerant started towards the end of 1993 only to become partly replaced by isobuthane already in 1995. Both refrigerants continue to be used in the production of household refrigerating/freezing appliances today. In estimating emissions, it was assumed that foreign producers began to introduce replacements for ozone-depleting CFCs at the same time as Slovenian producers, since they were predominantly from EU countries (Electrolux, Bosch-Siemens, etc.) and that the structure of their appliances was similar to those of Gorenje.

HFC-134a asserted itself as a substitute for CFC refrigerants also in commercial and industrial refrigerating equipment, air conditioners and in the production of mobile air-conditioning units. Abandoning the application of CFCs in accordance with the Montreal Protocol, in commercial and industrial refrigerating mixes of HFCs, such as HFC-404a, HFC-407c in HFC-507 began to be used.

Commercial and industrial refrigerating equipment includes refrigerated counters, cabinets, show-cases, ice machines, shock freezers, equipment for refrigerating fluids for varied purposes, all the way to custom-made industrial equipment for the food-processing, pharmaceutical, chemical and other process industries. The biggest producer of commercial equipment is LTH. This refrigerating equipment is of compact execution, includes a refrigerant, and is exported as such. Other producers predominantly carry out the filling at the moment of installation and their equipment is mostly exported without refrigerant. Consequently, in estimating emissions only the export of refrigerants in equipment produced by LTH has been taken into account and based on the market share of LTH the imports of HFCs in commercial equipment have been estimated.

There is no production of compact industrial refrigerating equipment in Slovenia, only installation of midsize and large equipment, either from domestic production or imported, mostly not containing a refrigerant. Filling is performed at installation time. As a substitute for CFCs, HCFC-22 is used, while HFCs will gradually substitute CFCs and HCFCs. In estimating emissions, it was assumed that there is no import or export of HFCs in products.

Small-size air conditioners for general consumption, e.g. window- or wall-mounted air conditioners, are filled with refrigerant at the time of assembly. There is no production of air conditioners in Slovenia; their sale on the Slovenian market may be equalled to their import. The number of importers is considerable, over 100. For a long time, only HCFC-22 was used and import of air conditioners with HFC began in 2000.

A/C systems in motor vehicles are filled with refrigerant at the moment of their installation into a vehicle. Car air conditioners are usually installed during vehicle assembly, although retrofitting is possible. HFC-134a began to be used in Slovenia in 1994, but some imported vehicles have been equipped with such air conditioners already since 1991. In the production of buses, HFC-134a began to be used in 1996. The production and sales of cars with air conditioning has risen sharply in recent years.

HFC-134a is used also in the production of polyurethane foam for the footwear industry (polyurethane shoe soles) and PU assembly foams, which are used in construction industry. The finished products, PU shoe soles, do not contain HFC, since the emission of HFC in the production of these foams amounts to 100%.

HFC is used for the production of PU assembly foams for export (Germany, Italy). All HFC used for the production of PU assembly foam is exported in products. Foams produced for the Slovenian market use hydrocarbons (propane, butane) as propellant or blowing agent. Imported products also contain hydrocarbons, not HFCs; there is no HFC in the imported products. Other users of HFC were not recorded, producers of soft PU foams, building boards, and polyethylene insulation foams have replaced CFC by introducing other substances, for instance hydrocarbons, CO2, HCFC, etc.

Fire extinguishing agents, that are used generally, include dust, CO2, or water. However, halon-filled systems are still in existence and they will be gradually replaced. It is supposed that halon systems will be allowed to be used until the end of their lifetime. They will be replaced by various alternatives, such as HFC-227ea.

The use of HFC and PFC as propellant in sprays has not been recorded. In sprays, CFC propellants have been replaced by hydrocarbons, partly also by HCFCs.

HFCs/PFCs had not become widely used as solvents; HCFCs and other substances have substituted CFCs.

Scope of application of SF6

SF6 is used as a soundproofing agent in the production of multiple glazing and at the same time as an insulating agent and fire-extinguishing agent in electrical installations, in middle voltage and high voltage (110 kV and 400 kV) gas insulated switchgear and circuit breakers (HV equipment).

Since there is no production of SF6 in Slovenia, it is either imported as gas or as a constituent of products or equipment.

SF6 is used by manufacturers of multiple glazing or windows. In Slovenia, it began to be used for this purpose in 1990. There are no data about emissions but experts judgements are that they are negligible. Glasses filled with SF6 are sold on the domestic market for subsequent production of windows and doors, to be sold on the domestic market or exported. The majority of these products are exported, as at present the demand for soundproof windows filled with SF6 on the Slovenian market is very minor. Considering the fact that this product is mostly destined for export, there are no emissions from this source in Slovenia.

Figure 4.15.16: HFC and SF6 emissions from different sources in CO2 eq./year.

Electric distribution companies also use SF6. Gas (SF6) insulated switchgear and circuit breakers were first used in Slovenia in 1976. The trend is on the increase, and particularly after 1993, the use of equipment with SF6 as insulating gas has increased strongly. This type of equipment is not produced in Slovenia and there is no export of SF6 in equipment.

The use of SF6 for first filling of tyres on motor vehicles has not been recorded.

### Methodological issues

HFC EMISSIONS

The research project that covered the use of HFC for the period 1995-1997 has calculated potential emissions according to Tier 1a and 1b methods, as well as actual emissions according to Tier 2 method.

For subsequent years, only actual emissions have been calculated. Emissions of substances have been estimated according to individual product categories.

Refrigerants and AC

Emissions of HFC refrigerants are calculated according to the following equation:

Et = Eassembly. t + Eoperation. t + Edisposal. t

where:

E assembly, t = E charge, t * (k/100)

| E assembly, t | = | Emissions during system manufacture/assembly in year t |
| --- | --- | --- |
| E charge, t | = | The amount of HFC refrigerant charged into new systems in year t |
|  | = | Production/assembly losses (%) |

In calculating emissions, all systems produced in the country are considered, regardless of whether they were produced for the domestic market or for export. Systems that are imported precharged are not considered in calculation.

E operation. t = E stock. t * (x/100)

| E operation. t | = | Amount of HFC emitted during system operation in year t |
| --- | --- | --- |
| E stock. t | = | Amount of HFC stocked in existing systems in year t |
| x | = | Annual leakage rate (in per cent of total HFC/ charge in the stock) |

In calculating emissions, all systems in operation have been considered, regardless of whether they were of domestic origin or imported.

E disposal, t = Ez charge (t-n)*(y/100)*(100-z)/100

| E disposal, t | = | Amount of HFC emitted at system disposal in year t |
| --- | --- | --- |
| Ez charge (t-n) | = | Amount of HFC refrigerants initially charged into new systems installed in year (t-n) – initial charge |
| n | = | Average equipment lifetime (years) |
| y | = | Amount of HFC/PFC in systems at time of disposal in per cent of initial charge |
| z | = | Amount of HFC/PFC recovered in per cent of actual charge (“recovery efficiency”) |

In estimating the initial charge of refrigerant, both systems produced for the domestic market and systems imported precharged have been taken into account. Systems made for export have not been considered.

Emissions have been calculated from acquired data on production, import and export as well as charging and our own estimates.

In estimating emissions from industrial equipment, the approach chosen was to start from the data from the Statistical Office of the Republic of the Slovenia (SY) on installed refrigerating units and the estimated charge of 1.5 – 2 kg of refrigerant per installed kW. In determining individual factors (assembly losses, leakage...). factors given in the IPCC methodology were applied, while considering the experiences of domestic experts.

As stated in previous chapters, in Slovenia there are producers of household refrigerating/freezing appliances, commercial equipment, and vehicles (passenger cars, buses) with A/C units. For industrial equipment, it was assumed that equipment is charged with refrigerant at assembly. It such cases emissions occur in manufacturing/assembly. Air conditioners for general consumption are imported charged; consequently, there are no emissions in manufacturing.

Leakages occur in operation of all refrigerating equipment, particularly in mobile air conditioning in vehicles and industrial equipment.

Emissions occur at the disposal of equipment. It was considered that equipment lifetime of refrigerating units and air conditioners, except in vehicles, amounted to 15 years. For car air conditioners, it was assumed that their lifetime amounted to 12 years. Since the production of equipment using HFCs began only at the end of 1993, in the period until 2004, emissions at disposal or discontinued operation were not expected.

There were no significant changes in the production emission between 1995 and 1997; however, the consumption emission keeps rising, mostly due increased consumption of cars with air conditioning. Owing to this, the total emissions are rising.

Table 4.15.1: Emission factors, used in refrigeration and A/C appliances.

|  | production | use |
| --- | --- | --- |
| Domestic refrigeration | 2 | 1 |
| Commercial refrigeration | 3 | 5 |
| Industrial refrigerations | 4 | 15 |
| Stationary AC | 2 | 1 |
| Mobile AC | 0.5 | 20 |

Foams

In calculating HFC emissions for blowing foams the following formula was applied:

Et = Eproduction, t + Eoperation, t + Edisposal, t

where:

Eproduction, t = Econsumption, t * (k/100)

| Eproduction, t | = | Emissions in production in year t |
| --- | --- | --- |
| Econsumption, t | = | Amount of HFC used for foam production in year t |
| K | = | Fraction loss during production (%) |

Eoperation, t = Estock, t * (x/100)

| Eoperation, t | = | Amount of HFC emitted from foam during operation in year t |
| --- | --- | --- |
| Estock, t | = | Amount of HFC in used products in year t |
| X | = | Annual loss in operation of products (%) |

Edisposal, t = Econsumption (t-n)*(y/100)

| Edisposal, t | = | Amount of HFC emitted at disposal of blown foam in year t |
| --- | --- | --- |
| Econsumption (t-n) | = | Amount of HFC used for production of foam in year (t-n) |
| N | = | product lifetime (in years) |
| Y | = | Share of HFC in products to be disposed of in % of the amount used for their production |

Emissions of HFC in the manufacturing of insulating foam for household refrigerating/freezing appliances amount to 3 to 5 %, for calculations we have used average value 4%. We assumed that there are no emissions from the insulation foam (close cell) during operation of household refrigerating/freezing appliances and that emissions occur at the time of disposal of appliances on solid waste disposal sites (100 %) at the end of their lifetime.

Emissions in the production of PU shoe soles amount to 100% and therefore the finished products do not contain HFCs. There are no emissions during their use and disposal.

Emissions during the production of PU assembly foams amount to less than 1%. HFC performs the function of propellant and blowing agent. Part of HFC is emitted during application of the product, for instance during installation of windows or doors, within the time of a year, but a part of HFC remains in the foam and is probably slowly released during the following 20-25 years. Considering the fact that this product is entirely destined for export, there are no emissions from application of the product on the domestic market and emissions arise in the importing countries.

In the period 1995-1997, the actual emission from blowing foam diminished because the use of HFC for PU insulation in household refrigerating/freezing appliances was discontinued.

Fire Extinguishing Agents

In calculating emissions of HFC/PFC, IPCC methodology and the therein-stated assumption that emissions amount to 35% of the quantities used in new stationary systems were applied. This assumption is derived from experience with the use of halon systems and is supposed to be appropriate also for estimating HFC emissions, unless there is a requirement to replace the existing systems.

It was assumed that halon systems in Slovenia would be allowed to be used until the end of their lifetime; replacement would take place gradually and various substances would be used as alternatives, among them HFC.

SF6 EMISSIONS

In 2006 a research covering all high-voltage equipment has been done and a estimation of SF6 emissions in the period 1986-2005 have been improved.

Emissions have been calculated according to Tier 3b method. where

Et = Emanufacturing, t + Einstallation, t + Euse, t + Edisposal, t

As production of SF6 in Slovenia doesn’t exist and until now no equipment including SF6 has been disposed, emissions occurred only during installation and use.

Equipment installation emissions have been estimated by subtracting the nameplate capacity of all new equipment filled from the actual amount of SF6 used to fill new equipment.

Equipment use emissions are determined by the amount of SF6 used to service equipment. SF6 which has been recovered from equipment before servicing and returned after servicing is not included in the estimate.

Activity data and emissions have been obtained from all users of HV equipment and are presented in the paper: Izdelava evidenc emisij žveplovega heksa-fluorida (SF6) v slovenskem elektroenergetskem sistemu (EES) za obdobje 1986-2005, available only in Slovenian language.

Table 4.15.2: Amount of SF6 emissions in high-voltage equipment.

|  |  | 1986 | 1990 | 1995 | 2000 | 2001 | 2002 | 2003 | 2004 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| total amount of SF6 | kg | 4666 | 4922 | 5492 | 10640 | 11460 | 12863 | 14179 | 16774 |
| new amount of SF6 | kg | 10 | 0 | 54 | 1318 | 820 | 1403 | 1316 | 2595 |
| installation emissions | % | 1.0 |  | 1.7 | 0.1 | 0.6 | 0.4 | 0.1 | 0.1 |
| use emissions | % | 9.2 | 8.8 | 8.8 | 6.2 | 5.8 | 5.6 | 5.3 | 4.5 |
| total emissions | kg | 429 | 431 | 482 | 659 | 674 | 725 | 750 | 766 |

### Source-specific planned improvements

Calculating emissions from the consumption of HFC is difficult, particularly in terms of data that are needed for the calculation. Data on the usage of these gases have not been gathered in a systematic and organized manner until now. Owing to the EC Regulation on fluorinated gases which has been adopted in the previous year in EU and which will regulate the HFC usage, in our view the situation in this field will undergo a significant change. In our opinion, up to the year 2007, an overview of all major sources of HFC emissions will be at our disposal and emissions will fall significantly due to increased recycling, recovery efficiency and other measures.

### Recalculations

We have recalculated all HFC emissions from mobile AC using updated default values from GPG. For first-fill emission rate we have used 0.5% instead of 4% and for MAC system emission rate we have chosen 20% (before 30%).

We have also recalculated all SF6 emissions using new activity data and actual emissions data from companies owing HV equipment.

# SOLVENT AND OTHER PRODUCT USE (CRF sector 3)

## Solvent use

### Source category description

Consumption of coatings is an important source of greenhouse gases, particularly due to emissions of NMVOCs since no important quantities of other gases are emitted in this activity. This category includes emissions arising in the consumption of paints, lacquers, solvents, diluents, and other coatings. All of the above-enumerated products contain significant quantities of NMVOCs. Emissions arise due to the evaporation of volatile compounds during application of these products, NMVOCs arise in various activities, which are scattered over a wide area. Therefore, such sources are called area sources, since emissions arise with a large number of small consumers and not in large industrial premises.

This category should also include emissions that arise from evaporation during the use of other products. This report includes only emissions of N2O, which have been estimated at the Chamber of Commerce and Industry of Slovenia (Leban J., Lebar A., Fece V., 1999). For other sources, there is no suggested relevant methodology to calculate this type of emissions; consequently, this report will include only NMVOCs emissions from the consumption of coatings and emissions from the consumption of N2O.

### Methodological issues

NMVOC EMISSIONS

The basic method, which is recommended in IPCC guidelines, is related to each individual source and necessitates data on activity (quantity of produced or used coating) as well as an emission factor, given per unit of consumption or production.

No method for calculation of emissions has been proposed for the consumption of coatings in IPCC guidelines. Consequently, the methodology proposed in EMEP-CORINAIR (Co-operative Programme for Monitoring and Evaluation of the Long Range Transmission of Air Pollutants in Europe and The Atmospheric Emission Inventory for Europe: Atmospheric Emission Inventory Guidebook, 1996. p. B610 – SNAP 060100 - 060108) has been applied. Emission factors from National Emission Inventories (Rode B. Državne emisijske evidence, MOP 1998) have been applied.

Emissions may be calculated in two ways:

- On the basis of data on production of products for which coatings are used;

- On the basis of consumption of coatings.

In this report, the coating-consumption approach has been applied, since data for the production approach are available only for major industrial consumers, while the majority of sources of emissions of NMVOCs from the consumption of coatings are small and widely scattered. Typical for both methods is the high uncertainty due to considerable variation in the mode of application and type of coating applied and hence the use of generic emission factors is appropriate only for a first assessment of emissions.

In this sector, emissions from the consumption of coatings, diluents, and solvents in industry as well as from the consumption of coatings and diluents in construction and in households have been calculated.

Since 1997, the Statistical Office of the Republic of Slovenia does not gather and does not report the consumption of coatings, diluents, and solvents in industry. The consumption of these products has therefore been estimated by multiplying the consumption in the last year with the growth index of the industrial production, which is published annually in the Statistical Yearbook. Emission factors from national emission inventories (Rode 1998, p. 3) have been applied. Emission factors in National Emissions Inventories (Rode 1998, p. 3) are specified for already diluted coatings (mixtures of coatings, diluents and solvents). Consequently, data on consumption of coatings, diluents, and solvents must be summed up and emissions calculated from the totals.

Table 5.1: Emission factors for consumption of coatings, solvents, diluents, natural pigments, and printing inks in industry.

|  | emission factor (kg NMVOCs/ton) |
| --- | --- |
| Consumption of Coatings, Solvents, Diluents in Industry | 500 |
| Consumption of Natural Pigments in Industry | 30 |
| Consumption of Printing Inks in Industry | 200 |

Data on the consumption of coatings and diluents in construction have been taken from the publications of the Statistical Office of the Republic of Slovenia. Only joint data for coatings and diluents are given, therefore it is not possible to estimate separate items. DEE does not specify any emission factors for calculation of emissions of NMVOCs from the consumption of paints, solvents, and diluents in construction. Consequently, emission factors from EMEP-CORINAIR documents (Co-operative Programme for Monitoring and Evaluation of the Long Range Transmission of Air Pollutants in Europe and The Atmospheric Emission Inventory for Europe: Atmospheric Emission Inventory Guidebook, 1996. p. B610) have been used in calculations.

Table 5.2: Emission factors for consumption of coatings, solvents. and diluents in construction.

|  | emission factor (kg NMVOCs/ton) |
| --- | --- |
| Consumption of Coatings, Solvents, Diluents in Construction | 300 |

Data on the consumption of coatings and diluents in households are not available. In a report titled IPCC Inventory of Greenhouse Gases for Slovenia 1990 (Seljak, 1997), the consumption was estimated on the basis of data from the Questionnaire on the Consumption in Households of the Statistical Office of the Republic of Slovenia. In 1994, the mode of carrying out this questionnaire has changed and so data are not available in a corresponding form any more. It also proved impossible to obtain those data by monitoring retail sales. To ensure data comparability for the entire period of all those years, a single methodology of direct calculation of emissions of NMVOCs from the consumption of paints, solvents and diluents in households was applied, as recommended by the EMEP- CORINAIR documents (Co-operative Programme for Monitoring and Evaluation of the Long Range Transmission of Air Pollutants in Europe and The Atmospheric Emission Inventory for Europe: Atmospheric Emission Inventory Guidebook, 1996). This methodology is also recommended in National Emissions Inventories (Rode 1998, p. 3).

Table 5.3: Emission factors for consumption of coatings and diluents in households.

|  | Emission Factor (kg NMVOCs/inhabitant) |
| --- | --- |
| Households | 2.5 |

CO2 EMISSIONS

NMVOCs emissions oxidise in a relatively short period and therefore in Consumption of Coatings and in Industrial Processes. In previous submissions NMVOCs emissions have been transformed to CO2 emissions. In these two chapters there was deliberate double counting of emissions. A mass unit of NMVOCs contributes 3.19 mass units of CO2.

During review process we couldn’t make any explanation how this factor was derived. As we also didn’t find any method for calculation of CO2 from NMVOC in IPCC guidelines, we excluded this source from whole inventory for all period.

## Other products use

| dairy cattle | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | no | N2O | 0.41 |  | 30 |
| 2004 | Trend | N2O | 0.20 | 0.56 | 36 |

N2O EMISSIONS

Figure 5.2.17: Consumption of N2O in tons.

This category presents N2O emissions that have been estimated by the Chamber of Commerce and Industry of Slovenia (Leban J., Lebar. A., Fece V. 1999). Emissions have been estimated for 1986 and the period 1993 to 1998. Since 1999, there is no N2O production in Slovenia. Consumption is calculated from data on import and export, N2O is mostly used in the health service, and to a lesser extent also in food industry.

In previous submission data on import for 1998 was very low and imported amount in 1999 extremely high. We have corrected this data to use average value of this two value to estimate import in 1998 and 1999.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 30%.

Uncertainty of emission factor amounts to 80%

### Source-specific planned improvements

In 2003, the Environmental Agency began organizing a database on the consumption of solvents in dry cleaning and industry. The database with data for year 2004 has not been completed yet, but we hope to be able to include these data into our report.

### Source-specific recalculations

NMVOCs emissions oxidise in a relatively short period and therefore in Consumption of Coatings and in Industrial Processes. In previous submissions NMVOCs emissions have been transformed to CO2 emissions. In these two chapters there was deliberate double counting of emissions. A mass unit of NMVOCs contributes 3.19 mass units of CO2.

During review process we couldn’t make any explanation how this factor was derived. As we also didn’t find any method for calculation of CO2 from NMVOC in IPCC guidelines, we excluded this source from whole inventory for all period.

Considering use of N2O there was a mistake in activity data for 1998 and 1999. We have corrected mistake and calculated new emissions for 1998 and 1999.

# AGRICULTURE (CRF sector 4)

In agricultural activities, emissions of GHGs are generated from a variety of different sources. This section includes the quantification of CH4 emissions from enteric fermentation and manure management, N2O emissions from manure management, direct and indirect emissions from agricultural soils.

Figure 6.0.1: CH4 and N2O emissions from agriculture activities in Gg CO2 eq..

Burning crop residues is, generally, not practiced in Slovenia, therefore emissions of greenhouse gases from this source have not been considered in this report (Verbič, Sušin, Podgoršek 1999, p. 9).

There are no ecosystems in Slovenia that could be considered natural savannas or rice fields; consequently, no greenhouse gas emissions therefore exist for this sub-category.

## CH4 Emissions from Enteric Fermentation

| dairy cattle | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 2.13 |  | 10 |
| 2004 | Level, Trend | CH4 | 1.32 | 2.17 | 14 |

| non-dairy cattle | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 1.50 |  | 15 |
| 2004 | Level, Trend | CH4 | 1.72 | 0.60 | 10 |

Swine, goats, horses, sheep

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

CH4 emissions from enteric fermentation in animals result from methane being produced as a by-product of microbial fermentation in the digestive system. This process occurs especially in the rumen of ruminant animals, but also in smaller quantities in monogastric animals (swine, horses, birds and rabbits) where feedstuffs ferment in the large intestine. The estimates in this inventory include only emissions in farm animals. Emissions from wild animals and semi-domesticated game are not quantified and neither are emissions from humans or pet animals.

CH4 emission from enteric fermentation is a key source, both by level and trend. Dairy cows and non-dairy cattle are significant sources: dairy cows represent 40% of total CH4 emissions from enteric fermentation while a non-dairy cattle represents about 52% of total CH4 from enteric fermentation. Jointly, cattle are responsible for more than 92% of total CH4 emissions from enteric fermentation.

Figure 6.1.18: CH4 emissions from enteric fermentation in Gg.

The contribution of all other animals to methane emissions from enteric fermentation, e.g. swine, sheep, horses, and goats, listed here according to the importance of their contribution, is less than 8%. No methodology for calculating CH4 emission from poultry is available in IPCC guidelines.

### Activity data and methodological issues

All necessary data are obtained from the Statistical Office of the Republic of Slovenia (SORS).

Before 1997, SORS collected data on the number of livestock by ownership and as of December 31. Data on livestock in agricultural enterprises were collected and reported in the Annual Report on Livestock Production while data on livestock on family farms were collected in the Sample Survey on the Number of Livestock. Data on the number of livestock on family farms as of December 31, 1991 were estimated on the basis of the 1991 Census of Population, Households, Housing, and Agricultural Holdings. Some data on the number of livestock were taken from other administrative sources (Secretariat for Agriculture, Forestry and Food).

In 1997, SORS started collecting data on the number of livestock several times a year: data on the number of pigs were collected three times a year (April 1, August 1, and December 1), data on cattle twice a year (June 1 and December 1) and data on sheep and goats once a year (December 1). Data on family farms and agricultural enterprises were collected by mail, separately for cattle, pigs, sheep, and goats. All agricultural enterprises were covered, but only those family farms that were selected in the sample. In 1997, the date of monitoring the number of animals changed: because of the harmonisation with EU standards, we no longer collect these data as of December 31, but as of December 1. Data on animal output up to 1997 are therefore not entirely comparable with data since 1997.

Since 2000, SORS has been collecting data on the number of livestock twice a year (June 1 and December 1) for cattle and pigs, and once a year (December 1) for sheep and goats, and poultry. To facilitate presentation of data on the structure of agricultural holdings, the number of animals is shown by where they are stabled and not by ownership as was the case until 2000. In the December 2002 survey, the data on the number of animals were collected with fieldwork and not by mail; this accounts for some differences between the individual periods.

In 2003, SORS has published revised data on livestock numbers and production for the period 1991-2002. These data have been published in Rapid Reports No. 256. The main purpose of that revision was the methodological harmonisation of data and methods of estimating data for the mentioned period. This methodology is harmonised with recommendations of the Statistical Office of the European Communities.

Figure 6.1.19: Number of cattle, diary and non-diary in thousands.

Corrections refer to livestock number and production, while the data on total number of animals did not change. Not all published data have been revised. Some remained the same. Explanations of corrections are therefore valid only for the revised data. Data were revised on the basis of the 2000 Census of Agriculture.

Explanations of corrections were divided into two parts: number of livestock and livestock production. Data on cattle, pigs, poultry and sheep and goats were mostly revised in the same way. Differences appear because of particularities of individual species, the previous method of data collection for individual species of domestic animals and the quality of collected data.

In all monitored years, data on the total number of animals were obtained with statistical surveys and have not been corrected, but changes have been made to the year that data refer to. Before 1997, each year data about animal population were collected on December 31 and in the past have been applied to the next year, but now SORS considers these data to be valid for the current year. Because SORS did this only for data from 1991 (the year Slovenia became an independent state), we have to change all data prior to 1991 in the same way by ourselves.

Figure 6.1.20: Number of sheep, goats and horses in thousands.

But discrepancies with FAO data still exist. In the FAO database, livestock numbers have been grouped in 12-month periods, ending on September 30 of the year stated in the tables. Our Statistical Office collects data on animal population in December and reports them in the current year. In the FAO database, these data are applied to the next year. Considering this explanation, all data on animals in the FAO database and in our statistical database are the same. The only difference is in the number of poultry, where our entire poultry population is shown in the FAO database as chicken population.

IPCC methodology provides two different methods for estimating the quantity of methane from enteric fermentation. A more detailed method (Tier 2) for calculating emissions is used for cattle because of the comparatively large population and considerable emission per head and the default methodology (Tier 1) is used for other animals.

#### Dairy Cattle

The starting-point for this method is data on cattle population, daily energy consumption, and the conversion factor for the transformation of feed in methane. The accuracy of the estimate is improved by disaggregating the animals into several subgroups with regard to productivity. Disaggregating was done on the basis of milk recording data which is performed by the national Cattle breeding service (Verbič, Sušin, Podgoršek 1999, p. 3).

IPCC methodology is used in estimating gross energy intake. The net energy is estimated on the basis of the needs of animal maintenance, lactation, and pregnancy. In case of grazing, additional energy requirements for this activity has been taken into account. The requirements of animals for net energy have been calculated for all subgroups and then transformed to estimated gross energy intake.

Annual emissions of methane (EM ENTER) have been calculated on the basis of estimated gross energy intake (BE) according to an equation taken from the pages of IPCC Guidelines(1996)

Figure 6.1.21: Milk production per cow  in l milk/head/day.

A detailed description of methodology for EF calculation is given in Annex 3.

According to data on emission factors from period 1985-1996 an equation was developed that is based only on the data on average milk yield, where EM is methane emission in kg per animal per year, and M the average annual milk yield of dairy cows.

EM= 75.7+0.0038 × M

This equation has been applied for calculation of emissions for whole period 1985-2004. The average milk yield of dairy cows has been calculated in that the data on the amount of produced milk, according to the Statistical Yearbook of the Republic of Slovenia, was divided by the number of dairy cows. The amount of produced milk includes all produced milk, even the milk that is fed to calves. But this number does not include milk that the calves suckle from cows. In the previous research project, this milk was taken into account by increasing the amount of produced milk for every cow with annual milk yield below 2000 kg, by 600 kg of milk. The correction has been done only until 1996. Thereafter it was considered that these cows were transferred into category of suckler cows. In our opinion, this change in methodology does not affect the comparability of data from the years 1985-1987 with data for years after 2000, since the number of cows with less than 2000 kg of milk per year at that time was similar to the present number of suckler cows.

#### Non-dairy cattle

This group comprises young cattle (cattle for fattening, breeding heifers), suckling cows and sires (breeding bulls). This group is much more homogeneous as to the intensity of production than the dairy-cattle group. Data on the number of head of non-dairy cattle according to different categories are reported by the Statistical Office.

Methane emissions of growing cattle depend on weight gain. For 1986 it has been assumed that the weight gain of growing heifers from their sixth month of age to first mating amounts to 600 g per day, and of pregnant heifers to 500 g per day. The remaining total weight gain of cattle in Slovenia (according to the Statistical Office) has been equally distributed across other growing categories of cattle and thus the average daily weight gain for young bovine animals for fattening has been determined. In the same way as for dairy cows, according to equations laid down by IPCC Guidelines (1996) for individual categories, the requirements for net energy were calculated, which were then converted into gross energy. For growing heifers, we also took into account the energy that is needed for pregnancy. The required feed digestibility for individual categories have been estimated on the basis of recommendations by Kirchgessner (1985) and equations for converting the feed digestibility factors into metabolisable energy factors (INRA, 1989). For bovine animals for fattening, it has been considered that the required energy digestibility amounts to 61.5%, for growing heifers to 58.5%, for breeding bulls to 56,5%, for non-classified cattle (grown up steers and other cattle at maintenance level) to 54.5%.

For the year 2005, more precise average daily gains for young bovine animals for fattening were obtained. They were calculated on the basis of data on slaughtering date and carcass weight from slaughter houses and on the basis of birth dates of individual animals which are recorded in the database of Cattle breeding service (Verbič and Jeretina, 2006, unpublished). It was found that average daily gain in fattening animals increased from 714 g in 1986 to 840 g in 2005. Average daily gains between 1986 and 2005 were estimated by interpolation. For breeding heifers the same daily gains as for 1986 was taken into account.  Due to higher daily gains also the required energy digestibility for fattening animals has been increased from 61,5 % in 1986 to 66,4 % in years after 2000.

Emissions have been estimated on the basis of IPCC methodology (methane conversion factor 0.06) (Verbič, Sušin, Podgoršek 1999, p. 5-6)

Since in younger animals the rumen does not function normally yet, calves up to the age of 3 months have not been considered. Until the year 2000, in calculating methane emissions, only ½ of the category of calves of up to 6 months, after 2000, ¾ of young bovine animals up to 1 year have been considered. The modification of calculation is due to slight change in the manner of official presentation as to the age structure; previously, calves under the age of 6 months were presented separately while now they are all presented in the up to 1 year group.

After 1990, non dairy  cattle category includes also suckler cows. Previously they were not reported separately. Statistical Office of the Republic of Slovenia differentiate between dairy cows and suckler cows from the year 1996, the figure for the year 1995 is based on the number of paid premiums for suckler cows and for years 1991-1994 the numbers were estimated by the expert (Volk, personal communication). To allow for this alteration in reporting by the Statistical Office of the Republic of Slovenia, we have adapted our methods for estimating methane emissions arising from enteric fermentation in cows. For suckler cows, it was considered that the amount of emitted methane was equal to the amount that in the previous research project had been attributed to cows with 1500 to 2000 kg of milk per lactation, i.e. 80.4 kg per cow per year.

#### Sheep and Goats

The Statistical Office of the Republic of Slovenia has recently changed its methodology of estimating the population of sheep, and started to publish data on the number of goats (Statistical Information, No. 197, 1998), data that so far have not been published in the Statistical Yearbook. For breeding sheep, re-established data from 1992 to 1997 are available. The total number of sheep has been estimated on the basis of data on breeding sheep for the period 1992 to 1997 by applying the interacting ratio between breeding sheep and all sheep in 1997. For the time prior to 1992, the numbers from old statistical yearbook  has been taken. The number of goats has been estimated in the same way as the number of sheep. As goats have not been count before 1992 we consider the number of 10.000 heads as an estimate.

Considering the rather small number of sheep and goats, coefficients from the IPCC method have been used for estimating methane emissions; 8 kg of methane annually per head for sheep, 5 kg of methane for goats (Verbič, Sušin, Podgoršek 1999, p. 6)

#### Horses

The number of horses has been taken from the Statistical Yearbook, methane emissions have been estimated by applying generic emission factors according to IPCC methodology 18 kg per year (Verbič, Sušin, Podgoršek 1999, p. 6).

#### Swine

The population of swine is divided into three segments:

a) commercial pig farms,

b) market oriented family farms, and

c) small scale family farms.

Data published by the Statistical Office of the Republic of Slovenia allow a breakdown of the entire herd into commercial pig farms and family farms for the period 1985-2002. For the years 2003 and 2004 the herd was allocated to both segments on the basis of ratio in 2002.  Family farms were further divided into market oriented and small scale. In 1986 the estimate of production for market oriented family farms was based on the data on acquisition of pigs from market oriented family farm production, which was published by the Statistical Office of the Republic of Slovenia. The number of swine in small scale family farm production has been estimated from the difference between the entire herd and market oriented production (commercial and market oriented family farms). This type of estimating agreed rather well with the results of the 1991 regular census. For 2000, the number of pigs in the small scale family farm production has been estimated on the basis of the census of family farms. Pigs that were bred on farms with up to 10 pigs have been considered as small scale family farm production, pigs on family farms that breed more than 10 pigs have been considered as market oriented family farm production. From the year 1986 to year 2000, the fraction of pigs in small scale family farm production has kept diminishing. In the period between 1986 and 2000 the proportion of small scale production was obtained by interpolation. For years 2001 to 2004 extrapolation was used by taking into account the same trend of diminishing small scale production as in period 1986-2000.

Figure 6.1.22: Number of swine in thousands.

Based on data from the Statistical Office of the Republic of Slovenia (SY) on total weight gain, herd size in commercial pig farms and private sector as well as estimated average weight gains in commercial conditions (Šalehar et al., 1998) it is estimated that the weight gain in swine on small scale family farms does not differ significantly from weight gains in market oriented family farm production (approximately 600 g/day). Consequently, in estimating emissions, only the difference in the mass at the time of slaughter is taken into account. For swine on commercial pig farms and market oriented family farms, the emission factor suggested by IPCC has been taken into account - 1.5 kg methane per head annually, while for swine on small scale family farms, the emission factor of 2.33 kg of methane per head annually has been taken into account (estimated on the basis of body mass differences) (Verbič, Sušin, Podgoršek 1999, p. 6).

#### Poultry

IPCC guidelines do not define or require estimates of quantities of methane from enteric fermentation.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 10%.

Uncertainty of emission factor amounts to 20%.

The combined uncertainty, calculated according to IPCC GPG Tier 1 methodology amounts to 22.36%.

### Source-specific recalculations

All emission estimates for cattle have been recalculate according to new data of daily gains. There was also allocation of emissions from suckler cows which have been transferred to non-dairy cows.

### Future improvements

No improvements is planned for this category.

## CH4 Emissions from Manure Management

| dairy cattle | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 0.83 |  | 22 |
| 2004 | Level, Trend | CH4 | 0.65 | 0.49 | 23 |

| non-dairy cattle | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | no | CH4 | 0.46 |  | 28 |
| 2004 | Level, Trend | CH4 | 0.69 | 0.60 | 22 |

| swine | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 1.15 |  | 20 |
| 2004 | Level, Trend | CH4 | 0.82 | 0.90 | 19 |

goats, horses, sheep, poultry

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

In storing solid and/or liquid manure, both methane and N2O are emitted. Emissions depend largely on the type of manure storage. Methane arises in significantly larger amounts when manure is managed as slurry, while N2O prevails in storage of solid manure.

Figure 6.2.23: CH4 emissions from manure management in Gg.

Significant quantities of methane are emitted during the decomposition of animal excreta. Under anaerobic conditions, methane-producing bacteria convert organic matter into methane. The quantities of produced methane are largely dependent on the type of manure management system and environment temperature.

Figure 6.2.24: The average yearly temperature in Slovenia.

(http://www.arso.gov.si/podro~cja/vreme_in_podnebje/napovedi_in_podatki/temperaturna_karta.html)

Storing manure in lagoons or as slurry produces significantly greater quantities of methane compared to grazing on pasture or solid manure storage.

To estimate the amount of methane produced during manure management, it is necessary to know the quantities of excreted volatile solids (VS), methane-producing capacity of manure (B0, in m3 per kg of VS), and the manner of manure management (Verbič, Sušin, Podgoršek 1999, p. 7). The climate in Slovenia is cool (average yearly temperature is bellow 15°C for the whole area).

### Methodological issues

#### Cattle

The annual quantities of decomposable organic matter have been estimated by means of data gathered while estimating the extent of enteric fermentation. For dairy cows, on the basis of these data we have calculated an equation (Tomšič et al., 2000), which enables a direct estimation of the amount of excreted decomposable organic matter on the basis of annual milk yield M (in kg per year).

VS (in kg/year) = 1269 + 0.0414 × M

For suckler cows, the value of 80.4 kg VS per day has been considered, which in the previous research project (Verbič et all. 1999) was estimated for dairy cattle with 1500 to 2000 kg of milk per lactation.

For other categories of bovine animals, we have applied an equation that is laid down by IPCC (1996) and which, through nutrient requirements, is directly linked to the production intensity (to daily weight gain).

VS (kg/day) = intake (MJ/day) x (1 kg/18.45 MJ) x (1 - DE%/100) x (1-ash%/100)

The annual emitted amount of methane (EM MANURE) has then been estimated according to the equation:

EM MANURE = VS (kg/day) × 365 days/year × B0 (m3/kg VS) × 0.67 kg/m3 × KF

As for the methane producing capacity of manure B0 for dairy cows, the value of 0.24 m3/kg VS has been considered, for other bovine animals 0.17 m3/kg VS (IPCC, 1996). The methane conversion factor KF MANURE, which tells us what fraction of methane producing capacity of manure is actually used, has been calculated on the basis of fractions of individual types of manure storage and partial manure conversion factors for cool climate, which were found in appropriate tables (IPCC, 2000).

The fraction of individual manure management systems has been estimated on the basis of the results of a farm census done in 2000. Since manure management systems were not reported in the census, data on size and structure of cattle-breeding farms were used instead. It was considered that all farms with less than 10 head of bovine animals have solid manure storage systems, that of farms with 10-19 head of animals, 30% have liquid manure systems and 70% solid manure storage systems, and that all farms with 20 cows or more have liquid manure storage systems. For 1990, manure storage systems have been estimated according to a similar procedure, but in this case using the results of the 1991 census. Linear regression was used to estimate the changes in manure management systems in the period 1990-2000. The fraction of grazing bovine animals for 1990 has been estimated on the basis of data on grazing animals on mountain pastures end expert estimate on the scale of grazing on intensive grasslands (Verbič et al., 1999). In 2000, all grazing animals on mountain and other pastures have been recorded. This census showed that in 2000, one way, or another, 21% of animals were grazing. This data have been corrected with regard to the length of the grazing season, considering the fact that animals on mountain pastures on the average will graze for 141 days, and on other pastures for 210 days. The estimate for 1990 was used for the period 1985-1990 and the estimate for 2000 was used for the period 2000-2004. For the period 1991-1999 the data on grazing were obtained by linear regression which was calculated on the basis of data for the years 1990 and 2000. It has been estimated that the fraction of grazing animals and the fraction of liquid manure management systems have increased while the fraction of bovine animals in straw based systems has decreased.

Data on the number of livestock were the same as those used for calculating methane emissions from enteric fermentation.

#### Swine

Annual emissions of methane (EM MANURE) have been estimated according to the IPCC method. Quantities of excreted decomposable organic matter (VS) have been calculated for commercial pig farms and market oriented family farms, considering that each swine daily excretes 0.5 kg of VS (IPCC, 1996). For small scale family farm production, considering the higher body weight, 0.775 kg VS/day has been taken into account. For the methane-producing capacity of manure B0, the value for swine (0.45 m3/kg VS; IPCC, 1996) has been applied. The average manure conversion factor (KF MANURE) has been estimated with regard to the type of manure management system and partial manure conversion factors that had been laid down for various systems by IPCC (2000). In doing so, the following has been taken into account:

Commercial Pig Farms

From 1985 to 1994 – using old-style separators on commercial farms, app. 20% of organic matter was separated from liquid manure. For this portion, the partial KF for solid manure has been taken into account. The remainder (80%) has been disaggregated into lagoons (75%) and liquid manure (25%), taking into account a KF as suggested by IPCC (2000). The division into lagoons and liquid manure is founded on actual estimates of the extent of production on commercial farms, where the liquid portion of manure after separation is applied to fields and grassland. Considering the ratio between solid phase and liquid manure, which was either led into lagoons or used for fertilization, the average KF MANURE = 0.62 has been calculated.

The time from 1995 to 1999 was a period of introducing new separators and the beginning of operation of an anaerobic digester in the Farm Ihan. Introducing new separators on commercial farms increased the estimated portion of separated solid phase to 40%. Since the construction of a new wastewater treatment plant in Farm Ihan, it has been considered that on that commercial farm the mechanic separation separated 80% of decomposable organic matter while the remainder (20%) was captured as biogas. For large commercial farms, it is generally considered that the ratio between the liquid part, which flows off to lagoons, and the liquid part, which is used as fertilizer, is the same as prior to 1995 (3:1). The estimated average manure conversion factor KF MANURE for that period was 0.378.

Due to new farm reconstructions which led to improved slurry separation and introduction of additional capacity of anaerobic digesters, the methane conversion factor KF MANURE on big farms after 1999 decreased to 0.213.

Market oriented family farm production

For market oriented family farm production, it is considered that 95% of animal excreta are collected in the form of liquid manure and 5% in the form of solid manure. Based on the ratio between liquid manure and solid manure, the average manure conversion factor KF MANURE = 0.371 (IPCC, 2000) has been used for calculations.

Small scale family farm production

For small scale family farm production, it is estimated that 95% of pigs is reared in solid manure storage systems and 5% in liquid manure systems. For this type of production the average manure conversion factor KF MANURE = 0.029 was calculated on the basis of IPCC (2000) guidelines.

#### Sheep, Goats, Horses, and Poultry

Excreta of these animals contribute only a comparatively small portion of total emission of methane in Slovenia. In the estimating process, average values as suggested by IPCC (1996): 0.19 kg/sheep, 0.12 kg/goat, 1.4 kg/horse and 0.078 kg per layer or broiler (Verbič, Sušin, Podgoršek 1999, p. 8) have been considered.

Figure 6.2.25: Number of poultry in thousands.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 10%.

Uncertainty of emission factor amounts to 30%.

Combined uncertainty amounts to 31.62%.

### Source-specific recalculations

All emissions for all years have been recalculated because new value from IPCC GPG for MCFs gave been used for liquid system and for anaerobic digesters.

All emission estimates for cattle have been recalculate according to new data of daily gains. Emissions have been also recalculate There was also allocation of emissions from suckler cows which have been transferred to non-dairy cows.

### Future improvements

No improvements is planned for this category.

## N2O Emissions from Manure Management

| Solid storage and dry lot | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | N2O | 1.29 |  | 18 |
| 2004 | Level, Trend | N2O | 0.77 | 1.38 | 20 |

Anaerobic lagoons, liquid systems, other

Key sector - Base year: 	no

Key sector - Year 2004:	no

### Source category description

Farm animals emit directly very little nitrous oxide and have not been considered in estimating emissions of greenhouse gases. A considerable amount of nitrous oxide evolves during storage of animal waste – and is attributed to livestock production. Nitrous oxide emitted from urine and excreta of grazing animals in the pasture is attributed to emissions from agricultural soils.

### Methodological issues

Emissions of nitrous oxide, arising during manure storage, have been estimated on the basis of the data on the number of farm animals in Slovenia and nitrogen excretion rates of individual animal species and categories. The nitrogen excretion rates for cattle and pigs were harmonized with the methodology for ammonia emissions (Verbič, 2004).

In dairy cows the nitrogen excretion has been linked to productivity, i.e. milk production (M).  The following equation was used:

N excretion (kg/year) = 52.5 + 0.0105 × M (kg/year)

It has been considered that suckler cows excrete annually 78 kg of nitrogen, which corresponds to productivity of 2400 kg of milk per year. For other bovine animals 35 kg of N per year was considered. For sows and pregnant gilts 36 kg of N per year was used by taking into account consideration that this category also covers the N excreted by un-pregnant gilts, boars and piglets which were therefore not taken into account separately. For fattening pigs the excretion of 14 kg per animal was used.  It has been considered that  sheep, goats, horses and poultry excrete 20, 25, 25 and 0,6 kg of N per year respectively (IPCC, 1996).

Figure 6.3.26: Nitrogen excretion per different AWMS in tons.

Emission factors, which tell us how much of N from animal excreta is lost to the atmosphere in the form of N2O, have been taken from IPCC (2000). For anaerobic lagoons, anaerobic digesters, and liquid systems, the emission factors applied amounted to 0.001, for solid manure storage systems to 0.02 and for poultry manure without bedding 0.005.

The fraction of manure nitrogen produced in different animal waste management systems for bovine animals and swine has been estimated on the basis of the methodology for methane calculation.

Figure 6.3.27: Allocation of manure in different AWMS in the year 2004.

For goats, sheep, and horses the proportions of grazing animals were estimated by the expert (Verbič, 2004). It was estimated that during the grazing season all sheep, 80% of goats and 50% of horses are grazed. Two hundred and fifty days of grazing season has been considered for sheep and 210 for goats and horses. For the remaining period it has been considered that these animals were in straw based systems.

For poultry, floor system on bedding was assumed for broilers, and combined floor system (1/4) and battery-cage systems (3/4) were assumed for layers. For battery-cage system for layers it was assumed that in 50% the manure is removed daily and stored in tanks (liquid system) while in 50 % it is collected under the batteries (i.e. poultry manure without bedding).

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 50%.

Uncertainty of emission factor amounts to 100%.

Combined uncertainty amounts to 111.80%.

### Source-specific recalculations

All emissions from cattle and swine have been recalculated using nitrogen excretion rates which are now harmonized with the methodology for ammonia emissions.

### Future improvements

No improvements is planned for this category.

## Emissions from Agricultural Soils

Three sources of N2O are distinguished in the IPCC methodology: direct emissions from agricultural soil, direct soil emissions from animal production (grazing animals) and N2O emissions indirectly induced by agricultural activities.

Figure 6.4.28: N2O emissions from agricultural soil in Gg.

### Direct N2O Emissions from Agricultural Soil

| direct soil emissions | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | N2O | 2.01 |  | 12 |
| 2004 | Level | N2O | 1.85 | 0.44 | 9 |

#### Source category description

Major sources of nitrogen, causing direct emissions of nitrous oxide into the atmosphere, are the following:

- Mineral fertilizers

- Organic fertilizers (solid and liquid manure) from animal husbandry

- Biological fixation of nitrogen

- Crop residue

- Cultivation of high-organic content (peat) soil

#### Methodological issues

Nitrous oxide from mineral fertilizers

This estimate is based on the amount of N in mineral fertilizers that are annually consumed in Slovenia. The consumption of nitrogen from mineral fertilizers on agricultural soil in Slovenia has been obtained from the Statistical Yearbook.

SORS collect data on fertilisers used in enterprises, companies and co-operatives involved in crop production. Likewise, they are taking into account the data on import, export, and production. The difference between all fertilizers sold in this country and the amount that is used by enterprises, is the consumption of mineral fertilizers on family farms. Fertilizers that are not appropriate for agricultural production (mineral fertilizers for balcony flowers, lawns and similar) are not included.

The consumption of macro plant nutrients per hectare of fertilised utilised agricultural area in Slovene agriculture has been steadily decreasing from 1999 to 2003, the decrease in that period amounting to 17.6%.

Of the total estimated quantity of emitted N, the N, which is dispersed into the atmosphere in the form of ammonia and NOX (10%; IPCC, 1996), was subtracted. The emission of nitrous oxide was then calculated according to the default method from IPCC GPG Equation 4.22 by multiplying the quantity of the remaining N with emission factor 0.0125 kg N2O-N/kg N (IPCC, 1996).

Nitrous oxide from animal manure and liquid/slurry

The estimate is based on the amount of N in solid manure and liquid manure/slurry, which is annually used for fertilizing crops. The amount of N in the manure applied to soil has been calculated on the basis of methodology described in chapter 6.3 (N2O Emissions from Manure Management).

We estimated emissions according to the default method from IPCC GPG Equation 4.23. Of the total estimated quantity of emitted N, we subtracted the N that is emitted on the pasture, and N that is dispersed into the atmosphere in the form of ammonia and NOX. The emission of nitrous oxide has been calculated by multiplying the quantity of N from animal manure with the emission factor of 0.0125 kg N2O-N/kg N (IPCC, 1996).

Nitrous oxide from biological fixation of N

The amount of nitrogen assimilated by legumes has been estimated on the basis of the production data (Statistical Office of the Republic of Slovenia) while taking into account the ratio between crop residue and edible (usable) part of the crop, dry matter fraction and N fraction (Table 6.1). We have used default values from GPG if available; for other crops we obtained data from our Agriculture Institute.

To calculate emissions of nitrous oxide we used Tier 1b method and Equation 4.26 from IPCC GPG. The same emission factor as for N from mineral and organic fertilizers (0.0125 kg N2O-N/kg N; IPCC, 1996) has been applied.

Table 6.1: Dry matter fraction, residue/crop product ratio and N fraction for legumes.

|  | Dry Matter Fraction | Residue/Crop Ratio | N Fraction |
| --- | --- | --- | --- |
| Fodder peas | 0.85 | 1.5 | 0.0142 |
| Dry beans | 0.895 | 2.1 | 0.03 |
| Soya | 0.86 | 1.4 | 0.023 |
| Clover and Lucerne | 0.85 | 0 | 0.03 |
| Grass-clover mixtures | 0.85 | 0 | 0.03 |
| Peas | 0.15 | 1.2 | 0.0142 |
| French beans | 0.15 | 1.2 | 0.03 |

Emissions of nitrous oxide from crop residue mineralisation

Table 6.2: Dry matter fraction, residue/crop product ratio and N fraction.

|  | dry matter fraction | Residue/Crop Ratio | N fraction |
| --- | --- | --- | --- |
| Wheat and spelt | 0.86 | 1.3 | 0.0028 |
| Rye | 0.86 | 1.6 | 0.0048 |
| Barley | 0.86 | 1.2 | 0.0043 |
| Oats | 0.86 | 1.3 | 0.007 |
| Maize for grains | 0.86 | 1 | 0.0081 |
| Triticale | 0.86 | 1.6 | 0.015 |
| Millet | 0.86 | 1.4 | 0.007 |
| Buckwheat | 0.86 | 1.4 | 0.015 |
| Other cereals | 0.86 | 1.3 | 0.015 |
| Fodder peas | 0.85 | 1.5 | 0.0142 |
| Dry beans | 0.895 | 2.1 | 0.03 |
| Potatoes | 0.19 | 0.4 | 0.011 |
| Sugar beet | 0.25 | 1.4 | 0.015 |
| Fodder beet | 0.15 | 0.3 | 0.0228 |
| Fodder carrot | 0.16 | 0.3 | 0.015 |
| Fodder turnip | 0.12 | 0.3 | 0.015 |
| Fodder pumpkins | 0.1 | 0.4 | 0.015 |
| Oil turnip/rape | 0.92 | 1.7 | 0.015 |
| Sunflowers | 0.86 | 1.3 | 0.015 |
| Soya | 0.86 | 1.4 | 0.023 |
| Pumpkins for oil | 0.9 | 5 | 0.015 |
| Hops | 0.89 | 0.3 | 0.015 |
| Other industrial plants | 0.89 | 0.3 | 0.015 |
| Tomatoes | 0.063 | 1 | 0.015 |
| Cucumbers | 0.037 | 1 | 0.015 |
| Sweet | 0.1 | 1 | 0.015 |
| Carrots | 0.1 | 1.5 | 0.015 |
| Garlic | 0.354 | 1 | 0.015 |
| Onion | 0.142 | 1 | 0.015 |
| Beetroot | 0.1 | 0.3 | 0.015 |
| Peas | 0.15 | 1.2 | 0.0142 |
| French beans | 0.15 | 1.2 | 0.03 |
| Other vegetables | 0.0854 | 1.2 | 0.015 |

An important source of emissions of nitrous oxide into atmosphere is nitrogen from crop residue mineralisation that remains or is returned to soil (kg N/year). We have decided to estimate crop residue on the basis of data on the production of individual arable crops, vegetables, fodder plants and industrial plants while considering the ratio between the edible (usable) part and that part which remains on the fields, dry matter fraction and N fraction (Table 6.2).  We have used default values from GPG if available; for other crops we obtained data from our Agriculture Institute.

For calculations we have used Tier 1b method and Equation 4.29 from GPG. IPCC methodology envisages that burned crop residue has to be subtracted from the amount of crop residue. Since emissions from burning crop residue have been ignored, burning has not been taken into account here. We also do not use crop residue as a fuel, construction or fodder. To calculate emissions of nitrous oxide, the same emission factor as for N from mineral and organic fertilizers as well as biological N-fixation (0.0125 kg N2O-N/kg N, IPCC, 1996) has been applied.

Emissions of nitrous oxide due to cultivation of organic soils

Cultivation of soils with high contents of organic material (histosols) causes a release of a long-term bound N. The surface of organic soil in Slovenia has been obtained from the pedology map of the Centre for Pedology and Environmental Protection at the Department of Agronomy of the Biotechnical Faculty in Ljubljana, but only the surface of the peat soil of the low moor has been considered. This surface, according to data from year 1998, amounts to 4,320 ha. IPCC (1996) envisages that due to cultivation of organic soils, there is an annual emission of 5-10 kg N2O-N/ha. In our estimate, even if all soil with high contents of organic material were ploughed, the emission from that source would present only 0.6% of the total emissions of nitrous oxide. Because of its small fraction, this source has not been considered. An additional argument against considering this source is the fact that in Slovenia, the cultivation of organic soil is linked to hydro technical amelioration of soils that is causing a decrease in the natural emission of GHG methane, which in the suggested methodology is not subtracted.

#### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 10%.

Uncertainty of emission factor amounts to 500%.

Combined uncertainty amounts to 500.10%.

### Recalculations

Recalculations in manure management sector have influenced also on nitrous oxide emissions from animal manure and liquid/slurry

### Future improvements

No improvements is planned for this category.

### Nitrous oxide emissions from grazing animals

Key sector - Base year: 	no

Key sector - Year 2004:	no

#### Methodological issues

The estimate is based on the amount of N in manure, which is annually applied to soil during grazing. This amount has been calculated on the basis of methodology described in chapter 6.3 (N2O Emissions from Manure Management).

IPCC methodology (1996) suggests using the same emission factor (0.02 kg N2O-N/kg of emitted N) for all grazing animals, regardless of their species and the climatic conditions.

#### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 10%.

Uncertainty of emission factor amounts to 500%.

Combined uncertainty amounts to 111.80%.

### Recalculations

No recalculations have been performed in this category.

### Future improvements

No improvements is planned for this category.

### Indirect N2O emissions from Agricultural Soil

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | N2O | 1.65 |  | 14 |
| 2004 | Level | N2O | 1.52 | 0.36 | 11 |

#### Source category description

The most important indirect sources of nitrous oxide are:

- Volatilisation of ammonia and nitrogen oxides (NOx)

- Nitrogen leaching and runoff

- Municipal sewage (quantities have been estimated, but included in the chapter on waste treatment)

#### Methodological issues

Nitrous oxide arising due to volatilization of ammonia (NH3) and nitrogen oxides (NOX)

In fertilizing agricultural soils with nitrogen fertilizers, some N volatilises in form of ammonia and nitrogen oxides (NOx). This N has not been considered in determining emissions from fertilizing with mineral fertilizers (2.3.1.1) and organic (2.3.1.2) fertilizers. This nitrogen is deposited by precipitation and particulate matter on agricultural soil, in forests and waters and thus indirectly contributes to emissions of N2O. Emissions are attributed to the place of origin of ammonia and NOx, not to the place where N is re-deposited, causing N2O emissions.

Emissions from mineral fertilizers

Indirect emissions of nitrous oxide from mineral fertilizers depend to a large extent on the fraction of N that volatilises during fertilizing. The amount of volatilised N depends very strongly on the type of fertilizer as well as on weather conditions and the manner of application. In Slovenia, data on the consumption of various nitrogen fertilizers are not available, and also IPCC methodology (1996) does not lay down different emission factors. It has been considered that 10% of N from mineral fertilizers volatilises (IPCC, 1996). For calculating indirect emissions of nitrous oxide, the emission factor of 0.01 kg N2O-N/kg NH3 and NOX-N (IPCC, 1996) has been considered.

Emissions from animal manure:

Numerous factors influence the fraction of volatilised N in form of ammonia and nitrogen oxides, such as: the ratio between N excreted in dung and N excreted in urine, the manner of slurry storage, the manner of slurry application etc. In spite of differences, IPCC (1996), for the time being, suggests a generic emission factor; 20% of the excreted N are supposed to volatilise in form of ammonia and nitrogen oxides. Emissions of nitrous oxide have been calculated by multiplying the estimated quantities of volatilised N with emission factor of 0.01 kg N2O-N/kg NH3-N and NOX-N (IPCC, 1996).

Nitrous oxide from leaching and runoff of nitrogen compounds into surface waters, groundwater, and watercourses

The nitrogen that enters groundwater and watercourses, mainly in the form of nitrates, is there subjected to nitrification and denitrification. This gives rise to some nitrous oxide, which is diffused into the atmosphere. Denitrification takes place mostly in groundwater, riverine sediments, and estuarine sediments. This nitrogen, which enters watercourses, contributes to emissions of nitrous oxide also during the course of nitrification. Algae and aquatic plants assimilate nitrates into organic matter, which in decomposing release ammonia that is quickly nitrified in rivers, giving rise to some nitrous oxide in the process.

Surface runoff or leaching of N into groundwater, surface waters, and watercourses due to mineral fertilizers:

It has been considered that 30% of N from mineral fertilizers are leached and run off into the groundwater and watercourses. In calculating emissions of nitrous oxide, it has been considered that for every kg of leached/run-off nitrogen, 0.025 kg of N2O-N is emitted (IPCC, 1996). The applied emission factor is a sum of partial factors (denitrification in soil or in groundwater 15 g N2O-N/kg N, denitrification in river sediments 2.5 g N2O-N/kg N, nitrification in rivers 5 g N2O-N/kg N, nitrification in estuaries 2.5 g N2O-N/kg N).

Nitrogen leaching and runoff into groundwater, surface waters, and watercourses due to animal manure:

It has been considered that for every kg of N, which is excreted by farm animals, 0.3 kg of N run off to watercourses and groundwater (IPCC, 1996). The methodology of estimating annual quantities of N, excreted by individual kinds and categories of animals, has been already described under 2.1.4. With regard to the defined methodology, all N has been considered, i.e. N excreted in indoor housing and N excreted on pasture. To improve the estimate, it might be advisable to particularize the methodology in the future by considering a higher leaching and runoff factor for big farms and areas with intensive livestock production than for areas with unintensive livestock production. In calculating emissions of nitrous oxide, the same emission factor has been considered as in the case of nitrogen leaching/run-off due to mineral fertilizers (0.025 kg N2O-N/kg of leached/run-off N).

#### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 50%.

Uncertainty of emission factor amounts to 70%.

Combined uncertainty amounts to 86.02%.

### Recalculations

Recalculations in manure management have influenced also on indirect emissions from animal manure.

### Future improvements

No improvements is planned for this category.

# LULUCF (CRF sector 5)

## Source category description

Forests exert a strong influence on the carbon cycle while the biomass influences the removal and emissions of CO2, which is one of the major greenhouse gases (70%). In forests, large quantities of carbon are stored in the vegetation and soil, CO2 is built into tree-cells or the building blocks of the cell-structure of trees in the organic compounds of the protoplast and apoplast. According to Brown, emissions and removals of CO2, occurring in the decomposition and burning of woody biomass, and its incorporation into plants in the successive reactions of the Calvin cycle are, on a global scale, in equilibrium. A new research project on emissions and sink of greenhouse gases for forestry as well as land-use change was assigned too the Slovenian Forestry Institute in the end of 2005 (Krajnc. N., Piškur M., and Simončič P., Hočevar M., Kobler A., Kušar G.). While the project is still in the first phase we are still using data from previous projects and official data (Slovenian forestry service) but with some country specific modifications (according to GPG for LULUCF 2003)

Removal of CO2 arising due to the difference between the annual increment in wood mass and cut for the year in question amounted to approximately a fourth part of the Slovenian emissions of CO2. The woody biomass used in woodworking industry shares the fate of wood residues and is either burnt or accumulated in wood products (secondary raw materials), interrupting the carbon cycle and the exchange between the atmosphere and the vegetation as well as between the vegetation and the soil.

The chapter with guidelines for calculation of emissions caused by human activity includes two processes (IPCC Guidelines for National Greenhouse Gas Inventories: Workbook, Reference Manual, 1996). Land-use change caused by forest clearing for agricultural use, including biomass on-site burning, which is particularly important in rapidly developing countries of Latin America, Africa, and in some Asian countries. In Slovenia, we are witnessing an opposite process of regrowth of abandoned managed lands. In addition to the process of land-use change, the CO2 balance for forestry necessitates also a calculation of the biomass accumulated in the woody biomass in the forests of Slovenia.

## Methodological issues

In calculating the balance of CO2 for forests in Slovenia, the existing inventories (Slovenian forestry service), official statistics and some project results (Slovenian forestry service. Statistical office of R Slovenia) have been used. A comparative analysis of data on forest areas, timber-growing stock and annual felling has been carried out. That analysis has shown certain discrepancies between data from different sources.

Considering the principles of IPCC methodology (GPG 2003), which is based on the use of existing inventory data as well as limited time periods, the adjustment (improvement) of not fully reliable data for the past has been limited to the most stringent necessities, while choosing the most reliable of the existing data. The criteria in choosing data were the following: continuity, quality, and reciprocal comparability as well as accessibility of sources.

Table 7.2.1: Forest areas in Slovenia (1880 - 2004)

| Year | Forest Area ha |  | Year | Forest Area ha |  | Year | Forest Area ha |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1880 | 783,646 |  | 1961 | 961,000 |  | 1986 | 1,064,200 |
| 1885 | 820,739 |  | 1962 | 968,222 |  | 1987 | 1,067,400 |
| 1886 | 823,398 |  | 1963 | 975,444 |  | 1988 | 1,070,600 |
| 1887 | 826,057 |  | 1964 | 982,667 |  | 1989 | 1,073,800 |
| 1888 | 828,716 |  | 1965 | 989,889 |  | 1990 | 1,077,000 |
| 1889 | 831,375 |  | 1966 | 997,111 |  | 1991 | 1,081,300 |
| 1890 | 834,034 |  | 1967 | 1,004,333 |  | 1992 | 1,085,601 |
| 1891 | 833,711 |  | 1968 | 1,011,556 |  | 1993 | 1,089,901 |
| 1892 | 833,389 |  | 1969 | 1,018,778 |  | 1994 | 1,094,201 |
| 1893 | 833,066 |  | 1970 | 1,026,000 |  | 1995 | 1,097,929 |
| 1894 | 832,743 |  | 1971 | 1,027,900 |  | 1996 | 1,098,844 |
| 1895 | 832,421 |  | 1972 | 1,029,800 |  | 1997 | 1,109,710 |
| 1896 | 832,098 |  | 1973 | 1,031,700 |  | 1998 | 1,111,006 |
| 1900 | 830,807 |  | 1974 | 1,033,600 |  | 1999 | 1,115,661 |
| 1905 | 838,479 |  | 1975 | 1,035,500 |  | 2000 | 1,134,227 |
| 1910 | 846,151 |  | 1976 | 1,037,400 |  | 2001 | 1,142,869 |
|  |  |  | 1977 | 1,039,300 |  | 2002 | 1,149,633 |
|  |  |  | 1978 | 1,041,200 |  | 2003 | 1,157,824 |
|  |  |  | 1979 | 1,043,100 |  | 2004 | 1,163,812 |
|  |  |  | 1980 | 1,045,000 |  |  |  |
|  |  |  | 1981 | 1,048,200 |  |  |  |
|  |  |  | 1982 | 1,051,400 |  |  |  |
|  |  |  | 1983 | 1,054,600 |  |  |  |
|  |  |  | 1984 | 1,057,800 |  |  |  |
|  |  |  | 1985 | 1,061,000 |  |  |  |

Data in table 7.2.1. are taken from official data from forestry management plans (annual reports of Slovenian forestry service for the period of 1994-2004 and from Statistical office of R Slovenia 1961-1993).

Forest: In 1953, 1956 and 1958, all areas grown over with forest trees as well as smaller incorporated forest areas were shown as forest. Since 1961 the term forest covers areas larger than 0.05 ha that are covered with forest trees, irrespective of their height and growth. Trees lining paths, parks in inhabited areas, forest nurseries and groups of trees on areas smaller than 0.05 ha are not considered as forest.( Statistical Yearbook of the Republic of Slovenia 2005)

Data on forestry reserves (area, growing stock and annual gross increment) for 1953, 1956 and 1958 were supplied by the district forestry authorities, whereas for 1961 they were supplied by the forestry authorities of municipal assemblies. Since then the data were drawn from the inventories for the forestry management plans.

### Changes in Forest and Other Woody Biomass Stocks

In Slovenia mixed forests prevail. Consequently, in Slovenian forestry, it is not the surfaces that are split for being deciduous or coniferous, but we divided total forest area according to relations in total wood stock.

Considering the adjusted values of increases, calculated by the control method, the following values for the current annual increase in tonnes of dry matter have been obtained:

Table 7.2.1: Values of the current annual increase of deciduous and coniferous trees for the period 1986-1996 (According to IPCC 1996)

| Year | Coniferous Trees t dm | Deciduous Trees t dm |
| --- | --- | --- |
| 1986 | 1,046,000 | 1,611,600 |
| 1987 | 1,045,113 | 1,610,233 |
| 1988 | 1,063,371 | 1,638,364 |
| 1989 | 1,124,019 | 1,731,806 |
| 1990 | 1,186,214 | 1,827,632 |
| 1991 | 1,236,940 | 1,945,367 |
| 1992 | 1,264,231 | 2,028,291 |
| 1993 | 1,255,621 | 2,053,767 |
| 1994 | 1,306,576 | 2,177,545 |
| 1995 | 1,292,453 | 2,263,195 |
| 1996 | 1,346,350 | 2,246,476 |

Table 7.2.2: Values of the current annual increment of deciduous and coniferous trees for the period 1995-2004 (According to GPG 2003)

|  | Coniferous | Deciduous |
| --- | --- | --- |
| years | Tonnes d.m.yr-1 | Tonnes d.m.yr-1 |
| 1995 | 1,708,907 | 2,768,235 |
| 1996 | 1,734,238 | 2,811,100 |
| 1997 | 1,733,002 | 2,845,392 |
| 1998 | 1,739,180 | 2,850,536 |
| 1999 | 1,760,186 | 2,913,119 |
| 2000 | 1,912,171 | 3,238,037 |
| 2001 | 1,928,235 | 3,261,184 |
| 2002 | 1,970,247 | 3,354,631 |
| 2003 | 2,008,552 | 3,463,508 |
| 2004 | 2,048,711 | 3,540,666 |

In table 7.2.1 data for average annual increment above and below ground are calculated. All presented data are from official statistic of Slovenian forestry service. Data for the period before 1995 will be recalculated in next report. We are planning to carry out national forestry inventory (NFI) in 2007 to improve the quality of data (plots on 4 *4 km grid).

Table 7.2.3: Step one of calculations in the submodule “Changes in Forest and Other Woody Biomass Stocks”

|  | Felling (merchantable volume) (underbark) (k m3) |  |  |  | Carbon loss due to fellings (Gg C) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Years | SUM | Coniferous | Deciduous | Correc. decid.*1 | Coniferous | Deciduous | Sum |
| 1995 | 1751 | 1042 | 709 | 1013.9 | 377.9 | 492.5 | 870.41 |
| 1996 | 1990 | 1277 | 713 | 1019.6 | 463.1 | 495.3 | 958.41 |
| 1997 | 2207 | 1172 | 1035 | 1480.1 | 425.0 | 719.0 | 1144.03 |
| 1998 | 2132 | 1187 | 945 | 1351.4 | 430.5 | 656.5 | 1086.94 |
| 1999 | 2068 | 1147 | 921 | 1317.0 | 415.9 | 639.8 | 1055.77 |
| 2000 | 2253 | 1209 | 1044 | 1492.9 | 438.4 | 725.3 | 1163.70 |
| 2001 | 2257 | 1240 | 1017 | 1454.3 | 449.7 | 706.5 | 1156.18 |
| 2002 | 2283 | 1272 | 1011 | 1445.7 | 461.3 | 702.3 | 1163.62 |
| 2003 | 2591 | 1549 | 1042 | 1490.1 | 561.7 | 723.9 | 1285.60 |
| 2004 | 2551 | 1556 | 995 | 1422.9 | 564.3 | 691.2 | 1255.49 |

Note *1: We corrected harvest figures for deciduous trees in private forests. The correction factor used is 1.43 (only for harvest in private). The factor was calculated according to data from national census of Agricultural households in 2000 carried out by Slovenian statistic office (SURS). We expect to have more reliable data about harvest in private forests in the end of 2006 (measurements on permanent plots carried out by SFS (Slovenian forestry service).

In calculation Method 1 (default method) was used. We are considering to change or to combine the method after the results of NFI (in 2007).

Table 7.2.4: Factors used in calculation (according to GPG 2003)

|  | D | BEF1 | R | BEF2 | CF |
| --- | --- | --- | --- | --- | --- |
| Coniferous | 0.407 | 1.20 | 0.32 | 1.35 | 0.5 |
| Deciduous | 0.567 | 1.15 | 0.26 | 1.36 | 0.5 |

D: basic wood density was calculated from table 3A,1.9 (GPG 2003) according to tree species in growing stock in Slovenia

BEF1: was taken from 3A,1.10 as default values

R: was taken from GFRA 2005 (report for Slovenia)

BEF2: was calculated with:

BEF2=ki∗BEF1

ki is national factor for recalculating merchantable volume of trees to above ground biomass (including bark):

k1 – for coniferous is 1.1765

k2 – for deciduous is 1.13636

CF: default factor is 0.5

In our calculation (Equation 3.2.7 GPG 2003) of annual carbon loss due to felling we didn’t consider factor fBL. Main reason is that we don’t have reliable national specific data about dead wood carbon stocks. We are planning to have more data after NFI (2007). Belowground biomass losses were calculated as a part of carbon loss due to fellings.

Table 7.2.5: Annual change in carbon stock in living biomass in forest land remaining forest land (default method GPG 2003)

| v gG CO2 |  |  |  |
| --- | --- | --- | --- |
| Year | Annual increase in carbon stocks | Annual carbon loss due to fellings | Annual change in carbon stocks in living biomass |
| 1995 | 8,208.09 | 3,191.51 | 5,016.58 |
| 1996 | 8,333.12 | 3,514.17 | 4,818.95 |
| 1997 | 8,393.72 | 4,194.77 | 4,198.96 |
| 1998 | 8,414.48 | 3,985.46 | 4,429.02 |
| 1999 | 8,567.73 | 3,871.14 | 4,696.59 |
| 2000 | 9,442.05 | 4,266.89 | 5,175.16 |
| 2001 | 9,513.94 | 4,239.33 | 5,274.60 |
| 2002 | 9,762.28 | 4,266.60 | 5,495.68 |
| 2003 | 10,032.11 | 4,713.88 | 5,318.23 |
| 2004 | 10,247.19 | 4,603.47 | 5,643.72 |

### Regrowth of Abandoned Managed Lands

In calculating the submodule “Regrowth of Abandoned Lands”, the “Grasslands” category has been ignored, since in the climate conditions prevailing in Slovenia the forest as the climax type of vegetation, not grasslands, is the final result of regrowth of abandoned agricultural areas. To avoid double counting of forest areas, the calculation of the submodule “Forest and Grassland Conversion” has been omitted. The reason for this is that clearing of forest areas (regardless of the cause) and regrowing of abandoned managed lands have already been taken into account in the changes in forest areas in submodule “Regrowth of Abandoned Managed Lands”.

Table 7.2.6: Forest surfaces under regrowth for the period 1986 – 1996.

|  | Surfaces under Regrowth |  |  |
| --- | --- | --- | --- |
| Year | ha | m3 | t dm |
| 1986 | 69,501 | 243,253 | 121,626 |
| 1987 | 69,270 | 242,444 | 121,222 |
| 1988 | 69,039 | 241,636 | 120,818 |
| 1989 | 68,808 | 240,827 | 120,414 |
| 1990 | 68,577 | 240,019 | 120,009 |
| 1991 | 68,266 | 238,932 | 119,466 |
| 1992 | 67,956 | 237,846 | 118,923 |
| 1993 | 67,646 | 236,760 | 118,380 |
| 1994 | 67,335 | 235,673 | 117,837 |
| 1995 | 67,066 | 234,731 | 117,366 |
| 1996 | 67,000 | 234,500 | 117,250 |

The process of land-use change, forest clearing as well as conversion to pastures and grassland causes a temporary increase of CO2 emissions, while on the contrary, regrowth of abandoned managed lands functions as a CO2 sink. According to IPCC methodology, the submodule “Regrowth of Abandoned Managed Lands” encompasses calculations of CO2 emissions due to regrowth of abandoned managed lands or forest clearing (which have been included in our calculations), and an estimate of emissions due to on-site or off-site burning of biomass, as well as an estimate of release of CO2 due to the decay of aboveground biomass. In Slovenia, a process of regrowth of abandoned managed lands (pastures, meadows) is in course, contributing to the increase of aboveground and belowground woody biomass. Step five of the submodule puts forward the calculation of carbon released from biomass due to forest clearing (forest  grasslands).

The increase of biomass due to regrowth is calculated for abandoned lands, i.e. the ones having been overgrown. These are areas, which have not yet been classified as forests proper or lands, which started to be overgrown 20 or less years ago. In 1996, these surfaces amounted to 67,000 hectares. For prior years prior no comparable data are available. Surfaces under the process of regrowth between 1986 and 1995 have been calculated by correspondingly adjusting the data for 1996 to the size of non-forest surfaces in a particular year. As mentioned before, forest areas have not been disaggregated into coniferous and deciduous forests and therefore the increase of biomass on the newly won forest areas for coniferous in deciduous trees has been calculated together. In calculating the absolute increase (m3), the value for the current annual increase of woody biomass on areas under regrowth has been taken into account, which had been given in an expertise by L. Žgajnar, amounting to 3.5 m3/ha. Considering the total conversion factor of 0.5 t dry matter per m3, the following values for regrowth have been obtained:

Table 7.2.7: Step one of calculations in the submodule “Regrowth of Abandoned Managed Lands” for the last 20 years

|  | A Area under Regrowth in the Last 20 Years (k ha) | B Biomass Growth Increment (t dm) | D Carbon Fraction in Aboveground Biomass | E Uptake of C/yr into Aboveground Biomass (kt C) |
| --- | --- | --- | --- | --- |
| 1996 | 67 | 117 | 0.5 | 59 |
| 1995 | 67 | 117 | 0.5 | 59 |
| 1994 | 67 | 118 | 0.5 | 59 |
| 1993 | 68 | 118 | 0.5 | 59 |
| 1992 | 68 | 119 | 0.5 | 60 |
| 1991 | 68 | 119 | 0.5 | 60 |
| 1990 | 69 | 120 | 0.5 | 60 |
| 1986 | 70 | 122 | 0.5 | 61 |

Table 7.2.8: Step three of calculations in the submodule “Regrowth of Abandoned Managed Lands” for the last 20 years

|  | L  (L= E) Carbon Uptake on Land under Regrowth (kt C) | M  (L x (44/12) Total CO2 Uptake on Lands in Question (Gg CO2) |
| --- | --- | --- |
| 1996 | 59 | 216 |
| 1995 | 59 | 216 |
| 1994 | 59 | 216 |
| 1993 | 59 | 216 |
| 1992 | 60 | 220 |
| 1991 | 60 | 220 |
| 1990 | 60 | 220 |
| 1986 | 61 | 224 |

### CO2 Emission Due to Land-use Change

The last submodule is about the calculation of CO2 emissions and uptake by soils from land-use change and management. IPCC methodology (1996) includes three processes in this calculation: changes in carbon stored in soil and litter of mineral soils due to changes in land-use practices (grassland  forest); CO2 emissions from organic soils converted to forests (plantation forestry) and vice versa; CO2 emissions from liming of agricultural soils. After yearlong unchanged land use, the input of carbon due to decaying organic matter and the loss of carbon due to decomposition of organic matter for forest, brushwood, and grasslands are believed to be balanced and, consequently, also emissions and removal of CO2 of the ecosystem in question. The criterion for including carbon balance in soil into the total calculation is that there was in the last twenty years an occurrence of a statistically significant change in the stocks of carbon in soil for the area in question. In Slovenia, forest areas have increased in the 1986-1996-inventory period by 34,644 hectares. The steps of this submodule include calculations of emissions/removal of CO2 due to land-use change (regrowth of abandoned managed lands – first step; emissions of carbon from agricultural areas with intensive cultivation – second step, which is not considered) as well as emissions from liming of agricultural soils. Finally, in the fifth submodule, the sum total of CO2 emissions or removals is calculated; this is positive in case of emissions of CO2, or negative case of CO2 removal.

On the analogy of the calculation of increase of woody biomass due to regrowth, the change in the content of carbon in mineral soils of the land under regrowth has been calculated (see table). Based on an expert estimate, 25 % of soil in the areas under regrowth has been classified as less, and 75% as soil that is more active. Sources of data for the last submodule were the Statistical Office of the Republic of Slovenia (1997) and an expert evaluation (oral source; Mihelič R., 1999). Calculations included previously used data on forest areas (see appendices). This module would actually require detailed data, but unfortunately, that was not possible. Factors suggested in IPCC guidelines have been used in calculations.

Table 7.2.9: Step one of submodule “CO2 Emissions due to Land-use Change and Liming”

|  | B Soil Activity by Soil Type | C Soil Carbon acc. to IPCC (Mg C/ha) | D Forest Area under Regrowth in the Last 20 yr. (Mha) | H Change of Carbon Content (Tg/20 years) | H Total Change of Carbon Content (Tg/20 years) |
| --- | --- | --- | --- | --- | --- |
| 1996 | High | 95 | 0.050 | 4.75 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.03 |
| 1995 | High | 95 | 0.050 | 4.75 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.03 |
| 1994 | High | 95 | 0.051 | 4.85 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.12 |
| 1993 | High | 95 | 0.051 | 4.85 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.12 |
| 1992 | High | 95 | 0.051 | 4.85 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.12 |
| 1991 | High | 95 | 0.051 | 4.85 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.12 |
| 1990 | High | 95 | 0.051 | 4.85 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.12 |
| 1986 | High | 95 | 0.052 | 4.94 |  |
|  | Low | 75 | 0.017 | 1.28 | 6.22 |

Table 7.2.10: Step one of submodule “CO2 Emissions due to Land-use Change and Liming”

|  | A Quantity (Mg) | B Conversion Factor for Carbon | C Emissions of C due to Soil Liming (Mg C) |
| --- | --- | --- | --- |
| 1996 |  100,000 | 0.120 |  12,000 |
| 1986 |  100,000 | 0.120 |  12,000 |

Figure 7.2.291: Step three of Submodule “CO2 Emissions due to Land-use Change and Liming”

|  | A | B Carbon Conversion Factor t | C Total Annual Emissions of C (Gg) | D Conversion to Annual Emission of CO2* 44/12 (Gg / year) |
| --- | --- | --- | --- | --- |
|  | 6.03 | - 50 | - 302 |  |
| 1996 |  12,000 | 0.001 | 12 | -1,061 |
|  | 6.03 | - 50 | - 302 |  |
| 1995 |  12,000 | 0.001 | 12 | -1,061 |
|  | 6.12 | - 50 | - 306 |  |
| 1994 |  12,000 | 0.001 | 12 | -1,078 |
|  | 6.12 | - 50 | - 306 |  |
| 1993 |  12,000 | 0.001 | 12 | -1,078 |
|  | 6.12 | - 50 | - 306 |  |
| 1992 |  12,000 | 0.001 | 12 | -1,078 |
|  | 6.12 | - 50 | - 306 |  |
| 1991 |  12,000 | 0.001 | 12 | -1,078 |
|  | 6.12 | - 50 | - 306 |  |
| 1990 |  12,000 | 0.001 | 12 | -1,078 |
|  | 6.22 | - 50 | - 311 |  |
| 1986 |  12,000 (III/V) | 0.001 | 12 | -1,094 |

## Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 20%.

Uncertainty of emission factor amounts to 30%.

## Source-specific planned improvements

The LUCF sector will in future require by far the highest number of improvements if we are to be sure that calculations are brought up to date and their accuracy is comparable to that of other sectors. The review process performed by the UNFCCC secretariat in 2004 confirmed this.

A new research project on emissions and sink of greenhouse gases for forestry as well as land-use change was assigned too the Slovenian Forestry Institute in the end of 2005 In this project a group of experts from forestry and agricultural sector was established. While the project is still in the first phase we are still using data from previous projects and official data (Slovenian forestry service) but with some country specific modifications (according to GPG for LULUCF 2003). Planned activities in this project are:

- National forestry inventory: test in 2006 (16 * 16 km grid), realization of NFI in 2007 (4 * 4 km grid)

- Analysis of carbon in forest soil and litter (16 * 16 km grid) in 2006

- Recalculation of time series (1986-2006)

- Analysis of carbon stock in harvested wood products (Appendix 3A.1)

- Assessment of country specific factors (basic density)

- Annual change in carbon stock caused by Land-use change

Special emphasis will be given to analysis of annual change in carbon stock in living biomass in forest land remaining forest land because it planned to be a key category for Slovenia (Tier 2 or 3).

We are aware of the fact that this sector is of particular importance for Slovenia because sinks, due to the abundance of forests in Slovenia, play a crucial role in achieving the goals that have been set by the Kyoto protocol.

The new methodology, described in Good Practice Guidance and LULUCF, which was published in 2003, proves very challenging, both in terms of the data required as well as its extensiveness. At the same time, it calls for a multidisciplinary approach in calculating emissions and sinks with the cooperation of experts from the fields of forestry, agriculture, ecology, and climatology. Furthermore, it will be necessary to secure data that will be obtained on the basis of verified methodological procedures with minimum errors and maximum accuracy, and that will make temporal traceability possible. A significant improvement of calculations from this sector is planned for the year 2006 and 2007.

# WASTE (CRF sector 6)

Waste management and treatment of industrial and municipal wastes are sources of GHGs emissions. The inventory covers CH4 emissions resulting from solid waste disposal on land and treatment of liquid wastes. This section also includes estimates of emission of N2O from municipal sewage.

Figure 8.0.1: Emissions from solid waste disposal on land and from wastewater handling in Gg CO2 eq.

## CH4 Emissions from Solid Waste Disposal sites

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 1.48 |  | 16 |
| 2004 | Level, Trend | CH4 | 2.08 | 1.62 | 8 |

### Source category description

Methane is emitted during anaerobic fermentation of degradable organic substances in solid waste disposal sites in processes, which may last several decades. If waste were not disposed of on solid waste disposal sites, the degradation would take place in aerobic conditions without methane formation. Methane emissions from waste disposal are thus of anthropogenic origin and, consequently, a constituent part of national GHG inventories in accordance in IPCC methodology.

### Methodological issues

Methodology

In terms of methodology, IPCC lays down two levels of ascertaining methane emissions from waste management. The simpler, default tier is based on the assumption that all methane is released in the year the waste is disposed of. The default time-dependent emission profile of methane is thus of zero order. This simplified tier will give a fair description of methane emissions of time-dependence only in case if the amount and composition of deposited waste have been constant or slowly varying over a prolonged period of time.

IPCC guidelines describe a more precise method, which considers that methane emissions from disposed-of waste are released over a longer period of time. The so-called First Order Decay (FOD) method is based on the assumption of an exponential time-dependent decline of emissions. Annual emissions are thus partial sums of emissions from waste disposed of in previous periods. The FOD method is more precise, but requires data on quantity, composition and disposal conditions for a period of 20 to 30 years prior to the year for which emission are determined. At the same time, it is necessary to know the half-life of methane generation.

As methane emissions from SWDS are our key source we have to use FOD method to calculate these emissions.

This method can be represented by the equation (1) and (2):

CH4 generated in year t (Gg/yr) =Σx [(A·k·MSW(x) ·LO(x)) ·e –k(t-x) ]             (1)

for x = initial year to t

where:

t = year of inventory

x = years for which input data should be added

A = (1 – e-k)/k; normalisation factor which corrects the summation

k = methane generation rate constant (1/yr)

MSW(x) = total municipal solid waste landfilled (Gg/yr)

L0 = methane generation potential (Gg CH4/Gg waste)

Methane generation rate k has been taken from GPG, 2000 and is 0.05. This value is appropriate for countries with average humid climate and for mixture of slow and fast degradable components in the waste. The half time period is 14 years.

The methane generation potential (Lo) depends upon the composition of waste, on waste disposal practices and on the physical characteristics of the SWDS. It is estimated by the formula:

Lo = MCF  DOC  DOCF  F  16/12

where:

MCF = CH4 correction factor (fraction)

DOC = degradable organic carbon (fraction) (Gg C/Gg waste)

DOCF = fraction DOC dissimilated

F = fraction by volume of CH4 in landfill gas

16/12 = conversion from C to CH4

The estimation of CH4 emitted each year, results from equation (2):

CH4 emitted (Gg/yr) = [CH4 generated – R] . (1 – OX)            (2)

where:

R = CH4 recovered (Gg/yr)

OX = oxidation factor (fraction)

Activity data and parameters

Presently, most waste in Slovenia is landfilled (69.4%) and only to a lesser degree reused or recycled (30.1%). There is no thermal processing of waste, except for minor quantities of some types of waste (0.5%). In 2004, the recovery of landfill gas has been set up on some major solid waste disposal sites, and its use for generating energy has been started at three largest solid waste disposal sites (Ljubljana-Barje, Maribor-Pobrežje, Celje-Bukovžlak), which cover some 30% of the entire population.

##### The amount of waste in the period 1964 – 1994

There are no data on the amount of waste prior to 1995. The first regulated municipal solid waste disposal site, the Ljubljana Barje SWDS, started its operation in 1964. An estimate for the period 1964 - 1994 arrived on presumption that in 1964 50% of population was included in municipal waste collection system and that this percentage have slightly increased end reach 60% in 1977 and 76% in 1995. The composition of which was deemed same all time. We are fully aware that this is a rough estimate, but since methane emissions from that period exert a significantly smaller influence on emissions in the reported period, this error is not considerable.

We have also assumed that in that period an amount of waste per person was 470 kg/year. For 1995 on we have used actual data on amount of waste.

##### The amount of waste in the period 1964 – 1994

For that period, we considered that the covering (the inclusion of population into public waste-removal systems) gradually increased from 60% in 1977 and reached 76% in 1995. Consequently, we increased data on the quantity of dumped waste to match the covering.

##### The amount of waste in the period 1995 – 2000

According to data provided by the SURS (data submitted to EUROSTAT), over 1 million tons of municipal waste were produced in that period. It is inferred that that amount, due to unclear definition of municipal waste, included industrial waste as well. The calculation of quantities in the table above takes into account the assumption that all collected municipal waste was landfilled (which for Slovenian circumstances is quite accurate).

##### The amount of waste in the period 2001 - 2004

The initial parameters for calculating methane emissions are the total annual amount of municipal waste and the fraction of landfilled municipal waste. In calculating, we used data of the Environmental Agency of the Republic of Slovenia, which on a regular basis collects data on the formation and handling all types of waste in Slovenia. Data are collected by means of forms which are set down by the law and which the reporting agent must fill in once a year (for the preceding year). Beside the quantities of collected mixed waste and separate fractions of municipal waste from households, reporting agents also provide data on the quantities of collected mixed and separate fractions of municipal waste and related waste which is produced in economic and service activities.  On the basis of these data, the Statistical Office of the Republic of Slovenia generate its annual reports on waste handling.

Table 8.1.1: Quantities of landfilled waste in the period 1964 -- 2004.

| Year | Municipal waste (t) | Industrial waste | Total waste(t) | Year | Municipal waste (t) | Industrial waste | Total waste(t) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1964 | 383,180 | 300,000 | 683,180 | 1985 | 622,376 | 259,309 | 881,684 |
| 1965 | 387,847 | 300,000 | 687,847 | 1986 | 633,037 | 263,278 | 896,316 |
| 1966 | 400,205 | 300,000 | 700,205 | 1987 | 644,144 | 266,804 | 910,947 |
| 1967 | 405,318 | 300,000 | 705,318 | 1988 | 655,907 | 269,672 | 925,579 |
| 1968 | 416,386 | 300,000 | 716,386 | 1989 | 664,069 | 276,142 | 940,211 |
| 1969 | 418,907 | 300,000 | 718,907 | 1990 | 671,980 | 282,862 | 954,842 |
| 1970 | 430,074 | 300,000 | 730,074 | 1991 | 681,580 | 287,894 | 969,474 |
| 1971 | 432,961 | 300,000 | 732,961 | 1992 | 687,897 | 296,209 | 984,105 |
| 1972 | 444,532 | 300,000 | 744,532 | 1993 | 694,418 | 304,319 | 998,737 |
| 1973 | 448,243 | 300,000 | 748,243 | 1994 | 702,108 | 311,260 | 1,013,368 |
| 1974 | 460,768 | 300,000 | 760,768 | 1995 | 707,000 | 317,000 | 1,024,000 |
| 1975 | 482,226 | 300,000 | 782,226 | 1996 | 725,000 | 336,000 | 1,061,000 |
| 1976 | 504,485 | 300,000 | 804,485 | 1997 | 743,000 | 351,000 | 1,094,000 |
| 1977 | 518,699 | 245,933 | 764,632 | 1998 | 761,000 | 357,100 | 1,118,100 |
| 1978 | 533,040 | 246,223 | 779,263 | 1999 | 780,000 | 371,100 | 1,151,100 |
| 1979 | 546,537 | 247,357 | 793,895 | 2000 | 800,000 | 271,157 | 1,071,157 |
| 1980 | 559,969 | 248,557 | 808,526 | 2001 | 820,000 | 263,707 | 1,083,707 |
| 1981 | 572,769 | 250,389 | 823,158 | 2002 | 822,339 | 273,918 | 1,096,257 |
| 1982 | 583,024 | 254,766 | 837,789 | 2003 | 844,606 | 272,352 | 1,116,958 |
| 1983 | 593,592 | 258,829 | 852,421 | 2004 | 810,647 | 221,210 | 1,031,857 |
| 1984 | 604,686 | 262,366 | 867,053 |  |  |  |  |

The quantities of municipal waste have marked a slight decrease in recent years. There are several possible explanations for this, one of them being that the quantities in previous years have mostly been arrived at by estimation, whereas in the last three years we had at our disposal very accurate data from all SWDS. At the same time, the area where waste is collected separately and then recycled is getting ever wider.

Figure 8.1.30: Population served by solid waste collection systems

Methane correction factor (MCF) accounts for the effect of management practices on CH4 generation. Unmanaged disposal sites present lower methane-generating potential, because a larger fraction of waste decomposes aerobically in the top layers of unmanaged SWDS.

The IPCC guidelines describes managed SWDS site as site with one off the following:

- cover material

- mechanical compacting

- levelling of waste

In 1964, the Ljubljana-Barje SWDS started to operate as our first managed solid waste disposal site. Other existing solid waste disposal sites were unmanaged at that time. In our estimate, roughly half of the waste was collected at managed SWDS, half at unmanaged SWDS. As the depth of unmanaged SWDS at that time is unknown, we considered them as non-categorised and assumed a MCF of 0.6 for them, while assuming a MCF of 1 for managed SWDS . For the entire period 1964-1976, emissions have been calculated with an average value of MCF, i.e. 0.8.

1977 saw an accelerated rate of controlled placement of waste, with the result that three quarters of waste were disposed of on managed solid waste disposal sites in that year, and we therefore assumed a MCF of 0.90. Following that year, all other solid waste disposal sites progressively introduced managing practices and since 1986 all other SWDS in Slovenia may be classified as managed SWDS. Accordingly, MCF was increased linearly for the period from 1977 to 1986 MCF, and since 1986 it is equal to 1.

Figure 8.1.2: Methane correction factor

Degradable organic carbon (DOC) is the organic carbon that is accessible to biochemical decomposition. It is a function of the composition of waste and can be calculated from a weighted average of carbon content of various components of waste.

DOC = (0.4  A) + ( 0.17  B ) + ( 0.15  C ) + ( 0.3  D )

where:

A = fraction of waste that is paper and textiles

B = fraction of waste that is garden waste, park waste or other non-food organic putrescibles

C = fraction of waste that is food waste

D = fraction of waste that is wood or straw

For mixed waste, which represents the major part of municipal and similar types of waste, we assumed the composition or fractions A, B, C in D, as stated in the Operational programme of waste disposal to be the same for the entire period: A:12%, B:5%, C:25%, D:5%.

The composition of waste and the composition of its biodegradable part is taken from the results of certain screening analyses in Slovenia.

In the year 2005 and partly in the year 2006 a new screening analyze of mixed municipal waste have been performed. The results have been as followed: 25.7% A, 21.5% C in 8.1% D, or summed together 55.3% of degradable wastes. We have used these data also for the year 2004. When we considered all waste disposed in the SWDS and fraction of degradable waste in other types of disposed wastes we have estimate the following composition of waste for 2004: A:24%, C:20% and D:8%. The fraction of waste that is garden waste or park waste is zero because of legislation which prohibited the deposition of such type of waste on SWDS. We have interpolated the values for the period 2001-2003.

For the composition of industrial waste we have no data for the years before 2001. In last four years because of legislation no degradable industrial waste is allowed to be disposed on MSWD. This type of industrial waste is mostly transformed to compost in aerobic process without methane formation. We decided to take conservative approach not considering degradable industrial waste the same composition as was ascribed to municipal waste, whereas for the last three years the composition was calculated on the basis of data provided by ARSO.

Table 8.1.2: Fractions of degradable waste in municipal waste and degradable organic carbon (DOC).

| year | A paper textiles | B garden waste | C food waste | D wood straw | Degradable waste in % | DOC |
| --- | --- | --- | --- | --- | --- | --- |
| 1964-2000 | 12 | 5 | 25 | 5 | 47 | 10.9 |
| 2001 | 14 | 4 | 24 | 6 | 48 | 11.7 |
| 2002 | 18 | 3 | 22 | 7 | 50 | 13.1 |
| 2003 | 20 | 2 | 21 | 7 | 50 | 13.6 |
| 2004 | 24 | 0 | 20 | 8 | 52 | 15.0 |
| 2005 | 24 | 0 | 20 | 8 | 52 | 15.0 |

Fraction of degradable organic carbon dissimilated (DOCF) is an estimate of the fraction of carbon that is ultimately degraded and converted into landfill gas, and reflects the fact that some organic carbon does not degrade, or degrades very slowly, when deposited in SWDS.

The IPCC Guidelines, 1996 provide a default value of 0.77 for DOCF. Based on a review of recent literature, it appears that this default value may be an overestimate. It should only be used if lignin C is excluded from the DOC value. It is a good practice to use a value of 0.5-0.6 as the default. In our calculations, we assumed a DOCF value of 0.55 as a median value of the recommended interval.

Process Fraction of CH4 in landfill gas (F) reflects the fact that biogas is mainly consisted of CH4 and CO2 (usually considered half of each gas). We use 0.5 as most usually taken value in our estimates.

CH4 recovery (R) is the amount of CH4 generated at SWDS that is recovered and combusted (e.g. flared or used for energy). The amount of methane that is recovered and flared or oxidised in gas engines is subtracted from the annual methane emissions. Data on the quantities of recovered methane from 2000 on were provided by the Waste Sector (ARSO) and for previous years directly from disposal sites. Since there are no data on the amount of recovered methane for 2001, an interpolated value was used in calculation.

Oxidation factor (OX) reflects the portion of CH4 from SWDS that is oxidised in the soil or other material covering the waste. The amount of CH4 that oxidises turns primarily to CO2. If the OX is zero, no oxidation takes place, and if OX is 1 then 100% of CH4 is oxidised. Well-managed disposal sites tend to have higher oxidation results than unmanaged dump sites with no cover or where large amounts of CH4 can escape through cracks in the cover. We assumed that, in our case, OX was very close to zero.

Figure 8.1.3: Methane recovery in tons.

Emissions estimates according to default method (Tier1) and first order decay method are presented in the Figure 8.1.4.

Figure 8.1.4: Methane emissions from SWDS in Gg CO2.

The essential difference between the results obtained by these two methods is in emissions of the reference year and in the response of certain emissions to waste-management measures. The quantities of disposed of waste have been rising in the last 30 years and therefore emissions calculated according to the first order decay method for the reference year are, as a rule, lower. The first order decay method takes into account also the contribution of waste disposed of in the past, therefore the reduction of quantity of disposed of biodegradable waste in certain emissions shows only after a certain time delay.

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 30%.

Uncertainty of emission factor amounts to 40%

### Source specific recalculations

In former submission we have used the same composition data for industrial waste as for municipal waste. Experts from review team have not agreed with this assumption as industrial wastes usually don’t consist of so many degradable wastes as municipal wastes.  After checking all available data we have decided to not taken industrial waste in our calculation. This assumption is good for the last years but maybe underestimate our emissions in the base year. We have recalculated estimates from entire period for this reason.

### Future improvements

No improvements is planned for this category.

## Emissions from Wastewater Handling

### Source category description

IPCC methodology recommends separate handling of municipal and industrial wastewater. Wastewater handling produces methane, provided the disposal is anaerobic. IPCC guidelines specify methods for calculation of emissions of methane from wastewater handling. The research project on emissions of methane from wastewater handling for Slovenia has been carried out by the Ministry of Environment, Spatial Planning, and Energy (Dolenc, Žitko-Štemberger, 1999). This report brings merely the basic procedures and results of calculations that demonstrate the relations with IPCC methodology. Detailed calculations and all procedures are specified in that source.

Nitrous oxide from human sewage is included in this chapter as well.

Figure 8.2.31: Emissions from wastewater handling in Gg CO2 eq..

### Methodological issues

#### Municipal Wastewater

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 0.56 |  | 24 |
| 2004 | Level | CH4 | 0.48 | 0.22 | 25 |

CH4 EMISSIONS

IPCC methodology has been used in calculating the emission of methane from municipal wastewater handling

As a first step, it is necessary to determine the quantity of total organic decomposable matter in the wastewater (TOWdom) and sludge (TOSdom)

TOWDOM=P∗DDOM∗1−DSDOM

TOSDOM=P∗DDOM∗DSDOM

P – Population (in 1000 persons)

DDOM – Degradable organic component in wastewater per 1000 persons annually

DSDOM = Fraction of degradable organic component removed as sludge

Secondly, the emission factor for wastewater and sludge is estimated. Emission factor in kg CH4/kg of degradable organic component for:

EFi=B0∗x(WSIx∗MCFix)	for wastewater

EFj=B0∗y(SSjy∗MCFjy)	for sludge

B0 – Maximum methane-producing capacity for the wastewater or sludge

Wsix = Fraction of wastewater from source i treated in system x

MCFix = Methane conversion factor for an individual type of wastewater handling system

SSjy = Fraction of sludge from source j treated in system y

MCFjy = Methane conversion factor for sludge handling

Total emissions of methane from wastewater and sludge handling are:

WM=i(TOWi∗EFi−MRi)

SM=j(TOWj∗EFj−MRj)

MRi/i - Quantity of methane recovered or flared from a wastewater or sludge

Total emission of methane from municipal wastewater treatment and from handling of sludge from municipal wastewater amounts to:

TM=WM+SM

Population

Dispersed settlement and a large number of communities with less than 2,000 inhabitants exert a strong influence on the extent and structure of municipal infrastructure as well as on the organisation of municipal activities. In 2004, only about 60% of inhabitants were connected to the public sewage system and only for some 30% of inhabitants a sewage system ended in a wastewater treatment plant. Wastewater treatment is done in various ways. Some 12% of municipal wastewater undergoes only coarse mechanical treatment, 3% primary treatment, only 15% secondary, and 1% tertiary wastewater treatment.

With regard to CH4 emissions, municipal wastewater that undergoes only coarse treatment may be regarded as untreated water.

The number of inhabitants according to various types of municipal wastewater treatment is shown in table 8.2.1. population fractions in figure 8.2.1. Data are taken from the database on municipal wastewater treatment plants at ARSO.

Figure 8.2.32: Fraction of inhabitants included into various types of municipal wastewater treatment (septic tanks (pit), primary treatment, secondary/tertiary treatment and no treatment) in percentage.

Table 8.2.1: Number of inhabitants included into various types of municipal wastewater treatment (septic tanks, primary treatment, secondary/tertiary treatment, etc.) in thousands.

| Year | Ppit | Pprim | Psek/ter | Pw/o |
| --- | --- | --- | --- | --- |
| 1986 | 967 | 54 | 228 | 732 |
| 1987 | 957 | 54 | 238 | 740 |
| 1988 | 947 | 54 | 248 | 751 |
| 1989 | 937 | 54 | 258 | 750 |
| 1990 | 928 | 54 | 268 | 749 |
| 1991 | 900 | 54 | 295 | 753 |
| 1992 | 900 | 54 | 295 | 747 |
| 1993 | 900 | 54 | 295 | 742 |
| 1994 | 900 | 54 | 295 | 740 |
| 1995 | 900 | 54 | 295 | 739 |
| 1996 | 900 | 54 | 295 | 742 |
| 1997 | 900 | 54 | 299 | 734 |
| 1998 | 900 | 54 | 303 | 726 |
| 1999 | 900 | 54 | 307 | 725 |
| 2000 | 900 | 54 | 311 | 725 |
| 2001 | 900 | 54 | 315 | 723 |
| 2002 | 900 | 54 | 319 | 723 |
| 2003 | 900 | 54 | 323 | 719 |
| 2004 | 900 | 54 | 323 | 721 |

Degradable organic component: in biochemical oxygen demand per person is overall default 60 g BOD/person/day or 21,900 kg BOD/1000 person/year

Fraction of degradable organic component removed as sludge:

The results of mass balances of operation of biological wastewater treatment plants show that the fraction of degradable organic compounds which will decompose as sludge is around 40%.

Methane conversion factor:

The calculated amount of generated methane is dependent on the methane conversion factor, which tells us what fraction is actually transformed into methane. MCF is 0 for completely aerobic systems and 1 for completely anaerobic systems. Wastewaters are mostly handled aerobically, only for secondary treated wastewaters we assumed that MCF equalled 0.05, which was to account for slight irregularities in wastewater treatment in biological wastewater treatment plants.

In calculating emission factor for sludge, we assumed a value of 0.8 for MCF in handling sludge from wastewater treatment plants and in calculating the contribution of methane emissions from septic tanks. For untreated or only mechanically dehydrated sludge, which is disposed of on waste disposal sites, we assumed that the value of MCF was 1. In our estimate, in the period in question approximately 39% of sludge from wastewater treatment plants was disposed of on solid waste disposal sites, while 61% was treated in digestion plants.

Compared to anaerobic decomposition processes on solid waste disposal sites, guided processes in digestion plants proceed faster. However, while the process at a solid waste disposal site after a prolonged period leads to complete decomposition of organic matter, the process in digestion plants in wastewater treatment plants stops at a certain level of sludge stabilisation. For this reason, the MCF for sludge from solid waste disposal sites is higher than the CF for methane from managed digestion plants. In our estimate, the fraction of organic matter in sludge, which is anaerobically decomposed during anaerobic stabilisation, amounts only to several ten percent. However, the stabilised sludge, which is disposed of on open solid waste disposal sites still presents a source of methane, because in spite of the access of atmospheric oxygen, further processes are not entirely aerobic. In all probability, the value of 0.8 may be somewhat too high, but because of the small quantity of sludge from wastewater treatment plants the influence of this source on the total emissions from sludge amounts to 20% at the most and the influence of this error to 5% at the most.

Maximum methane producing capacity (B0)

The IPCC Guidelines provide only one default value of B0 that had to be applied to both COD and BOD. This is not consistent with the observed differences between BOD and COD levels in raw sewage where COD (ml/l) is 2 to 2.5 higher than BOD (mg/l). Thus, it is good practice to use a default value of 0.25 kg CH4/kg COD or a default value of 0.6 kg CH4/kg BOD. In our case we use B0 as 0.6 kg CH4/kg BOD.

Methane recovery

In calculating methane emissions, the quantity of methane recovered in sludge treatment in wastewater treatment plants is subtracted from the total methane production. This amount of methane generated in treating technological  wastewaters, which are cleaned in municipal wastewater treatment plants, too. We assumed the quantities of recovered methane remained constant from 1996, although they kept rising in recent years, but we did not have any precise data.

Figure 8.2.33: CH4 emissions from municipal wastewater treatment in Gg of CO2 equivalent.

N2O EMISSIONS

IPCC methodology (1996) assumes that emissions of nitrous oxide from human sewage and the use of that sewage for fertilizing of agricultural land are negligible. For Slovenia, this does not quite hold true since in rural areas sizeable amounts of sewage are used as fertilizer.

Table 8.2.2: Population. protein consumption and N2O emissions in the period 1986-2003.

| year | population in 000 | protein in kg/person/year | N2O in Gg |
| --- | --- | --- | --- |
| 1986 | 1980.7 | 38.0 | 0.189 |
| 1987 | 1989.5 | 37.2 | 0.186 |
| 1988 | 2000.0 | 36.8 | 0.185 |
| 1989 | 1999.4 | 36.8 | 0.185 |
| 1990 | 1998.1 | 38.2 | 0.192 |
| 1991 | 2001.8 | 35.9 | 0.181 |
| 1992 | 1995.8 | 28.8 | 0.144 |
| 1993 | 1990.6 | 32.6 | 0.163 |
| 1994 | 1988.9 | 34.3 | 0.172 |
| 1995 | 1987.5 | 35.5 | 0.177 |
| 1996 | 1991.2 | 37.4 | 0.187 |
| 1997 | 1986.8 | 36.8 | 0.184 |
| 1998 | 1982.6 | 36.2 | 0.180 |
| 1999 | 1985.6 | 39.2 | 0.196 |
| 2000 | 1990.3 | 38.2 | 0.191 |
| 2001 | 1992.0 | 37.4 | 0.187 |
| 2002 | 1995.7 | 37.4 | 0.188 |
| 2003 | 1995.7 | 37.4 | 0.188 |
| 2004 | 1997,5 | 37,4 | 0,188 |

Calculation of emissions of N2O is based on the number of inhabitants in a certain country as well as average consumption of protein per inhabitant in any individual year. The model presumes 0.16 kg N/kg protein as a default fraction of nitrogen in protein as well as an emission factor of 0.01 kg of N2O-N/kg of consumed nitrogen.

For emission calculation statistical data on the population and FAO data on the consumption of protein have been used.

#### Industrial Wastewater

|  | KS | Gas | Contribution to Level % | Contribution to Trend % | Rank KS level |
| --- | --- | --- | --- | --- | --- |
| Base Year | Level | CH4 | 0.48 |  | 27 |
| 2004 | no | CH4 | 0.43 | 0.13 | 28 |

CH4 EMISSIONS

Emissions of methane from industrial wastewater are calculated for the chosen industrial sectors with a large output of wastewater and high content of degradable organic components. In Slovenia, these are in particular the pulp and paper industry, meat processing and leather industry, while minor quantities of organically degradable organic components are produced also by some other sectors.

Calculation of emissions of methane has been performed according to a procedure similar to that for municipal wastewater.

Firstly, the total output of organic components in wastewater for each individual industry (TOWind) and of sludge for each individual industry (TOSind) has to be determined.

TOWind(kgKPKyear)=W∗O∗Dind1−DSind

TOSind(kgKPKyear)=W∗O∗Dind∗DSind

W - Quantity of wastewater / unit of product (m3 / unit of product)

O - Production of individual industry in tonnes

Dind - Concentration of organic component in the wastewater for an individual industry

DSind - Fraction of a degradable organic component removed as sludge

Further, the emissions are calculated in the same way as for municipal wastewater.

Values W and Dind for an individual industry are estimated on the basis of data from reports on operational monitoring, on the basis of cooperation with representatives of individual industries or theoretical values outlined in guidelines. The table 8.2.3 specify values W and Dind, which are used in subsequent calculations.

The sources of data on production are the publications of the Statistical Office of the Republic of Slovenia (results of research for individual years). A detailed presentation of utilized data is compiled in the research project of the Ministry of the Environment and Physical Planning (Dolenc, Žitko-Štemberger 1999).

Table 8.2.3: Quantities of Wastewater per Unit of Product and Concentrations of Organic Components in Wastewater for Selected Industries

| Industry Type | Wastewater Produced (m3/t of product) | COD Value (kg COD/m3 wastewater) |
| --- | --- | --- |
| Prod. of Pulp | 145.5 | 2.0 |
| Prod. of Paper | 37.0 | 1.5 |
| Prod. of Leather | 50.0 | 5.8 |
| Prod. of Fruit. Vegetables | 9.0 | 5.0 |
| Prod. of Milk | 2.8 | 1.5 |
| Prod. of Meat | 11.0 | 2.9 |
| Prod. of Oil | 1.6 | 0.3 |
| Prod. of Starch | 10.0 | 21.0 |
| Prod. of Beer | 6.0 | 2.6 |
| Prod. of Soft Drinks | 4.0 | 0.4 |
| Prod. of Alcoholic. Beverage | 2.6 | 4.0 |

Table 8.2.4: Sectors Included in the Calculation of Emissions of Methane from Industrial Wastewater

| IPCC Classification |  | Classification used in Slovenia |
| --- | --- | --- |
| Food & Beverage | Canneries |  |
|  | Beer | Production of Beer |
|  | Wine | Production of Wine and Alcoholic Beverages |
|  | Meatpacking | Production. Processing and Preservation of Meat |
|  | Dairy Products | Processing of Milk and Production of Milk Products |
|  | Sugar |  |
|  | Fish Processing |  |
|  | Oil & Grease | Production of Vegetable and Animal Oils and Cooking Fats |
|  | Coffee |  |
|  | Soft drinks | Production and Bottling of Soft Drinks |
|  | Other | Production of Starch; Processing of Fruit and Vegetables |
| Pulp and paper | Paper | Production of Paper, Cardboard, and Pasteboard |
|  | Pulp | Production of Pulp |
|  | Other | Production of Leather and Fur |

Table 8.2.5: Fraction of Wastewater and sludge in Individual Industries Treated

|  | primary | secondary | municipal | sludge |
| --- | --- | --- | --- | --- |
| Prod. of Pulp | 0.84 | 0.00 | 0 | 0.40 |
| Prod. of Paper | 0.84 | 0.00 | 0 | 0.40 |
| Prod. of Leather | 0.02 | 0.98 | 0 | 1.00 |
| Prod. of Fruit. Vegetables | 0.20 | 0.06 | 0.47 | 0.73 |
| Prod. of Milk | 0.28 | 0.11 | 0.09 | 0.48 |
| Prod. of Meat | 0.46 | 0.04 | 0 | 0.50 |
| Prod. of Oil | 0.93 | 0.00 | 0 | 0.93 |
| Prod. of Starch | 0.11 | 0.00 | 0 | 0.11 |
| Prod. of Beer | 0.26 | 0.00 | 0 | 0.26 |
| Prod. of Soft Drinks | 0.10 | 0.35 | 0 | 0.45 |
| Prod. of Alcohol Beverage | 0.10 | 0.35 | 0 | 0.45 |

For further calculations, the values of the methane conversion factor are required:

- For primary handling of water or sludge, the value of MCF = 0

- For secondary wastewater handling, the value for MCF ranges from 0.05 to 0.06

- For treatment in municipal wastewater handling systems, MCF amounts to 0.05

- The methane conversion factor for methane in sludge handling from treated wastewater amounts to 0.8.

Emissions in the period 1986-2004 mainly follow the trends in the industrial production. The greater part of emissions was contributed by sludge treatment, while wastewaters contributed merely a good percent.

Table 8.2.6: Methane emissions in 2004 with regard to various industries

|  | CH4 from water | CH4 from sludge | CH4 total in Gg | Share in % |
| --- | --- | --- | --- | --- |
| Prod. of Pulp | 0.000 | 1.245 | 1.245 | 34.00 |
| Prod. of Paper | 0.000 | 1.520 | 1.520 | 41.51 |
| Prod. of Leather | 0.029 | 0.315 | 0.344 | 9.38 |
| Prod. of Fruit. Vegetables | 0.008 | 0.117 | 0.125 | 3.41 |
| Prod. of Milk | 0.006 | 0.062 | 0.067 | 1.84 |
| Prod. of Meat | 0.002 | 0.240 | 0.242 | 6.61 |
| Prod. of Oil | 0.000 | 0.001 | 0.001 | 0.03 |
| Prod. of Starch | 0.000 | 0.009 | 0.009 | 0.24 |
| Prod. of Beer | 0.000 | 0.087 | 0.087 | 2.38 |
| Prod. of Soft Drinks | 0.001 | 0.010 | 0.011 | 0.31 |
| Prod. of Alcohol. Beverages | 0.001 | 0.010 | 0.011 | 0.30 |
| Total | 0.046 | 3.616 | 3.662 | 100.00 |

### Uncertainties and time-series consistency

Uncertainty estimates based on expert judgement.

Uncertainty of activity data amounts to 30%.

Uncertainty of emission factor for municipal wastewater amounts to 20%. for industrial to 30%.

### Source-specific recalculations

No recalculations have been performed in this category.

### Source-specific planned improvements

We expect to obtain individual data on methane recovery on wastewater treatment plants until next year, and to prepare an estimate on how to divide this amount into municipal and  technological wastewater.

Moreover, it is our objective to acquire data on the consumption of water per unit of product and the COD value for industry, which makes up for the largest share in terms of emissions from wastewater. We plan to update data on ways of treating industrial wastewaters and on sludge handling.

# OTHER

No emissions have been reported under other.

# RECALCULATIONS AND IMPROVEMENTS

## Explanations and justifications for recalculations

In 2005 Slovenia implemented many improvements in the inventories in accordance with the findings of the in-country review 2004 and centralized review 2005 but some amendments may be still expected in the future.

Recalculations have improved GHG inventory considering completeness, consistency and accuracy. A brief description of recalculations is in the chapter 10.4, as detailed descriptions

## Implications for emission level

The total CO2 equivalent emissions w/o LULUCF in the base year 1986 decreased by 21.5 Gg CO2 eq. (-0.11%) compared to the last submission.

The most relevant changes are:

- CO2: +232.4 Gg CO2 in the Energy sector (1A) due to improved AD in manufacturing industry

- CH4: +250.6 Gg CO2 eq. in the Agriculture (4B) due to improved methodology and new data

- CH4: -159.9 Gg CO2 eq. in Waste sector (6A) due to the improved solid waste disposal data

- N2O: -107.7 Gg CO2 eq. and -212.5 Gg CO2 eq. in Agriculture (4B and 4D) due to improved estimates of nitrogen in manure.

In the 2003 CO2 equivalent emissions w/o LULUCF decreased by 253.5 Gg CO2 eq. (1.28%) compared to the last submission.

The most relevant changes are:

- CO2: -87.2 Gg CO2 in the Energy sector (1A) due to improved AD in manufacturing industry

- CH4: +307.8 Gg CO2 eq. in the Agriculture (4B) due to improved methodology and new data

- CH4: -190.7 Gg CO2 eq. in Waste sector (6A) due to the improved solid waste disposal data

- N2O: -68.7 Gg CO2 eq. and -148.8 Gg CO2 eq. in Agriculture (4B and 4D) due to improved estimates of nitrogen in manure.

All changes are presented in CRF Tables 8.

## Recalculations, including in response to the review process and planned improvements to the inventory

In energy sector we improve activity data considering coke and petroleum coke combustion in manufacturing industry. For 1997-2003 a disaggregation between iron and steel and non-ferrous metals has been done. But the major improvement happened in fugitive, where we all emissions from transport, distribution and use of natural gas, have been recalculated, using emission factors from German study and supplemented activity data from companies.

In industrial processes the main improvement was developing a CS EF for CO2 emissions from lime production. There were also improvements considering F-gasses; all HFC emissions from mobile AC using updated default values from GPG and SF6 emissions using new activity data and actual emissions data from companies owing HV equipment.

In agriculture sector there were many reasons for recalculation: the new data about daily gains for cattle, updated value for MCFs for liquid systems and for anaerobic digesters from GPG and new value of nitrogen excretion rates for cattle and swine have leaded to recalculations of GHG estimates in all agriculture sub sectors.

Some changes in estimations of sinks from LULUCF have also been made but the project

is still in the first phase. Momentary we are using data from previous projects and official data from Slovenian forestry service but with some country specific modifications (according to GPG for LULUCF 2003)

There were also recalculations in waste sector because of correction of historical data set about amount and composition of waste disposed to the SWDS.

For 2007 we are planning three big improvement of GHG inventory:

- final implementation of AC procedures

- independent peer review of inventory

- new uncertainty estimates

- CRF tables for 1987-1989

# SOURCES AND LITERATURE

- Breskvar B., Torkar M.: Določitev emisij toplogrednih plinov pri proizvodnji aluminija, železa in jekla ter ferozlitin, Inštitut za kovinske materiale in tehnologije, Ljubljana 1999

- Dolenc T., Žitko – Štemberger N.: Ocena emisije metana pri čiščenju odpadnih voda, Uprava za varstvo narave, Ljubljana 1998

- EMEP, CORINAIR: Atmospheric Emission Inventory Guidebook, EMEP-CORINAIR 1995

- EMEP, CORINAIR: Atmospheric Emission Inventory Guidebook, EMEP-CORINAIR 1996

- Grilc V., Husić M., Ignjatović D.: Določitev emisij toplogrednih plinov pri ravnanju z odpadki, potencijala za njegovo zmanjšanje in projekcij emisij, Kemijski Inštitut, Ljubljana 1999

- INRA Ruminant nutrition, Recomended allowance & feed tables, Paris, INRA, 1989, 389 pages

- Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reference manual, UNEP-OECD-IEA-IPCC, Bracknell 1996

- Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Reporting instruction, UNEP-OECD-IEA-IPCC, Bracknell 1996

- Intergovernmental Panel on Climate Change: Greenhouse Gas Inventory - Workbook, UNEP-OECD-IEA-IPCC, Bracknell 1996

- Intergovernmental Panel on Climate Change: Good Practice Guidance and Uncertainty Management in National Greenhouse Gas Inventories -, IGES, Japan 2000

- Kahnt G., Gruen dungung, DLG Verlag, Frankfurt, 1998, 146 pages

- Kirchgessner M., Tierernaehrung, Frankfurt, DLG Verlag, 1985, 488 pages

- Jamnik B., Bordon C., Kus J.: Projekcije emisije metana pri ravnanju s komunalno odpadno vodo, JP Vodovod-kanalizacija, Ljubljana 1999

- Leban J., Lebar A., Fece V.:Določitev emisij HFC, PFC in SF6 iz industrijskih procesov, potencialov za njihovo zmanjševanje ter priprava projekcij emisij, Gospodarska zbornica Slovenije, Ljubljana 1999

- Ministrstvo za gospodarske dejavnosti: Letopis energetskega gospodarstva RS 1986, Ministrstvo za gospodarske dejavnosti, Ljubljana 1986-2003

- Urad RS za statistiko: Statistični letopis RS 1987, Urad RS za statistiko, Ljubljana 1986-2003

- Gasperič M., Dornik M.: Določitev emisijskega faktorja CO2 pri energetski izrabi ter emisijskega faktorja CH4 pri transportu in distribuciji zemeljskega plina, Ljubljana: Inštitut za energetiko, 1998

- Gasperič M., Dornik M.: Disagregacija porabe goriv na komercialno institucionalni sektor in gospodinjstva, Ljubljana: Inštitut za energetiko, 1998

- Šuštaršič A. idr.: Ocena emisij snovi v zrak in rezultati merjenj emisijskih koncentracij termoelektrarne Šoštanj v letu 2003, Elektroinštitut Milan Vidmar, Ljubljana 2004

- Šuštaršič A. idr.: Ocena emisij snovi v zrak in rezultati merjenj emisijskih koncentracij Termoelektrarne Toplarne Ljubljana v letu 2003. Elektroinštitut Milan Vidmar, Ljubljana 2004

- Šuštaršič A. idr.: Ocena emisij snovi v zrak in rezultati merjenj emisijskih koncentracij Termoelektrarne Trbovlje v letu 2003, Elektroinštitut Milan Vidmar, Ljubljana 2004

- Šuštaršič A. idr.: Ocena emisij snovi v zrak in rezultati merjenj emisijskih koncentracij Termoelektrarne Brestanica v letu 2003, Elektroinštitut Milan Vidmar, Ljubljana 2004

- JQ Annual Questionnaire- Coal, Liquid, Gassous, Renevables, Statistični urad RS, 2004

- Baza podatkov o registriranih motornih vozilih v R Sloveniji; Ministrstvo za notranje zadeve,2004

- COPERT III, Computer programme to calculate emissions from road transport - Methodology and emission factors, Leonidas Ntziachristos and Zissis Samaras ETC/AEM

- Nacionalni emisijski faktor za lignit iz premogovnika Velenje; št. Referata 1686; EIMV, HSE;april 2004

- Zapušek A., Orešnik K., Avberšek F: Določitev emisijskih faktorjev metana pri izkopu premoga za leto 1986 in leta v obdobju 1990-1996, ERICo - Inštitut za ekološke raziskave, Velenje 1999

- Prometnotehnični inštitut pri Fakulteti za arhitekturo, gradbeništvo in geodezijo: Določitev emisij toplogrednih plinov cestnega motornega prometa, Prometnotehnični Inštitut, Ljubljana 1999

- Ministrstvo za okolje, prostor in energijo, Operativni program odstranjevanja odpadkov s strategijo zmanjševanja odloženih količin biološko razgradljivih odpadkov za obdobje od 2003-2008

- Paradiž B.: Ocena emisij metana in priprava njihovih projekcij pri ravnanju z odpadki, Ministrstvo za okolje in prostor, Ljubljana 2000

- Rode B.: Priporočeni emisijski faktorji za Slovenijo (za IPCC emisijske evidence plinov tople grede)

- Ministrstvo za okolje in prostor – Hidrometeorološki zavod RS, Ljubljana 1998

- Simončič P., Kobler A., Robek R., Žgajnar L.: Ocena emisij oz. ponora TGP za gozdarstvo ter spremembe rabe zemljišč, Gozdarski inštitut Slovenije, Ljubljana 1999

- Šalehar A., I. Štuhec and M. Kovač, Nekatere presoje in možnosti za povečanje kakovosti in gospodarnosti slovenske prašičereje, Sodobno kmetijstvo 31 (1998), pages 344-349

- Torelli N.: Ekološki, surovinski in energetski pomen gozda in lesa, Gozdarski inštitut, Ljubljana 1996

- Urad RS za statistiko: Letni pregled industrije – delovna gradiva 1997-2003, Urad RS za statistiko

- Urad RS za statistiko: Rezultati raziskovanj - Letni pregled industrije 1986-1996, Urad RS za statistiko, Ljubljana 1989-1997

- Verbič J., Sušin J., Podgoršek P.: Emisije toplogrednih plinov v kmetijstvu – ocene in možnosti za zmanjšanje, Kmetijski inštitut Slovenije, Ljubljana 1999

- Verbič J., Cunder T.. Podgoršek P.: Ocena potencialnih zmanjšanj izpustov toplogrednih plinov v sektorju kmetijstvo ob upoštevanju kvot, ki smo jih dosegli v predpristopnih pogajanjih z Evropsko unijo, Kmetijski inštitut Slovenije, Ljubljana 2003

- Verbič J., Cunder T., Podgoršek P.: Dodatek k poročilu: Ocena potencialnih zmanjšanj izpustov toplogrednih plinov v sektorju kmetijstvo ob upoštevanju kvot, ki smo jih dosegli v predpristopnih pogajanjih z Evropsko unijo, Kmetijski inštitut Slovenije, Ljubljana 2003

- Verbič J., Sušin J.: Poenostavljena metodika za oceno izpustov toplogrednih plinov iz kmetijstva, Kmetijski inštitut Slovenije, Ljubljana 2003

- Verbič J.: Izpusti amonijaka v kmetijstvu – ocene za leto 2002 in napovedi do leta 2020, Ljubljana 2004

Other sectors represent emissions that are caused by fuel combustion in households, commercial sector, services, forrestry and agriculture.