| New baseline and monitoring methodology or methodological tool recommendation FORM (Version 02.0) |  |
| --- | --- |
| Information to be completed by the Methodological Expert Panel (mep) |  |
| Date and number of the MEP meeting | 30/01/2026 |
| Type of standard | New baseline and monitoring methodology |
| Unique reference number | A6.4-PMM006 |
| Title of the methodology or methodological tool | Fertilizer production with renewables-based ammonia |
| Title of underlying project activity or programme of activities | UGF- Uberaba Green Fertilizer Project |
| History of submission | 07 - 27 January 2026: Open for public input |
| Is a similar methodology(ies) already under review/approved under the Article 6.4 mechanism? Is the methodology similar to an approved CDM methodology? | A6.4-PNM001 - Production of Ammonia through electrolysis of water, air separation and synthesis of hydrogen and nitrogen |
| Section I. Information about the new baseline and monitoring methodology |  |
| Short description of the types of Article 6.4 activities covered by the methodology |  |
| This methodology is applicable to activities producing renewables-based ammonia for fertilizer production replacing conventional ammonia production that is based on fossil fuel feedstocks to produce the required hydrogen. Accordingly, CO2 emissions from the fossil fuel feedstock conversion process are avoided and the use of electricity from renewable sources will further decrease GHG emissions of the entire production process. 4. Manufacturing industries Ammonia production as an industrial process requiring substantial energy input. This methodology is globally applicable to project activities that produce ammonia, that is less carbon-intensive than conventional ammonia production, primarily with renewable electricity (RE) for: - Production of hydrogen (H2) using electrolysers to separate water into H2 and O2; - Production of nitrogen (N) through Air Separation Plants or other facilities; - Synthesis of hydrogen and nitrogen to produce ammonia (Haber Bosch Process). The methodology allows under certain conditions for up to 10% of the electricity consumed to stem from the grid instead of dedicated renewable energy plants. |  |
| Conditions under which the methodology is applicable |  |
| - The activity produces renewables-based ammonia for fertilizer production; - No fossil fuels are used as feedstock in the production of hydrogen and ammonia; - Hydrogen under the activity is produced from electrolysis and the entire production of hydrogen is used for the renewables-based ammonia production; - Nitrogen is produced from air separation; - Electricity used to produce hydrogen, nitrogen and ammonia is either sourced from one or a combination of the following sources: - from dedicated renewable energy sources connected directly to the ammonia production facility; - from dedicated renewable energy sources connected via the electricity grid to the ammonia production facility; - from the grid, where the grid emission factor (i.e. the combined margin) is equal or less than 0.2 tCO2e/MWh and it can be demonstrated that the share in total electricity production was expanded over the last 5 years prior to the start date of the activity; - From the grid, where the grid emission factor (i.e. the combined margin) is above 0.2 tCO2e/MWh and it can be demonstrated that the share in total electricity consumption from the grid is below 15%. - The activity shall demonstrate that the planned use of electricity from the grid will not result in increased grid instability and constraints negatively impacting other consumers of grid electricity; - Ammonia is produced by the Haber-Bosch process using the hydrogen and nitrogen produced as per the previously described conditions; reductions; - Only the share of ammonia used for fertilizer production within the host country of the activity is applicable for generating emission reductions; - The activity shall use no more than 5 % of the drinking water available locally, to ensure that the water used in the electrolysis will not displace other uses; - Activities must demonstrate minimal impact on water needed for drinking, agriculture, livelihood and ecosystems as per Appendix II; - Activities must demonstrate that water utilization will not have an impact on ecosystems resulting in an increase of emissions (e.g. loss in biomass or soil carbon stocks); - This methodology is applicable to activities implemented in existing ammonia and fertilizer production facilities replacing fossil-fuel-based hydrogen and ammonia production (brownfield) or to the construction of new hydrogen and ammonia production facilities (greenfield activities); - For brownfield activities, it must be possible to distinguish all production steps, related energy inputs and emission sources between the existing production pathway and the activity production pathway. This may be demonstrated based on the entire production facility or on the level of entire production lines; - For brownfield activities, it must be demonstrated that replaced equipment and production facilities are scrapped and not used elsewhere; - It can be demonstrated that there is no risk of double counting as per chapter 6 of this methodology. |  |
| Summary of the methodology (including project boundary, baseline scenario, baseline approach, additionality, baseline emissions or removals, project emissions or removals, leakage, emission reductions or removals, non-permanence and reversals and monitoring) |  |
| Project Boundary: The activity boundary encompasses the production facilities required to produce renewables-based ammonia including: - Production of hydrogen by electrolysis; - Production of nitrogen by air separation; - Production of renewables-based ammonia (Haber-Bosch process); - Incremental transportation of feedstock within and between the above-mentioned production facilities; - The electricity and energy sources which will be used in the above-mentioned production facilities. |  |
| Baseline approach & baseline scenario: The baseline scenario is determined as follows: - Greenfield activities and brownfield activities after the end of the remaining lifetime: The BAT approach is selected from paragraph 36 of the RMPs to set the baseline for activities eligible under this; - Brownfield activities until the end of the remaining lifetime: The baseline approach shall be based on existing actual or historical emissions. |  |
| Additionality: The demonstration of additionality contains the following steps detailed in the methodology: - STEP 1. Regulatory Analysis; - STEP 2. Avoidance of locking-in the level of emissions; - STEP 3. Investment analysis; - STEP 4. Common practice analysis (for the production of renewable-based ammonia in the country). |  |
| Baseline emissions or removals: Selection of the baseline approaches from paragraph 36 of the rules, modalities and procedures: - Greenfield activities and brownfield activities after the end of the remaining lifetime: The BAT approach is selected from paragraph 36 of the RMPs to set the baseline for activities eligible under this; - Brownfield activities until the end of the remaining lifetime: The baseline approach shall be based on existing actual or historical emissions. |  |
| Baseline downward adjustment: Downward adjustment in the calendar year of the start date of the first crediting period (for approaches based on 36(iii)): based on the uncertainty factor or the minimum downward adjustment. Downward adjustment in subsequent years: based on the maximum between 1% per year and the yearly adjustment for achieving the Party’s net zero target. |  |
| Business-as-usual scenario: The identification of the conservative business-as-usual (BAU) scenario shall follow the provisions and requirements of section 8 of the baseline standard. The activity proponent shall consider the following alternatives for the purpose of determining the BAU scenario and justify the choice, including how it ensures conservativeness: - Continuation of the historical situation (pre-activity scenario); - Establishment of an economically viable technology and/or practice; - A scenario combining (a) for the remaining lifetime of the existing equipment and/or practice, followed afterwards by (b). Where several scenarios are plausible, the most conservative scenario shall be chosen as the BAU scenario. |  |
| Activity emissions or removals: Activity emissions refer to the total direct and indirect greenhouse gas (GHG) emissions resulting from the implementation of the Green Ammonia production project. These emissions must be compared with baseline emissions to determine the net mitigation impact. Activity emissions sources include: - Direct onsite emissions from hydrogen and ammonia during ammonia production, including physical leaks of hydrogen; - Indirect emissions resulting from electricity consumption during the ammonia production process; - Emissions from transportation of ammonia to the fertilizer production facility (if applicable); - Emissions from land clearing for facility construction. |  |
| Leakage: Activity proponents shall consider the provisions of the leakage standard as applicable. As a result of the applicability conditions of this methodology, only emissions from international transport as a result of greenfield activities where fertilizer is exported from the host country needs to be quantified as leakage emission source. |  |
| Emission reductions or net removals: Emission reductions are calculated as the difference of baseline, activity and leakage emissions. |  |
| Non-permanence and reversals: N/A |  |
| Monitoring: 𝑁𝐻3,Act,y Quantity of Green Ammonia produced in PACM project activity in year y; LKH2,y Quantity of hydrogen physically leaked from the production facility in year y; ECgrid Quantity of renewable electricity generated and supplied to the green ammonia production facility in year y; FCi,y t Quantity of fuel type i combusted in process j during the year y; Df trip distance between the origin and destination of freight; FR,f,i,y mass of freight transported; Wc,i,y fraction of carbon in fuel type i in year y; 𝝆,𝒊,𝒚 Density of fuel type i in year y; NCVi,y Weighted average net calorific value of the fuel type i in year y; EFCO2,e,y Weighted average CO2e emission factor of fuel type i in year y. |  |
| Other methodologies principles: >> |  |
| Section II. Information about the new methodological tool |  |
| Conditions under which the methodological tool is applicable |  |
| >> |  |
| Summary of the methodological tool (including the calculation of baseline emissions or removals, project emissions or removals, leakage, emission reductions or removals, non-permanence and reversals and monitoring) |  |
| >> |  |
| Section III. Recommendations to the Supervisory Body |  |
| Approve the proposed new methodology or methodological tool (“A case”) Reject the proposed new methodology or methodological tool (“C case”). |  |
| Reasons for rejection if the recommendation is C case |  |
| The reasons may include among others: (i) Misalignment with existing standards; (ii) Risks of over-crediting; (iii) Additionality-related risks; (iv) If applicable: Addressing the risk of non-permanence; (v) Other environmental integrity risks; (vi) Issues with ensuring the outcomes are real, in particular regarding monitored and non-monitored parameters; (vii) Other risks identified; (viii) Inadequate justifications. |  |
| Any other issues arising from the assessment of the proposed methodology |  |
| The reasons may include among others: (i) Misalignment with existing standards; (ii) Risks of over-crediting; (iii) Additionality-related risks; (iv) If applicable: Addressing the risk of non-permanence; (v) Other environmental integrity risks; (vi) Issues with ensuring the outcomes are real, in particular regarding monitored and non-monitored parameters; (vii) Other risks identified; (viii) Inadequate justifications. |  |
| Section IV. Clarifications or modifications requested Please use this section only when the form is used for requesting clarifications or modifications from the proponent according to the procedure “Development, revision and clarification of baseline and monitoring methodologies and methodological tools” |  |
| Section; Para; Clarification Questions. 5. Applicability; 8a; Please explain the reasons for limiting the use of ammonia to fertilizer production. Response: The methodology is limited to renewables-based ammonia production for fertilizer only, as this relates to a very clear baseline scenario which is production of fertilizers based on conventional processes based on fossil fuels used as feedstock, mainly natural gas bases steam reforming process. To broaden the applicability of ammonia use for other purposes such as use as fuel for heavy/maritime transport for example would result in the requirement for in-depth baseline scenario determination as the baseline for ammonia used for transport is likely use of conventional fossil fuel such as bunker fuels. Likewise, any other use of ammonia as energy carrier would require a full baseline scenario assessment. Furthermore, additional questions regarding project boundary and potential leakage emissions might arise. To enable renewables-based ammonia production as soon as possible, we proposed the limitations as presented in the submitted methodology. Nevertheless, we would be supportive of broadening the scope in a later revision or consolidation of ammonia related methodologies.. 8b; The condition maintains no fossil fuel consumption, though there is a component of fossil fuel consumption in the activity emissions. Please clarify whether limited auxiliary fossil fuel consumption (e.g. for start-up, back-up generation, or safety systems) is permitted under the applicability conditions, and if so, how such consumption is distinguished from prohibited fossil fuel use for eligibility purposes and consistently accounted for within activity emissions. Response: the intension was to fully replace fossil fuel input as feedstock in ammonia production while minor fossil fuel consumption as source of auxiliary energy to operate a machine or vehicles in the production process is possible as long as fully accounted as activity emission source. The methodology has been revised to clarify this aspect.. 8e- i,ii; Please clarify the acceptable approaches for demonstrating that renewable electricity used by the activity does not displace renewable electricity that would otherwise be available to other users, including whether captive generation, dedicated facilities, contractual instruments, or grid-based additionality tests are permitted, and under what conditions. Response: The issue is addressed in two ways within the proposed methodology. Firstly, by defining conditions in Appendix 1 of the methodology to apply a grid emission factor of zero for renewables, and secondly by application of the grid emission factor for any electricity consumed beyond the consumption as defined in Appendix 1. Generally, any activity based on using electricity (e.g. e-mobility, heat pumps, etc.) is subject to such a question. By referring to the application of a grid emission factor “overuse” of renewables would result in an increase of the grid emission factor and as such be reflected by increasing activity emissions.. 8eiii, iv; Please clarify whether the condition related to grid electricity emission factors constitutes an eligibility requirement or is intended solely for emission accounting purposes, noting that any emissions associated with grid electricity consumption would already be reflected in activity emissions. Response: The proposed methodology introduced thresholds for grid emission factors as applicability requirements. This was deemed important to ensure the possibility of using grid electricity while at the same time ensuring that the related electricity grid is already fed by a high share of renewable energy sources. The methodology has been revised to clarify this aspect.. 8f; It needs to be clarified as to how it can be ensured that condition 8f of the applicability conditions, relating to demonstrating that the planned use of electricity does not result in increased grid instability, is complied with. Response: The proposed methodology has been revised to provide additional guidance on how to demonstrate compliance with the condition. Reference to operational licences as issued by the host country has been added to strengthen host country responsibilities and ownership.. 8h; Please clarify if all of the hydrogen consumed at the H-B process in the boundary is produced by electrolysers under the activity? Response: For the purpose of renewables-based ammonia production under the proposed methodology, all hydrogen consumed must be from electrolysis produced within the boundary of the proposed activity. The methodology has been revised to clarify this.. 8l and 8m; Please provide a clear and operational definition of “brownfield” activities, specifying whether it includes: (a)complete replacement of existing hydrogen production facilities; (b) partial replacement of production units; or (c) parallel operation of existing and new production pathways. In addition, please clarify how the existing and activity production pathways are distinguished for boundary setting, baseline determination, and emission accounting purposes. Response: The methodology was revised by adding definitions on brownfield and greenfield activities clarifying the potential use cases (entire facility versus production lines) applicable under this methodology and provided related guidance on the conditions (i.e. it is possible to distinguish related energy consumption and emissions).. 8 Activity Boundary/ 12 Leakage; 17; Please clarify how upstream emissions associated with water supply and treatment, and transport of key inputs are treated within the system boundary, and provide justification for any exclusions in light of the materiality of these emission sources. Regarding transport emissions, the current text indicates only transport within the boundary and transport of ammonia to the fertilizer plant. Response: The methodology has been revised to clarify this aspect. Any emissions related to upstream emissions associated with the water supply and treatment and any net increase of transportation of input materials is now clearly considered in the activity emissions quantification.. 10.2 baseline scenario; 39; Please clarify whether the default global best available technology (BAT) benchmark applies uniformly to both greenfield and brownfield activities. In particular, please explain how this is reconciled with the approach described in paragraph 51, which indicates that brownfield baselines are determined based on existing actual or historical emissions. Response: The global default value was intended to be applicable under the BAT approach for greenfield activities only. The methodology has been revised to clarify this aspect.. Supplementary information; Has this information been subjected to external technical assessment? If so, please provide evidence and a summary of the findings. Response: The Gold Standard (GS) has launched a public consultation on their proposed new methodology and the related supplementary information in July 2025. According to the published results there was no submission received that put the presented values into question. We reached out to GS to understand their current status of approving their methodology with the underlying values from the supplementary information and would be able to share any update we will receive on this. For the development of our proposed methodology, we have checked the information as well against scientific literature and found the referenced values as appropriate. For example, the proposed default value in the submitted methodology is well below the default values identified under the Carbon Border Adjustment Mechanism (CBAM) of the EU.. |

- - - - -

Document information

| Version | Date | Description |
| --- | --- | --- |
| 02.0 | 20 August 2025 | Revision to incorporate new sections and sub-sections in line with current standards. |
| 01.0 | 18 December 2024 | Initial publication of form template. |
| Decision Class: Regulatory Document Type: Form Business Function: Methodology Keywords: A6.4 mechanism, developing methodologies and tools, recommendations |  |  |