|  | PROPOSED New baseline and Monitoring methodology oR methodological tool FORM FOR EMISSION REDUCTIONS ACTIVITIES (Version 02.0) |  |
| --- | --- | --- |
| Information to be completed by the secretariat and Methodologies Expert Panel |  |  |
| Type of standard |  | Choose an item. |
| Unique reference number and title of the proposed new methodology or new methodological tool |  | >> |
| Date when this form was received at UNFCCC secretariat: |  | Click or tap to enter a date. |
| Date of posting in the UNFCCC A6.4 web site for global stakeholder consultation |  | Click or tap to enter a date. |

Information to be completed by the submitter (read before filling the form)

This form is required at the “submission of proposed new methodology or methodological tool” stage and is submitted together with ‘New baseline and monitoring methodology and methodological tool proposal form (A6.4-FORM-METH-001).

Instructions for using this form In using this form, please follow the guidance established in the following documents: - Fill out all relevant sections of the form in clear print or typing; - Provide your input after the >> indicator in the space provided; - Leave blank sections which are found to be not applicable. Formatting Instructions: - Do not modify any part of this form, including headings, logo, format or font; - The form provides the formatted headings which should be used throughout the document; - Please use word equation editor to write equations; - Please format figures, tables and footnotes to update automatically; - Please note the footnotes have a separate format (Times New Roman - size 10). - Please clearly distinguish between proper methodology text, tables and equations and explanatory notes, using the following colour coding: - Methodology text shall be written in black fonts. - Guidance from the UNFCCC is provided in blue fonts and can be deleted. - Explanatory notes shall be written in grey fonts. Please note that explanatory notes are solely for the sake of methodology submission and consideration. Do not include guidance to activity participant in explanatory notes. Please note upon methodology approval, explanatory notes will be deleted.

- Summary and applicability of the baseline and monitoring methodology or methodological tool

Title, submission date and version

Title: Production/Manufacture of Low Carbon Intensive Ammonia through electrolysis of water, air separation and synthesis of hydrogen and nitrogen.

Submission date: 05th March 2025

Version: 1.0

Submission date: 21st April 2025

Version: 1.1

Submission date: 11th August 2025

Version: 1.2

Submission date: 26th August 2025

Version: 1.3

If this methodology or methodological tool is based on a previous submission or an approved Article 6.4 mechanism methodology or methodological tool, please state the reference numbers here. Explain briefly the main differences and their rationale.

This methodology is not based on a previous submission or an approved Article 6.4 mechanism methodology.

However, it is a revised version incorporating inputs and feedback from MEP, which completed its initial assessment on April 8, 2025. (Initial_Assessment_Production_Low_Carbon_Ammonia.pdf)

Summary description of the methodology or methodological tool, including major baseline and monitoring methodological steps.

This methodology is globally applicable to project activities that produce ammonia, that is less carbon-intensive than conventional ammonia production, primarily with renewable electricity for

- Production of hydrogen (H2) using electrolysers to separate water into H2 and O2.

- Production of nitrogen (N) through Air Separation Plants or other facilities that use renewable energy only.

- Synthesis of hydrogen and nitrogen to produce ammonia (Haber Bosch Process).

The baseline scenario is the conservative among the following three:

(i)  Best Available Technology

(ii) Ambitious benchmark and

(iii) Existing actual or historical emissions corrected for downward adjustment.

However, the baseline can be chosen from either one or two of the approaches, in cases when data and information is available for conducting these approaches. All the subprocesses to produce ammonia are to be powered mainly by renewable electricity sourced from the greenfield renewable power plant. The RE supply, use of fossil fuels, and leakages arising out of diversion of RE or water, all parameters and information that is related to baseline and project scenario are comprehensively addressed within the Monitoring framework and are thoroughly accounted for in the subsequent sections on Methodology, Recording and Verification. Life cycle emissions are considered both in baseline and activity emissions.

- Proposed new baseline and monitoring methodology or methodological tool

- Introduction

- The following table presents a summary of the key elements of a methodology:

Table 1. Methodology key elements

| Type of GHG mitigation measure(s) | Fuel/feedstock switch Technology switch GHG destruction GHG formation avoidance Engineered carbon dioxide removal Nature based carbon dioxide removal |
| --- | --- |
| Types of mitigation outcomes achieved under this methodology | Emission reductions Removals |
| Are the mitigation outcomes under this methodology at risk of reversal? | Yes No |
| Typical projects eligible under the methodology | Construction and operation of green ammonia plants where all subprocesses are greenfield and powered by renewable energy, or projects where the electrolyser is greenfield and other subprocesses are existing or retrofitted, with the entire production system powered by renewable energy to enable low carbon/green ammonia production. |

- Scope and entry into force

Scope

This methodology applies to project activities that reduce greenhouse gas (GHG) emissions through the low carbon production of ammonia by substituting conventional, fossil-fuel-intensive processes with processes powered by renewable electricity. The typical projects eligible under this methodology are those that integrate:

- Hydrogen production via electrolysis powered by renewable energy

- Nitrogen production through air separation using renewable electricity

- Ammonia synthesis through the Haber-Bosch process using the renewable energy.

The methodology is applicable to:

- Greenfield ammonia plants specifically designed for low-carbon ammonia production.

- Brownfield retrofitting projects that replace fossil-based hydrogen production (e.g., via steam methane reforming of natural gas or other fossil fuels) with green hydrogen generated from renewable electricity.

Entry into force

- [For the UNFCCC secretariat to complete - Leave blank]

Applicability of sectoral scopes

Designated operational entities validating and verifying Article 6.4 activities that use this methodology shall apply sectoral scope(s):

- [For the UNFCCC secretariat to complete – leave blank]

- Definitions

In addition to the definitions contained in the ‘Article 6.4 mechanism Glossary of Terms’, the following definitions apply for the purpose of this methodology:

Air separation plants - A plant/process that separates atmospheric air into its primary components, typically nitrogen and oxygen, and sometimes also argon and other rare inert gases. Air separation plants are constructed in different forms depending on what products are produced, production capacity, and purity.

Ammonia (NH3) - Ammonia is a colourless gas that is used a feedstock in the production of fertilizers, chemicals and refrigerants.

Conventional ammonia production - Production of NH3 that involves fossil fuels in the production of hydrogen and nitrogen separation from air.

Electrolyser hydrogen production plant - A facility that produces hydrogen from the electrolysis of water. Under this methodology, the hydrogen production plant includes the desalted water station, electrolytic cell, hydrogen compressor, gas-liquid processor, hydrogen purification unit and other ancillary equipment.

Gasification of coal - an industrial process where coal is converted into syngas through a process of gasification.

Green Ammonia - Refers to ammonia (NH₃) produced using renewable energy sources for all upstream energy requirements, without any use of fossil fuels. The production process involves:

- Electrolytic hydrogen production using electricity from renewable energy (e.g., solar, wind, hydro),

- Nitrogen separation from air, typically using cryogenic or pressure swing adsorption (PSA) processes, and

- Ammonia synthesis via the Haber-Bosch process powered by renewable electricity.

Green Hydrogen - Hydrogen (H₂) produced through the electrolysis of water, where the electricity used for electrolysis is derived exclusively from renewable energy sources.

Steam methane reforming (SMR) process - In these plants, the steam reforming process for ammonia production uses natural gas to produce hydrogen, which is then combined with nitrogen to create ammonia. It is the process through which hydrogen is generated from natural gas. High-temperature steam ranging between 700°C–1,000°C is used to produce hydrogen from a methane source, such as natural gas. It is to be noted that the process is endothermic, i.e., heat must be supplied for the initiation of the reaction.

Steam reforming of natural gas - an industrial process where natural gas reacts with steam in the presence of a catalyst to produce hydrogen, carbon monoxide and releases carbon dioxide as a by-product.

Steam reforming of syngas - an industrial process where the syngas produced from the gasification of coal or oil reacts with steam in the presence of a catalyst to produce hydrogen, carbon monoxide and releases carbon dioxide as a by-product.

- Normative references

This proposed baseline and monitoring methodology is based on the following proposed new methodologies and/or approved or consolidated methodologies:

Feed switch in integrated Ammonia-urea manufacturing industry”. AM0050 / Version 03.0.0/ Sectoral scope: 05/EB 68

Hydrogen production from electrolysis of water, AM0124 / Version 1.1 / Sectoral scope: 1,5 / EB 119

This methodology also refers to the latest approved versions of the following methodological tools/standard:

A6.4-STAN-METH-005 – Addressing leakage in mechanism methodologies (v.01.0)

A6.4-STAN-METH-001 – Application of the requirements of Chapter V.B (Methodologies) for the development and assessment of Article 6.4 mechanism methodologies (v.01.1)

A6.4-STAN-METH-004 - Setting the baseline in mechanism methodologies (v.01.0)

This methodology is based on the following sources of information:

Ammonia Production Processes from Energy and Emissions Perspectives: A Technical Brief Banafsheh Jabarivelisdeh1, Enze Jin1, Phillip Christopher2, Eric Masanet1,31 Bren school of Environmental Science and Management, University of California Santa Barbara, CA, USA 2 Department of Chemical Engineering, University of California Santa Barbara, CA, USA 3 Department of Mechanical Engineering, University of California Santa Barbara, CA, USA

Product carbon footprint methodology for ammonia production by conventional steam reforming–A case study: European Journal of Sustainable Development Research 2024, 8(1), em0241 e-ISSN: 2542-4742

2019 refinement to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories /Section 3.2 / Chapter 3: Chemical Industry Emissions

Techno-Economic Challenges of Green Ammonia as an Energy Vector / Chapter 4: Ammonia Production Technologies https://doi.org/10.1016/B978-0-12-820560-0.00004-7

Hinico - Carbon Footprint Methodology, AEA Annual Conference, November 13 – 15

- Applicability

The methodology is applicable under the following conditions:

  - Production of green ammonia using renewable energy to power the associated processes, including the electrolyser, air separation unit, synthesis of hydrogen and nitrogen via the Haber-Bosch process, and other related utilities.

  - This methodology applies to project activities that encompass the following scenarios:

- (i) All subprocesses involved in green ammonia production are greenfield, OR

- (ii) The electrolyser plant is greenfield, while other subprocesses are existing or retrofitted.

  - In the case of a brownfield or retrofitted project activity, where the electrolyser plant is greenfield and all subprocesses are existing or retrofitted, the output(s) or level(s) of service must remain identical to those in the pre-PACM project condition.

  - This methodology is limited to the production and storage of ammonia within the manufacturing facilities.

  - The electricity used by the ammonia production plant shall primarily originate from greenfield renewable power plants or renewable electricity supplied through the grid.

  - The renewable energy (RE) plants supplying electricity must be either captive or identified RE generation and/or storage facilities, delivering either dispatchable or intermittent electricity. All components of the associated RE assets must have commenced operation within two years of the start date of the project.

  - All identified RE generators/suppliers, whose operation start date exceeds 180 days prior to the start of the ammonia production project activity must demonstrate that they did not have any long-term renewable electricity supply contracts with other end users and that their supply has been redirected to the project activity.

  - The use of grid electricity in ammonia production and any of its subprocesses shall not exceed 10% of the total electricity consumption on an annual basis, and the designated operational entity (DOE) shall confirm compliance with this requirement.

  - The renewable energy generation, its storage (if applicable), and its use within the project activity may be co-located or located at different sites, including supply through the grid, provided that the renewable electricity is physically delivered to the project facility—meaning the electricity flows to the facility through the grid, dedicated lines, or wheeling arrangements—and is supported by a verified contractual arrangement (e.g., Power Purchase Agreement with a renewable energy generator on the same grid), ensuring verifiable and exclusive use of renewable electricity by the project.

  - Project activities may consist of modular green ammonia production equipment, in the vicinity where ammonia will be used or supplement the ammonia production in existing manufacturing facilities.

  - In cases where green ammonia production supplements an existing facility, the project must ensure clear separation and robust monitoring/metering to distinguish green ammonia from conventional output. The designated operational entity (DOE) shall confirm proper implementation by verifying the monitoring system and ensuring accurate tracking and reporting of green ammonia production.

  - The project shall limit the use of locally available water (whether surface or groundwater) for electrolysis to no more than 5%, ensuring minimal impact on water needed for drinking, agriculture, or livelihoods. Compliance with this requirement will be verified during validation and at each crediting period renewal using project data and official sources.

  - If the project activity involves installing a greenfield electrolyser within an existing ammonia production facility, the existing equipment used for hydrogen production must be decommissioned, destroyed, or properly disposed of. Failure to do so will require that emissions associated with its continued use be fully accounted for. The Designated Operational Entity (DOE) shall be responsible for verifying compliance with this requirement.

  - Only the quantity of ammonia that is produced using renewable hydrogen and nitrogen inputs, and is verifiably exported or sold, shall be considered eligible for emission reduction calculations. Supporting evidence may include sales invoices, dispatch records, or delivery logs.

- Avoidance of double-counting

The project shall

- Ensure emission reductions are claimed only by the green ammonia producer through enforceable contracts preventing double claims by users or suppliers.

- Not be part of any mandatory domestic mitigation scheme; and

- Include safeguards to avoid overlap with national targets or policies.

- The above-mentioned conditions shall be verified by the DOE at validation/verification stage as well as during renewal of each crediting period.

- Demonstration of alignment with the policies, options and implementation plans with regard to the NDC and LT-LEDS of the host Party and the long-term temperature goal of the Paris Agreement and long-term goals of the Paris Agreement

Activity participants shall provide to the DOE responsible to perform the validation of the Article 6.4 project an assessment, undertaken by the DNA of the host Party, of the activity’s consistency with Decision 3/CMA.3 paragraph 40 (c) and paragraph 27 (a) as part of the host Party’s approval”.

- Activity Boundary

The project boundary includes:

- The specific location and grid from which renewable electricity is generated and supplied.

- The location where the hydrogen production facility consisting of electrolyser stacks; water desalination/treatment and pumping.

- The location where the Air Separation plants are located to produce nitrogen.

- The location where the hydrogen is combined/ synthesised with nitrogen to produce the green ammonia.

- The boundary of the project activity is limited to the premises of production facility(ies) and the supply sources of renewable energy.

Table 2. Emissions sources and sinks included in or excluded from the activity boundary

|  | - Source | - GHG |  |  | - Justification / Explanation |
| --- | --- | --- | --- | --- | --- |
| - BASELINE | - Ammonia Production, excluding transportation | - CO2 | Included - Not included | Controlled Related to - Affected by | - Main emission source |
|  |  | - CH4 | Included - Not included | Controlled Related to - Affected by | - Minor emission source |
|  |  | - N2O | Included - Not included | Controlled Related to - Affected by | - Minor emission source |
|  |  | - ----- | Included - Not included | Controlled Related to - Affected by | - Not Applicable |
|  | - Hydrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
|  | - Nitrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
| - ACTIVITY | - Low carbon intensive ammonia production, excluding transportation. | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
|  | - Hydrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
|  | - Nitrogen production | - CO2 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not Applicable |
| - LEAKAGE | - Transferred baseline equipment | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Resource competition | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Diversion of production | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |
|  | - Indirect environmental emissions | - CO2 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - CH4 | Included Not included | Controlled Related to Affected by | - Main emission source |
|  |  | - N2O | Included Not included | Controlled Related to Affected by | - Minor emission source |
|  |  | - ----- | Included Not included | Controlled Related to Affected by | - Not applicable |

- Demonstration of additionality

Regulatory analysis

The activity participant of the project shall demonstrate that the mitigation activity is not a legal requirement. However, the activity participant shall ensure that all the rules and regulations related to the ammonia production process is mandatorily adhered to. The statutory documents (Environmental Impact Assessment, license to operate and other terms and conditions etc) shall be made available during the validation process.

The activity participant shall obtain the Host Party approval (through its DNA), If the host Party approves the project, the approval shall include:

- Confirmation that, and information on how, the project fosters sustainable development in the host Party.

- Approval of any potential renewal of the crediting period, if the Party intends to allow the project to continue beyond the first crediting period, where the Party has specified that the crediting periods of A6.4 activities that it intends to host may be renewed pursuant to paragraph 27(b) of the RMPs;6

- Explanation of how the project relates to the implementation of its NDC and how the expected GHG emission reductions or net GHG removals contribute to the host Party’s NDC, and the purposes referred to in Article 6, paragraph 1.

- Authorization of the activity participants

Avoidance of locking-in the level of emissions

A paragraph Locking-in emissions refers to project activities that commit a system to a high-emission pathway (e.g., fossil fuel infrastructure with long asset lifetimes), making future decarbonization harder. Ammonia production process, based on electrolysis powered by dispatchable RE, inherently avoids lock-in. However, to rigorously demonstrate this, the following process may be applied:

Definition of Lock-In Emissions Criteria

- Lock-in risks are identified by assessing whether the project:

- Creates dependency on fossil fuels (e.g., requires fossil-based backup systems).

- Extends the lifespan of high-emission infrastructure (e.g., retrofits coal plants instead of replacing them).

- Adopts technologies with no decarbonization pathway (e.g., irreversible design choices).

- Triggers market/regulatory shifts favouring fossil fuels (e.g., crowding out renewables).

Process to Identify Lock-In Emissions Risks

- Step 1: Project Component Analysis

| Project Component | Lock-In Risk Assessment | Required Mitigation and Evidence |
| --- | --- | --- |
| Electrolysis Technology | If designed with fossil-based backup (e.g., grid-tied without RE guarantee), may create long-term reliance on fossil grid power.. | Electrolysers are RE-powered and modular. Submit technical specifications and supplier declarations showing 100% RE use. |
| Air Separation Unit (ASU) | If powered by fossil grid or installed with fossil backup systems, it may lock in carbon-intensive nitrogen production over time.. | Use RE-powered ASU only. Provide energy consumption breakdown and evidence of RE source. Confirm that no fossil-fired backup compressors are installed. |
| Haber-Bosch Ammonia Synthesis | Traditionally optimized for fossil-based hydrogen; risk of integrating fossil fuel preheating or steam inputs. | PFD (process flow diagram) showing no fossil heating or steam usage. Specify energy source for reactor heat integration (e.g., electrical or RE-derived thermal). |
| Dispatchable RE Source | If backed by diesel or grid-based fossil electricity during non-RE generation hours, may lock in backup fossil usage | Provide energy generation profiles and dispatch strategy demonstrating 24/7 RE-based operation. Include storage design documentation. |
| Grid Interconnection | If connected to a fossil-dominated grid without a clean energy contract or tracking, it may lead to indirect fossil fuel reliance. | Submit proof of exclusive RE supply through PPAs or wheeling agreements. Demonstrate no net fossil imports. |
| Auxiliary Systems | If auxiliary compressors/pumps rely on fossil fuels, long-term carbon emissions could be locked in.. | Submit equipment specifications showing use of electrically driven, zero-emission systems only. |

Step 2: Qualitative Checklist for Project Developers

The methodology includes the following checklist to be completed and substantiated with evidence during validation:

| Criterion | Requirement | Evidence Type |
| --- | --- | --- |
| No fossil fuel combustion in any process | Required | Process flow diagram, energy input data |
| Modular and upgradable RE infrastructure | Required | Technical specs, supplier declarations |
| No long-term fossil input contracts | Required | Fuel/power procurement contracts |
| Alignment with host country’s decarbonization pathway | Required | Policy documents, official targets |

Addressing Edge Cases:

- In cases where temporary fossil backup systems (e.g., diesel generators) are used:

- Their use must be limited to less than 5% of total annual energy consumption.

- A clear and time-bound phase-out plan must be provided.

- Such systems shall not be structurally integrated into the project’s long-term operation.

Investment analysis, Barrier analysis and Common practice analysis

Investment analysis

To demonstrate additionality in accordance with the A6.4-STAN-METH-003 standard, this methodology requires the demonstration of financial additionality using investment analysis to assess whether the Article 6.4 activity is not financially viable in the absence of revenues from A6.4ERs.

Project proponents shall undertake a detailed financial assessment to demonstrate that the proposed activity is not financially attractive in the absence of carbon revenue from the sale of Article 6.4 emission reductions (A6.4ERs), and that such revenue is instrumental in the investment decision.

The methodology requires that the financial analysis be conducted based on data and assumptions consistent with those used in investment decision-making, reflecting conditions at the start date of the activity.

Type of Investment Analysis: The methodology requires activity participants to apply one of the following types of investment analysis as suitable to the project context, in line with paragraphs 34–36 of A6.4-STAN-METH-003:

- Benchmark analysis: comparing the financial viability of the project to a conservative financial benchmark such as the Weighted Average Cost of Capital (WACC).

- Investment comparison analysis: comparing the project’s financial attractiveness to alternative options.

- Simple cost analysis: for cases where the activity generates no financial return other than from A6.4ERs.

The type of analysis shall be justified by the activity participants, based on the nature of the activity (e.g., whether alternatives are available, whether it is a modification to an existing facility, or a greenfield investment).

Financial Indicators and Carbon Revenue Impact: The financial analysis shall calculate appropriate financial indicators (e.g., IRR, NPV, payback period), both with and without the expected carbon revenues from A6.4ERs. The results must clearly show that:

- The project is not financially viable without A6.4ER revenue (e.g., IRR below benchmark or NPV negative); and

- The inclusion of carbon revenue materially improves the financial viability, such that it becomes attractive to investors.

This comparison is essential to demonstrate that the carbon finance is a decisive factor in moving forward with the investment.

Data Requirements and Transparency: All financial data (e.g., capital investment, operational and maintenance costs, electricity/fuel revenues, carbon price assumptions, discount rates) must be based on publicly available sources, feasibility studies, or actual investment documents. These shall be transparently documented, with all key assumptions disclosed.

Where relevant, a sensitivity analysis shall be conducted on key variables (e.g., carbon price, input cost, product price) to assess the robustness of the additionality claim.

Barrier analysis

Barrier analysis is not applicable in this case, as the Green Ammonia project does not relate to activities outlined in paragraphs 52 and 53 of A6.4-STAN-METH-004 - such as those implemented at individual households or by small entities without access to commercial or public third-party finance.

Common practice analysis

This methodology applies the draft Common Practice Assessment Tool (A6.4-MEP07-A01) under the Article 6.4 Mechanism to determine whether the proposed activity is common practice in the applicable geographical area to assess the additionality of the mitigation project activity.

The following parameters shall guide the implementation of the common practice analysis:

| Parameter | Guidance / Value | Justification of the Choice |
| --- | --- | --- |
| Approach used | Approach B – Target market size and market penetration | Approach B is selected because the ammonia sector lacks enough directly comparable green ammonia facilities, which limits the effectiveness of Approach A. Approach B allows quantifying the diffusion of the green ammonia production practice within the defined market. |
| Indicator type | Capacity/output-based – Annual ammonia production (tonnes/year), considering either installed design capacity or actual output. | Ammonia production is best measured through capacity or actual output, as it is directly tied to project scale and performance. This aligns with available industry data and regulatory reporting norms. |
| Data approach | Stock-based – All operational facilities started commercial operation before the project design document (A6.4-PDD) is published for global stakeholder consultation or before the start date of proposed project activity, whichever is earlier for the proposed project activity. | A stock-based approach ensures a comprehensive assessment of existing operational facilities at the time of project planning, representing the real state of technology deployment prior to the proposed project activity. |
| Geographical area | Should be the entire host country. If the project participants opt to limit the applicable geographical area to a specific geographical area (such as province, region, etc.) within the host country, then they shall provide justification on the essential distinction between the identified specific geographical area and rest of the host country. | Applying the analysis at the national level provides a holistic view of technology penetration and avoids regional data bias. If projects apply a sub-national scope, they must justify how the selected area is distinct in terms of market structure, infrastructure, or policy support. |
| Capacity/output range | Include projects with a production capacity or output within ±50% of the total design capacity or output of the proposed project activity. | The ±50% range aligns with the guidance provided in the Tool and ensures inclusion of comparable-scale projects |
| Common practice threshold (Fmax) | 20% – Default value as per the Common Practice Tool | Although the Common Practice Tool prescribes a default threshold of 3% for Approach B, a threshold of 20% is adopted to reflect the early-stage market of green ammonia. Green ammonia production is currently in a nascent phase globally and in the host country, with limited diffusion due to high costs, lack of infrastructure, and policy uncertainty. Using a higher threshold (20%) provides a more meaningful assessment of whether technology is genuinely widespread. This adjustment aligns with the tool’s allowance for deviation when justified by sectoral characteristics and enhances environmental integrity by avoiding over-crediting of technologies. |
| Market penetration calculation | Share of green ammonia production capacity relative to total ammonia capacity within the defined geographical area | Market penetration is best captured through the share of green ammonia capacity in total ammonia production, as this clearly distinguishes the proposed low-emission technology from conventional ammonia production (e.g., SMR, coal gasification). |
| Validation responsibility | DOE to validate all assumptions, parameter selection, and data sources at validation stage and at each crediting period renewal | As per Tool guidance, Designated Operational Entities (DOEs) are required to validate all methodological applications. |

The outcome of the common practice analysis, conducted as per the guidance in this methodology and using Tool A6.4-MEP07-A01, shall be used to determine whether the proposed project activity is a widely adopted practice in the applicable geographical area. If the calculated market penetration of the proposed technology, measure, or practice is below the common practice threshold (Fmax), the activity shall be deemed as not common practice and may be considered additional under the Article 6.4 Mechanism.

- Baseline scenario

Selection of the baseline approaches from paragraph 36 of the rules, modalities and procedures

For each emission reduction component, the baseline can be set using one of the following approaches, depending on data availability and project circumstance

Best available technologies that represent an economically feasible and environmentally sound course of action, where appropriate.

An ambitious benchmark approach where the baseline is set at least at the average emission level of the best performing comparable activities providing similar outputs and services in a defined scope in similar social, economic, environmental and technological circumstances.

An approach based on existing actual or historical emissions, adjusted downwards to ensure alignment with paragraph 33 of the RMP.

Approach 1:  To determine the Best Available Technology (BAT) for ammonia production, that is both economically feasible and environmentally sound, a structured methodology is delineated below. This method considers the current state of technology, economic viability (including carbon pricing), and compliance with national regulations. The goal is to establish a baseline for deep decarbonization while ensuring practical implementation.

If the host country has evaluated and notified Best Available Technology for Ammonia Production, the same may be adopted. Otherwise,

Below is a step-by-step outline for determining the BAT for ammonia production:

Step 1: Define Objectives and Scope

      - Objective: Identify the most economically viable and environmentally sound technology for ammonia production that aligns with deep decarbonization goals.

      - Scope: Focus on ammonia production via hydrogen and nitrogen synthesis.

      - Include technologies for hydrogen production (e.g., SMR, electrolysis) and nitrogen separation (e.g., cryogenic air separation).

      - Consider both existing and emerging technologies.

      - Incorporate compliance with national regulations and standards.

      - Account for carbon pricing (zero or domestic carbon price/tax).

Step 2: Establish Evaluation Criteria

- Define the key criteria for evaluating technologies:

- Economic Viability:

- Capital expenditure (CAPEX) and operational expenditure (OPEX).

- Levelized cost of ammonia (LCOA) production.

- Sensitivity to carbon pricing (if applicable).

- Payback period and return on investment (ROI).

- Environmental Performance:

- Greenhouse gas (GHG) emissions intensity (tonnes CO₂ per tonne NH₃).

- Energy efficiency (GJ per tonne NH₃).

- Resource use (water, raw materials).

- Waste generation and byproduct management.

- Regulatory Compliance:

- Alignment with national environmental regulations (e.g., emissions limits, energy efficiency standards).

- Compatibility with international agreements (e.g., Paris Agreement, net-zero targets).

- Technical Feasibility:

- Scalability and maturity of the technology.

- Availability of infrastructure (e.g., natural gas pipelines, renewable energy sources).

- Operational reliability and maintenance requirements.

- Social and Strategic Factors:

- Job creation and local economic impact.

- Energy security and reliance on imports.

- Public acceptance and alignment with national energy transition strategies.

Step 3: Identify Candidate Technologies

- List all potential technologies for each stage of ammonia production:

- Hydrogen Production

- Steam Methane Reforming (SMR): Dominant technology; low-cost but high GHG emissions.

- Autothermal Reforming (ATR): Improved efficiency over SMR; lower emissions.

- Electrolysis (Green Hydrogen): Powered by renewable energy; zero emissions but higher costs.

- Blue Hydrogen: SMR or ATR with Carbon Capture and Storage (CCS); reduced emissions.

- Nitrogen Production

- Cryogenic Air Separation: Standard technology; highly efficient but energy intensive.

- Pressure Swing Adsorption (PSA) : Suitable for smaller-scale operations.

- Ammonia Synthesis

- Haber-Bosch Process: Dominant technology; mature and reliable.

- Electrochemical Ammonia Synthesis: Emerging technology; low temperature and pressure but not yet commercially viable.

Step 4: Conduct Comparative Analysis

- Perform a detailed comparison of candidate technologies using the evaluation criteria:

- Cost Analysis:

- Estimate CAPEX and OPEX for each technology.

- Calculate LCOA for different candidate technologies:

- Environmental Impact Assessment:

- Quantify GHG emissions for each technology (e.g., lifecycle analysis).

- Evaluate energy efficiency and resource use.

- Regulatory Compliance Check:

- Verify adherence to national emissions limits, energy efficiency standards, and other relevant regulations.

- Feasibility Assessment:

- Assess technical readiness levels (TRL) and scalability.

- Evaluate infrastructure requirements and availability.

Step 5: Select the Best Available Technology (BAT)

- Based on the comparative analysis, select the technology that meets the following criteria:

- Economic Viability: Lowest LCOA under the given carbon pricing scenario.

- Environmental Soundness: Lowest GHG emissions and highest energy efficiency.

- Regulatory Compliance: Meets or exceeds national environmental and safety standards.

- Technical Feasibility: Proven at scale and compatible with existing infrastructure.

Note: The selected Best Available Technology (BAT) shall satisfy Criteria 1, 2, 3, and 4.  Of the two technologies evaluated if one meets the criteria 1, 3, and 4, and the other meets the criteria 2, 3, and 4 then the technology with the lowest GHG emissions shall be chosen.

Approach 2: In this option, the activity participant shall select an ambitious benchmark, where the baseline is set at least at the average emission level of the best performing comparable activities, providing similar outputs and services, in a defined scope in similar social, economic, environmental and technological circumstances.

If the host country has evaluated and notified an ambitious benchmark for Ammonia Production (in its Standardized Baseline), the same may be adopted. Otherwise,

For baseline emissions calculations, the following steps and equations shall be applied:

Step 1: Define the Scope of Comparable Ammonia Plants

- Technology Type:

- Include ammonia plants utilizing natural gas (SMR), coal gasification and other fossil fuel-based processes.

- Exclude older and inefficient technologies that do not represent best practices.

- Geographical Coverage:

- Consider national level best-performing plants while ensuring alignment with regional economic and environmental conditions.

- Use publicly available databases and industry reports to source efficiency data.

- Operational Timeframe:

- Only plants commissioned within the last five years (to ensure relevance to Best Available Technology).

- Data Sources:

- Industry databases (e.g., IFA, IHS Markit, Argus, UNFCCC CDM databases, corporate sustainability reports).

- Peer-reviewed literature and verified third-party assessments.

- If direct plant-level data is unavailable, use modelled estimates based on known best-performing technologies.

Step 2: Collect and Rank Specific Emissions Data

- Data Collection:

- Gather specific CO₂ emissions data (SEi, in tCO₂/tNH₃) for all identified ammonia plants that meet the scope criteria.

- The emissions data should reflect full lifecycle emissions, including hydrogen production and ammonia synthesis.

- Rank by Efficiency:

- Sort the ammonia plants in ascending order based on their specific CO₂ emissions (SEi).

- The most efficient plant has the lowest SEi,

- Select the Top 10% Most Efficient Plants:

- The criteria for the benchmark selection will be as follows:

- The benchmark is set at the average emission level of the top 10% most efficient ammonia plants worldwide or in the geographical region.

Approach 3: In this option, the activity participant shall select an approach based on existing actual or historical emissions, adjusted downwards to ensure alignment with paragraph 33 of the RMP.

Step 1: Determine Historical Specific Emissions

- Historical specific emissions are based on actual or past emissions data from comparable ammonia plants that have commenced operation over a defined period (typically 3–5 years)

Step 2: Apply Downward Adjustment

- According to paragraph 33 of the RMP, the baseline must be adjusted downward to avoid over-crediting.

Step 3: Final Adjusted Baseline Emissions

Application of the selected approach, prior to implementation of a downward adjustment

Procedure for the identification of the baseline scenario

The stepwise approach has already been explained in B.5.5.1. Based on this approach, the identification of the baseline scenario will be as follows:

- BEconserv,y= min⁡(BEBAT,BEbenchmark, BEhistorical,)                 Equation (1)

The above is comparison and selection of the most conservative Baseline Emissions.

Calculation of baseline emissions prior to downward adjustment

Approach 1 – For baseline emissions calculations, that applies Best Available Technologies (BAT) for ammonia production, the following equations can be applied:

- BEBAT,y = SEBAT,y* PNH3,y                               Equation (2)

With:

i) If project activity supplements an existing conventional ammonia facility

- PNH3,y= QNH3,total*fgreen                               Equation (3)

ii) If project activity is a new standalone green ammonia facility

- PNH3,y= QNH3,total                                      Equation (4)

Where:

| BEBAT,y | = | Baseline CO2 emissions using BAT in year y (tCO2) |
| --- | --- | --- |
| SEBAT,y | = | Specific emissions of the best available ammonia production technology in year y (tCO2/tNH3) |
| PNH3,y | = | Quantity of Green Ammonia produced in PACM project activity in year (tNH3) |
| QNH3,total | = | Total quantity of ammonia produced and verifiably exported by the project activity during the year (t NH₃) |
| fgreen | = | Fraction of ammonia produced by the new project activity that is classified as green, based on the use of renewable hydrogen and nitrogen inputs (%) |

Approach 2: The benchmark specific emissions factor is the average emissions intensity of the top 10% most efficient ammonia plants:

- If there are N total plants, select the top 10%:

- Nbenchmark=0.10*N                                 Equation (5)

Nbenchmark​, represents the best-performing ammonia production facilities.

SEbenchmark,y​= i=1NbenchmarkSEi​Nbenchmark                                 Equation (6)

Where:

| SEbenchmark,y | = | Ambitious benchmark specific emissions factor (tCO2/(tNH3 ) |
| --- | --- | --- |
| SEi | = | Specific emissions of the ith plant in the top 10% best-performing ammonia plants (tCO2/tNH3) |

BEbenchmark,y = SEbenchmark,y* PNH3,y                 Equation (7)

Where:

| BEbenchmark,y | = | Baseline CO2 emissions using the benchmark approach in year y (tCO2) |
| --- | --- | --- |
| SEbenchmark,y | = | Specific emissions of best-performing ammonia production plants globally (in year y tCO2/tNH3) |

Approach 3: For baseline emissions calculations, that apply an approach based on existing actual or historical emissions for ammonia production, the following steps and equation shall be applied:

BEhistorical = SEhistorical* PNH3,y                            Equation (8)

Where:

| BEhistorical,y | = | Baseline CO2 emissions based on historical data in year y (tCO2) |
| --- | --- | --- |
| SEhistorical | = | Historical Specific emissions of ammonia production (tCO2/tNH3) |

Step 1: Determine Historical Specific Emissions

SEhistorical,y​= t=1TECO2,y​t=1TPNH3,y​                             Equation (9)

Where:

| SEhistorical,y​ | = | Average historical emissions intensity (tCO2/tNH3) |
| --- | --- | --- |
| ECO2,y | = | Actual CO2 emissions from historical ammonia production in year y (tCO2) |

Step 2: Apply Downward Adjustment

A conservative factor (AF) is applied:

BEhistorical,y= SEhistorical,y -  AF                                 Equation (10)

AF = SEhistorical,yYNet Zero - YSE                                              Equation (11)

Where:

| AF | = | Adjustment Factor to align with Net Zero Target of Host Party |
| --- | --- | --- |
| SEhistorical,y | = | Sectoral Emissions determined in year y |
| YNet Zero | = | Year in which the host country has committed to achieving net-zero greenhouse gas (GHG) emissions |
| YSE | = | Year in which historical sectoral emissions were determined |

Final Baseline Emissions with Adjustments:

BEadjusted,y= BEhistorical,y* PNH3,y                              Equation (12)

Where:

| BEadjusted,y​ | = | Adjusted Baseline Emissions (tCO2) |
| --- | --- | --- |
| BEhistorical,y | = | Baseline CO2 emissions based on historical data (tCO2) in year y (tCO2/tNH3) |

Final Baseline Emissions:

BEfinal,y= BEconserv,y = min⁡(BEBAT,y,BEbenchmark,y , BEadjusted,y)    Equation (13)

Other key factors for Baseline Determination

- Scientific Accuracy: The methodology relies on peer-reviewed literature, industry reports, and international best practices.

- Transparency: All data sources, assumptions, and calculations are clearly documented.

- Conservativeness: Where multiple data sources exist, the most conservative (lowest) baseline emissions value is selected.

- Comparability: The selected baseline represents comparable ammonia production plants operating under similar economic and technological conditions.

Data Sources and Assumptions

Primary Data Sources: Whenever possible, primary data is collected from:

- Best performing commercial ammonia plants (past 5 years)

- Industry reports (IEA, IPCC, IRENA, Ammonia Energy Association, etc.)

- Environmental Impact Assessments (EIA) of ammonia production facilities

- Regulatory emissions data (EU ETS, US EPA, etc.)

- Secondary Data Sources: Where primary data is unavailable, credible secondary sources are used, ensuring they meet the following criteria:

- Reliability: Data is obtained from authoritative sources (peer-reviewed research, government agencies, industry databases).

- Geographical & Technological Relevance: The data reflects production conditions like the project location.

- Conservativeness: The most conservative value is used if multiple datasets exist.

Calculation of the downward adjusted baseline

As this methodology requires that all three approaches be chosen and conservative amongst these as the baseline scenario, it includes host countries’ ambition over time, and it is conservative and below Business as Usual (BAU). It aligns with long term temperature goal as approach 3 has a downward adjustment factor. Leakages are explicitly accounted in subsequent sections.

Identification of the conservative BAU scenario

The BAU scenario reflects the most plausible and conservative set of processes, technologies, and regulatory requirements for ammonia production in the host country or relevant region, in the absence of the Article 6.4 activity.

The determination follows the guidance under Section 8 of A6.4-STAN-METH-004 and the methodology considers the following alternatives for the identification of the Business-as-Usual (BAU) scenario:

- Continuation of the historical situation (pre-activity scenario).

- Establishment of an economically viable technology and/or practice.

- A scenario combining (a) for the remaining lifetime of the existing equipment and/or practice, followed by (b).

- Other relevant scenarios justified in line with the applicable principles.

Among the options listed above, this methodology selects Option (a): Continuation of the historical situation (pre-activity scenario) as the most appropriate and conservative approach for determining the BAU scenario. The justification for the selection and exclusion of the other options is provided below:

This option reflects the dominant ammonia production technologies and practices observed in the host country or region during the most recent three-year period.

This approach is supported by global production trends. The global nameplate capacity for ammonia is estimated to be between 180 and 190 million tonnes per annum (Mtpa) , with approximately 150 Mtpa of actual production in 2023 and 2024 according to the U.S. Geological Survey (USGS). The International Fertilizer Association (IFA) projects that capacity will grow to 207 Mtpa by 2028, but most of the current production continues to rely on conventional fossil-fuel-based technologies. This production is spread across approximately 490 to 550 plants globally, indicating high technological lock-in and relatively slow turnover of infrastructure.

Given this widespread deployment of conventional ammonia production, it is conservative and appropriate to define the BAU scenario based on the continuation of historical practices.

Additional conservativeness is ensured by:

- Excluding emerging or experimental technologies (e.g., electrolysis-based hydrogen or novel synthesis methods) unless they surpass a 5% national production share.

- Incorporating legally binding regulatory constraints (e.g., energy efficiency mandates or emissions limits) through the Regulatory Adjustment Factor (RA).

- Using recent verifiable data (last three years) to avoid reliance on forward-looking projections.

Option (b): Establishment of an economically viable technology and/or practice (Not selected) - This option was considered but not selected because it would require assumptions about future technology trends or cost-effectiveness, which may not reflect current market realities. In the context of ammonia production, certain low-emission technologies (e.g., electrolysis using renewable electricity) may be economically promising but are not yet dominant or widely adopted. Selecting this option could underestimate baseline emissions and compromise conservativeness.

Option (c): Combined scenario (Not selected) - This option is more applicable to situations involving partial continuation of existing facilities followed by technology upgrades. However, the green ammonia activities targeted under this methodology are typically greenfield projects or involve a complete transition from conventional methods. Therefore, this scenario is not relevant to the context of the methodology.

Option (d): Other relevant scenarios (Not selected) - This fallback option is only applicable when the above alternatives are unsuitable. Since Option (a) sufficiently reflects current sectoral conditions in a conservative manner, Option (d) is not applicable.

Thus, Option (a) ensures a realistic, transparent, and conservative determination of the BAU scenario consistent with the requirements of A6.4-STAN-METH-004.

Definition of BAU Subprocesses: Ammonia production under BAU is characterized by the dominant production pathway observed over the past three years. This typically includes:

- Hydrogen Production

- Steam Methane Reforming (SMR) or Coal Gasification, depending on the region’s primary fossil fuel resources (e.g., natural gas or coal).

- Electrolysis is excluded unless it accounts for >5% of national ammonia production.

- Nitrogen Production:

- Cryogenic Air Separation, the industry standard for high-purity nitrogen extraction.

- Ammonia Synthesis:

- Haber-Bosch Process, universally used to combine hydrogen and nitrogen under high pressure/temperature.

- Direct Ammonia Production (single-step processes like electrochemical synthesis) is excluded from BAU due to lack of commercial-scale adoption in the past three years.

Data Collection for BAU Determination

- Technology Prevalence:  Identification of the most widely deployed technologies (e.g., SMR vs. coal gasification) based on national industry surveys, production reports, or peer-reviewed studies.

- Energy Source Dependency:  Analysis of the primary feedstock (e.g., natural gas, coal, or grid electricity) used in hydrogen production, aligned with the country’s resource availability.

- Regulatory Compliance: Incorporation of legally enforced efficiency standards or technology mandates (e.g., waste heat recovery requirements, emission limits) that were binding during the three-year baseline period.

Adjustments for Regulatory and Efficiency Mandates: In accordance with Paragraphs 13–15 of Appendix 1 of the Baseline Standard, this methodology ensures that quantified emission reductions are attributed solely to the implementation of the Article 6.4 activity. A systematic assessment of uncertainties arising from exogenous factors is conducted, and conservative approaches are applied accordingly. Emission reductions are adjusted using a Regulatory Adjustment Factor (RA) to account for exogenous influences such as:

- National or subnational policy interventions (e.g., renewable energy mandates, fuel-switching regulations)

- Sector-wide decarbonization trends or Technology Adoption trends

- Data limitation

| Exogenous Factor | Uncertainty | Conservativeness Measure |
| --- | --- | --- |
| Policy or regulatory changes | Timing, enforcement, and impact not certain | Only include policies with clear, enforceable mandates; apply conservative RA values |
| Sector-wide trends for decarbonization or Technology Adoption trends | Difficult to isolate impact from Article 6.4 activity | Use lower-bound emission reduction estimates or apply discount factors |
| Data limitations | Incomplete or outdated information | Use standardized, verified data sources |

All Regulatory Adjustment Factors (RA), including those applied due to policy changes, sector-wide technology transitions, or data limitations, shall be determined using conservative and evidence-based assumptions. These assumptions—such as policy enforcement timelines, projected adoption rates of emerging technologies, and data reliability—shall be transparently documented and subject to cross-check during the validation stage and at each renewal of the crediting period. This ensures that emission reductions attributed to the Article 6.4 activity remain conservative, credible, and not influenced by unrelated exogenous factors.

Exclusion of Emerging Technologies: The BAU scenario excludes:

- Green Hydrogen-Based Ammonia:  Production pathways using renewable hydrogen (e.g., electrolysis powered by wind/solar) unless they account for >5% of national production.

- Novel Synthesis Methods:  Experimental technologies (e.g., plasma catalysis, electrochemical synthesis) lacking commercial-scale adoption.

Conservativeness and Compliance: The BAU scenario is designed to be conservative and replicable:

- Temporal Scope:  Based on the most recent three years of verifiable data to avoid speculative assumptions about future trends.

- Data Sources:  Relies on auditable data (e.g., national industry reports, energy statistics, regulatory filings).

- Adjustment Safeguards:  Downward adjustments are made only for enforced regulations (not voluntary initiatives) to prevent overestimation.

Methodological Steps to Determine BAU:

- Step 1: Collect data on ammonia production technologies, energy sources, and subprocess prevalence (last three years).

- Step 2: Identify the dominant production pathway (e.g., SMR + Haber-Bosch).

- Step 3: Adjust for binding regulations (e.g., efficiency standards) that alter the baseline emissions/intensity.

- Step 4: Exclude emerging technologies not yet mainstream.

- Step 5: Validate against national statistics and third-party industry analyses.

Key Output: A BAU emissions intensity (e.g., tCO₂e/ton NH₃) reflecting:

- Dominant subprocesses.

- Energy source carbon intensity.

- Regulatory-compliant efficiency levels.

Calculation of the conservative BAU emissions

BAU emission is calculated as follows: The BAU emissions formula estimates the total greenhouse gas (GHG) emissions from ammonia production under standard conditions (before any decarbonization measures).

- BAUEmission,y = (EH2,y + EN2,y + Esynthesis) * RA                             Equation (14)

Where:

| EH2,y | = | Emissions from Hydrogen production in year y |
| --- | --- | --- |
| EN2,y | = | Emissions from Nitrogen production in year y |
| Esynthesis,y | = | Emissions from Ammonia Synthesis process in year y |
| RA | = | Regulated Adjustment Factor to account for policy mandated efficiency improvements |

Hydrogen Production Emissions (EH2)

- EH2,y = Hreq,y * IH2                          	                      Equation (15)

Where:

| Hreq,y | = | Mass of pure hydrogen generated by the project activity at the hydrogen production plant during year y (tonnes of H₂) |
| --- | --- | --- |
| IH2 | = | GHG intensity of hydrogen production (tCO₂e/tonH₂) |

Calculation of IH2 :

1. IH2 for SMR: For SMR, the emissions are mainly from:

- Natural gas combustion (energy for process heating).

- Process emissions (CO₂ from methane reforming reaction).

- Electricity consumption (if any).

- IH2= ECH4+ Ecombustion,SMR+ Eelectricity,SMR                         Equation (16)

- With

- ECH4 = Qnatural gas * EFNG                                 Equation (17)

Where:

| ECH4 | = | Methane feedstock emissions |
| --- | --- | --- |
| Qnatural gas | = | Quantity of natural gas consumed (tons) |
| EFNG | = | Emission factor for natural gas used (tCO2) |

Ecombustion,SMR = Qfuel,SMR * EFfuel,SMR                      Equation (18)

Where:

| Qfuel,SMR | = | Quantity of fuel consumed in SMR (tons) |
| --- | --- | --- |
| EFfuel,SMR | = | Emission factor for fuel used in SMR (tCO2) |

Eelectricity,SMR = Qelectricity,SMR * EFgrid,SMR                   Equation (19)

Where:

| Qelectricity | = | Quantity of electricity consumed in SMR (MWh/tH2) |
| --- | --- | --- |
| EFgrid | = | Emission factor of grid electricity used in SMR (tCO2/MWh) |

2.  IH2 for Coal gasification:

IH2= Ecoal+Ecombustion+ Eelectricity                    Equation (20)

With:

Ecoal = Qcoal * EFcoal                                   Equation (21)

Where:

| Qcoal | = | Quantity of coal consumed (tons) |
| --- | --- | --- |
| EFcoal | = | Emission factor for coal (tCO2) |

Ecombustion,coal = Qfuel * EFfuel                            Equation (22)

Where:

| Ecombustion,coal | = | Emissions from combustion of fossil fuel in coal gasification |
| --- | --- | --- |
| Qfuel,coal | = | Quantity of fossil fuel consumed in coal gasification (tons) |
| EFfuel,coal | = | Emission factor of fossil fuel used in coal gasification (tCO2) |

Eelectricity,coal = Qelectricity,coal *  EFgrid,coal                  Equation (23)

Where:

| Eelectricity,coal | = | Emissions from electricity consumption in coal gasification (tCO2/MWh) |
| --- | --- | --- |
| Qelectricity,coal | = | Quantity of electricity consumed for coal gasification (MWh) |
| EFgrid,coal | = | Emission factor for grid electricity consumed in coal gasification tCO₂e/MWh |

Nitrogen Production Emissions (EN2)

EN2,y = N2energy,y * EFgrid,N2                          Equation (24)

Where:

| EN2,y | = | Emissions from nitrogen production in year y (tCO₂e/tonN2) |
| --- | --- | --- |
| N2energy,y | = | Energy consumption for nitrogen production in year y (MWh/tN₂) |
| EFgrid,N2 | = | Grid emission factor (tCO₂e/MWh) |

Ammonia Synthesis Emissions (Esynthesis)

Esynthesis,y = NH3energy,y* EFgrid,synthesis                   Equation (25)

Where:

| Esynthesis,y | = | Emissions from ammonia synthesis (tCO₂e/ton NH₃) |
| --- | --- | --- |
| NH3energy,y | = | Energy consumption for ammonia synthesis (MWh/tNH₃) |
| EFgrid | = | Grid emission factor (tCO₂e/MWh) |

Calculation of the Regulatory Adjustments Factor (RA)

- a) Policy-Based RA

- If applicable national or subnational regulations are scheduled or in force and are expected to reduce emissions during the crediting period:

- RA=1−X%                                 Equation (26)

- Where,

- X% reflects the expected emissions reduction impact due to the policy, based on published regulatory documents or government targets.

- b) Sector-wide trends for decarbonization or Technology Adoption Trend Adjustment

- Where sector-wide decarbonization trends or the uptake of emerging technologies (e.g., blue hydrogen, CCS retrofits) are anticipated to partially displace conventional production methods, the Regulatory Adjustment Factor (RA) shall reflect the projected level of adoption as follows:

- RA = (Adoption share) × Emission intensity of BAU techEmission intensity of transition tech  +  (1−Adoption share) Equation (27)

- c) Data Quality Adjustment (Due to Data Limitations)

- Where baseline parameters or emission factors are derived from incomplete, outdated, or non-project-specific data, a standardized conservativeness factor shall be applied to account for uncertainty due to data limitations.

- RA = 0.95

- A fixed 5% discount is applied (i.e., CF = 0.95), unless higher-quality, project-specific or nationally validated data is available. This ensures conservativeness in cases where data reliability is uncertain

Comparison of the downward adjusted baseline and the conservative business-as-usual baseline

Activity participants shall calculate the difference between the baseline emissions or removals and BAU emissions as an annual and total amount concerning the crediting period:

- ∆d= BAUEmissions,y- BEfinal,y                               Equation (28)

Where:

| ∆d | = | Difference between BAU and baseline emissions in year y (tCO2/year) |
| --- | --- | --- |
| BAUEmissions,y | = | Business As Usual Emissions in year y (tCO2e/tNH3) |
| BEfinal,y | = | Final Baseline Emissions based on the most conservative factor in year y (tCO2) |

- Activity scenario

Calculation of activity emissions

Activity emissions refer to the total direct and indirect greenhouse gas (GHG) emissions resulting from the implementation of the Green Ammonia production project. These emissions must be compared with baseline emissions to determine the net mitigation impact.

Activity emissions sources include:

- Direct onsite emissions from hydrogen, nitrogen and ammonia production, including physical leaks all the gases.

- Indirect emissions resulting from electricity consumption during the ammonia production process.

Data Sources and Assumptions

- Primary Data Sources (Preferred)

- Where possible, real-time operational data from the project facility is used, including:

- Onsite monitoring of energy consumption (electrolysers, compressors, synthesis reactors, etc.).

- Renewable energy supply and grid mix data (for indirect emissions calculations).

- Hydrogen production efficiency (electrolysis efficiency, renewable energy input, etc.).

- Physical leaks of hydrogen and ammonia in green ammonia production value chain.

- Secondary Data Sources (If Primary Data is Unavailable)

- IPCC Emission Factors Database (for fuel combustion, industrial processes and GWP value).

- International Energy Agency (IEA) reports on ammonia and hydrogen production emissions.

- Ammonia Industry Reports (IFA, Ammonia Energy Association, etc.).

- Regional grid hourly emission factors (from IEA, EPA, or country-specific data sources).

Ensuring Data Conservativeness:

- If multiple datasets exist, the most conservative (highest emissions) value shall be selected.

- Data sources shall be selected based on geographical and technological relevance.

Steps for Calculating Activity Emissions in Green Ammonia Production

- Step 1: Define System Boundary for Activity Emissions

- Activity emissions are calculated for the following key process stages:

- Hydrogen Production (Electrolysis); Desalination/Treatment of water and Pumping

- Air Separation for Nitrogen

- Ammonia Synthesis (Haber-Bosch Process)

- Step 2: Calculate Emissions from Electricity Use

AEelectricity = Celec,grid,y* EFelec,grid+ Celec,RE,y​×EFelec,RE,y​   Equation (29)

Where:

| AEelectricity | = | Activity emissions from the consumption of electricity from sources other than the renewable power plant in the ammonia production project activity in year y (tCO2e). |
| --- | --- | --- |
| Celec,grid,y | = | Electricity consumed in the project activity from grid in a year y (MWh/year) |
| EFelec,grid | = | Emission factor for grid electricity (tCO2/MWh) |
| Celec,RE,y | = | Electricity consumed from renewable energy sources in year y (MWh/year) |
| EFelec,RE,y | = | Emission factor for renewable electricity (tCO2/MWh) |

- If the renewable electricity is sourced from solar or wind power plants, its emission factor shall be considered zero. However, if it is sourced from hydro or pumped hydro storage, project participants shall account for associated CH₄ and CO₂ emissions from the reservoirs, using default values or project-specific estimates as applicable.

EFelec,RE,y= EFRes                                 Equation (30)

Where:

| EFRes | = | Default emission factor for emissions from reservoirs of hydro power plants/ PSP (kg CO2e/MWh) |
| --- | --- | --- |

- Step 3: Calculate Emissions from fossil fuel Use (if any)

Efuel,y = Fconsumed,y* EFfossil-fuel                         Equation (31)

Where:

| Efuel | = | Activity emissions from the consumption of fossil fuel in the ammonia production project activity in year y (tCO2e) |
| --- | --- | --- |
| Fconsumed,y | = | Fuel consumed by the project activity in a year y (tons/year) |
| EFfossil-fuel | = | Emission factor for fuel used (tCO2/ton of fuel) |

- Step 4: Calculate emissions from physical leaks of Hydrogen and Ammonia

Eleaks,y = EH2-leaks,y+ ENH3-leaks,y                              Equation (32)

Where:

| Eleaks, y | = | Activity emissions due to physical leaks in ammonia production project activity in a year y |
| --- | --- | --- |
| EH2-leaks, y | = | Activity emissions due to physical leaks of hydrogen in the project activity in a year y (tCO2e) |
| ENH3-leaks, y | = | Activity emissions due to physical leaks of ammonia in the project activity in a year y (tCO2e) |

- Step 5: Emissions Due to Physical Leaks of Hydrogen

- EH2-leaks = Hreq,y * PLH2 * GWPH2                                          Equation (33)

Where:

| Hreq,y | = | Mass of pure hydrogen generated by the project activity at the hydrogen production plant during year y (tH₂) |
| --- | --- | --- |
| PLH2,y | = | Physical leaks of hydrogen in hydrogen value chain as a percentage of the total production in a year y (%) |
| GWPH2 | = | Global warming potential of hydrogen (tCO2e/tH2) |

- Step 6: Emissions Due to Physical Leaks of Hydrogen

- ENH3-leaks = PNH3,y * PLNH3 * fNH3 →NOx* GWPNO2                 Equation (34)

Where:

| PNH3,y | = | Quantity of Green Ammonia produced in PACM project activity in year y (tNH3) |
| --- | --- | --- |
| PLNH3,y | = | Physical leaks of ammonia in ammonia value chain as a percentage of the total production in a year y (%) |
| GWPNO2 | = | Global warming potential of nitrous oxide (tCO2e/tNH3) |
| fNH3→NOx | = | Fraction of volatilized ammonia (NH₃) that undergoes atmospheric oxidation to form nitrogen oxides (NOₓ) (%) |

- Note: Based on the some of the research, fraction of volatilized ammonia that undergoes atmospheric oxidation to from nitrogen oxides (NOx) is very minimal (i.e.: lesser than 1%). Hence this can be neglected if country specific oxidation factor is not available.

- Step 6: Calculate Total Activity Emissions

- Total activity emissions are calculated as:

AEy = AEelectricity,y+AEfuel,y+AEleaks,y               Equation (35)

Where:

| AEy | = | Activity emissions in year y (tCO2) |
| --- | --- | --- |

- Leakage

Identification of leakage emission sources

In accordance with the requirements of the Standard for Addressing leakage in mechanism methodologies (A6.4-STAN-METH-005) (Version 1.0), this methodology identifies all potential sources of leakage relevant to Green Ammonia production project activities.

Leakage emissions sources in green ammonia production includes:

- Transfer of Baseline Equipment

- Competition for Resource Use

- Diversion of Existing Production or Outputs

- Environmental GHG Release

Avoidance or minimization of leakage

Transfer of Baseline Equipment (Applies to the brownfield project activities):

- Leakage may occur if fossil-based ammonia or hydrogen production equipment replaced by the project is reused elsewhere. In accordance with paragraph 82(b) of the Standard, the methodology requires either

- (a) proof of permanent decommissioning or scrapping of the baseline equipment, or

- (b) quantification of continued emissions if reused.

- Where the baseline equipment continues to operate, leakage is estimated based on operating hours, fuel use, output, and emission factors. If dismantled and repurposed in a disaggregated manner, no leakage is considered. Thus, leakage is avoided or minimized through decommissioning or quantification measures.

Competition for Resource Use:

- Risks of leakage from increased electricity or water consumption are minimized through applicability conditions: at least 90% of electricity must come from new renewable power plants (avoiding displacement of existing clean capacity) and project water use is capped at 5% of local availability, thereby preventing significant diversion from other users and avoiding the leakage from the competition from the resource use.

Leakage from Diversion of Production (applies only to brownfield activities)

- For brownfield project activities, leakage from diversion of production is avoided by mandating that the project delivers the same output/service level as the baseline. The applicability conditions of the methodology specify that the output(s) or level(s) of service delivered by the project must remain identical to those in the pre-PACM project condition. As this requirement ensures there is no diversion of production or change in service level, calculation of leakage emissions related to such changes is not required. Therefore, the risk of leakage related to production diversion is effectively mitigated and can be excluded from the leakage assessment for simplification purposes.

Leakage from Indirect Environmental Emissions

- This provision applies to greenfield project activities. If, prior to the implementation of the PACM project, the land was used for agricultural purposes or if any land use change has occurred, the associated emissions must be accounted for and calculated as per the methodology requirements. This ensures that potential leakage from indirect land-use change is properly managed and minimized.

Addressing leakage emissions

Where leakage is identified as significant, it must be quantified using conservative assumptions and, where available, default emission factors or project-specific data.

- The total leakage emissions (LEtotal) for a given year shall be calculated as:

LEy​=LEequip,y​+LEres,y​+LEdiv,y​+LEenv,                                                 Equation (36)

Where:

| LEy | = | Total leakage emission in a year y (tCO2) |
| --- | --- | --- |
| LEequip,y | = | Leakage from transferred baseline equipment in a year y (tCO2) |
| LEres,y | = | Leakage from resource competition in a year (tCO2) |
| LEdiv,y | = | Leakage from diversion of production in a year (tCO2) |
| LEenv,y | = | Leakage from indirect environmental emissions in a year (tCO2) |

Leakage from Transferred Baseline Equipment (LEequip,y): This provision applies only to brownfield project activities where the baseline equipment is not disposed of or scrapped and continues to be used for the same purpose as it was prior to the pre-PACM project activity.

LEequip,y​=Qoutput,y​×EFequip​                                Equation (37)

Where:

| Qoutput,y | = | Output from transferred equipment (tH2/year) |
| --- | --- | --- |
| EFequip | = | Emission factor of the baseline conventional ammonia production method baseline equipment (tCO₂e/ton output) |

Leakage from Resource Competition (LEres,y):  Risk of leakage from resource competition are minimized through applicability condition. Therefore, the risk of leakage related to resource competition is effectively mitigated and can be excluded from the leakage assessment.

- LEres,y​ = 0                                              Equation (38)

Leakage from Diversion of Production (LEdiv,y): Risk of leakage from Diversion of Production are minimized through applicability condition. Therefore, the risk of leakage related to diversion of production is effectively mitigated and can be excluded from the leakage assessment.

- LEdiv,y = 0			                 Equation (38)

Leakage from Indirect Environmental Emissions (LEenv,y): This provision applies to greenfield project activities. If, prior to the implementation of the PACM project, the land was used for agricultural purposes or if any land use change has occurred, the associated emissions must be accounted for and calculated as below

LEenv,y​=g∑​(Qg,y​×GWPg​)                                           Equation (39)

Where:

| Qg,y | = | Quantity of GHG released (in tons/year) |
| --- | --- | --- |
| GWPg | = | Global Warming Potential of GHGs |

- Emission reductions

For each year of the crediting period, emission reductions are calculated by applying the baseline, activity, and leakage formulations already defined, using monitored annual green ammonia production data and verified emission factors.

For each year y of the crediting period, emission reductions are calculated as

- ERy = BEfinal,y – AEy - LEy                                         Equation (40)

Where:

| ERy | = | Emission Reduction in year y (tCO2) |
| --- | --- | --- |

- Data and parameters not monitored

| Data/parameter | IH2 |  |  |  |
| --- | --- | --- | --- | --- |
| Description | GHG intensity of hydrogen production in BAU scenario |  |  |  |
| Data unit | tCO2eq/ton of hydrogen produced |  |  |  |
| Equations referred | Equation 15 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Data sourced from internationally recognized literature (e.g., IEA, IPCC, peer-reviewed scientific publications) providing lifecycle GHG intensity of hydrogen production through conventional processes such as steam methane reforming (SMR) or coal gasification. |  |  |  |
| Treatment of uncertainty | Conservative value will be chosen from literature sources (preferably upper bound of GHG intensity) to avoid over-estimation of emission reductions. Cross-verification of multiple sources (IPCC guidelines, IEA hydrogen outlook) will be carried out. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Qnatural gas |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Quantity of natural gas consumed in the SMR Process in BAU scenario |  |  |  |
| Data unit | Tons |  |  |  |
| Equations referred | Equation 17 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of project-specific measurement data for BAU plants, the value is taken from internationally recognized literature sources such as IEA Hydrogen Outlook, IPCC Guidelines, or peer-reviewed engineering studies. If regional statistics are available (e.g., national energy agency, industry association reports), they will be prioritized. |  |  |  |
| Treatment of uncertainty | For baseline emissions, conservative values are applied by using the lower end of natural gas consumption reported in credible sources to avoid overestimation of emission reductions. Data is cross-checked with multiple literature references to minimize bias. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFNG |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for natural gas consumed in the SMR Process in BAU scenario |  |  |  |
| Data unit | tCO2 |  |  |  |
| Equations referred | Equation 17 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 2.75 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default emission factor for natural gas is sourced from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Volume 2 (Energy Sector). This source is internationally recognized and widely applied in GHG accounting |  |  |  |
| Treatment of uncertainty | For baseline emissions, conservative values are applied by using IPCC default factors (internationally accepted lower-tier values) to avoid overestimation of emission reductions |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Qfuel,SMR |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Quantity of fuel consumed in SMR |  |  |  |
| Data unit | GJ/tH2 |  |  |  |
| Equations referred | Equation 18 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of plant-specific baseline measurement, data will be obtained from internationally recognized references such as IPCC Guidelines, IEA reports, or peer-reviewed studies |  |  |  |
| Treatment of uncertainty | For baseline emissions, uncertainty is addressed by applying conservative assumptions, i.e., using the lower range of fuel consumption values reported in credible literature to avoid overestimation of emission reductions. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFfuel,SMR |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for fuel consumed in SMR |  |  |  |
| Data unit | a. If Qfuel,SMRis in GJ, then take unit as tCO₂e/GJ. b. If ​Qfuel,SMR is in m³, then take unit as​ in tCO₂e/m³. |  |  |  |
| Equations referred | Equation 18 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 0.0561 tCO₂e per GJ |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default CO₂ emission factor for natural gas fuel is taken from the 2006 IPCC Guidelines (Volume 2, Chapter 2, Table 2.2). This factor is widely used and internationally recognized for GHG accounting |  |  |  |
| Treatment of uncertainty | For baseline emissions, uncertainty is minimized by using IPCC default emission factors, which are conservative and internationally accepted. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Qelectricity,SMR |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Quantity of electricity consumed in SMR |  |  |  |
| Data unit | MWh/tH2 |  |  |  |
| Equations referred | Equation 19 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of project-specific BAU data, electricity consumption values are taken from recognized technical literature, IEA hydrogen energy outlooks, or peer-reviewed engineering studies. Where possible, regional benchmarks (e.g., national energy efficiency statistics) are used. Conservative values (lower end of electricity consumption) are selected to avoid overestimation of baseline emissions. |  |  |  |
| Treatment of uncertainty | For baseline emissions, uncertainty is addressed by selecting the lower end of electricity consumption values reported in credible literature. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Qcoal |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Quantity of coal consumed in gasification process |  |  |  |
| Data unit | Tons |  |  |  |
| Equations referred | Equation 21 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of project-specific BAU measurements, coal consumption values are sourced from internationally recognized studies and guidelines (e.g., IPCC, IEA, and peer-reviewed engineering data). If regional statistics are available, they will be prioritized |  |  |  |
| Treatment of uncertainty | For baseline emissions, uncertainty is addressed by selecting the lower end of coal consumption values reported in credible literature to ensure conservativeness and prevent overestimation of emission reductions. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFcoal |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for coal consumed in coal gasification in BAU scenario |  |  |  |
| Data unit | tCO₂ |  |  |  |
| Equations referred | Equation 21 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 1.81 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default CO₂ emission factor for coal is taken from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Volume 2, Chapter 2 – Stationary Combustion, as updated by the 2019 Refinement to the 2006 Guidelines. These defaults are internationally recognized, peer-reviewed, and widely applied in national inventories and project-level |  |  |  |
| Treatment of uncertainty | Uncertainty is minimized by relying on the IPCC default emission factors (2006, refined in 2019), which are scientifically established and internationally accepted. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Qfuel |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Quantity of fossil fuel consumed in coal gasification in BAU scenario |  |  |  |
| Data unit | Tons |  |  |  |
| Equations referred | Equation 22 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied |  |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default/representative values for coal consumption in conventional coal gasification systems are used from credible literature or feasibility study data. If actual plant-level measurements are available, they should be preferred for greater accuracy. |  |  |  |
| Treatment of uncertainty | Uncertainty arises from variations in gasification technology, fuel quality, and operating conditions. To minimize uncertainty, conservative and widely accepted reference values are applied |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFfuel |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for fuel used in coal gasification in BAU scenario |  |  |  |
| Data unit | tCO₂e |  |  |  |
| Equations referred | Equation 22 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied |  |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default CO₂ emission factor for fuel is taken from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Volume 2, Chapter 2 – Stationary Combustion, as updated by the 2019 Refinement to the 2006 Guidelines. These defaults are internationally recognized, peer-reviewed, and widely applied in national inventories and project-level |  |  |  |
| Treatment of uncertainty | Uncertainty is minimized by relying on the IPCC default emission factors (2006, refined in 2019), which are scientifically established and internationally accepted. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | Qelectricity,coal |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Quantity of electricity consumed for coal gasification in BAU scenario |  |  |  |
| Data unit | MWh |  |  |  |
| Equations referred | Equation 23 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In absence of direct plant-level measurements, representative or conservative values are adopted from feasibility study reports, peer-reviewed literature, or recognized databases. Where available, site-specific measured consumption data will be prioritized. |  |  |  |
| Treatment of uncertainty | Uncertainty arises due to variability in gasification technology, efficiency levels, and electricity grid composition. To minimize this, conservative estimates or standardized reference values are applied. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFgrid,coal, EFgrid,N2 and EFgrid,synthesis |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor for grid electricity consumed in coal gasification, nitrogen production and synthesis of ammonia in BAU scenario |  |  |  |
| Data unit | tCO₂e/MWh |  |  |  |
| Equations referred | Equation 23, Equation 24 and Equation 25 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | The grid emission factor shall be based on the Combined Margin (CM) approach, which is a weighted average of the Operating Margin (OM) and the Build Margin (BM). The weights shall be applied as per internationally accepted conventions (e.g., UNFCCC CDM Tool to calculate the emission factor for an electricity system, or equivalent approaches). The CM factor shall be sourced from the most authoritative and publicly available dataset in the host country (e.g., official electricity authority). |  |  |  |
| Treatment of uncertainty | Uncertainty is minimized by prioritizing official host-country datasets |
| Additional comments | NA |  |  |  |

| Data/parameter | N2energy,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Energy consumption for nitrogen production in year y in BAU scenario |  |  |  |
| Data unit | MWh/tN2 |  |  |  |
| Equations referred | Equation 24 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of direct plant-level measurements, conservative or representative values shall be adopted from feasibility study reports, peer-reviewed literature, or international databases |  |  |  |
| Treatment of uncertainty | Default values are applied conservatively; if measured data are used, uncertainty is addressed through QA/QC and calibration of monitoring equipment. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | NH3energy,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Energy consumption for ammonia synthesis in BAU scenario |  |  |  |
| Data unit | MWh/tNH3 |  |  |  |
| Equations referred | Equation 25 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied |  |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of direct plant-level measurements, conservative or representative values shall be adopted from feasibility study reports, peer-reviewed literature, or international databases |  |  |  |
| Treatment of uncertainty | Default values are applied conservatively; if measured data are used, uncertainty is addressed through QA/QC and calibration of monitoring equipment. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | SEBAT,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Specific emissions of the best available ammonia production technology (tCO2/tNH3) in year y |  |  |  |
| Data unit | tCO₂e/ tNH3 |  |  |  |
| Equations referred | Equation 2 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied |  |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of direct plant-level measurements, conservative or representative values shall be adopted from feasibility study reports, peer-reviewed literature, or international databases |  |  |  |
| Treatment of uncertainty | Default values are applied conservatively; if measured data are used, uncertainty is addressed through QA/QC and calibration of monitoring equipment. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | N |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Number of plants that represent the 10% of the most efficient plants in the sector |  |  |  |
| Data unit | Nos |  |  |  |
| Equations referred | Equation 5 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Determined through sectoral benchmarking. Data shall be collected from recognized industry databases, national statistics, or other credible sources representing sector-wide performance. Only plants operating under comparable conditions should be considered. |  |  |  |
| Treatment of uncertainty | Uncertainty minimized by relying on a representative dataset of plants in the sector |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | SEi |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Specific emissions of the ith plant in the top 10% best-performing ammonia plants |  |  |  |
| Data unit | tCO₂/ tNH3 |  |  |  |
| Equations referred | Equation 6 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of direct plant-level measurements, conservative or representative values shall be adopted from feasibility study reports, peer-reviewed literature, or international databases |  |  |  |
| Treatment of uncertainty | Default values are applied conservatively; if measured data are used, uncertainty is addressed through QA/QC and calibration of monitoring equipment. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | SEhistorical,y |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Historical Specific emissions of ammonia production (tCO2/tNH3) |  |  |  |
| Data unit | tCO₂/ tNH3 |  |  |  |
| Equations referred | Equation 8 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | In the absence of direct plant-level measurements, conservative or representative values shall be adopted from feasibility study reports, peer-reviewed literature, or international databases |  |  |  |
| Treatment of uncertainty | Default values are applied conservatively; if measured data are used, uncertainty is addressed through QA/QC and calibration of monitoring equipment. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFfossil-fuel |  |  |  |
| --- | --- | --- | --- | --- |
| Description | CO2 emission factor of the fossil fuel |  |  |  |
| Data unit | tCO2/ton of fuel |  |  |  |
| Equations referred | Equation 31 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied |  |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Default CO₂ emission factor for fuel is taken from the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Volume 2, Chapter 2 – Stationary Combustion, as updated by the 2019 Refinement to the 2006 Guidelines. These defaults are internationally recognized, peer-reviewed, and widely applied in national inventories and project-level |  |  |  |
| Treatment of uncertainty | Uncertainty is minimized by relying on the IPCC default emission factors (2006, refined in 2019), which are scientifically established and internationally accepted. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | YNet Zero |  |  |  |
| --- | --- | --- | --- | --- |
| Description | The earliest net-zero target year, established by the host country or under an applicable international agreement. |  |  |  |
| Data unit | NA |  |  |  |
| Equations referred | Equation 11 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | - |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Sourced from official government publications, National policy documents, UNFCCC submissions, Paris Agreement commitments. The choice is justified as it represents the legally recognized national or international net-zero commitment. |  |  |  |
| Treatment of uncertainty | Uncertainty is minimal, as the target year is officially declared. Any changes to national or international commitments will be updated in the methodology. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | GWPH2 |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Global warming potential of Hydrogen |  |  |  |
| Data unit | tCO2/tonH2 |  |  |  |
| Equations referred | Equation 33 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 5.8 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC AR4 WG1 as under chapter 2.10.3.6 |  |  |  |
| Treatment of uncertainty | Uncertainty is minimized by relying on the IPCC default value, which are scientifically established and internationally accepted. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | GWPNO2 |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Global warming potential of Nitrous Oxide |  |  |  |
| Data unit | tCO2/tonNH3 |  |  |  |
| Equations referred | Equation 34 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 273 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | IPCC Sixth Assessment Report, 2020 (AR6) |  |  |  |
| Treatment of uncertainty | Uncertainty is minimized by relying on the IPCC default value, which are scientifically established and internationally accepted. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | fNH3→NOx |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Fraction of volatilized ammonia (NH₃) that undergoes atmospheric oxidation to form nitrogen oxides (NOₓ) (%) |  |  |  |
| Data unit | % |  |  |  |
| Equations referred | Equation 34 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 273 |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Values shall be obtained from recognized peer-reviewed literature, IPCC Guidelines (2006, Vol. 4, Ch. 11, Agriculture), or country-specific defaults where available |  |  |  |
| Treatment of uncertainty | If default/IPCC factors are applied, they are conservative and globally applicable. Where national inventory factors exist, they should be used to reduce uncertainty. Uncertainty assessment should follow IPCC good practice guidance. |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFRes |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Default emission factor for emissions from reservoirs |  |  |  |
| Data unit | kgCO2e/MWh |  |  |  |
| Equations referred | Equation 30 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied | 90 kgCO2e/MWh |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Decision at CDM EB 23 |  |  |  |
| Treatment of uncertainty | NA |  |  |  |
| Additional comments | NA |  |  |  |

| Data/parameter | EFequip​ |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Emission factor of the baseline equipment (tCO₂e/ton output) |  |  |  |
| Data unit | % |  |  |  |
| Equations referred | Equation 37 |  |  |  |
| Purpose of data | Baseline emissions | Project emissions |  | Leakage emissions |
| Value(s) applied |  |  |  |  |
| Source of data | Measured |  | Other sources |  |
| Choice of data or measurement methods and procedures | Data sourced from recognized literature and benchmarks to ensure accuracy and represent best available technology standards. |  |  |  |
| Treatment of uncertainty | NA |  |  |  |
| Additional comments | NA |  |  |  |

- Data and parameters monitored

| Data/parameter | Fconsumed,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Fuel consumed by the project activity in a year y |  |  |  |  |
| Data unit | Tons |  |  |  |  |
| Equations referred | Equation 31 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Based on inventory balance records, purchase orders, and delivery notes of the fuel; updated monthly and cross-checked annually. |  |  |  |
| Measurement methods and procedures | Fuel consumption will be determined from purchase invoices, stock records, and inventory balances maintained by the project entity. The net fuel consumed during the monitoring period will be calculated as: |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Weighbridge (for solid fuels) / calibrated flow meter (for liquid fuels), cross-verified with purchase invoices. |  |  |
|  | Accuracy class |  | Minimum ±1% of measured value. |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | At project site fuel storage / receiving point. |  |  |
| QA/QC procedures | - Cross-check fuel consumption data with purchase records, supplier invoices, and stock registers. - Reconciliation of measured data with invoices to avoid errors. - Periodic verification of measurement records through internal audits. - Backup of electronic data and maintenance of hard copies of invoices and stock records. |  |  |  |  |
| Treatment of uncertainty | - Uncertainty minimized through use of calibrated instruments - Default values for emission factors taken from IPCC 2006 guidelines or national standards to reduce methodological uncertainty. - Any missing or inconsistent data will be conservatively treated to avoid overestimation of emission reductions |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | QNH3,total |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Total quantity of ammonia produced and verifiably exported by the project activity during the year |  |  |  |  |
| Data unit | tNH3 |  |  |  |  |
| Equations referred | Equation 3 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Measured continuously by flow meters/weighbridge and recorded in daily logs; aggregated monthly and reported annually. |  |  |  |
| Measurement methods and procedures | Green ammonia production and export will be determined based on continuous readings from calibrated flow meters at the ammonia synthesis outlet or weighbridge records (for bulk transfers). These measurements will be cross-checked with production records, dispatch notes, and stock balances. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Calibrated flow meter (continuous measurement) / weighbridge (batch-wise transfer), verified against production logs. |  |  |
|  | Accuracy class |  | Minimum ±1% of measured value. |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | At project site, downstream of ammonia synthesis unit / dispatch point. |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data on a monthly basis. - Annual third-party verification of records and calibration certificates. - Electronic data backed up and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | - Uncertainty minimized through use of calibrated instruments - Default values for emission factors taken from IPCC 2006 guidelines or national standards to reduce methodological uncertainty. - Any missing or inconsistent data will be conservatively treated to avoid overestimation of emission reductions |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | fgreen |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Fraction of ammonia produced by the new project activity that is classified as green, based on the use of renewable hydrogen and nitrogen inputs |  |  |  |  |
| Data unit | % |  |  |  |  |
| Equations referred | Equation 3 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Measured continuously by flow meters/weighbridge and recorded in daily logs; aggregated monthly and reported annually. |  |  |  |
| Measurement methods and procedures | Green ammonia production and export will be determined based on continuous readings from calibrated flow meters at the ammonia synthesis outlet or weighbridge records (for bulk transfers). These measurements will be cross-checked with production records, dispatch notes, and stock balances. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Calibrated flow meter (continuous measurement) / weighbridge (batch-wise transfer), verified against production logs. |  |  |
|  | Accuracy class |  | Minimum ±1% of measured value. |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | At project site, downstream of ammonia synthesis unit / dispatch point. |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data monthly. - Annual third-party verification of records and calibration certificates. - Electronic data backed up and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | - Uncertainty minimized through use of calibrated instruments - Any missing or inconsistent data will be conservatively treated to avoid overestimation of emission reductions |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | Celec,grid,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Electricity consumed in the project activity from grid in a year y |  |  |  |  |
| Data unit | MWh/year |  |  |  |  |
| Equations referred | Equation 29 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous monitoring via flow meters/weighbridge; aggregated monthly, reported annually. |  |  |  |
| Measurement methods and procedures | Electricity import from the grid is measured using utility-grade energy meters installed at the grid interconnection point. These meters record total electricity drawn from the grid for use in hydrogen production, nitrogen separation, ammonia synthesis, and other project operations. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Utility-grade energy meters integrated with SAP/SCADA for real-time data capture and reporting |  |  |
|  | Accuracy class |  | Class 0.2S or Class 0.5 as per national metering standards |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier 3. Responsible entity: Accredited third-party calibration agency / internal QA team |  |  |
|  | Location |  | Grid interconnections point at project boundary |  |  |
| QA/QC procedures | - Routine calibration and maintenance as per SOP - Monthly cross-verification with utility bills - Data archived digitally via SCADA/SAP and retained for auditing |  |  |  |  |
| Treatment of uncertainty | Minimized through calibrated instruments; missing data treated conservatively. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | Celec,RE,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Electricity consumed from renewable energy sources in year y (MWh/year) |  |  |  |  |
| Data unit | MWh/year |  |  |  |  |
| Equations referred | Equation 29 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous monitoring via utility-grade meters; data aggregated monthly and reported annually |  |  |  |
| Measurement methods and procedures | Electricity supplied from renewable energy sources (onsite or wheeled) is measured at the RE generator export point and/or at the project site import point using utility-grade bidirectional meters. These meters record total renewable electricity supplied for hydrogen production, nitrogen separation, ammonia synthesis, and other project operations. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Utility-grade bidirectional energy meters integrated with SCADA/SAP |  |  |
|  | Accuracy class |  | Class 0.2S or Class 0.5 as per national metrology standards |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier 3. Responsible entity: Accredited third-party calibration agency / internal QA team |  |  |
|  | Location |  | RE generator export point; Project site import point (for wheeled power) |  |  |
| QA/QC procedures | - Monthly reconciliation of RE generator invoices and SLDC schedules - Calibration logs and utility billing cross-checks - Real-time SCADA-based data logging and archival for verification |  |  |  |  |
| Treatment of uncertainty | Uncertainty minimized through calibrated meters; in case of missing data, conservative assumptions will be applied |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | Hreq,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Total hydrogen production |  |  |  |  |
| Data unit | tons (weight) |  |  |  |  |
| Equations referred | Equation 15, Equation 33 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Measured continuously by flow meters/weighbridge and recorded in daily logs; aggregated monthly and reported annually. |  |  |  |
| Measurement methods and procedures | Hydrogen production will be determined based on continuous readings from calibrated flow meters at the hydrogen transfer area or weighbridge records (for bulk transfers). These measurements will be cross-checked with production records, dispatch notes, and stock balances. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Digital Electronic Flowmeter interfaced with SAP or Load Cells |  |  |
|  | Accuracy class |  | Class 0.5, Class 1, or Class 1.5 |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | At project site, Hydrogen transfer area |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data on a monthly basis. - Annual third-party verification of records and calibration certificates. - Electronic data backed up and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | - Uncertainty minimized through use of calibrated instruments - Any missing or inconsistent data will be conservatively treated to avoid overestimation of emission reductions |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | PLNH3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Physical leaks of ammonia in ammonia value chain as a percentage of the total production |  |  |  |  |
| Data unit | % |  |  |  |  |
| Equations referred | Equation 34 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous monitoring via flow meters/weighbridge; aggregated monthly, reported annually. |  |  |  |
| Measurement methods and procedures | Determined through calibrated flow meters / weighbridge records; cross-checked with production, dispatch, and stock records. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Gas leak detection sensors (e.g., ammonia detectors) are integrated with SAP or SCADA systems for real-time monitoring and alerts |  |  |
|  | Accuracy class |  | Sensor-specific; typically, qualitative or ppm-level accuracy |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | Ammonia production and storage areas |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data monthly. - Annual third-party verification of records and calibration certificates. - Electronic data backed up and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | Minimized through calibrated instruments; missing data treated conservatively. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | PLH2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Physical leaks of Hydrogen in hydrogen value chain as a percentage of the total production |  |  |  |  |
| Data unit | % |  |  |  |  |
| Equations referred | Equation 33 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | Continuous monitoring via flow meters/weighbridge; aggregated monthly, reported annually. |  |  |  |
| Measurement methods and procedures | Determined through calibrated flow meters / weighbridge records; cross-checked with production, dispatch, and stock records. |  |  |  |  |
| Entity/person responsible for the measurement | Plant operator / project activity manager. |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | Gas leak detection sensors (e.g., ammonia detectors) are integrated with SAP or SCADA systems for real-time monitoring and alerts |  |  |
|  | Accuracy class |  | Sensor-specific; typically, qualitative or ppm-level accuracy |  |  |
|  | Calibration requirements |  | 1. Calibration procedures: As per manufacturer’s specifications / national standards. 2. Calibration frequency: At least once every year or as per manufacturer’s recommendation, whichever is earlier. 3. Responsible entity: Accredited third-party calibration agency / internal QA team. |  |  |
|  | Location |  | Hydrogen production and storage areas |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data monthly. - Annual third-party verification of records and calibration certificates. - Electronic data backed up and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | Minimized through calibrated instruments; missing data treated conservatively. |  |  |  |  |
| Additional comment | NA |  |  |  |  |

| Data/parameter | Qg,y |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Description | Quantity of GHGs released due to land-use change (if the plant is constructed on land with prior vegetation/biomass) |  |  |  |  |
| Data unit | tGHGs |  |  |  |  |
| Equations referred | Equation 39 |  |  |  |  |
| Purpose of data | Baseline emissions |  |  | Project emissions | Leakage emissions |
| Measurement and updating frequency |  | One-time estimation prior to project commissioning based on land clearance and soil disturbance. |  |  |  |
| Measurement methods and procedures | Determined through calibrated flow meters / weighbridge records; cross-checked with production, dispatch, and stock records. |  |  |  |  |
| Entity/person responsible for the measurement | Project developer / environmental consultant |  |  |  |  |
| Measuring instrument(s) | Type of instrument |  | - |  |  |
|  | Accuracy class |  | Determined by standard IPCC LULUCF procedures |  |  |
|  | Calibration requirements |  | Not applicable for this parameter (emission factors from IPCC guidelines are used) |  |  |
|  | Location |  | Project site prior to construction |  |  |
| QA/QC procedures | - Cross-check measured production data with dispatch notes, invoices, and stock records. - Internal reconciliation of production, dispatch, and inventory data monthly. - Annual third-party verification of records and calibration certificates. - Electronic data backed up and hard copies of invoices/records archived securely |  |  |  |  |
| Treatment of uncertainty | Conservative assumptions applied: maximum biomass and soil carbon loss considered to avoid underestimation of baseline emissions |  |  |  |  |
| Additional comment | NA |  |  |  |  |

Frequency of submission of monitoring reports

This methodology does not involve activities that result in removals or emission reductions with a risk of reversal. Therefore, the provisions related to the maximum permissible interval between the start date of the crediting period and submission of the first monitoring report, as well as the interval.

- Methodologies principles

Encouraging ambition over time

This Mechanism methodology shall apply increasingly ambitious baselines to encourage ambition of activities over time, while considering host Party circumstances. and should facilitate the deployment of low carbon solutions and technologies.

Mechanism methodologies should facilitate the deployment of technologies or measures that are not widely used or available in specific locations, to facilitate knowledge transfer and encourage deployment of technologies or measures that reduce the cost of decarbonization and unlock investment in low-carbon solutions.

Mechanism methodologies should contain provisions that facilitate the inclusion of progressively more efficient and less greenhouse gas (GHG)-intensive technologies, replicable and scalable mitigation activities, expanding the user base, broader geographic coverage, and greater penetration of low-carbon solutions after initial deployment

In all the three approaches of baseline scenario determination, the ambition of the host party is clearly embedded, Over and above the methodology, makes provision for most conservative amongst these. The difference with BAU and locking-in emissions are quantified.

Contributing to the equitable sharing of mitigation benefits between participating Parties

In this methodology, the conservative approach among all baseline scenarios ensures that emission reductions exceed BAU, directly contributing to the host Party’s NDCs and beyond. The green ammonia project lifetime is ~25 years, double the crediting period, ensuring that a significant portion of emission reductions (~70%) benefits the host Party beyond the crediting period. Provisions of the methodology incorporate conditions specified by the Designated National Authority (DNA) of the host Party to align with national priorities. Mitigation benefits to the host Party are estimated using conservative assumptions, including adjustments for exogenous factors and crediting period alignment, and are explicitly linked to sustainable development objectives through the application of the SD Tool in project design and implementation. This approach ensures equitable sharing of mitigation benefits between participating Parties while tangibly supporting the host Party’s sustainable development goals.

Implementation and Verification:

- Credit period and project lifetime: The crediting period is clearly defined and shorter than the project lifetime, ensuring continued benefits to the host Party beyond the crediting period.

- DNA provisions: All conditions specified by the host Party’s DNA are incorporated during project design and implementation.

- Sustainable development linkage: Mitigation benefits are tied to SD objectives using the SD Tool; impacts are monitored and reported in accordance with the methodology.

- Conservative estimation of benefits: Emission reductions and associated benefits to the host Party are calculated using conservative assumptions and adjustments for exogenous factors.

- Review and validation: Equitable sharing provisions and SD alignment are verified during validation and each crediting period renewal to ensure compliance and transparency.

The Designated Operational Entity (DOE) shall verify the equitable sharing provisions, estimation of mitigation benefits, and SD alignment during project validation and verification, as applicable.

Encouraging broad participation

This methodology is designed to encourage broad participation by being accurate, clear, and straightforward, ensuring that a wide range of project participants and host Parties - including those with limited scientific infrastructure or financial capacity - can implement it. It explicitly considers the context on the ground in host Parties, particularly in least developed countries and small island developing States, allowing flexibility to accommodate local conditions. The methodology recognizes and incorporates the knowledge and practices of local communities and Indigenous Peoples where relevant. To address potential data gaps, it permits the use of multiple data sources, conservative default values, and benchmarked data from comparable regions as applicable. Furthermore, the methodology is written in simple, inclusive, gender-sensitive language to ensure accessibility to a wide range of stakeholders, including local communities and Indigenous Peoples, thereby promoting equitable participation in the implementation of Article 6.4 activities

Attributability of emission reductions or net removals to the Article 6.4 activity

Attributability of emission reductions to the Article 6.4 activity is ensured as described in paragraph 73 (‘Identification of the conservative BAU scenario’), which accounts for exogenous factors and applies conservative adjustments in line with Section 5 of Appendix 1 of the Baseline Standard.

Potential perverse incentives

This methodology identifies the potential for a perverse incentive whereby activity participants may increase the production of ammonia to maximize the volume of emission reductions credited under the Article 6.4 mechanism.

Increased output could, in principle, inflate the calculated emission reductions if the mechanism rewards additional production that would not have occurred in the absence of the incentive.

However, this methodology ensures that such perverse incentives are avoided, through the following provisions:

- The installed production capacity of the ammonia plant is fixed at the time of validation and registration under the Article 6.4 mechanism. Any increase in capacity or significant change in output requires a design change request and revalidation, in accordance with the applicable A6.4 rules and procedures.

- 1. Only ammonia that is produced entirely from renewable hydrogen and nitrogen sources is eligible under the methodology.

- 2. Any increased production must be justified by actual demand and must occur within the constraints of available renewable inputs (e.g., electrolyser capacity, renewable power supply).

- 3. All input and output data are subject to third-party monitoring and verification to ensure that only eligible volumes are credited.

Through these design features, the methodology ensures that emission reductions cannot be artificially inflated and that output levels reflect legitimate activity expansion, not gaming of the mechanism.

Rebound effects

The issue of rebound effects is not relevant in the context of this methodology, which applies to the production of green ammonia from renewable energy sources. While rebound effects are typically a concern where increased efficiency or reduced costs could lead to an increase in demand (e.g., lower electricity bills leading to higher appliance usage), the following considerations apply to green ammonia project activity:

- 1. No impact on market prices:

- Green ammonia, especially in its early stages of market development, generally has higher production costs than fossil-based ammonia. As such, its implementation is not expected to reduce ammonia prices in the short to medium term. This significantly limits any potential for increased consumption due to price effects.

- 2. Policy and regulatory alignment:

- The scaling up of green ammonia is often driven by climate policies, mandates, and premium markets (e.g., shipping fuel, green hydrogen mandates, or export requirements), rather than price incentives for farmers or industrial consumers.

- 3. Fixed capacity and output constraints:

- The methodology requires that eligible projects define installed capacity and operate under permits and operational constraints. Any increase in demand would not automatically lead to higher production unless new capacity is added, which would require a separate baseline scenario and methodological applicability check.

- Therefore, rebound effects do not apply or are negligible for this methodology.

Process description and flowcharts

1. Conventional Ammonia Production Process

2. Conventional Hydrogen Production Process

The CO2 emissions per kg of hydrogen produced can vary greatly depending on the method of production used.

    - Steam Methane Reforming (SMR) is the most common method of hydrogen production, and it typically results in CO2 emissions of around 8-10 kg CO2 per kg of hydrogen produced. This is because the process involves reacting methane (natural gas) with steam to produce hydrogen and CO2. The emissions from this process are directly proportional to the carbon content of the natural gas used.

    - Coal Gasification is another method of hydrogen production, it typically results in CO2 emissions of around 14-15 kg CO2 per kg of hydrogen produced. This is because the process involves reacting coal with steam to produce hydrogen and CO2. This method is highly polluting and energy intensive.

    - Steam reforming of natural gas offers an efficient, economical, and widely used process for hydrogen production, and provides near- and mid-term energy security and environmental benefits. The efficiency of the steam reforming process is about 65% to 75%, among the highest of current commercially available production methods. Natural gas is a convenient, easy to handle, hydrogen feedstock with a high hydrogen-to-carbon ratio. It is also widely available from sources in most of the countries across the world.

    - During the production of hydrogen, CO2 is also produced. The SMR process in centralized plants emits more than twice the CO2 than hydrogen produced.

    - Carbon Captue: To avoid emission of CO2 into the atmosphere, CO2 can be concentrated, captured, and sequestered; sequestration concepts and technologies are relatively new and there is no long-term test evidence to prove that these technologies will be successful. Sequestration in oceans is controversial because of the possible adverse impact on the aquatic environment by the reduction of ocean water pH.

    - Biomass Gasification is a method of hydrogen production that typically results in CO2 emissions of around 2-3 kg CO2 per kg of hydrogen produced. This is because the process involves reacting biomass (such as wood, agricultural waste, or sewage) with steam to produce hydrogen and CO2. The emissions from this process are considered to be low, as the CO2 produced is considered to be "carbon neutral" since it comes from a renewable source.

    - Electrolysis of water is a method of hydrogen production that typically results in CO2 emissions of around 1-2 kg CO2 per kg of hydrogen produced. The emissions from this process are directly proportional to the carbon content of the electricity used.

    - Photolysis is a method of hydrogen production that typically results in CO2 emissions of around 0 kg CO2 per kg of hydrogen produced. This is because the process involves using solar energy to split water into hydrogen and oxygen and does not produce any CO2 emissions.

    - Overall, the “cleanest” hydrogen production methods in terms of CO2 emissions per kg of hydrogen are electrolysis of water using renewable energy or photolysis.

    - The shift to a low-carbon, hydrogen-based economy is prompting the energy industry to explore more cost-effective and sustainable technologies, including methane pyrolysis (turquoise hydrogen), which offers significantly lower carbon emissions than steam methane reforming (SMR) and provides a more economical alternative to existing electrolysis-based green hydrogen production.

    - Methane pyrolysis's cost-effectiveness can be further enhanced through the effective utilization of the solid carbon byproduct, which none of the other competing technologies produce.

    - The solid carbon produced has potential applications across diverse sectors, such as electronics, energy storage systems, tire production, agricultural additives, and construction materials. Currently, emerging companies are at the forefront of methane pyrolysis research, development, and commercialization. Research encompasses thermal, thermocatalytic, and plasma decomposition methods for methane cracking, with each method offering unique advantages.

    - This study opens by offering a comparative analysis of methane pyrolysis with conventional hydrogen production technologies (SMR and water electrolysis). It covers multiple aspects of hydrogen production through methane pyrolysis, providing an overview of the thermal, thermocatalytic, and plasma pyrolysis processes.

The study evaluates each method's strengths and challenges and highlights the pioneering companies in each segment. In addition, it offers insight into the technology's driving forces and challenges and provides a techno-economic analysis of the various processes associated with methane pyrolysis. It also covers the patent landscape and offers a comprehensive analysis of the growth opportunities projected to play a pivotal role in driving the adoption of methane pyrolysis technology.

3. Conventional Nitrogen Production Process

The PSA process: The steps in the PSA process of nitrogen generation involve passing compressed air through a tower containing an adsorbent material such as a carbon molecular sieve that adsorbs oxygen in preference to nitrogen. The oxygen in the compressed air is removed by the adsorbent material, leaving behind nitrogen. When the adsorbent material is saturated, the oxygen in it is desorbed. In practice, two towers with adsorbent material are used alternately so that the production of nitrogen is continuous: Step 1 Nitrogen production using a pressure swing adsorption process.

Air separation plants extract gases from the air by distillation, primarily producing oxygen and nitrogen. Some facilities also recover argon, carbon dioxide, helium, neon, krypton, and xenon. These plants can be designed to produce either liquid or gaseous products and customized to extract specific gases while discarding others.

4. Emissions from ASUs

Air separation plants generate minimal emissions, mainly consisting of air gases and impurities returned to the atmosphere. However, accidental spills or hazardous releases must be reported. Potential emissions include:

- Air gases: Separated components like nitrogen and oxygen.

- Impurities: Naturally occurring substances in the air that are released back.

- Combustion emissions: Some backup systems use fired heaters, which may produce minor emissions.

Air Separation Methods

- Fractional distillation: The most common method.

- Cryogenic air separation units (ASUs): Used for producing nitrogen, oxygen, and sometimes argon.

- Membrane separation: Extracts a single component from the air.

- Pressure swing adsorption (PSA): Separates specific gases.

- Vacuum pressure swing adsorption (VPSA): Another method for isolating gases.

Overall, air separation processes produce very low emissions. Some cryogenic plants use small amounts of natural gas or electricity to heat pre-purifiers, but emissions remain minimal.

- - - - -

Document information

| Version | Date | Description |
| --- | --- | --- |
| 0.2.0 | 19 August 2025 | Revision to incorporate new sections and sub-sections in line with current standards; provide completion instructions, realign their sequence, and allow inclusion of explanatory notes. |
| 01.0 | 18 December 2024 | Initial publication of form template. |
| Decision Class: Regulatory Document Type: Form Business Function: Methodology Keywords: A6.4 mechanism, developing methodologies and tools |  |  |

- Format for footnotes.