Information on the results of the checks performed on the consistency of the emissions reported in the greenhouse gas inventories with the verified emissions reported under Directive 2003/87/EC

CONTENTS

LIST OF TABLES	2

LIST OF FIGURES	3

1	EU-ETS versus National Inventory Report emissions from Energy Sector	4

2	EU-ETS versus National Inventory Report emissions from Industrial Processes and Product Use Sector	4

2.1	Clincker production	4

2.2	Lime Production	6

2.3	Glass Production	7

2.4	Other process uses of carbonates	8

2.5	Ammonia production	9

2.6	Nitric acid production	10

2.7	Iron and Steel Production	11

3	Comments on the comparison between EU-ETS and NIR	13

# LIST OF TABLES

Table 2.1 The CO2 emissions from Clinker Production in the NIR and EU-ETS industrial Sector for the 2023 year	5

Table 2.2 The CO2 emissions from Lime Production in the NIR and EU-ETS industrial Sector for the 2023 year	6

Table 2.3 The CO2 emissions from Glass Production in the NIR and EU-ETS industrial Sector for the 2023 year	7

Table 2.4 The CO2 emissions from Other process uses of carbonates in the NIR and EU-ETS industrial Sector for the 2023 year	8

Table 2.5 The CO2 emissions from Ammonia Production in the NIR and EU-ETS industrial Sector for the 2023 year	9

Table 2.6 The N2O emissions from Nitric Acid Production in the NIR and EU-ETS industrial Sector for the 2023 year	10

Table 2.7 The CO2 emissions from Iron and steel production in the NIR and EU-ETS industrial Sector for the 2023 year	12

# LIST OF FIGURES

Figure 2.1 The CO2 emissions from Clinker Production in the NIR and EU-ETS industrial Sector for the 2023 year	5

Figure 2.2 The CO2 emissions from lime production in the NIR and EU-ETS industrial Sector for the 2023 year	6

Figure 2.3 The CO2 emissions from Glass Production in the NIR and EU-ETS industrial Sector for the 2023 year	7

Figure 2.4 The CO2 emissions from Other process uses of carbonates in the NIR and EU-ETS industrial Sector for the 2023 year	8

Figure 2.5 The CO2 emissions from Ammonia Production in the NIR and EU-ETS industrial Sector for the 2023 year	9

Figure 2.6 The N2O emissions from Nitric Acid Production in the NIR and EU-ETS industrial Sector for the 2023 year	10

Figure 2.7 The CO2 emissions from Iron and steel Production in the NIR and EU-ETS industrial Sector for the 2023 year	12

Based on the requirement in the Art. 26.3 and Annex V Part 1 (i) of the Regulation (EU) no. 1999/2018, corroborated with Art. 14.2 of the Regulation (EU) no. 1208/2020, please find below the elements on the results of the checks performed on the consistency of the emissions reported in the greenhouse gas inventories with the verified emissions reported under Directive 2003/87/EC.

# EU-ETS versus National Inventory Report emissions from Energy Sector

Analysis implemented between data from Greenhouse gas inventory and data on verified emissions under Directive 2003/87/EC on the energy sector, relieved some differences between the two datasets. In general, the differences are due to the use of different reporting requirements and the way of reporting report emissions of combustion and process by CRT categories.

# EU-ETS versus National Inventory Report emissions from Industrial Processes and Product Use Sector

For the Industrial processes the comparison between the NIR list and EU-ETS list it was done based on raw material consumption (eg. limestone and dolomite use) and production (eg. cement, lime, glass, iron and steel production).

# Clincker production

- In accordance with EU-ETS methodology, the data used are coming from the installations with a production capacity exceeding 500 tonnes of clinker /day. CO2 emissions are calculated based on clinker production and bypass dust production;

- For NIR the data regarding the national production of clinker was used, provided directly by the companies. CO2 emissions are calculated based on clinker production and bypass dust production.

Table 2.1 The CO2 emissions from Clinker Production in the NIR and EU-ETS industrial Sector for the 2023 year

| CO2 emissions from Clinker Production (tonnes CO2) | 2023 |
| --- | --- |
| EU-ETS data | 3,394,076.24 |
| NIR data | 3,721,391.11 |

Figure 2.1 The CO2 emissions from Clinker Production in the NIR and EU-ETS industrial Sector for the 2023 year

## Lime Production

- In accordance with EU-ETS methodology, the data used are coming from the installations with a production capacity exceeding 50 tonnes of lime /day including the captive producers. CO2 emissions are calculated based on consumption of row materials (one operator reported CO2 emissions by production of lime);

- For NIR the annual activity data will be used (calcium quicklime quantity) collected from economic operators based on questionnaires. CO2 emissions are calculated based on production of lime.

Table 2.2 The CO2 emissions from Lime Production in the NIR and EU-ETS industrial Sector for the 2023 year

| CO2 emissions from Lime production (tonnes CO2) | 2023 |
| --- | --- |
| EU-ETS data | 317,546.68 |
| NIR data | 325,490.10 |

Figure 2.2 The CO2 emissions from lime production in the NIR and EU-ETS industrial Sector for the 2023 year

  - Glass Production

- In accordance with EU-ETS methodology, the data used are coming from the installations with a production capacity exceeding 20 tonnes of glass /day. CO2 emissions are calculated based on the consumption of raw materials (limestone, dolomite, soda ash);

- For NIR CO2 emissions are calculated based on the amount of melted glass and the cullet ratio of glass reintroduced into the system provided by the economic operators.

Table 2.3 The CO2 emissions from Glass Production in the NIR and EU-ETS industrial Sector for the 2023 year

| CO2 emissions from Glass Production (tonnes CO2) | 2023 |
| --- | --- |
| EU-ETS data | 59,020.29 |
| NIR data | 61,409.43 |

Figure 2.3 The CO2 emissions from Glass Production in the NIR and EU-ETS industrial Sector for the 2023 year

  - Other process uses of carbonates

- In accordance with EU-ETS methodology, the data regarding the limestone and dolomite use are coming from the ceramics plants and flue gas desulphurization plants; CO2 emissions are calculated based on the consumption of raw materials;

- For NIR the data provide from pulp and paper producers, ceramics plants and flue gas desulphurization plants. For NIR the data regarding the soda ash use are provided from pulp and paper producers, chemicals producers, flue gas desulphurization plants, water treatment, soap and detergents producers. CO2 emissions are calculated based on the consumption of raw materials (limestone, dolomite, clay, fly ash and other additives consumptions).

Table 2.4 The CO2 emissions from Other process uses of carbonates in the NIR and EU-ETS industrial Sector for the 2023 year

| CO2 emissions from Other process uses of carbonates (tonnes CO2) | 2023 |
| --- | --- |
| EU-ETS data | 190,904.40 |
| NIR data | 197,127.83 |

Figure 2.4 The CO2 emissions from Other process uses of carbonates in the NIR and EU-ETS industrial Sector for the 2023 year

  - Ammonia production

- In accordance with EU-ETS methodology, the data regarding ammonia production are coming from the installations that produce the ammonia. CO2 emissions are calculated based on natural gas consumption (non-energy use) from which are subtracted the CO2 amount used for production of CaCO3 precipitate obtained in the technological process of producing complex fertilizers; In accordance with Regulation (EU) No 2066/2018 on the monitoring and reporting of greenhouse gas emissions pursuant to Directive 2003/87/EC of the European Parliament and of the Council and amending Regulation (EU) no. 601/2012 of the Commission, for Ammonia Production, the CO2 emissions that are used for urea production are not deducted from the total emissions of the installation, the related amount of CO2 shall be considered as emitted by the installation producing the CO2. From CO2 emissions from ammonia production are subtracted the CO2 amount used for production of CaCO3 precipitate obtained in the technological process of producing complex fertilizers.

- For NIR the data regarding ammonia production are provided directly from Ammonia Production plant. CO2 emissions are calculated based on natural gas consumption (energy and non-energy use). From CO2 emissions from ammonia production are subtracted the CO2 amount used for production of CaCO3 precipitate obtained in the technological process of producing complex fertilizers.

Table 2.5 The CO2 emissions from Ammonia Production in the NIR and EU-ETS industrial Sector for the 2023 year

| CO2 emissions from Ammonia Production (tonnes CO2 equiv.) | 2023 |
| --- | --- |
| EU-ETS data | 39,904.58 |
| NIR data | 98,080.99 |

Figure 2.5 The CO2 emissions from Ammonia Production in the NIR and EU-ETS industrial Sector for the 2023 year

  - Nitric acid production

- In accordance with EU-ETS methodology, the data regarding nitric acid production are coming from the installations that produce the nitric acid. N2O emissions are calculated based on nitric acid production;

- For NIR the data regarding nitric acid production are coming from the installations who produce the nitric acid. N2O emissions are calculated based on nitric acid production.

Table 2.6 The N2O emissions from Nitric Acid Production in the NIR and EU-ETS industrial Sector for the 2023 year

| N2O emissions from Nitric Acid Production (tonnes CO2 equiv.) | 2023 |
| --- | --- |
| EU-ETS data | 14,722.03 |
| NIR data | 14,722.03 |

Figure 2.6 The N2O emissions from Nitric Acid Production in the NIR and EU-ETS industrial Sector for the 2023 year

## Iron and Steel Production

- In accordance with EU-ETS methodology, the data used are coming from the installations with a production capacity exceeding 2.5 tonnes of pig iron or steel /hour. CO2 emissions are calculated based on mass balance (inputs minus outputs);

- For NIR, the data regarding the national production of iron and steel was used provided directly by the companies. The emissions of CO2 were calculated with the Tier 3 method based on the data provided by the economic units (on mass balance). For EAF, the emissions from the reduction agents used was not take into account, except the electrode consumption. These emissions are calculated and reported under Energy sector.

Table 2.7 The CO2 emissions from Iron and steel production in the NIR and EU-ETS industrial Sector for the 2023 year

| Iron and Steel Emissions (tonnes CO2) | 2023 |
| --- | --- |
| EU - ETS data | 1,296,632.43 |
| NIR data | 1,243,106.85 |

Figure 2.7 The CO2 emissions from Iron and steel Production in the NIR and EU-ETS industrial Sector for the 2023 year

# Comments on the comparison between EU-ETS and NIR

- For the Clinker Production, in both cases (NIR list and EU-ETS list) CO2 emissions are calculated based on clinker production and bypass production. The difference between emissions from the GHG inventory and verified emissions are due to different emission estimation methodologies. For NIR, the emission factor for clinker is calculated with the formula: EF clinker = 0.785 x CaO content (weight fraction) in clinker + 1.091 x MgO content (weight fraction) in clinker, which differs from the EU-ETS Monitoring Reports. For EU-ETS emissions, the conversion factor is also taken into account.

- For the Lime Production, the CO2 emissions are different (NIR list and EU-ETS list) because in the EU-ETS methodology case CO2 emissions are calculated based on consumption of row materials and for NIR CO2 emissions are calculated based on production of lime. For NIR, the quantity of lime declared by a captive lime producer under EU-ETS was taken into account.

- For the Glass Production, the CO2 emissions are different (NIR list and EU-ETS list) because in the EU-ETS methodology case the production capacity was applied and CO2 emissions are calculated based on the consumption of raw materials (limestone, dolomite, soda ash) and for NIR CO2 emissions are calculated based on the amount of melted glass and the cullet ratio of glass reintroduced into the system.

- For the Other process uses of carbonates, the CO2 emissions are different (NIR list and EU-ETS list) because in the EU-ETS methodology case the data regarding the limestone and dolomite use are coming from the ceramics plants and flue gas desulphurization plants; CO2 emissions are calculated based on the consumption of raw materials. For NIR the data provide from pulp and paper producers, ceramics plants and flue gas desulphurization plants and the data regarding the soda ash use are provided from pulp and paper producers, chemicals producers, flue gas desulphurization plants, water treatment, soap and detergents producers. CO2 emissions are calculated based on the consumption of raw materials (limestone, dolomite, clay, fly ash and other additives consumptions).

- For the Ammonia Production, the CO2 emissions are different (NIR list and EU-ETS list) due to the different emission estimation methodologies.

- For NIR, according to the IPCC 2006 methodology, CO2 emissions for 2B1 category are calculated based on energy and non-energy use of natural gas. For 2023, the data on the amount of natural gas used as fuel in the ammonia production plant was taken directly from the operators. CO2 emissions from the use of natural gas as fuel were calculated on the basis of the emission factors and net calorific values country specific from the Energy sector. From CO2 emissions from ammonia production are subtracted the CO2 amount used for production of CaCO3 precipitate obtained in the technological process of producing complex fertilizers (reported under 3.G.1 category).

For 2023:

Total CO2 emissions = 50.508 kt (CO2 emissions from non-energy use) + 58.176 kt (CO2 emissions from energy use) – 10.604 kt (the CO2 amount used for production of CaCO3 precipitate obtained in the technological process of producing complex fertilizers)

Total CO2 emissions = 98.081 kt

The amount of natural gas used as fuel in the ammonia production process, which led to the amount of CO2 emissions of 58.176 kt, was deducted from the Energy sector of category 1A.2.c to avoid double counting.

- For EU-ETS, CO2 emissions for 2B1 category are calculated based on non-energy use of natural gas from which are subtracted the CO2 amount used for production of CaCO3 precipitate obtained in the technological process of producing complex fertilizers.

CO2 emissions resulting from the energy use of natural gas in ammonia production are allocated to category 1A2c - Energy - Chemicals.

Total CO2 emissions = 50.508 kt – 10.604 kt = 39.904 kt

In accordance with Regulation (EU) No 2066/2018 on the monitoring and reporting of greenhouse gas emissions pursuant to Directive 2003/87/EC of the European Parliament and of the Council and amending Regulation (EU) no. 601/2012 of the Commission, for Ammonia Production CO2 emissions that are used for urea production are not deducted from the total emissions of the installation, the related amount of CO2 shall be considered as emitted by the installation producing the CO2.

- For the Nitric Acid Production, the N2O emissions are similar (NIR list and EU-ETS list) because in both cases are calculated based on Nitric Acid Production.

- For the Iron and Steel Production, the CO2 emissions are similar (NIR list and EU-ETS list); in the EU-ETS methodology case the production capacity was applied and CO2 emissions are calculated based on mass balance (inputs minus outputs).

For NIR the emissions of CO2 were calculated with the Tier 3 method based on the data provided by the economic units (on mass balance). For EAF, the emissions from the reduction agents used was not take into account, except the electrode consumption. These emissions are calculated and reported under Energy sector.