Annex III: Detailed description of the reference approach (including inputs to the reference approach such as the national energy balance) and the results of the comparison of national estimates of emissions with those obtained using the reference approach (related to a non-mandatory provision as per para. 36 of the MPGs)

# 3.1. Category description

According to the IPCC documents (“IPCC 2006 Guidelines”), two separate approaches have to be applied in order to estimate the emissions from fuel combustions activities.

In calculating GHG emissions from the Energy Sector, were used two methods (Reference Approach (RA) and Sectoral Approach (SA)).

The Reference Approach (RA) is a top-down methodology, which uses a national balance (taking into account the non–energy use of fuels), calculated from the quantities of the Production, Import and export, Stock changes and International bunkers. The RA is a method for estimating CO2 combustion emissions using a simplified methodology and the apparent consumption of each fuel is calculated.

The Sectoral Approach (SA) is a more detailed methodology (a bottom–up method), using the fuel consumption for each of the subsectors 1.A.1.Energy Industries, 1.A.2. Manufacturing Industries and Construction, 1.A.3. Transport, 1.A.4. Other Sectors and 1.A.5. Other.

# 3.2. CO2 Reference Approach

The National Institute for Statistics annually provides the statistics on fuels, through the published document the Romanian Energy Balances (see Annex V.6). The applied methodologies are in accordance with the IPCC 2006 Worksheets provisions.

For the Reference Approach CO2 emissions estimation, were considered fuels for which there was reported both energy and non-energy consumption.

- LIQUID FUELS:

  - Crude oil;

  - Natural gas liquids;

  - Additives oxygenates;

  - Refinery feedstocks;

  - Other hydrocarbons;

  - Refinery gas;

  - LPG;

  - Naphtha;

  - Motor gasoline;

  - Aviation gasoline;

  - Kerosene type jet fuel;

  - Other kerosene;

  - Transport diesel;

  - Heating and other gasoil;

  - Residual fuel oil;

  - White spirit;

  - Lubricants;

  - Bitumen;

  - Paraffin waxes;

  - Petroleum coke;

  - Other products.

- SOLID FUELS:

  - Anthracite;

  - Coking coal;

  - Other bituminous coal;

  - Sub-bituminous coal;

  - Lignite/brown coal;

  - Peat;

  - Coke oven coke;

  - Patent fuel;

  - BKB/PB.

- GASEOUS FUELS:

  - Natural gas.

- OTHER FUELS:

  - Industrial wastes;

  - Municipal wastes (non-renewable).

The biomass (solid, liquid and gas) was calculated and reported separated.

The apparent consumption for different reported fuels was calculated according to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Vol. II, Ch. 6 Reference Approach, p. 6.6, and Annex 1 Worksheets, p. A1.16, taking into account the production for the primary fuels, import, export, international aviation and marine bunkers and the variation of the stock.

Since the Energy Balance provides the variation of the stock calculated as the difference between the opening stock level and closing stock level for stocks held on national territory (a stock build is shown as a negative number and a stock draw is shown as a positive number - see the documentation of the IEA/ EUROSTAT questionnaires), in the CRT the “stock change” parameter was introduced with the sign changed (in order to respect the methodology: the increasing of the stock must be subtracted from the calculation of the apparent consumption).

The conversion factor used to calculate the apparent energy consumption for solid fuels was obtained calculating the NCV weighted average from the NCVs of production, imports and exports provided through the Energy Balance–solid fuels for each fuel and each year (see table 1).

Table 1 Weighted arithmetic average of the Net Calorific Power for the solid fuels [MJ/t]

| Weighted arithmetic average of the Net Calorific Power for the solid fuels [MJ/t] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Year | Anthracite | Coking Coal | Other Bituminous Coal | Sub-bituminous Coal | Lignite/ Brown Coal | Coke Oven Coke | Patent Fuel | BKB/ PB | Peat |
| 1989 | 25533 | 21114 | 18384 | NO | 7243 | 25121 | 14654 | 14654 | NO |
| 1990 | 25533 | 27192 | 24442 | NO | 7638 | 27612 | 20860 | 14654 | NO |
| 1991 | 25533 | 27192 | 24241 | NO | 7638 | 27612 | 20860 | 14654 | NO |
| 1992 | 25533 | 27192 | 24241 | NO | 7638 | 27612 | 20860 | NO | 15348 |
| 1993 | 25533 | 27227 | 24241 | NO | 7638 | 27612 | 26372 | NO | 15348 |
| 1994 | 25533 | 27429 | 24241 | NO | 7638 | 27612 | 15348 | NO | 8791 |
| 1995 | 25533 | 26776 | 24241 | 22735 | 7638 | 27612 | 20860 | NO | 8793 |
| 1996 | 25533 | 26308 | 24241 | 23045 | 7638 | 27612 | NO | NO | 8785 |
| 1997 | 25533 | 27448 | 24241 | 19615 | 7638 | 27612 | NO | NO | NO |
| 1998 | 25533 | 27412 | 24241 | 18662 | 7638 | 27612 | NO | NO | 8793 |
| 1999 | 25533 | 27432 | 24241 | 18714 | 7638 | 27612 | 14763 | NO | 8792 |
| 2000 | 25533 | 27485 | 24241 | NO | 7638 | 27612 | NO | NO | 8790 |
| 2001 | 25533 | 27478 | 24241 | NO | 7638 | 27612 | NO | NO | 8717 |
| 2002 | 25533 | 27487 | 24241 | NO | 7638 | 27612 | NO | NO | 8790 |
| 2003 | 25533 | 27482 | 24241 | NO | 7638 | 27612 | NO | NO | 8790 |
| 2004 | 25533 | 27500 | 24241 | 24603 | 7638 | 27612 | NO | NO | 8790 |
| 2005 | 25533 | 27500 | 24241 | 24603 | 7638 | 27612 | NO | NO | 8790 |
| 2006 | 25533 | 27500 | 24241 | 24603 | 7638 | 27612 | NO | NO | 8790 |
| 2007 | 25533 | 27500 | 23550 | 24603 | 7753 | 28200 | NO | NO | 8790 |
| 2008 | 25533 | 27500 | 24958 | 24572 | 7638 | 26904 | NO | NO | 8790 |
| 2009 | 25533 | 27500 | 24572 | 24565 | 7647 | 27568 | NO | NO | 8790 |
| 2010 | 25533 | 27500 | 23885 | 24565 | 7514 | 27775 | NO | NO | 8790 |
| 2011 | 25533 | 27500 | 24283 | 24442 | 7412 | 21308 | NO | NO | 8790 |
| 2012 | 25533 | 27500 | 23397 | 24523 | 7398 | 25983 | NO | NO | 8790 |
| 2013 | 25533 | 27500 | 23843 | 24416 | 7543 | 23145 | NO | NO | 8790 |
| 2014 | 25533 | 27500 | 23535 | 24381 | 7552 | 27623 | NO | NO | 8790 |
| 2015 | 25533 | 27500 | 23479 | 24589 | 7462 | 23730 | NO | NO | 8790 |
| 2016 | 25533 | 27500 | 24049 | 24603 | 7361 | 20650 | NO | NO | 8790 |
| 2017 | 25533 | 27500 | 24212 | 24604 | 7061 | 18191 | NO | NO | 8790 |
| 2018 | 25533 | 27500 | 24464 | 24604 | 6986 | 18816 | NO | NO | 8790 |
| 2019 | 25533 | 27500 | 24459 | 24604 | 6898 | 17199 | NO | NO | 8790 |
| 2020 | 25533 | 27500 | 24798 | 24604 | 6711 | 16128 | NO | NO | 8790 |
| 2021 | 25533 | 27500 | 24180 | 24604 | 6509 | 17158 | NO | NO | 8790 |
| 2022 | 25533 | 27500 | 23497 | 24604 | 6196 | 14798 | NO | NO | 8790 |
| 2023 | 25533 | 27500 | 23275 | 24604 | 6036 | 14088 | NO | NO | 8790 |

For the liquid fuels, as conversion factors the average of net calorific values provided through the Energy Balance–liquid fuels are used to calculate the apparent energy consumption. For the liquid fuels reported on the EU–ETS monitoring reports, the national values for the corresponding NCVs were derived and used as averages, as follows: for the Romanian EU–ETS reporting period, 2007–2010 years, annual averages of the NCVs were used; for the rest of the time series the averages of the reporting EU–ETS period for the liquid fuels were used.

Therefore, for the liquid fuels, transport diesel, refinery gas, residual fuel oil, petroleum coke, heating and

other gasoil, national values of the net calorific power were derived from the EU-ETS reports. The NCVs used within the Reference Approach are included in Annex V.1 and Annex V.6.

For Natural Gas and Industrial Wastes was taken into consideration the energetic consumption provided through the Energy Balance (converting the gross to net consumption for natural gas).

For the fuels having reported consumption as non-energy, the carbon stored was calculated using the following values for the fraction of carbon stored (default values according to the 2006 IPCC Guidelines for National Greenhouse Gas Inventories, Vol. II, Ch. 1, p. 1.23, Table 1.4) (see table 2).

Table 2 Fraction of Carbon Stored

| Carbon stored in products – Fuel | Fraction of carbon stored |
| --- | --- |
| Lubricants | 1.00 |
| Bitumen | 1.00 |
| Naphtha | 1.00 |
| LPG | 1.00 |
| Refinery gas | 1.00 |
| Motor Gasoline | 1.00 |
| Kerosene Type Jet Fuel | 1.00 |
| Other Kerosene | 1.00 |
| Gas-Diesel Oil | 1.00 |
| Petroleum Coke | 1.00 |
| Residual Fuel Oil | 1.00 |
| Natural Gas as Feedstock | 1.00 |
| Other Products | 1.00 |
| Paraffin waxes | 1.00 |
| White spirit | 1.00 |
| Lignite/Brown Coal | 1.00 |
| Coal Oil and Tars (from coking coal) | 1.00 |
| Other Bituminous Coal | 1.00 |

The emission factors for the analyzed fuels were applied as default, using Tier 1 approach, according with the IPCC provision (2006 IPCC Guidelines for National Greenhouse Gas Inventories, Vol. II, Ch. 1, p. 1.23, Table 1.4).

For the following fuels were calculated and applied corresponding country specific emission factors, in the context of the using the Tier 2 approach. For the following fuels, based on the Study “Elaboration/documentation of national emission factors/other parameters relevant to National Greenhouse Gas Inventory (NGHGI) Sectors Energy, Industrial Processes, Agriculture and Waste, values to allow for the higher tier calculation methods implementation” and EU-ETS operators reporting, the carbon content country-specific emission factors were calculated, in the context of the using the Tier 2 approach:

  - Lignite;

  - Natural gas;

  - Refinery gas;

  - Other bituminous coal;

  - Coke oven coke;

  - Transport diesel;

  - Residual fuel oil;

  - Heating and other gasoil;

  - Petroleum coke;

  - Industrial wastes.

Country specific values NCVs and CO2 EFs have determined and used for the 2007–2023 period.

Tables 3 presents: the country-specific carbon content values used for estimating emissions within the Reference Approach for 1989-2006 period, averages of the EU-ETS related activity categories values: column of “2007-2010 averages”; the annual country-specific carbon content values used for estimating emissions within the Reference Approach for each year of 2007-2023 period, values based on weighted averages EU-ETS activity categories of the above period.

Table 4 presents the default carbon content values for estimating emissions for 1989-2023 period for non-EU-ETS activity categories.

Tables 3 Country specific carbon content values [t C/ TJ]

| Fuel type / Years | SOLID FUELS |  |  |  |
| --- | --- | --- | --- | --- |
|  | Other bituminous coal | Sub-bituminous coal | Lignite | Coke_Oven_Coke |
| 2007-2010 | 25.94 | 25.45 | 27.40 | 25.89 |
| 2007 | 25.43 | 25.57 | 28.21 | 25.77 |
| 2008 | 26.17 | 25.31 | 27.46 | 23.00 |
| 2009 | 25.96 | 25.59 | 26.81 | 26.14 |
| 2010 | 25.88 | 25.35 | 26.90 | 29.22 |
| 2011 | 25.04 | 25.28 | 25.84 | 26.51 |
| 2012 | 25.45 | 25.59 | 26.82 | 25.85 |
| 2013 | 25.14 | 25.71 | 26.88 | 26.01 |
| 2014 | 25.40 | 25.52 | 26.79 | 29.53 |
| 2015 | 25.26 | 25.13 | 26.75 | 28.97 |
| 2016 | 25.41 | 25.44 | 26.74 | 28.43 |
| 2017 | 25.89 | 25.33 | 26.75 | 26.23 |
| 2018 | 25.76 | 25.51 | 26.62 | 26.23 |
| 2019 | 25.74 | 25.75 | 26.41 | 26.29 |
| 2020 | 25.58 | 25.58 | 26.32 | 26.12 |
| 2021 | 25.71 | 25.58 | 26.12 | 25.85 |
| 2022 | 25.46 | 25.35 | 26.22 | 25.95 |
| 2023 | 25.79 | 25.51 | 26.81 | 25.82 |

| Fuel type / Years | LIQUID FUELS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Refinery gas | Transport diesel |  | Residual fuel oil | Heating and other gasoil |  | Motor gasoline* | Petroleum Coke |
| 2007-2010 | 15.33 | 19.99 |  | 21.32 | 20.23 |  | 19.53 | 25.54 |
| 2007 | 14.95 | 20.18 |  | 21.43 | 20.31 |  |  | 0.00 |
| 2008 | 14.88 | 19.73 |  | 21.00 | 21.24 |  |  | 25.73 |
| 2009 | 15.80 | 20.19 |  | 21.25 | 20.30 |  |  | 25.05 |
| 2010 | 15.75 | 19.84 |  | 21.73 | 20.09 |  |  | 25.64 |
| 2011 | 15.73 | 19.89 |  | 21.57 | 19.99 |  |  | 26.86 |
| 2012 | 15.49 | 20.06 |  | 21.61 | 20.20 |  |  | 26.41 |
| 2013 | 15.83 | 19.27 |  | 21.71 | 20.23 | For each year in part from the period 2013-2023, the weighted average value of 2007-2012 will be used. |  | 25.31 |
| 2014 | 15.64 | 19.93 | For each year in part from the period 2014-2023, the weighted average value of 2007-2013 will be used. | 21.90 | 20.23 |  |  | 25.29 |
| 2015 | 15.05 | 19.93 |  | 21.48 | 20.23 |  |  | 26.01 |
| 2016 | 15.37 | 19.93 |  | 21.55 | 20.23 |  |  | 26.21 |
| 2017 | 15.17 | 19.93 |  | 21.50 | 20.23 |  |  | 26.09 |
| 2018 | 15.27 | 19.93 |  | 21.76 | 20.23 |  |  | 25.78 |
| 2019 | 15.18 | 19.93 |  | 21.63 | 20.23 |  |  | 25.72 |
| 2020 | 15.47 | 19.93 |  | 21.37 | 20.23 |  |  | 25.65 |
| 2021 | 15.48 | 19.93 |  | 21.30 | 20.23 |  |  | 25.42 |
| 2022 | 15.59 | 19.93 |  | 22.57 | 20.23 |  |  | 25.41 |
| 2023 | 15.65 | 19.93 |  | 21.65 | 20.23 |  |  | 25.97 |

| Fuel type / Years | GASEOUS FUELS | OTHER FUELS |
| --- | --- | --- |
|  | Natural gas | Industrial Wastes |
| 2007-2010 | 15.14 | 22.66 |
| 2007 | 15.07 | 22.79 |
| 2008 | 15.18 | 22.73 |
| 2009 | 15.13 | 22.68 |
| 2010 | 15.17 | 22.53 |
| 2011 | 15.15 | 22.66 |
| 2012 | 15.16 | 22.66 |
| 2013 | 15.14 | 24.16 |
| 2014 | 14.97 | 24.90 |
| 2015 | 15.15 | 25.13 |
| 2016 | 15.20 | 25.41 |
| 2017 | 15.19 | 25.55 |
| 2018 | 15.16 | 25.32 |
| 2019 | 15.16 | 23.93 |
| 2020 | 15.16 | 23.90 |
| 2021 | 15.13 | 23.57 |
| 2022 | 15.15 | 24.78 |
| 2023 | 15.15 | 24.41 |

* motor gasoline country-specific carbon content is calculated based on the carbon content provided by the Romanian Ministry of Economy.

Emission factors – for the fuels not having determined country specific values, the default carbon content provided through the IPCC 2006 GL – Vol. 2.1, Chapter 1, are used.

Table 4 Default carbon content values [t C/ TJ]

| Years | 1989-2023 |  |  |
| --- | --- | --- | --- |
| Fuel Type | Default carbon content values | Fuel Type | Default carbon content values |
| Anthracite | 26.80 | Kerosene Type Jet Fuel | 19.50 |
| Coking Coal | 25.80 | Other Kerosene | 19.60 |
| Peat | 28.90 | Other Products | 20.00 |
| Patent Fuel | 26.60 | Naphtha | 20.00 |
| BKB/PB | 26.60 | White spirit | 20.00 |
| Crude Oil | 20.00 | Lubricants | 20.00 |
| Natural Gas Liquids | 17.50 | Bitumen | 22.00 |
| LPG | 17.20 | Paraffin waxes | 20.00 |
| Aviation Gasoline | 19.10 | Refinery Feedstocks | 20.00 |

The carbon content was calculated applying the following oxidation factors (2006 IPCC Guidelines for National Greenhouse Gas Inventories, Vol. II, Ch. 1, p. 1.23, Table 1.4) and is presented in the table 5. Oxidation factors – the full oxidation was assumed for all reported fuels, as it is provided through the IPCC 2006 GL.

Table 5 The values of the oxidation factor

| Oxidation factors |  |
| --- | --- |
| Coal | 1.00 |
| Oil and Oil Products | 1.00 |
| Gas | 1.00 |
| Peat for electricity generation | 1.00 |

# 3.3. Comparison between the Reference Approach (RA) and Sectoral Approach (SA)

Results of the comparison

Figure 4. 1 Comparison of the Reference Approach with the Sectoral Approach

Explanation of differences

A comparison between the RA and the SA indicates differences in both the energy consumption data and CO2 emissions, -4.58% in terms of energy consumption and -2.11% in terms of CO2 emissions for 2023.

One of the reasons for these differences refers to the fact that the Reference Approach deals with the non–energy uses of fuels as if they are combustion activities. A correction is done by subtracting the non–energy use from the apparent consumption of the fuels.

Thus, the consumption reported through the Energy Balances as being non–energetic in the sectoral activities, were subtracted from the Reference Approach.

In addition, the following processes consumption were subtracted from the Reference Approach:

- In 1.A.1.b Petroleum refining category, the reported quantities of the petroleum coke, on the entire time–series, were subtracted. Further to a dialog between energy sector experts and the operators from Refineries domain, it was concluded that the petroleum coke is reported in the Energy Balance as refinery fuel, in fact being the quantity of the "catalyst coke" deposited on the catalyst during refining processes and representing process emissions, which are accounted as fugitive emissions. The RA–SA difference was affected in the sense of decreasing of this difference. The petroleum coke was subtracted from 1.A.1.b Petroleum Refineries category, and reported under the Fugitive emissions 1.B.2.a.6 oil, other category.

- Due to the fact that Coke Oven Coke is used as reduction agent in Blast Furnace, Iron and Steel Production activity, this non–energy use of the fuel from the Reference Approach, was subtracted. The result is a balanced approach in respect of the used methodology for the CO2 emissions estimation in the Reference Approach in comparison with the Sectoral Approach.

- In 1.A.2.c Chemicals category, the reported quantities of the natural gas consumption, used in energy goal, on the entire time–series were subtracted. These quantities are reported in the IPPU sector in 2.B.1 Ammonia Production category, in according to the 2006 IPCC methodology;

- In 1.A.2.a category, the reported quantities of the coal is accounted in the IPPU sector, other than the coke_oven_coke used in Blast oxygen furnaces. Also, these quantities are not corrected in the RA.

The institution in charge with data provision to the inventory compiler, through the Energy Balances (the same data being transmitted to the international institutions–IEA/ EUROSTAT, UNECE) is the National Institute for Statistics. In the context of the inventory review under “2012 technical review of GHG emission inventories under the EU Effort Sharing Decision (ESD)”, during the developing of the project “Environmental Integrated Informational System (Romania)–International consultant for Key Category Analysis and Uncertainty Analysis for Romanian National GHG Inventory” by the representatives of Austrian Environmental Federal Agency–Umweltbundesamt and University of Graz, Institute for Systems Science, Innovation and Sustainability Research, the response is contained in the word files, Country_practice_template_Romania 2012, who clarifying the country practice in Energy Statistics and sent as reference to the “Regulation (EC) No 1099/2008 of the European Parliament and of the Council of 22 October 2008 on energy statistics provisions”. In addition, because the statistical differences are the main reason for the differences between RA and SA, in order to explain these differences, the methodological note of the domestic Energy Balance, provides the following note: Statistical differences are calculated as difference between “available for final consumption”– from which was subtracted the non-energy use – and “final energetic consumption” observed through statistical investigation. The statistical differences comprise the statistical registered stocks variations, the energy consumption in military purposes (excluding those for industrial production, which are included in the industrial activities) as well as the differences generated by the statistical investigation system: the energy producers are exhaustively registered while the consumers are surveyed based on a representative sample, being admitted a range of error. The statistical difference could be negative or positive as the consumption is lower or higher than the available supply in the reference period”. Overall, the estimation on the KP period – fuels is overestimated, due to the negative differences (Sectoral Approach higher than Reference Approach). The main reason for this are the differences in fuels consumption resulting from the significant amounts of refinery loses reported and the reported statistical differences. An explanation for the differences between the two approaches is provided by the Energy Balance: for some of years being reported a significant statistical differences which generated by the statistical investigation system (while the energy producers are exhaustive recorded, the consumers are inquired on census or on a sampling base, admitting a margin of error). Data are collected by country statistical offices (40 counties) and compiled to regional totals before being sent to the national agency. Electronic checking procedures allow to eliminate errors in compiling the national total. Statistical procedures allow to match missing data. The response rate is above 90%, however. Supply (from census) and consumption (from census and survey) are being reconciled by checking the energy balance. Transformation factors allow to assess losses, again input versus outputs are being checked. In reconciling, statistical errors are being corrected but company information is maintained. The highest differences between the two approaches are observed in the period 1990–1996, and most notably in 1990, 1993 and in 1995. The analysis showed that the main reason for this are the differences in liquid fuels consumption resulting from the significant amounts of refinery loses reported (6.1% of total refinery intake in 1995 was reported as refinery losses in comparison with the refinery intake observed reported consumption) and the reported statistical differences. For the fuels reported on the EU–ETS, the national parameter of the NCVs were determined and used: annually for the EU–ETS period (2007–2010 years) and average of the EU–ETS period for the rest of the back time series; it is the case of the following fuels: Transport Diesel, Refinery Gas, Petroleum Coke, Residual Fuel Oil, Heating and Other Gasoil.

The difference between the two approaches is influenced by the usage of the EU-ETS activity data in 1.A.1.a, 1.A.1.b, 1.A.1.c, 1.A.2.a, 1.A.2.b, 1.A.2.c, 1.A.2.d, 1.A.2.e, 1.A.2.f and 1.A.2.g categories for 2007–2023 period. This correction is not implemented in the RA. Another reason about the difference between the two approaches is due to the use of emission factors including the oxidation factors in the calculation of emissions for the sectoral approach (SA) compared to the use of emission factors excluding the oxidation factors to calculate CO2 emissions from the reference approach (RA), this difference is because the consumption of fuels is variable. The CO2 emission factors with oxidation is higher than CO2 emission factors without oxidation because they depend on the annual variation in the number of the economic operators under EU–ETS (the number of operators is decreasing) and on the variations in the fuel consumption of each economic operator.

The table 6 includes elements on the comparison of the energy consumption and the emissions, according to both approaches in terms of all fuels.

Table 6 Comparison of the RA with SA (all fuels)

| Year | Energy consumption [PJ] |  | Difference [%] | CO2 Emissions [ Gg] |  | Difference [%] |
| --- | --- | --- | --- | --- | --- | --- |
|  | Reference approach | Sectoral approach |  | Reference approach | Sectoral approach |  |
| 1989 | 2792.96 | 2515.11 | 4.12 | 183211.85 | 171992.14 | 6.52 |
| 1990 | 2611.97 | 2216.58 | 12.15 | 172737.97 | 149545.46 | 15.51 |
| 1991 | 2066.49 | 1825.70 | 6.63 | 134488.05 | 122968.61 | 9.37 |
| 1992 | 1866.65 | 1651.75 | 2.51 | 120195.80 | 115724.93 | 3.86 |
| 1993 | 1809.78 | 1530.80 | 8.75 | 118450.62 | 106635.15 | 11.08 |
| 1994 | 1705.79 | 1502.26 | 5.42 | 113365.80 | 104350.20 | 8.64 |
| 1995 | 1831.46 | 1540.72 | 10.61 | 120626.92 | 106742.72 | 13.01 |
| 1996 | 1821.38 | 1590.76 | 5.66 | 120069.27 | 111002.21 | 8.17 |
| 1997 | 1614.44 | 1452.28 | -0.13 | 104460.11 | 101166.58 | 3.26 |
| 1998 | 1460.37 | 1278.37 | 2.63 | 92834.42 | 87980.19 | 5.52 |
| 1999 | 1278.88 | 1084.59 | 5.22 | 80191.47 | 74644.97 | 7.43 |
| 2000 | 1294.58 | 1123.56 | 2.32 | 81523.34 | 78534.27 | 3.81 |
| 2001 | 1345.02 | 1185.89 | 1.48 | 85596.51 | 84060.64 | 1.83 |
| 2002 | 1401.12 | 1179.06 | 5.71 | 89514.84 | 82537.78 | 8.45 |
| 2003 | 1461.08 | 1230.21 | 6.52 | 93914.36 | 86044.81 | 9.15 |
| 2004 | 1382.61 | 1191.67 | 1.81 | 87073.66 | 83617.40 | 4.13 |
| 2005 | 1346.61 | 1160.04 | -0.92 | 82614.62 | 81522.34 | 1.34 |
| 2006 | 1430.00 | 1193.16 | 3.62 | 89695.08 | 84233.57 | 6.48 |
| 2007 | 1387.37 | 1226.89 | -1.64 | 89481.07 | 88775.77 | 0.79 |
| 2008 | 1322.09 | 1224.52 | -4.42 | 87008.65 | 89087.86 | -2.33 |
| 2009 | 1114.53 | 1057.60 | -6.09 | 72540.66 | 75879.78 | -4.40 |
| 2010 | 1100.77 | 1018.64 | -3.56 | 70908.29 | 71300.93 | -0.55 |
| 2011 | 1163.85 | 1106.73 | -5.83 | 75303.62 | 79004.42 | -4.68 |
| 2012 | 1117.78 | 1091.86 | -6.52 | 74640.29 | 77686.68 | -3.92 |
| 2013 | 981.68 | 966.33 | -7.41 | 64009.30 | 67922.50 | -5.76 |
| 2014 | 958.50 | 956.98 | -8.88 | 62637.13 | 67687.14 | -7.46 |
| 2015 | 973.39 | 965.61 | -7.00 | 64823.84 | 66445.90 | -2.44 |
| 2016 | 958.97 | 928.17 | -4.90 | 63029.68 | 64941.86 | -2.94 |
| 2017 | 1021.61 | 973.59 | -2.33 | 67748.86 | 67791.76 | -0.06 |
| 2018 | 1031.44 | 995.68 | -3.45 | 67806.67 | 67916.19 | -0.16 |
| 2019 | 1002.38 | 951.12 | -2.82 | 65305.93 | 64714.66 | 0.91 |
| 2020 | 960.34 | 908.76 | -3.29 | 59877.60 | 61948.57 | -3.34 |
| 2021 | 1032.59 | 982.15 | -3.15 | 65016.52 | 65516.27 | -0.76 |
| 2022 | 944.31 | 952.67 | -6.47 | 61293.72 | 63897.42 | -4.07 |
| 2023 | 908.92 | 908.24 | -4.58 | 59299.54 | 60576.89 | -2.11 |